National Iranian Oil Company Safety, Health and Environment Management A Guideline on Environmental Considerations of Flaring Reviewing Translation; Confirmed by; Publication Date of English Version. Davood Naderi/ Laleh H. Abbasi Training & Performance Measurement Department; Mani Abdollahzadeh Rad/ HSE Manager; September 2018. Responsibility; Prepared by; Corroborated by; Approved by; Publication date. Full Name; Seyed Emadeddin Alavi Environmental Advisor [Signed] Farzad Nejad Bahadori Environmental Advisor [Signed]; Siavash Derafshi [Signed] Head of Health, Safety, and Environment Department; Reza Khalili [Signed] Non-industrial operations Deputy; Ali Kardor [Signed] CEO; January 2017. Position. Document Code: NIOC-HSE-EN-GU-029-00 Revision date: January 29, 2017 Revision No.: 0

Table of Contents

7. References	14

2-2 Contactors monitoring	19

Appendix 3 - A Methodology for Estimation of Emissions from Flare (results of a research concerning the standardization of methods used to estimate air pollutant emissions in NIOC)	20

3-1 Flare Categorization for Emission Estimation	20

3-2  A Guide to Emission Factors Levels	20

3-3 Flare Emissions Estimation Using Constant Emission Factors	21

	 Operating Conditions and accuracy	21

	 Calculable pollutants	21

	 Necessary data	21

	 Calculation before using the right emission factor (description and steps)	21

4-3 Flare Emissions Estimation Using Parametric Relations	24

	 Operating Conditions and accuracy	24

	 Calculable pollutants	25

	 Necessary data	25

	 Calculation before using the right emission factor (description and steps)	27

Example C-2 - Use of Parametric Factors:	29

Appendix 4 - Annual Flare Reports	33

Health, Safety, and Environment (HSE) Department	34

#### 1. Introduction

###### One of the most important environmental issues faced by the petroleum industry deals with the proper disposal of waste hydrocarbon gases produced in fields and process units. Today, gas burning is one of the most common methods used for hydrocarbon gas disposal. Different techniques and equipment are used to burn waste gases depending on their nature. In the petroleum industry, flares are the most widely used equipment for waste gas burning. Flares can effectively burn waste gases and releases them into the environment.

###### The term “flare” refers to an open flame used to burn waste gases. This process mainly takes place in upstream oil and gas facilities, refineries, petrochemical industry, and other relevant industries. Flares can release a large amount of gas not the air in a rather short time.

Gas flaring management has two important aspects. First, the burnt gases are of a significant economic value. Second, gas flaring has some damaging effects on the environment (air pollution, water pollution, soil contamination, noise pollution, global warming, etc.). Given its strategic position in the oil and gas production and export market, Iran should pay special attention to gas flaring management and regularly review and update the associated procedures to keep up with the relevant ever-increasing technological advances.

According to the World’ Bank’s statistics, gas flaring associated with oil and gas production has dropped worldwide from 172 billion m3 in 2005 to 140 billion m3 in 2011. This approximately 20% decrease can be attributed to the relevant investments and the increasing cooperation between the countries enjoying rich sources of oil and gas. Despite the reduction, the figure is equivalent to about 25% and 30% of annual gas consumption in the united states and Europe, respectively. Given the many ongoing projects in different parts of the world, it is estimated that worldwide gas flaring is going to drop to 100 billion m3 by 2017. Thus, based on this global approach, the National Iranian Oil Company (NIOC) shall develop a comprehensive and detailed plan to recover flare and other waste process gases and reduce gas flaring to a satisfactory extent.

2. Objective

The present guideline aimed at introducing a method to be unwaveringly used to ensure all the environmental considerations have been observed in the flaring operations conducted by NIOC’s subsidiaries. This document establishes the minimum requirements to meet the conditions set by NIOC’s HSE Department. Fulfillment of these requirements is mandatory for all NIOC subsidiaries and associate companies and their departments.

3. Scope of Application

The present guideline applies to all NIOC’s subsidiaries and departments conducting flaring operations.

4. Legal Requirements

The guideline’s legal requirements are as follows:

- Environment Protection and Improvement Act 1980, and the relevant Amendment 2010

- Air Pollution Prevention Act 1995

- Kyoto Protocol 1998

- Code of Practice on

- CDM (Clean Development Mechanism) projects approval by the qualified Iranian authority within Kyoto Protocol Framework (12/21/2009)

- Paragraphs (B) and (C) of Iran’s Development Plan

- Iran’s environmental standards

5. Definitions and Terms

Organization:		Iranian Department of Environment

Main Company:	National Iranian Oil Company (NIOC)

Subsidiary/Department:	All subsidiaries and departments affiliated to NIOC

Vent Gas: 	All gases sent to the flare (except pilot and purge gases), and the steam and

the air used to contribute to combustion comprise the vent gas.

Flaring:	Flaring occurs when the flare produces flames other than the pilot flame.

Supplementary Fuel: Supplementary fuel mixes with waste gases headed towards the flare to enhance full combustion of the gas flow and prevent misfire.

Pilot gas:	Pilot gas is the gas sent to the flare so as to keep the pilot light burning.

#### 6. Roles and Responsibilities

- The highest authority within each subsidiary/department shall be responsible for the proper implementation of the present guideline at his/her organization.

- The main company’s HSE director is responsible for supervising the proper i implementation of the present guideline at NIOC.

- The HSE directors at Subsidiaries/departments are responsible for supervising the proper implementation of the present guideline at their organization.

- All subsidiaries/departments shall develop the instruction required to implement the present guideline. The instruction shall aim at addressing the environmental concerns associated with flaring.

#### 7. Design Requirements

###### When designing new units, the following requirements should be taken into consideration by all associated subsidiaries/departments:

7-1 All subsidiaries and departments should be familiar with the latest national and international rules and regulations as well as the environmental requirements and regulations set by the Iranian Ministry of Petroleum.

All national environmental standards and the requirements set in the international conventions Iran has committed to shall be taken as the criteria for action.

7-2 Operation units should be designed based on the “No Flaring” idea. Hydrocarbon flares or waste gas flares should be recovered and there should be a minimum need for flaring (only in emergency cases).

The recovery system should be designed while taking into consideration the latest available technologies and local conditions.

The flare is designed according to the project’s safety requirements and the latest international standards. The necessary measures (supplementary air/steam) should be taken to improve combustion efficiency and prevent misfiring and smoke.

The requirements of the environment management program (if EIA studies are conducted) should be fully observed in the design of the flare and construction of the necessary infrastructure.

7-4 When designing flares, the arrangements or infrastructures needed for qualitative and quantitative measurement of gases sent to the flare should be taken into account.

7-5 An EFD (Environmental flow Diagram) map of the gas production resources and routes leading to the flare should be made and presented as an engineering document.

The quality and quantity of gas (under normal and emergency conditions) flowing in each route should be indicated on the map.

7-6 The amount of gas emissions caused by flaring and their release pattern in normal and emergency situations should be modeled and documented for presentation.

7-7 Mass balance calculations or emission factors should be used to estimate and document the amount of greenhouse gas emissions produced through the flaring process.

7-8 Flares should be designed so as to prevent visible emissions (smoke). Visible emissions should not last longer than 5 minutes within two consecutive hours.

The amount of allowed visible emissions from flares is determined according to the mechanism described in Method 22 of the list of methods provided by the United States Environmental Protection Agency (Appendix 1).

7-9 Flares should be designed such that no emission can directly enter any aqueous or soil media (the methods used for control, collection, and disposal of liquids produced by seal drum and knockout drum should be determined).

7-10 The HSE office of the subsidiary/department should continually supervise the fulfillment of all the above-mentioned requirements.

#### 8. Manufacturing Requirements

###### When constructing new units, the following requirements should be taken into consideration by all associated subsidiaries/departments:

8-1 The subsidiary/department should carry out all flare manufacturing operations within the predetermined area and as agreed with the Iranian Department of Environment. It should refrain from trespassing other areas.

8-2 The subsidiary/department should take into consideration and fully observe all the environmental arrangements and infrastructures specified in the design stage (to recover waste gases and decrease flaring).

8-3 The subsidiary/department should take into consideration and fully observe all the supplementary arrangements and infrastructures specified in the design stage in order to improve combustion efficiency and prevent smoke and gas emissions.

8-4 The subsidiary/department should take the necessary measures to prevent emissions’ direct discharge and entry into aqueous or soil media (the facilities required for control, collection, and disposal of liquids produced by seal drum and knockout drum and other equipment with potential to emit should be determined).

8-5 The HSE office of the subsidiary/department should periodically (monthly) supervise the proper implementation of the specified arrangements and construction of environmental infrastructure based on the design documents. It should also prepare a report and present it to NIOC’s HSE Director (quarterly).

#### 9. Operation Requirements

###### When operating both old and new flare systems, the following requirements should be taken into consideration by all associated subsidiaries/departments:

9-1 All subsidiaries/departments should control their own operational process so that the gas is only sent towards the flare in emergency situations. In normal situations, other recovery methods taken into consideration in the design stage should be applied.

9-2 All subsidiaries/departments should develop a detailed program to equip old flares lacking a gas recovery mechanism with recovery systems corresponding to the type of flare gas used. The program should be prepared in at least 3 years.

9-3 All subsidiaries/departments should formulate programs aimed at controlling and monitoring leakage (especially in control and pressure relief valves). They should take the necessary measures required to fix such leakage in the first major scheduled maintenance.

9-4 All subsidiaries/departments should determine and document the following items:

- Identifying all major resources and equipment involved in the production of vent gases sent towards the flare

- Making an EFD map of the flows sent towards the flare as specified in Paragraph 7-1.

- Identifying the flow rate of gases sent towards the flare and comparing it with the that obtained during the design stage

- Identifying the chemical composition of gases sent towards the flare and comparing it with the chemical composition obtained during the design stage

9-5 All subsidiaries/departments should fully utilize all the predicted supplementary arrangements and infrastructures aimed at improving combustion efficiency. They should also control the flare combustion process in order to prevent the release of smoke and other emissions into the environment.

9-6 The pilot flame should always be lit and present. The presence of flame within the pilot of the flares equipped with an automatic flame generator should be checked using a thermocouple or other equipment1.

1Such as infrared sensors and ultraviolet beam sensors.

9-7 All subsidiaries/departments should systematically manage the potential emissions released from different parts of the flare (seal drum, knockout drum, and other equipment with potential to emit) and prevent emissions to be discharged and released into the environment.

9-8 The actual flare outflow velocity is calculated by dividing the volumetric flow rate to the unobstructed (free) cross sectional area (the gas flow escapes through this surface) of flare tip.

9-9 Gases containing halogen compounds should not be sent towards the flare.

#### 10. Flare Monitoring Requirements

10-1 All subsidiaries/departments dealing with flares should always monitor their flares’ performance and make sure they are being operated according to the design requirements and relevant standards. The flare monitoring program should be documented within the instruction prepared by the subsidiary/department according to the present guideline. The recommended flare monitoring methods and equipment are mentioned in Appendix 2.

10-2 The following monitoring actions should be daily performed on all flares systems:

- Monitoring the flow rate at the flare’s main header using one of the methods below:

- Using a flow meter (Appendix 2 contains a list of EPA-approved equipment and their general specifications)

- Mass balance

- Calculation and estimation

- Monitoring gas composition through on of the methods below:

- Manual sampling or use of an automatic sampler and delivering the sample to the relevant laboratory

- Using continuous analyzers

- Calculation and estimation

- Continuous video monitoring of flare and flame for flares receiving a large amount of gas (over 30,000 m3 in 24 hours).

10-3 All the following information should be monthly provided and documented for each flare:

- Source of the vent gas

- The reason for sending the gas toward the flare

- Volume of gases sent towards the flare (direct reading from the flow meter, mass balance, and calculation and estimation methods)

- Composition of gases sent towards the flare (direct reading from the analyzer, sampling and testing, calculation and estimation)

- Monthly comparison of the total volume of the gas sent towards the flare with relevant design requirements

- Estimation of the amount of emissions released into the environment  Estimation of the amount of greenhouse gas emissions released into the environment       Type and amount of pilot and purge gases used in the flare system

- Video archive

10-4 All subsidiaries/departments should calculate their annual emissions from flare systems (including carbon monoxide, sulfur dioxide, carbon dioxide, nitrogen oxides, unburnt hydrocarbons (UHCs), and methane) and report the results along with the mass values of gases annually sent towards the flare to NIOC. The calculations should be performed using the methods described in Appendix 3 - A Methodology for Estimation of Emissions from Flare (results of a research concerning the standardization of methods used to estimate air pollutant emissions in NIOC). The report should be submitted in the format provided in Appendix 4.

10-5 Flares are important sources of greenhouse gas emissions, especially CO2. As a result, flare gas reduction and recovery projects can be registered as international greenhouse gas emissions reduction projects (CDM/NAMA). All subsidiaries/departments should take the necessary measures in this regard.

#### 11. References

Human's Environmental Laws, Regulation, Criteria, and Standards (book)Standardization of Air Pollutant Emissions Estimation by NIOC (research project)Flare Monitoring and Environmental Requirements Guideline (National Petrochemical Company (NPC))

#### 12. Records

###### All records associated with this guideline shall be maintained by HSE offices of all subsidiaries/departments.

13. Appendices

13- 1 Appendix 1 - Visual determination of Smoke Emissions from Flares

13-2 Appendix 2 - Recommended Monitoring Methods and Equipment

13-3 Appendix 3 - A Methodology for Estimation of Emissions from Flare (results of a research concerning the standardization of methods used to estimate air pollutant emissions in NIOC)

13-4 Appendix 4 - Annual Flare  Reports

Appendix 1 - Visual determination of Smoke Emissions from Flares

##### 1. Introduction

This method involves visual determination of visible smoke emissions from flares used for combustion of waste or vented process gases. It can also determine the accumulated emission time (the amount of time that visible emissions occur during the observation period).  This method does not require that the opacity of emissions be determined. This procedure is only used to determine whether or not visible emissions have occurred. However, the observer should have received the minimum training with respect to the general procedures for determining the presence of visible emissions. At a minimum, the observer must be trained and knowledgeable regarding the effects of background contrast, ambient lighting, observer position relative to lighting, wind, and the presence of uncombined water (condensing water vapor) on the visibility of emissions. This training is to be obtained from written materials found in References 1 and 2 of the present appendix.

##### 2. Application and

##### Principles

2-1 Application: This method is used to determine emission frequency of visible smoke emissions from flares.

2-2 Principles: Smoke emissions from flares are visually determined by the observer and no tool or equipment is used in the process.

3. Definitions

3-1 Emission Frequency: the percentage of time that emissions are visible during the observation period.

3-2 Emission time: the accumulated amount of time that emissions are visible during the observation period.

3-3 Smoke emissions: a pollutant generated by combustion in a flare and occurring immediately downstream of the flame. Smoke occurring within the flame, but not downstream of the flame, is not considered a smoke emission.

3-4 Observation period: the accumulated time period during which observations are conducted (in hour for flares).

##### 4. Equipment

4.1 Stopwatch: two cumulative stopwatches (able to add up different time intervals) with unit divisions of at least 0.5 seconds.

##### 5. Instruction

5-1 Place: Survey buildings and facilities affecting line of sight and emissions source. Do not select a position where sunlight is shining directly in the observer's eyes.

5-2  Information to be recorded (Table 1): company name, industry, process unit, observer's name, observer's affiliation, and date. Record also the estimated wind speed, wind direction, and sky condition.  Sketch the process unit being observed, and note the observer’s location relative to the source and the sun. Indicate the potential and actual emission points on the sketch.

5-3 Observation: Record the clock time when observations begin. Use a stopwatch to monitor the duration of the observation period. Start the stopwatch when the observation period begins. If the observation period is divided into two or more segments by process shutdowns or observer rest breaks, stop the stopwatch when the segment ends and restart it when the next segment starts.

Stop the stopwatch at the end of the observation period. The accumulated time indicated by this stopwatch is the duration of observation period. Record the clock time when the observation period is complete.

During the observation period, continuously watch the emission source.  Upon observing an emission, start the second accumulative stopwatch; stop the watch when the emission stops. Note that condensed water vapor is not considered an emission. Stop the watch when the emission stops. Continue this procedure for the entire observation period. The accumulated elapsed time on this stopwatch is the total time emissions were visible during the observation period (i.e., the emission time).

5-3-1 Observation period:  Choose an observation period of sufficient length to meet the requirements for determining compliance with the relevant regulations. According to Paragraph 5-5 of Flare Monitoring and Environmental Requirements Guideline, observation period should not exceed 2 hours.

5-3-2 Observer Rest Breaks: Do not observe emissions continuously for a period of more than 15 to 20 minutes without taking a rest break (continuous observation should not exceed 15-20 minutes). For sources requiring observation periods of greater than 20 minutes (subject of the present guideline), the observer shall take a break of not less than 5 minutes and not more than 10 minutes after every 15 to 20 minutes of observation.  If continuous observations are desired for extended time periods (over 20 minutes), two observers can alternate between making observations and taking breaks.

5-3-3 Visual Interferences: Occasionally, emissions from other sources  (e.g., road dust) may prevent a clear view of the emission point. This may particularly become problematic during periods of high wind. If the view of the potential emission points is obscured to such a degree that the observer questions the validity of continuing observations, then the observations shall be terminated, and the observer shall clearly note this fact on the data form.

5-4 Recording Observations:  Record the accumulated time of the observation period on the data sheet as the monitoring period duration.

Record the accumulated time emissions were observed on the data sheet as the emission time. Record the clock time the observation period began and ended, as well as the clock time any observer break began and ended.

6. Calculations

The emission rate should be expressed as an emission frequency (in percent). Determine this value as follows: Divide the accumulated emission time (in seconds) by the duration of the observation period (in seconds) and multiply this quotient by 100.

## 7. References

7-1 Missan, Robert and Arnold Stein. Guidelines for Evaluation of Visible Emissions Certification, Field  Procedures,  Legal  Aspects,  and  Background  Material.  EPA Publication No. EPA-1/041-77-007. April 1777

7-2 Wohlschlegel, P. and D. E. Wagoner. Guidelines  for  Development  of  a  Quality Assurance  Program:  Volume  IX  --  Visual  Determination of  Opacity Emissions from Stationary Sources. EPA Publication No. EPA-4704/-7/-007-i. November 1777.

Table 1 - Report Form for Observation of Smoke Emissions Form Flare

| Visual Inspection of Smoke Emissions from Flares |  |
| --- | --- |
| Company: | Observer: |
| Location: | Affiliation: |
|  | Date: |
| Sky conditions: | Wind direction: |
| precipitation: | Wind speed: |
| Industry: | Process unit: |
| Sketch a process unit showing observer position relative to source, actual emission points and other potential emission points. |  |
| Accumulated emission time Observation period duration Clock Time Observations |  |
| min: sec Min: sec |  |
| Begin |  |
| Observation |  |
| End Observation -------------------------------- -------------------------------- -------------------------------- |  |

Appendix 2 - Recommended Monitoring Methods and Equipment

1. Flow rate monitoring: The device installed and used for continuous monitoring of vent gas flow rate should:

1-1 be able to determine gas with a minimum flow rate of 0.03 m/s (0.1 ft/s).

1-2 continuously detect flow rate changes ranging from 0.15 m/s to 85 m/s (0.5 to 275 ft/s).

1-3 be installed on the flare’s header or in a place where the entire flow can be measured.

1-4 have its accuracy as defined by the manufacturer set to  for flow rates ranging from 0.3 to 85 m/s.

1-5 Table 1 outlines the list of equipment and their specifications as approved by the United States Environmental Protection Agency. In addition, more specific instructions on the use of these devices and determination of volumetric flow rate in a case-by-case basis are provided in Reference Methods 2.2A, 2C, and 2D (available at http://www.epa.gov/ttn/emc/promgate.html).

Currently, ultrasonic flow rate meters are the most suitable device for monitoring vent gas flow rate in flares. These equipment, if properly installed and calibrated and information registration system thereof be appropriately programmed, can provide you with accurate and reliable data.

2. Vent gas composition:

Composition of flare gases can be monitored in a laboratory or by a continuous flow analyzer. The sample should be taken from a source representing the overall composition of the gas vented towards the flare. The samples taken from headers used by more than two flares are enough. In cases where the gas composition is determined in a laboratory, the sample can be taken both manually or with the help of an automatic sampler. The advantages and disadvantages of each method are discussed below.

Table 1 - Flow Rate Monitoring Equipment and their Specifications as Approved by by the United States Environmental Protection Agency

2-1 Sampling

Different technologies are used for sampling. Thus, companies should adopt the technology that best suits their needs. Sampling can be done both manually and automatically. Manual sampling is more cost-effective as it does not require the use of complex tools like the ones used in continuous analyzers. Preparing the sample in the laboratory can be hard and time-consuming. Short sampling intervals can make the process rather costly. Manual sampling should be conducted with more caution to ensure safety of the personnel involved in the process. In cases where flares are rarely operated, manual sampling would be a more logical choice.

An alternative method is uniform sampling, which is performed by automatic sampling equipment. A uniform sample is composed of a number of samples taken from the gas flow over time. For example, the automatic sampler takes ten 100-mm samples from the gas flow within a certain time period. These ten samples comprise a 1 liter sample known as the uniform sample. As these samples are taken over time, the uniform sample represents the changes made to gas composition during the same time period.

## 2-2 Contactors monitoring

Alternatively, online analyzers along with continuous gas composition monitoring can be used to determine gas composition. Gas composition can be determined using different methods and technologies (e.g., gas chromatography, non-dispersive infrared (NDIR), and flame ionization detectors). Continuous monitoring is costly as it requires design and installation of a sample preparation system. At the same time, presence of water, particulates, metal particles, etc. contributes to the complexities associated with this method.

## Appendix 3 - A Methodology for Estimation of Emissions from Flare (results of a research concerning the standardization of methods used to estimate air pollutant emissions in NIOC)

This appendix describes the method used to estimate emissions from flares. Emission factors obtained in a research conducted by Sharif University of Technology concerning the standardization of methods used to estimate air pollutant emissions in NIOC were taken as the estimation criteria. Two types of emission factors are used in the estimation of emissions from flares. They include constant emission factors and parametric emission factors. When using constant emission factors, the flared fuel’s flow rate is the only data needed. Here, the estimation is carried out quickly through a series of simple multiplications. Parametric emission factors lead to more accurate estimates. When using parametric factors, you need more data to complete the estimation. Given their available data, companies can choose a method and carry out their estimation accordingly.

## 3-1 Flare Categorization for Emission Estimation

There are different categorizations of flares.  When emissions from flares are to be estimated, flares are categorized into warm hydrocarbon flares and acid flares. Warm hydrocarbon flares are often used to flare hydrocarbons, while acid flares can burn a considerable amount of hydrogen sulfide.

## 3-2  A Guide to Emission Factors Levels

The following table introduces different emission factors tables by their accuracy level and shows how they should be used in situation where different data are available.

The quantities in parentheses are not mandatory. However, they can be used to effectively improve the estimation's accuracy.

Table 3-1 A Guide to Emission Factors Tables

| Overall estimation accuracy | Emission Factors Table | Available data | Flare Type |
| --- | --- | --- | --- |
| B | Table 3-2 | Flare flow rate (mass friction of hydrogen sulfide) or its heating value | Warm |
| D | Table 3-3 | Flare flow rate or fuel’s heating value | Acid |
| B | Table 3-4 | Flare flow rate - fuel analysis | Acid |
| A | Table 3-7 | Flare flow rate - fuel analysis - flare diameter - flare height - wind speed | Warm or acid |

## 3-3 Flare Emissions Estimation Using Constant Emission Factors

The fastest way to estimate emissions from flares is to use constant emission factors. The factor to be used is determined by the fuel’s mas and heating value. In this method, the estimate is obtained by multiplying the constant emission factors are by the mass flow rate. This section introduces the emission factors provided by the researches at Sharif University of Technology. These factors are associated with flares used in oil and gas upstream industries.

## Operating Conditions and accuracy

The emission factors used in this research apply to warm and acid flares with simple tube-shaped structures. The fuel inflow of these flares should not contain any liquid droplets. In this method, the accuracy of emission estimation depends on the fare's operating conditions. The estimations are rather accurate when dealing with common flares. As emission factors are the average emission factors of different operating conditions, they cannot be employed in cases where special flares with different operating conditions are used as they might result in errors. In this method, the only necessary data are the fuel’s flow rate and heating value. Mass friction factors of the constituent elements of the limiting reagent can improve the estimation's accuracy.

## Calculable pollutants

Emission factors can help you calculate carbon monoxide, carbon dioxide, unburnt hydrocarbons (UHCs), soot, nitrogen oxides, sulfur dioxides, and hydrogen sulfide emissions.

## Necessary data

In this method, the only necessary data are the fuel’s mass flow rate or the fuel inflow’s heating value.

## Calculation before using the right emission factor (description and steps)

Emission factors used in the estimation of emissions from flares are the same emission factors used in the estimation of emissions from other combustion equipment. These emission factors are obtained through numerical simulations. The emission factors used to estimate emissions from warm flares are presented in Table 3-2. The emission factors used to estimate emissions from acid flares can be found in Tables 3-3 and 3-4. For warm flares, if the flared fuel’s composition is known, the sulfur dioxide and hydrogen sulfide emissions are calculated more accurately.

For acid flares, if the flared fuel’s composition is known, the calculations will be more accurate and the data presented in Table 3-4 can be used. These data have a higher reliability.

Emission factors are presented in kg/kg (fuel’s mass flow rate) and g/GJ (heating value). Thus, booth quantities can be used to estimate emissions from flares.

Table 3-2 Emission factors for Pollutants Level 1 - Warm Flares

| Emission Factor Ranking | Pollutant’s average emission factor (g/GJ) | Pollutant’s average emission factor (kg/kg) | Pollutant |
| --- | --- | --- | --- |
| A | k53.2 | 0.97 | Carbon dioxide |
| B | 448 | 0.0179 | Carbon monoxide |
| B | 61.2 | 0.0024 | Nitrogen oxides |
| B | 34.3 | 0.0014 | Soot |
| B | 133.7 | 0.0053 | Unburned hydrocarbons (UHCs) |
| A(C) | 649 | yH 2S 1.825 | Sulfur dioxide* |
| A(C) | 13 | yH 2S 0.03 | Hydrogen sulfide* |

* If the weight percent of hydrogen sulfide (yH2S) in the consumed fuel is available, it will be more accurate to use the emission factors provided in the first column. These emission factors are obtained by averaging out the combustion efficiency of hydrogen sulfide in warm flares (97%). The emission factor of these two pollutants (based on the mass friction of hydrogen sulfide) would be A.If mass friction of hydrogen sulfide is not available, the default value (0.0145) would be used.

Table 3-3 Emission factors for Pollutants Level 1 - Acid Flares (Method 1)

| Emission Factor Ranking | Pollutant’s average emission factor (g/GJ) | Pollutant’s average emission factor* (kg/kg) | Pollutant |
| --- | --- | --- | --- |
| D | k111 | 0.91 | Carbon dioxide |
| D | 41 | 0.0003 | Carbon monoxide |
| D | 62.5 | 0.0005 | Nitrogen oxides |
| D | 17.5 | 0.0001 | Unburned hydrocarbons )UHCs( |
| D | k91 | 0.75 | Sulfur dioxide |
| D | k3.8 | 0.031 | Hydrogen sulfide |

Table 4-3 Emission factors for Pollutants Level 1 - Acid Flares (Method 2)

| Emission Factor Ranking | Pollutant’s average emission factor* (kg/kg) | Pollutant |
| --- | --- | --- |
| B | 2.53× yHC + yCO2 | Carbon dioxide |
| B | 0.098× yHC | Carbon monoxide |
| B |  | Nitrogen oxides |
| B | 0.024× yHC | Unburned hydrocarbons (UHCs) |
| B | 1.731× yH 2S | Sulfur dioxide |
| B | 0.08× yH 2S | Hydrogen sulfide |

* Pollutants’ and carbon dioxide’s emission factors are obtained by averaging out the combustion efficiency of hydrogen sulfide and hydrocarbons (97%) based on the principle of mass conservation.

Example C-1: Using constant emission factors

We have a warm flare with a gas burning ratio of 5000 kg/day in an oil site. The flare's gas inflow contains hydrogen sulfide with a weight percent of 2%.

Daily NOx, CO, and SO2 emissions are estimated using the following emission factors.

Based on the data provided in Table 3-2:

## 4-3 Flare Emissions Estimation Using Parametric Relations

In this method, parametric relations and quantities affecting the emissions from flares are used to estimate pollutant’s emissions based on the fuel’s mass. These parametric relations are obtained through numerical simulation of flares under different conditions and fitting the diagram to the changes in the pollutants’ emission factors based on the changes in the quantities affecting the pollution.

## Operating Conditions and accuracy

The parametric relations are associated with flares having simple tube-like structures and fuel inflows free from any liquid droplets. In this parametric study, the emission factors can be used for flares with diameters ranging between 0.1 and 1 m, flammability limits ranging between 3.6 and 9.4, outflow jet velocities (from the flares’s opening) ranging between 0.1 and 7 m/s, heating values ranging between 20 and 65 (Mj/m3), ambient wind speeds (at height) ranging between 1 and 20 m/s.

In this method, all the quantities affecting the combustion in flares are taken into consideration, resulting in acceptably accurate emission factors.

## Calculable pollutants

Parametric relations can be used to estimate unburnt hydrocarbons (UHCs), soot, nitrogen oxides, carbon monoxide, carbon dioxide, sulfur oxides, and hydrogen sulfide emissions.

## Necessary data

In this method, the

independent quantities required for estimation of emissions from flares include:  Fuel flow rate, wind speed (Uw), fuel’s analyzed hydrocarbon and hydrogen sulfide content, and flare’s diameter (d) and height (h). The dependent quantities include: outflow jet velocity (Vj), stoichiometry value of carbon dioxide (EFCO2,S), fuel’s lower heating value (LHV), momentum Ratio of R, and power parameter of λ.

The two latter quantities are defined as follows:

(3-1)

(3-2)

In the above relation, the fuel’s heating value is in Mj/m3. Other values are in SI Units.  The method used to calculate the mixture’s flammability limit of the mixture is somehow complicated. According to Le Chatelier's principle, the ratio of air-to-fuel in combustion gas mixtures within the flammability limit is equal to the total weighted ratio of air-to-fuel in each component of the mixture within the flammability limit [14].

(3-3)

The following relation can be simplified as follows:

(3-4)

The law is used to determine the upper and lower flammability limits of fuel mixtures. In most cases, the above relation gives us relatively accurate results. However, when a fuel contains compounds like hydrogen sulfide, and neutral compounds such as carbon dioxide, the results differ greatly from the those obtained in a laboratory environment. Moulnarn et al. [15] modified and rewrote the above relation so that the effect of different compounds can be taken into account when calculating with the flammability limit if the air-fuel mixture. When the nth compound is tconsidered as the equivalent for nitrogen, then:

(3-5)

Ne is the correction factor of compounds other than nitrogen. Ne’s values, provided by Moulnarn et al., are shown in the Table below.

When the hydrocarbon compound is a mixture of the above compounds, the correction factors are expressed based on the dominant hydrocarbon.

Table 3-5 Ne’s Correction Factors

| Compound | CO2 | H2O |
| --- | --- | --- |
| CH4 | 2.23 | 1.87 |
| C2H4 | 1.93 | 1.51 |
| C3H6 | 1.84 | 1.68 |
| C3H8 | 1.92 | 1.36 |

Different lower flammability limits for the flare’s components along with the heating value and density are provided in Table 3-6. The listed values are standard values. As the flares’ flow rate are mainly expressed in standard f3, standard density values can be used to accurately convert mass and volumetric flow rates. When the gas inflow’s temperature and pressure are available, it would be better to calculate the gas mixture’s density based on the ideal gas relations. In addition, the enthalpy resulting from the temperature difference with the standard conditions should also be taken into account.

Table 3-6 Properties of different compounds found in flare fuel analysis

| Density (kg/m3) | Heating value/volume (MJ/m3) | Low flammability limit (LFL) (volume percent) | Compound |
| --- | --- | --- | --- |
| 165.1 | - | - | Nitrogen |
| 842.1 | - | - | CO2 |
| 36.1 | 76.23 | 3.4 | H2S |
| 668.0 | 71.37 | 4.4 | methane (C1) |
| 1 | 51.66 | 3 | ethane (C2) |
| 882.1 | 94.05 | 1.2 | propane (C3) |
| 46.2 | 30.120 | 6.1 | butane (C4) |
| 3.04 | 49.148 | 1.5 | pentane (C5) |
| 3.65 | 07.177 | 1.1 | hexanes (C6) |

## Calculation before using the right emission factor (description and steps)

The parametric relations used to obtain the emission factors can be found in Table 3-7.Emissions can be estimated by calculating emission factors based on the fuel inflow’s mass and flow rate.

First, the fuel’s properties (e.g., heating value, flammability limit) should be calculated. After calculating λ and R values, emission factors become calculable. In the following example, emissions from a given flare are estimated.

Table 3-7 Parametric Pollutant Emission Factors for Flares

| Applicable range | Pollutant’s verage emission factor* (kg/kg) | Pollutant |
| --- | --- | --- |
|  |  | Carbon dioxide |
|  |  | Carbon monoxide |
|  |  | Nitrogen oxides |
|  |  | Soot |
|  |  | Unburned hydrocarbons |
|  |  | )UHCs( |
|  |  | Sulfur dioxide |
|  |  | Hydrogen sulfide |

## Example C-2 - Use of Parametric Factors:

Emissions from a flare with a diameter of 10″, height of 15 m, and flow rate of 0.15 million f3/day should be estimated. According to the fuel’s analysis, the mole percent of nitrogen, carbon dioxide, hydrogen sulfide, methane, ethane, propane, and butane are, 8%, 2%, 0.5% 80%, 5%, 4%, and 0.5%, respectively.

At the height of 10 m, the air speed is 10 mile per hour.

Step 1: First, mass friction values of fuel components (yi) are calculated.

Mass friction is obtained based on the mole fraction (xi) and mole mass (Mi) of each components as follows:

(3-6)

The mixture’s density is related to the

components’ mole volume through the relation below:

(3-7)

Assuming the conditions are standard, the heating value/fuel’s volume is estimated as follows:

###### (3-8)

Data pertaining to different components are presented in Table 1.

| Fuel component | xi | yi |  | xiLHVv,i |
| --- | --- | --- | --- | --- |
| Nitrogen | 0.08 | 0.11 | 0.098 | - |
| CO2 | 0.02 | 0.04 | 0.024 | - |
| H2S | 0.005 | 0.01 | 0.006 | 0.119 |
| methane (C1) | 0.8 | 0.65 | 0.976 | 30.186 |
| ethane (C2) | 0.05 | 0.08 | 0.076 | 3.325 |
| propane (C3) | 0.04 | 0.09 | 0.048 | 3.837 |
| i-butane (C4) | 0.005 | 0.01 | 0.006 | 0.601 |
| Total | 1 | 1 | 1.234 | 38.05 |

Hence, the fuel's density and volumetric heating value are estimated 0.81 kg/m3, respectively.

The fuel’s lower flammability limit is calculated as follows:

The stoichiometry value gives us the amount of carbon dioxide produced per one mole of fuel.

As the above relation suggests, 1.06 mole of carbon dioxide is produced per each mole of fuel. Given the fuel’s average atomic number (19.64) and carbon dioxide’s atomic number (44), the stoichiometry value of CO2 is estimated:

Step 2: Speed Calculations

The wind speed at a height of ZRef is estimated 10 mile per hour or 4.47 meter per second. The speed should be estimated at a height of 15 m.  As most flares are placed in open spaces with no high-rising building in their vicinity, reference height of Z0 = 0.03 can be used.

The outflow jet velocity is calculated based on the volumetric flow rate. The given flare’s volumetric flow rate is 0.15 MMSCFD. A 1 MMSCFD is equal to 0.327 m3/s, the volumetric flow rate is estimated 0.05 m3/s. The outflow jet velocity is obtained by dividing the volumetric flow rate by the flare’s cross sectional area:

Step 3: Constant calculations

The two constants, power parameter of λ and momentum ratio of R are calculated as follows

Step 4: Emission factor estimation

Carbon dioxide:



Carbon monoxide: The weight percent of hydrocarbons in the fuel inflow is 84%. The average mole weight of hydrocarbons is 18.26 gr/mole. Thus:

Nitrogen oxides:

Soot

| Unburned hydrocarbons (UHCs): Sulfur dioxide: Step 5: Emission estimation After obtaining the emission factors, the emission can be easily estimated by multiplying the mass flow rate by emission factors. The flare’s mass flow rate (ṁ) is obtained by multiplying the th volumetric flow rate by the gas outflow’s density. Thus: ṁ= 0.05× 0.81 = 0.0405 (kg/s) Emission (kg/s) is estimated by multiplying emission factors by the instantaneous flow rate. If the average annual flow rate is obtained, the annual emission can be estimated by multiplying the value by the number of seconds a year is composed of. In this example, the flare’s emission estimates are provided in the table below: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Emission factor/mass | Emission (kg/second) | Emission (tone/year) | Pollutant |  |
|  | 2.341 | 0.095 | 2989.9 | Carbon dioxide |  |
|  | 0.012 | 0.0005 | 15.3 | Carbon monoxide |  |
|  | 0.0019 | 0.00008 | 2.4 | Nitrogen oxides |  |
|  | 0.0011 | 0.00004 | 1.4 | Soot |  |
|  | 0.0045 | 0.00018 | 5.7 | Unburnt hydrocarbons (UHCs) |  |
|  | 0.0185 | 0.00075 | 23.6 | Sulfur dioxide |  |
|  | 0.00014 | 0.00001 | 0.18 | Hydrogen sulfide |  |

## Appendix 4 - Annual Flare Reports

| Year | Table of Environmental Flare Reports ........ Company ..…..… |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Flare processes: |  |  |  |  | Flare name: |  |
| Non assisted□ |  | Air assisted□ |  | Steam –assisted □ Flare type: |  |  |
| Max capacity percentage1: |  |  | Design capacity (MJ/MCM): |  |  | Flare’s operating hours: |
| Gas mass (ton/year) |  |  | Gas/pollutant type |  |  | Gases sent to the flare2: |
|  |  |  | CO |  |  | Flare’s pollutant emissions3: |
|  |  |  | CO2 |  |  |  |
|  |  |  | NOx |  |  |  |
|  |  |  | SOx |  |  |  |
|  |  |  | CH4 |  |  |  |
|  |  |  | THC |  |  |  |

1. The maximum capacity percentage is obtained from the following formula:

Maximum capacity percent = Actual CapacityMaximum Capacity × 100

2. In order to calculate the mass of gases sent to the flare, the composition and flow rate of the flow sent to the flare should be determined base on one of the methods described in Appendix 2.

3. The amount of the emissions released into the environment is calculated according to the method described in Appendix 3.

# Health, Safety, and Environment (HSE) Department

#### No. 17, Yaghma Alley, Jomhouri-e Eslami St. (btwn Hafez St. & Valiasr Ave.), Tehran, Tehran, Iran

Phone: +98 21 66726016, Fax: +98 21 6609723, SMS System: +98 10007615

HSEE@NIOC.IR	HTTP://HSE.NIOC.IR