NIOC’s Periodical Monitoring of Air Pollutants Guideline

B

Standardization of Air Pollutant Emissions Estimation by NIOC

Sharif University of Technology

Table of Contents

List of Tables	5

List of Figures	6

NIOC’s Instructions on Periodical Environmental Monitoring of Air Pollutants in Oil and Gas Upstream Industries	7

Chapter 1	9

Chapter 2	32

Chapter 3	48

Chapter 4	55

5. References	73

## List of Tables

Table 1-1 Outdoor Air Sampling Methods and their Corresponding Pollutants

Table 1-2 Outdoor Air Audit Methods (Direct Reading) and their Corresponding Pollutants

Table 1-3 Equipment Needed for Different Sampling Methods

Table 1-4 Parameters to be Considered when Monitoring the Environment Surrounding Flares and Burning Pits

Table 1-5 Parameters to be Considered when Monitoring the Environment Surrounding Evaporation Ponds

Table 1-6 Parameters to be Considered when Monitoring the Environment Surrounding Floating roof Tanks

Table 4-1 Layout of Measurement Points for Ducts with Rectangular Cross Sections

Table 4-2 Measurement Points in Chimneys with Circular Cross Sections

## List of Figures

Figure 2-1 Velocity Profile in a Specific Cross Section

Figure 4-1 A schematic View of the the point where the port is Installed on the Chimney

Figure 4-2 Number and Layout of Sampling Ports

Figure 4-3 A schematic View of the Sampling Port’s Dimensions

Figure 4-4 Determining the Minimum Number of Testing Points Required for Non-Particulates Measurement

Figure 4-5 A Sample Layout of Measurement pionts in the circular Cross Section

Figure 4-6 A Sample Layout of Measurement Points in the Rectangular Cross Section

# NIOC’s Instructions on Periodical Environmental Monitoring of Air Pollutants

in Oil and Gas Upstream Industries

The present instruction deals with the auditing operations to be conducted in NIOC’s upstream facilities. In industries (e.g., refineries, petrochemical facilities) where the sources of emission and their corresponding pollutants are similar to those found in the oil and gas upstream industries, the same instruction can be followed.

The table below was compiled based on the emission sources studied in the present research project. It can directly guide you to the intended audit method.

A guideline on audit instructions in NIOC’s upstream facilities

| Instruction’s Chapters | Audit method | Source of emission |
| --- | --- | --- |
| Refer to Chapter 1 | Direct reading | Flares |
| Refer to Chapters 1 and 2 | Direct reading and sampling | Burning pits |
| Refer to Chapter 4 | Direct reading | Combustion equipment |
| Refer to Chapters 1 and 2 | Direct reading and sampling | Evaporation ponds |
| Refer to Chapter 1 | Direct reading and sampling | Floating roof tanks |
| Refer to Chapter 3 | Direct reading and sampling | Potentially leaking equipment and fixed-roof tanks |

### Chapter 1

An instruction on Auditing Air Surrounding Unconventional Sources

In this chapter, the process of auditing air surrounding unconventional sources and its associated considerations are introduced. Here, two methods, i.e., sampling and direct reading, are employed in the air auditing process.

###### 1-1 Scope of Application

The present instruction was developed to direct the periodical auditing process of air surrounding unconventional sources using two methods of sampling and direct reading, and measuring the gaseous pollutants and particulates including volatile organic compounds (VOCs), methanol, NO, NO2, SO2, LEL (CH4), H2S, CO, CO2, PM1, PM2.5, PM7, PM10, TSP, and TVOCs. VOCs include linear and cyclic hydrocarbons (C6+) and total volatile organic compounds (TVOCs). The sources of emissions referred to in the present instruction include flares and burning pits, evaporation ponds, and storage tanks used in operation areas associated with the Iranian oil and gas upstream industries.

###### 1-2 A Brief Introduction to Methodology

The air surrounding unconventional sources is audited using two methods, i.e., sampling and direct reading. The samples of the air (gaseous samples) present at the areas associated with the oil and gas upstream industries are obtained using appropriate sampling equipment. These equipment include tedlar bags, sorbent tubes, and impingers (Table 1-1).

Table 1-1 Outdoor Air Sampling Methods and their Corresponding Pollutants

| Impinger | Sorbent tube | Tedlar Bag | Method |
| --- | --- | --- | --- |
| H2S | VOCs | VOCs Methanol | Gaseous pollutants |

In the tedlar bag method, air is sucked into a bag at a certain rate and with the help of a pump. After being transported and stored under appropriate conditions, the sample is transfered to the lab where it will be analyzed using the gas chromatography method [1].

In the sorbent rube method, sampling is done by sucking a certain volume of air into the sorbet tube. During the sucking process, the gases and vapors will be attracted to the sorbent surface. After being transported and stored under appropriate conditions, the sample is transfered to the lab where it will be analyzed using the gas chromatography method [1].

In the impinger method, the sorbent solution is injected into the impinger. Next, the air is sucked into the impinger through an inlet and passes through the sorbent solution. After being transported and stored under appropriate conditions, the solution injected inside the impinger is transfered to the lab where it will be analyzed using the spectrography method [2].

Appropriate equipment should be used to carry out the auditing process based on the direct reading method in areas associated with the oil and gas upstream industries.

Some of these equipment include Phocheck+5000, Met One, Aeroqual, Gasalert MAX XT IIT, and VRAE (Table 1-2).

Table 1-2 Outdoor Air Sampling

Methods and their Corresponding Pollutants

| Met one | VRAE | Aeroqual | GasAlert MAX XT II | Phocheck+5000 | Method |
| --- | --- | --- | --- | --- | --- |
| - | SO2, NO2, NO, LEL (CH4) | H2S | H2S | TVOCs | Gaseous pollutants |
| PM2.5, PM1, TSP, PM10, PM7 | - | - | - | - | Particulates |

###### 1-2-1-2 Equipment

Each method (tedlar bag, coal sorbent tube, and impinger) requires its own equipment (Table 1-3) [1, 2].

Table 1-3 Equipment Needed for Different Sampling Methods

| Impinger | Sorbent tube | Tedlar Bag | Method |
| --- | --- | --- | --- |
| Impinger tubes, sorbent solutions, pumps, glass bottles, tubes, sample labels | Sorbent tubes, pumps, pump-sorbent tube attachments, sealed lids, sample labels | Bags, PTFE tubes, sample labels | Equipment |

###### 1-3 Methodology

This section shows us how the auditing process is performed using two methods of sampling and direct reading and presents us with the considerations we should take into consideration during the process.

###### 1-3-1 Sampling

The following sampling instruction includes on-site operations and measures taken afterwards by the personnel to transport and storage the samples taken of the air surrounding unconventional sources.

###### 1-3-1-1 Sampling method

The requirements associated with the two phases of preparation and sampling are as follows:

###### Preparation

1. The sampling site, and the number and type of equipment should be first determined.

2. All the necessary equipment should be supplied and adjusted.

3. If necessary. All the supplied equipment should be cleaned and the pump should be calibrated.

4. An initial inspection of the site should be conducted and all safety considerations been taken into consideration.

###### On-site operations

- Tedlar Bag

In the tedlar bag method, a bag is attached to the PTFE tube. The PTFE tube comprises the path through which the gas flow passes. A pump is attached to the tube, sucking the air into the bag. The air sucked into the bag never leaves the pump.

The pump’s flow rate is usually 3 liter per minute [1].

In the tedlar bag method, a vacuum container made of metal or plastic is used to carry the case during the sampling, storage, and transport processes. The container’s pressure should be negative and at least 10 inH2O. It should also be at least 20% smaller than the bag. It should be noted that the container should be large enough to carry the required sample.

The tedlar bag method is rather simple; however, some considerations should also be taken into account. The bag should be strong and durable, and neutral (no reactivity) to a wide range of chemicals. The tube and the bag should be properly attached to one another. The tube should be clean to avoid any contamination being carried over to the bag. Clean the bag thoroughly before re-using it. The bag is usually cleaned by blowing pure air or nitrogen into it.

It is recommended that an analysis be done after the bag was cleaned to ensure it is thoroughly clean. After the sample was collected, the sample label should be attached to the bag’s top edge. Do not stock the label on the bag’s body as some substances found in the label might penetrate into the bag. The note on the label should be written with a pen as markers and other similar tools contain VOCs that might penetrate into the bag. Thus, the following are the steps to be taken when the tedlar bag method is used for sampling:

- First, the bag’s inlet valve should be opened.

- Next, the pump’s outlet should be attached to the bag’s inlet.

- Now, the pump is tuned on to fill the bag.

- When the bag was full enough, the pump should be turned off and the bag's valve be immediately closed.

- Sorbent tube

- First, the sorbent tube should be unsealed at both ends.

- Next, the activated carbon sorbent should be attached to the pump using the charcoal - pump connector.

- The pump’s timing should be adjusted to suit the sampling time (10-15 min) (In urban area, the sampling is done in approximately 8 hours; however, when monitoring is done in the areas surrounding the sources of emission, the sampling time is shortened due to the high concentration of pollutants available at the site).

- The sorbent tube should be placed on a bar. It should not be obstructed by any object.

- The required meteorological parameters should be recorded.

- Now, the pump should be turned on.

7. Leave the pump to operate for a certain duration before pushing the “fault” button to see if it is properly operating.

8. Turn off the pump when the sampling process is completed.

9. Detach the activated carbon sorbent from the pump.

10. Other measurement parameters should be recorded in the sampling worksheets.

11. Samples should be kept in labeled bags. The labels should contain the sample's ID, total volume, and the results obtained through the required analyses. If there is an interval of more than a week between the sampling and the analysis, the sample should be stored in a container at a refrigerator or low temperature.

Low flow rate and high temperature and humidity can reduce the surface’s capacity to absorb the substance. Contamination might spread from the frontal parts of the tube to other sections. Keeping the tube in a cool place can reduce the likelihood of such consequences [1]. Be careful while selecting the sorbent tube. There are different sorbent tubes for different pollutants.

- Impinger

1. First, the gas sorbent solution (e.g., H2S) should be supplied.

2. Next, the impinger should be filled with the solution to the right level.

3. Connect the pump to the impinger before turning it on.

4. Air is sucked into the impinger through an inlet and passes through the sorbent solution, before leaving the pump. Make sure the pump’s exhaust does not interfere with the air entering the impinger (the air leaving the pump should not enter the impinger).

5. Turn off the pump and record the time.

6. The sample solution should be transferred into a vial [2].

###### 1-3-1-2 Transfer and Storage of Samples

The following measures should be taken to maintain the quality of the sample at the desired level [1 and 2]:

- After sampling is done, the sample should be coded and carefully sealed (in the sorbent tube method, special plastic lids should be used. Rubber lids are strictly prohibited).

- The codded sample should be kept in a cold place.

- Make sure the sample is not exposed to photolysis.

- The required forms should be sent along the sample being transported.

- In each phase, the collected samples should be personally protected by the person taking the sample or any other individual supervising the process until they are safely transported to another phase or section (transfer to another lab or shipped to another place).

- Ensure samples do not get contaminated. Using appropriate seals will considerably help protect the samples against contamination.

- All on-site sampling documents should be kept is a safe place.

- All sample transport (e.g., air transport) documents should be protected and kept in a safe place.

###### 1-3-1-3 Analysis Method

The samples collected by the tedlar bags and sorbent tubes are analyzed using the gas chromatography method and based on the NIOSH standard (VOC: NIOSH 2549 and BTEX: NIOSH 1501).

In this method, the concentration of VOCs is determined both individually and as TVOCs. The analysis process consists of sample preparation, calibration, quality control and measurement phases. In preparation

phase, sample is kept at ambient temperature and moisture tests are conducted to ensure sample’s moisture content is adequate. Next the desorption operation is conducted on the sample. In the calibration and quality control phases, the mass spectrograph is set up according to the operational requirements defined by the manufacturing company. Here, the device should be set on “Blank” mode (at least once) and the Spike samples should be prepared. Finally, the mass spectrum of the compounds comprising the sample is measured [3].

Absorption spectrography is used to analyze the sample collected through the impinger method, In this method, the analysis is done based on the absorption of radiation by the element's atoms.

The absorbed radiation is proportional to the substance’s concentration. The analysis process includes electric discharge, injection of sample into the flare, and conducting the calculations based on the absorbed radiation and calibration curves.

###### 1-3-2 Direct Reading

The following sampling instruction includes on-site operations and measures taken afterwards by the personnel to perform the measurements though the direct reading method.

###### 1-3-2-1 Direct Reading Method

As the direct reading method is continuous and is done by using the device at the site, the procedure through which the required equipment are used is not significantly different form the one used through the sampling method. First, the device should be calibrated. The device is turned on and the pollutant’s concentration at the intended point is read using the predetermined settings specifically adjusted for the intended application. Whenever appropriate, the recorded data will be exported from the device. In the direct reading method, the measurement should be repeated (3 times).

This must be done to make sure the data are reliable.

###### 1-4 Quality Assurance

1. All data should be recorded on pre-prepared worksheets.

2. All documentation including equipment transport documents (e.g., air transport), sample transport documents, worksheets completed at the site, etc. should be protected and kept in a safe place.

3. All equipment and tools should operate under appropriate conditions and as defined by the manufacturing company (unless certain special conditions have been planned for the sampling process).

4. The equipment should be inspected before sampling is done.

5. The pump (sampling method) and measurement equipment (direct reading method) should be calibrated.

6. All relevant quality control tests should be performed. In the sampling method, the sample is divided into at least two homogeneous parts and each part is analyzed separately. These two parts are sent to two different labs for the quality control tests to be performed. In the direct reading method, a certain pollutant is monitored at one site using two separate devices. The monitoring is repeated with one device.

7. The data collected by the device (direct reading) should be exported and recorded whenever appropriate [1 and 2].

###### 1-5 Safety

When taking samples of the air surrounding the sources of emission and monitoring the emissions though the direct reading method and throughout the operations conducted afterwards, certain on-site protective and safety measures should be taken into consideration. These measures include [1 and 2]:

- The individual should know that certain compounds will be lost during the initial hours after the sample has been collected.

- The individual should wear appropriate gears to protect different parts of his/her body.

- Toxic and hazardous compounds should be dealt with and handled according to certain standards including the OSHA standard. In this regard, the following points should be considered:

1. Individuals should be informed of the toxic and hazardous compounds likely to be found at the site.

2. All individuals should know the allowable exposure limit of each toxic and hazardous compound.

3. The required cautions should be exercised when transporting samples (sampling method).

###### 1-6 Selecting Points and Parameters to be Audited

The criteria used to select sampling points is as follows (the criteria apply to all instances where different equipment are used to periodically monitor the air surrounding sources of emissions):

These criteria are based on the technical and environmental status of the operation areas.

1. The topographic conditions of the area surrounding the selected equipment (sites with simple topographies are preferred)

2. The distance between other sources of pollution and the selected equipment (the effects of other sources of pollution on the equipment should be countered so far as possible)

3. Possibility of conducting the sampling process in the area surrounding the selected equipment (easy monitoring and maintaining personnel’s health and safety)

4. Operational plan of the selected equipment (removing the options with inconsistent performance throughout the year)

5. Selecting options with the most detailed information for each equipment

Accordingly, the most appropriate sites and, consequently, the intended points are determined. The monitoring points are selected based on the type of the source of emissions. In the present project, the auditing process is conducted through both sampling and direct reading methods described earlier. The sampling method is used to audit the air surrounding evaporation ponds, storage tanks, potentially leaking equipment. The direct reading method is employed to audit air surrounding flares, burning pits and evaporation ponds.

The auditing parameters are selected based on a number of factors the most important of which include nature of the source of emissions (combustible, evaporative, etc.), the method used to estimate emissions (parameters used in the calculation of emissions), monitoring costs, feasibility of monitoring the intended parameters given the available facilities, expected estimation accuracy, and in general, the objectives of the monitoring project.

In the following sections, the method used for the selection of sampling points and parameters corresponding to each emissions source are described.

1-6-1 Flares and Burning Pits

The pollutant emissions surrounding flares and burning pits are measured according to the procedure described in section 1-2-3-1. The auditing points selected through this procedure are based on the wind upstream and downstream. Accordingly, a minimum of one point at the upstream and three points at the downstream are selected for measuring the emissions using the direct reading method.

The first measure to be taken when selecting the sampling point at the flare’s downstream section is to find the point where the concentration is at its maximum. This point can be initially found by error and trial and using different dispersion models. Next, the monitoring points should be selected. The highest density of monitoring points should be around the point with the maximum concentration (to record a more accurate gradant of the concentration). Other monitoring points are located at longer distances before and after the maximum concentration point. As the concentration of pollutants changes with changes in the altitude, if possible, it is recommended that some point at different altitudes also be selected beside the point on the ground’s surface. Figures 1-1 and 1-2 show the recommended points for measuring emissions surrounding flares and burning pits.

2σy

50 m

Xmax

Background point

50 m

Measuring at the ground level and at height

Measuring at height

Figure 1-1 A design showing the recommended measurement points around a flare

5 m

Burning

Pit

5 m

30 m

30 m

50 m

50 m

Measuring at the ground level

Measuring at height

Background point

Figure 2-1 A design showing the recommended measurement points around a burning pit

The parameters to be monitored around flares and burning pits are determined based on the criteria mentioned in Section 1-6 (Table 1-4).

Table 1-4 Parameters to be Considered when Monitoring the Environment Surrounding Flares and Burning Pits

| Burning pits | Flares | Type of equipment |  |
| --- | --- | --- | --- |
| CO2, H2S, PM, NOx, SOx, CO | H2S, PM, NOx, SOx, CO2, CO | Pollutants | Parameters |
| Wind speed and direction, temperature, pressure |  | Meteorological parameters |  |

1-6-2 Evaporation Tanks

In the present project, the evaporation pond is audited using two methods of sampling (1-3-1-2) and measurement (1-3-2-1).

The auditing points are selected based on the wind upstream and downstream. Accordingly, the measurement process is carried out at a point located around the evaporation pond along the downstream and at different altitudes (if necessary, at three sides of the evaporation pond). The pollutant’s concentration should be determined at least one point along the wind upstream.

Figure 1-3 shows the recommended points for measuring emission surrounding evaporation ponds. The parameters to be considered when conducting the auditing process around an evaporation pond are shown in Table 1-5.

Table 1-5 Parameters to be Considered when Monitoring the Environment Surrounding Evaporation Ponds

| Evaporation pond | Type of equipment |  |
| --- | --- | --- |
| BTEX TVOC Methanol, H2S | Pollutants | Parameters |
| Wind speed and direction, temperature, pressure, relative humidity | Meteorological parameters |  |

Figure 1-3 A design showing the recommended measurement points around an evaporation pond

1-6-3 Floating Roof Tanks

In the present project, the floating rood tank is audited using two methods of sampling (1-3-1-2) and measurement (1-3-2-1).

The selected sampling points are where the the floating roof tank’s vents (control valves) are located. The selected direct reading points ate determined based on the floating roof tank’s deck dimensions. These points are located around the tanks and at different altitudes.

Figure 1-4 shows the recommended points for measuring emission surrounding floating roof tanks.

The parameters to be considered when conducting the auditing process around a floating roof tank are shown in Table 1-5.

Table 1-6 Parameters to be Considered when Monitoring the Environment Surrounding Floating roof Tanks

| Floating roof tanks | Type of equipment |  |
| --- | --- | --- |
| VOC CH4 | Pollutants | Parameters |
| Velocity, temperature, pressure | Meteorological parameters |  |

Figure 1-4 A design showing the recommended measurement points on a tank’s roof

1-7 Disclaimer The materials contained in this chapter are solely presented for conducting the auditing process associated with the present project. Thus, all relevant safety considerations might not be included in this document.

All individuals who are using the present report as their guideline should take into consideration all the safety points and precautions associated with the safety and health of personnel before attempting to conduct the sampling process.

### Chapter 2

Wastewater Auditing Instruction

The present auditing instruction includes all on-site operations conducted to select the sampling points, sampling methods, and procedures for transport and storage of samples, and measuring wastewater flow rate by audit personnel.

###### 2-1 Scope of Application

This instruction deals with wastewater sampling and flow rate measurement in open channels (e.g., evaporation pond’s and burning pits’ wastewater inflows and outflows).

Wastewater sampling aims at determining the pollutants occurring in wastewater. These pollutants include TVOCs, oil, grease, methanol, and total petroleum hydrocarbons (TPHs), BTEX, and C6+. In addition, some other values such as pH, Total Suspend Solids (TSS), Total Dissolved Solids (TDS), and weight percent ratio of petroleum to total wastewater (oil layer ‘s thickness) are measured.

###### 2-2 Sampling

The present instruction deals with the wastewater sampling process and its associated considerations.

###### 2-2.1 Materials and tools

The following materials and equipment are used for collecting samples from oily wastewater [4]:

- Sampling hooks

- Glass bottles used for collection the sample with a capacity of 1 liter and PTTTFE screw lids

- Acid solution for reducing the pH level of wastewater sample

- Cooling box to store samples at low temperatures

- Sample labels for coding the collected samples

- Safety gloves resistant to chemical and petroleum substances

#### 2-2-2 Methodology

Samples from the depth of oily wastewater are collected using a sampling hook and a glass bottle. The sample is collected from a point where water flow is running.

Wastewater sampling is conducted in three phases, namely, preparation, on-site operations, and post-sampling measures. The requirements are as follows:

###### Preparation

- First, the sampling site should be determined.

- An initial inspection of the site should be conducted and all safety considerations been taken into consideration.

- Sampling containers are made prepared.

- If necessary, sampling equipment and containers used for storing the samples should be cleaned (the glass bottle used to store the sample should not be washed with the sample itself).

- The containers used to store the samples should be coded (sampling time, date and site should be specified).

- The acid solution should be supplied in the volume needed to reduce the sample’s pH level.

- All the individuals involved in the sampling process should wear appropriate protective gears.

Note: Right before the start of the sampling process, remove the bottle’s lid and refrain from touching the inside of the container used to store the sample with your hands. If you accidentally touch the inside of the bottle with your hands, replace the bottle with a new one.

###### On-site operations

Sampling is performed using a sampling hook or a glass bottle at the sampling site. When collecting the sample, refrain from using pH papers, agitators, and other material that substances might stick into. Thus, the acid should be supplied in the volume required to reduce the pH level before conducting the on-site operations. If it is likely that the sample contains extractable substances in a quantity greater than 500 mg/L or particles and detergents interfering with the extraction process, a smaller sample should be collected.

The volume of acid used in the process should be reduced proportionally. It is highly likely that the extractable substances stick into the sampling container. This will result in smaller volumes of oil, grease, and hydrocarbons to be measured. Thus, mixes samples would be hard to collects (mixes samples are made of different individual samples). Thus, samples should be individually collected with the help of a sampling hook.

If a mixed sample is required to be measured, the samples collected by the

sampling hook should be individually analyzed at certain intervals. At the end, the average concentration value should be calculated. Alternatively, samples can be collected at the site and be mixed in the lab. For example, four 250-mL samples can be collected over the day.

Thus, the sampling steps are as follows:

1. When using the sampling hook, it should be immersed into the wastewater in a swooping motion. The hook’s opening should face the upstream.

When using the glass bottle, the bottle should be placed along the wastewater flow. The sampling process ends before the bottle gets full.

2. In cases where there is no need to close the glass bottle’s lid, the lid should be kept in a clean place.

3. After the sample was poured into the glass bottle, the pre-prepared acid solution is added to the sample to reduce its pH level (to avoid microbial degradation of the wastewater sample)

4. The collected samples are stored in a cooling box before finally being transported to a lab for further analysis.

###### Post-Sampling measures

The following steps should be followed after the required samples are collected in the glass bottles:

1. The glass bottles containing the collected sample should be sealed.

2. If necessary, ice packs should be placed in the boxed used for carrying the samples to follow the lab’s standard methods. Samples are vertically placed in the cooling box. The ice is required in a volume two times (maximum) the volume of the sample. Glass containers should be fixed and separated from one another using clean ice packs, plastic bottles and other packaging stuff.

3. The worksheets required by the lab should be completed and be placed in plastic bags placed on containers or attached to their wall.

4. The sample should be kept in a cooling box before the analysis (0-6°C).

5. The sample should be sent to the lab on time (not later than 28 days after the day when the sample was collected).

Note: Before heading to the sampling site, contact the lab conducting the analysis to make sure they are fully prepared to receive the samples.

###### 2-2-3 Sampling Site

Sampling site is selected using the Standard 1664 method as described in EPA guideline [4]. Oil, grease and hydrocarbons found in the wastewater might be a film occurring on the surface, an emulsion, a solution, or a combination of all three. As collecting a representative sample for analyzing oil and grease is rather hard, the individual conducting the sampling should carefully inspect the sampling site. The best sampling site would be a site with the richest mixture can occur so that the sample would represent the actual properties of the wastewater. In case of disagreement concerning the sampling site, sampling should be conducted at all sites in question.

The sampling site could be any place where wastewater is concentrated (e.g., evaporation ponds) or any other site corresponding to the objectives intended for the research project. However, sampling is mostly conducted at points where inflows and outflows occur.

###### 2-2-3-1 Taking samples from the inflows

The best sampling site would be where the flow is the most turbulent and has the richest mixture. However, in many instances, good sampling sites are not accessible. It is recommended that the inflow samples be collected before lateral flows join the inflows.

###### 2-2-3-2 Taking samples from the outflows

Samples should be collected at a site where the flow represents the real conditions of the wastewater outflow. This site can be found downwards from all the wastewater inflows and before the point where the outflow is discharged into the environment. [4].

###### 2-2-4 Transfer and Storage of Samples

After being collected, the samples should be properly stored and transported to the trusted laboratory. The steps taken to properly store and transport the samples are as follows:

- In each phase, the collected samples should be personally protected by the person taking the sample or any other individual supervising the process until they are safely transported to another phase or section (transfer to another lab or shipped to another place).

- Always make sure the place where samples are stored for transport is properly and sufficiently ventilated

- Samples should be stored in a cool, dry place. They should be kept away from heat, water and not be exposed to direct sunlight and other combustible and flammable source.

- The required cautions should be exercised when transporting samples (sampling method).

- All on-site sampling documents should be kept is a safe place.

- All sample transport (e.g., air transport) documents should be protected and kept in a safe place [1 and 2].

###### 2-2-5 Analysis Method

A gas chromatography-mass spectrometry (GC-mass) device and infrared radiation are used to analyze the collected sample in the lab. Chromatograph is mainly used to identify and measure the properties of volatile substances (like VOCs) or substances which might become volatile when mixed with some reagents or certain solvents. Chromatograph is used to analyze BTEX, methanol, and VOCs based on ISO11423 standard. It is recommended that infrared method and EPA standard method be used to analyze TPHs and Togs.

2-2-6 Quality Assurance

If appropriate methods are used for sample collection, transport and storage, sampling error can be minimized. In general, the following points should be considered to ensure the quality of samples:

- Sampling points should be properly identified and samples should be transported and stored under appropriate conditions.

- In each section, different samples (with the same volume) should collected to ensure they are of proper quality and to guarantee the results.

- All sampling steps should be taken in accordance with the requirements defined in the sampling standard) unless certain conditions are planned for the sampling process in advance).

- The sample’s analysis should be repeated using the same device. The sample is divided into at least two homogeneous parts and sent to at least two different labs (the labs’ records will be studied with records related to operations common between the labs being given an extra consideration).

- The sample should be analyzed under standard conditions.

###### 2-2-7 Safety

The required on-site safety and protective measures should be taken when collecting wastewater samples. These measures include:

- Samples might contain a high concentration of toxic volatile compounds. Wear masks to avoid inhalation of organic vapors.

- Acid-resistant rubber gloves and chemical-resistant cloths and shoes should be uses.

- Toxic and hazardous compounds should be handled according to relevant standards such as OSHA. The following points should be considered in this regard:

1. Individuals should study the MSDS (Material Safety Data Sheet) related to the material they are dealing with.

2. Individuals should be properly aware of the chemicals’ storage, clean-up, and disposal methods.

3. Avoid unnecessary discharging or spilling of chemicals. The remaining chemicals should be safely cleaned-up and collected/

4. Mercury-in-glass thermometers should not be used in field activities.

###### 2-3 Flow Rate Measurement

Any decision on water, wastewater, and oily wastewater flows requires that the decision-makers be aware of the currents’ flow rates.

This section deals with the methods used to measure wastewater flow rate.

###### 2-3.1 Materials and Equipment

Materials and equipment used to measure wastewater flow rate are as follows:

- High-precision timers

- Particles with the same density of wastewater

- High resolution cameras for photography

- Rectangular or triangular weirs

2-3-2 Methodology

Based on the available facilities and operational conditions, two methods can be used to measure wastewater flow rate. In the first method, flows are photographed at certain time intervals. The second method involves the use of weirs. Both methods are described in the following sections.

###### 2-3-2-1 Wastewater flow rate measurement by taking photographs of flows at certain time intervals

At two different moments, two photographs are taken on the flow and the particles found in it. Obviously, the particles have moved proportional to the velocity of the flow between the two photography intervals. The flow’s velocity can be measured based on the particle’s movement between these two intervals.

If the flow is free from particles whose movement can be followed, objects with densities approximately equal to that of the wastewater should be used. These objects (e.g., a piece of wood) can be released into the flow’s different cross sections. It should be noted that these objects should not get deformed or solved into the wastewater. The distance traveled by a particle in a certain time period can be determined by photographing the particle in the same time period. Photographs are analyzed and the traveled distance is divided by the elapsed time to determine the velocity of flow at its different cross sections. In order to determine the velocity profile of a certain cross section of the flow (Figure 2-1) at three different heights, objects are released into the flow. The velocity is calculated at each height. Finally, the flow’s velocity profile is obtained.

Figure 2-1 Velocity Profile in a Specific Cross Section

By measuring the area of the cross section, the flow rate of the current passing through the channel can be easily calculated based on the obtained velocity profile.

###### 2-3-2-1 Wastewater flow rate measurement by using weirs

Weirs are among the instruments used to measure wastewater flow rate. Weirs are, in fact, barriers places in the way of a flow. The flow rate can be measured based on the width of the weir and the level of the water above the weir. Weirs are divided into two groups, i.e., broad-crested weirs and sharp-crested weirs.

In this method, a weir with a cross section similar in dimensions to that of the flow is first cut and curved. When placing the weir in the way of the flow, make sure the fluid does not penetrate the walls and the weir does not get empty of the fluid. Wait a moment so that the flow becomes stable. By obtaining the Bernoulli equation between a point on the weir’s crest (a point where water seepage can be considered insignificant) and a point on the weir’s shutter, water speed can be calculated based on the level of the wastewater behind the weir. Thus, these value along with the area of an element from the flow’s cross section can be used to measure the flow rate of the current passing through the element. Total flow rate can also be obtained through partial integration.

The points to consider when measuring the flow rate are as follows:

- If a weir is used to measure the flow rate, it should be durable so that it can withstand the wastewater flow.

- The weir should be made of a material that can be easily cut and adapted to the flow’s cross section.

- The cut weir should completely fill the cross section with no leaks from the sides.

2-3-3 Safety

- This method might involve hazardous substances, operation, and equipment. In this method, all safety issues are not identified. The user shall be obliged to establish procedures that help maintain the health and safety of the personnel and determine the usability of monitoring restrictions before the auditing process. The personnel should receive the required training on the monitoring restriction before conducting the auditing process.

- Many hazardous compounds associated with wastewater might cause irritation (e.g., heptane), be of a corrosive nature and toxic (e.g., benzene and methanol). The sources of such compounds should be identified and the necessary preventive measure be taken to minimize their negative impacts.

2-3-4 Quality Assurance

- Check all devices (e.g., timers) and floating objects to make sure they can operate properly and their specifications correspond to the ones mentioned in the previous section.

- Constant values associated with the weir (e.g., height and discharge coefficient) and used in the equations should be carefully determined before conducting the measurement.

- In an attempt to improve the accuracy of calculations, the velocity at each part of the flow’s cross section is measured three times. In each step, the distance traveled by particles is fixed; however, the mean measurement time is calculated to determine the velocity at that specif part of the flow’s cross section.

### Chapter 3

Potentially leaking equipment and fixed-roof tanks

###### 3-1 Scope of Application

In the potentially leaking equipment and fixed-roof tanks, VOCs and methane (gas) are studied. The potential sources of leak to be measured in order to determine the levels of VOCs and methane (gas) include valves, flanges, fittings, and open-ended lines. Fixed-roof tanks operate like valves and flanges due to the outlets installed on their roofs.

###### 3-2 A Brief Introduction to Methodology

First, sources of leak of identified using leak detection devices such as Phocheck and leak check is performed. Next, in order to identify sources of leak within potentially leaking equipment and fixed-roof tanks, a portable device is used to measure the concentration level of VOCs and methane in the area surrounding the sources of leak. Subsequently, in order to determine the emission ratio, equipment are sealed and vacuum pumps are employed to make use of activated carbon sorbent for a specified period of time. Finally, the emission is measures using the relation between emission ratio and the determined on-site concentration level.

###### 3-3 Definitions

Leak concentration determination: Leak concentration is determined based on the amount of VOCs and methane observed on the source’s surface. Leak determination through direct reading devices is done by measuring a reference gaseous compound in the area surrounding the sources of leak [6].

Absence of leak: If no reading is recorded by the leak detection devices installed on the surface of the source of leak, no concentration level is reported for VOCs and other pollutants [6].

Reference gaseous compound: The concentration value of, for example, VOCs is determined by device calibration (for example, if concentration value of 10,000 ppm is shown for methane and the device is calibrated on methane, the defined concentration value is 10.000 ppm and methane is regarded as the reference gaseous compound) [6].

###### 3-4 Supplies and Equipment

- VOC detectors and devices used for detecting other defined pollutants are adjusted based on the processes compounds [6].

- The device should be able to measure the concentration value (direct reading) based on the leak that has occurred.

- Some of the devices used for measurement in pilots are Phocheck, Gas Alert, VRAE, and Aeroqual.

- The device should be able to read the emissions surrounding the source of leak and on its surface.

- The rate of the sucked flow should be measured using a constant flow sucked into an electric vacuum pump.

- The sucked flow can have a flow rate of o.oo4 to 0.1 ft3/min. It can be measured by installing a filter on the probe (preventing contact).

- Tools such as activated carbon (charcoal) sorbents are used after the sealing process in order to estimate emissions.

- The activated carbon sorbents identifiable by special unique codes are sent to the lab for further analysis.

3-5 Measuring Sources of Emissions within Potentially Leaking Equipment

3-5-1 Leak Investigation and Detection Make sure the leak detector is calibrated. Next, bring the device close to one of sources of leak (flanges, valves, open-ended lines). If leak detectors are used on the source’s surface, the potential leak will be investigated. The level of VOCs and methane present will be read and the screen value (SV) and the leak point will be determined. Subsequently, the maximum and minimum leak rates will be estimated. In order to obtain a 95% confidence interval, 50% of the equipment should be checked.

3-5-2 Concentration Measurement around Sources of Emission and Fixed-Roof Tanks The equipment is cleaned using a cotton wool cleaning cloth. Two layers of parafilm are wrapped around the equipment. Glue is used to attach the edges of parafilms to the equipment. Now, the surrounding area is checked for potential leaks. If no leak is detected, we can conclude that the sealing was properly done. Next, small holes (with diameters smaller that of the activated carbon sorbent) are made into the parafilm. The leak is discharged through these small holes. Direct reading devices are used to record the amount of pollutants occurring at the site. The acquired data will be registered in a pre-prepared table. The activated carbon sorbent is ready to be inserted into the hole (the activated carbon sorbent is coded and unsealed). When this is done, glue is used to the sorbent in its place. Next, a pump is u used to fill the sorbent in the certain amount of time. After the time required to fill the sorbent is finished (normally 15-30 minutes), special plastic lids are used to seal it. The sampling time is recorded in the special notebook. Finally, the collected activated carbon sorbents are stored in a cold place (cold box or refrigerator) to be sent to the lab for further analysis.

- Valves: Valves are the most common source of emission. The sampling probe should be installed after the valve is tightly closed.

- Next, the valve is sealed and suction is carried out specific time intervals [6].

- Flanges and fittings: First, leak is detected by the device. Next, parafilms are wrapped around the flanges. Sorbent is used at one point and at certain time intervals.

- Pump and sorbent are use for suction and pollutant collection, respectively [6].

- Open-ended lines: The sampling probe is installed at inlet exposed to open air [6].

3-5-3 On-site Detectable Pollutants To determine the concentration level of VOCs at the site, the device is places 1-2 m away from the VOCs’ emission source. The device is randomly places in the wind direction or opposite the wind direction. In case of any interference with other sources of emissions, the device is moved closer to the intended source of emission in order to determine VOCs’ concentration level. The minimum allowed distance between the device and the source of emission is 25 cm.

###### 3-5-4 Other Leak Detection Methods

- Soap solution can also be used to detect leaks in sources of emissions. However, this method cannot be used on moving parts for long periods of time [6].

- High concentration sprays can also be employed for leak detection. All the points referred to for the soap solution apply to this method as well. High concentration detergents or spray can help detect leaks sat points where the system's internal pressure is high [6].

###### 3-6 Transfer and Storage of Samples

Samples collected from leaks are kept in the activated carbon sorbents. The sorbents are sealed and stored in a cool place.

The samples are sent to the lab for further analysis in normally less than a week [6].

###### 3-7 Quality Assurance

Calibration should be performed on time to ensure the device is operating properly. VOC monitoring device is calibrated as follows: Before resetting and calibrating the device, it is warmed up‌ and calibration gas is injected into it. Rereading settings are adjusted and the device is ready to be used in the upcoming monitoring operations. Devices are calibrated by relevant companies and factories [6].

###### 3-8 Safety

- This method might involve hazardous substances, operation, and equipment. In this method, all safety issues are not identified. The user shall be obliged to establish procedures that help maintain the health and safety of the personnel and determine the usability of monitoring restrictions before the auditing process [6].

- Many hazardous compounds associated with wastewater might cause irritation (e.g., heptane), be of a corrosive nature and toxic (e.g., benzene and methanol). The sources of such compounds should be identified and the necessary preventive measure be taken to minimize their negative impacts [6].

### Chapter 4

An instruction on Using Portable devices for Chimney Auditing

The present instruction provides us with directions as to how gaseous pollutants and the velocity of chimney exhaust should be measured. As the methods used for measuring gaseous pollutants and velocity are rather similar, the gases produced through the combustion process and the velocity of chimney exhaust are simultaneously dealt with in this chapter.

###### 4-1 Measuring Combustion Gases and Velocities

4-1-1 Scope of Application The method introduced in this chapter is based in the instructions 1[7], 2[8], 3[9], 6[11], and 7[11] provided by the United States Environmental Protection Agency (EPA). These instructions include the procedure through which gaseous pollutants are measured, the flow’s properties are identified, and its contents (e.g., oxygen and carbon dioxide) are estimated using portable devices inside the chimney. This document can be referred to when measuring gas flows in channels, chimneys and ducts. It cannot be used in the following cases: 1) non-uniform or circular flows, and 2) chimneys with diameters smaller than 0.3m and cross sections smaller than 0.071 m2.

4-1-2 A Brief Introduction to Methodology This method is designed to measure the volumetric flow and concentration level of combustion gases released from a stationary source. The measurement point is selected within a certain part of the ongoing flow. At this point, the chimney’s cross-section is divided into a number of sections with equal areas. Measurement points are located in these sections. The gas flow is extracted and transferred to portable device user to analyze combustion gases. This portable device is used to measure the concentration level of the gas flow, determine its content and properties resulting from the combustion of gas inside the chimney.

###### 4-1-3 Definitions

Measurement system: Measurement system consists of equipment used to measure velocity and concentration level of chimney’s exhaust gases. These equipment include:

Sample Connectors: Sample connectors are parts of a system used to extract, transfer, and treat the sample before it enters the analyzer.

Analyzer: Analyzer is part of the system used to analyze the sample. It can comprise of electrochemical cells or be of an optical nature.

Sensitivity limit: Sensitivity limit is the minimum detection limit. It depends on the device and its accuracy.

Sensitivity limit should not be less than 0.1 of the nominal limit.

Nominal range: Nominal range is the range determined for measuring each component of the sample gas.

Nominal range should be defined so that all measurements will be within 20 and 80 percent of the analyzer’s overall measurement range.

###### 4-1-4 Supplies and Equipment

All portable measurement systems used to estimate velocity and measure combustion gas outflows from chimneys having measurement limits defined based on sensitivity limits and nominal range proportional to e concentration level of gases and chimney’s exhaust flow rate and having MCERT, EPA, and TUV certificates can be used. Lancom, TESTO and Kimo are among such measurement systems.

###### 4-1-5 Methodology

4-1-5-1 Locating the appropriate site for collecting samples of air pollutants and measurement of chimney's flow rate

The sampling site should be at least 8d away from the chimney’s gas inlet and at least 2d away from the chimney’s gas outlet (d = average diameter of chimney). In rectangular chimneys, the equivalent diameter is calculated based on (L = Chimney's length, Q = Chimney's width). In chimneys equipped with a scrubber, the sampling point should be at least 1.5d higher than the scrubber's end (chimney’s diameter above the scrubber).

Along this pathway (10d), chimney should not be bent, the diameter should not change, and no elbows or flame should be visible. If the above pathway is longer than 10d, the sampling site should be selected between 8d from the inlet and 2d from the outlet. The site is determined at the sampler’s discretion. Sampling conditions, the  possibility of installing a bench and sampling points govern this decision. The objective is to collect the sample from the chimney’s steady gas flow without making any turbulence.

In horizontal chimneys, sampling points are located between 8d from the inlet and 2d from the outlet, in the upper part of the chimney [7]. Figure 4-1 is a schematic view of the points where sampling ports are installed.

Figure 4-1 A schematic View of the point where the port is Installed on the Chimney

In short chimneys, it might not be possible to install the port 8d from the inlet and 2d from the outlet. Under such circumstances, the port is installed at least 4d from the inlet and at least 1 d from the outlet. In extremely short chimneys, it might not even be possible to install the port 4d from the inlet and 2d from the outlet. Under such circumstances, the port should be installed at least 1d from the outlet.  In short and extremely short chimneys, uncertainty over the calculated flow velocities is higher. Thus, based on how much the results are rationally acceptable, such uncertainties can be reduced by repeating the measurement process (at least three times) and increasing the number of measurement points. Scientific reviews and analyses (e.g., fluid dynamics modeling) should be performed to obtain the actual conditions of the flow as determined by the measurements.

###### 4-1-5-2 Number of sampling ports

If the chimney's diameter is between 30 and 244 cm, two points at a 90 angle are needed. If the chimney’s diameter is longer than 244 cm, four points each with a right angle to others are needed. If two ports cannot be installed on the chimney, at least one port would suffice. Figure 4-2 shows the number of port and their layout.

Figure 4-2 Number and Layout of Sampling Ports

###### 4-1-5-3 Specifications of sampling ports

The sampling port should have an exterior diameter of 10 cm and its opening should consist of a flange with a diameter of 15 cm. The distance between the port's opening and the chimney’s walls should be 7.6 cm [7]. Figure 4-3 shows a schematic view of the sampling port’s dimensions.

Figure 4-3 A schematic View of the Sampling Port’s Dimensions

###### 4-1-5-4 Sampling bench

A sampling bench is installed in front of each sampling point. The bench serves as a safe scaffold where samples are measured, and equipment are transferred and deployed. The sampling bench should have the following specifications:

The distance between sampling points and the bench's bottom should be 1.2 m. The bench's height and width should be both 1.1 m. For cylindrical chimneys with diameters longer than 1.1 m or big rectangular chimneys, the bench’s width should be as follows:

- For chimneys with diameters 1.2≤d ≤1.8: 1.56 m

- For chimneys with diameters 1/8≤d ≤2/4: 1.86 m

- For chimneys with diameters <2.4: 2.2 m

A safe ladder with fencing attached should be installed to make it available for the user to climb to the bench’s level. A lift should also be supplied for transporting measurement equipment to the sampling bench. The lift should have a minimum load capacity of 50 kg. 220-Volt electricity (15-20 A) should be supplied to the sampling bench so that devices can be operated when deployed there. Sufficient lighting should be provided to conduct sampling at night hours [7].

###### 4-1-5-5 Determining the number of measurement points

When the sampling site is located at least 8d away from the chimney’s gas inlet and at least 2d away from the chimney’s gas outlet, the number of measurement points is determined as follows:

- 12 sampling points if the duct’s cross-section is circular or rectangular with a diameter (or equivalent diameter) longer than 0.61 m (24 inch).

- 8 sampling points if the duct's cross-section is circular with a diameter between 0.3 and 0.61 m (12 and 25 inch).

- 9 sampling points if the duct's cross-section is triangular with an equivalent diameter between 0.3 and 0.61 m (12 and 25 inch).

When the sampling site cannot be located 8d away from the chimney’s gas inlet and at least 2d away from the chimney’s gas outlet, the minimum number of measurement points is determined according to Figure 4-4. The distances between measurement points and nearest upstream and downstream interferences are determined. The distances are divided by the duct's diameter or equivalent diameter to determine the number of measurement points.

Figure 4-4 Determining the Minimum Number of Testing Points Required for Non-Particulates Measurement

###### 4-1-5-6 Determining the location of measurement points

After the number of measurement point is determined, the points are lay out based on Table 4-1 (for chimneys with rectangular cross section) and Table 4-2 (for chimneys with circular cross sections). It should be noted that the numbers provided in Table 4-2 are a percentage of the chimney's diameter extending from the chimney's wall to the measurement point. In Figure 4-5 and Figure 4-6, a sample showing the layout of measurement points for rectangular and circular cross sections is provided.

Table 4-1 Layout of Measurement Points for Ducts with Rectangular Cross Sections

| 49 | 42 | 36 | 30 | 25 | 20 | 16 | 12 | 9 | Number of measurement points |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7×7 | 7×6 | 6×6 | 6×5 | 5×5 | 5×4 | 4×4 | 4×3 | 3×3 | Layout of points |

Figure 4-5 A Sample Layout of Measurement points in the circular Cross Section

##### (B)

Figure 4-6 A Sample Layout of Measurement Points in the Rectangular Cross Section

Table 4-2 Measurement Points in Chimneys with Circular Cross Sections

| Code of symetrial measurement points on a signle point | Number of symmetrical measurement points on a single diameter |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2 | 4 | 6 | 8 | 11 | 12 | 14 | 16 | 18 | 21 | 22 | 24 |
| 1 | 14.6 | 6.7 | 4.4 | 3.2 | 2.6 | 2.1 | 1.8 | 1.6 | 1.4 | 1.3 | 1.1 | 1.1 |
| 2 | 85.4 | 25.1 | 14.6 | 11.5 | 8.2 | 6.7 | 5.7 | 4.9 | 4.4 | 3.9 | 3.5 | 3/2 |
| 3 |  | 75.1 | 29.6 | 19.4 | 14.6 | 11.8 | 9.9 | 8.5 | 7.5 | 6.7 | 6.1 | 5.5 |
| 4 |  | 93.3 | 71.4 | 32.3 | 22.6 | 17.7 | 14.6 | 12.5 | 11.9 | 9.7 | 8.7 | 7.9 |
| 5 |  |  | 85.4 | 67.7 | 34.2 | 25.1 | 21.1 | 16.9 | 14.6 | 12.9 | 11.6 | 11.5 |
| 6 |  |  | 95.6 | 81.6 | 65.8 | 35.6 | 26.9 | 22.1 | 18.8 | 16.5 | 14.6 | 13.2 |
| 7 |  |  |  | 89.5 | 77.4 | 64.4 | 36.6 | 28.3 | 23.6 | 21.4 | 18.1 | 16.1 |
| 8 |  |  |  | 96.8 | 85.4 | 75.1 | 63.4 | 37.5 | 29.6 | 25.1 | 21.8 | 19.4 |
| 9 |  |  |  |  | 91.8 | 82.3 | 73.1 | 62.5 | 38.2 | 31.6 | 26.2 | 23.1 |
| 11 |  |  |  |  | 97.4 | 88.2 | 79.9 | 71.7 | 61.8 | 38.8 | 31.5 | 27.2 |
| 11 |  |  |  |  |  | 93.3 | 85.4 | 78.1 | 71.4 | 61.2 | 39.3 | 32.3 |
| 12 |  |  |  |  |  | 97.9 | 91.1 | 83.1 | 76.4 | 69.4 | 61.7 | 39.8 |
| 13 |  |  |  |  |  |  | 94.3 | 87.5 | 81.2 | 75.1 | 68.5 | 61.2 |
| 14 |  |  |  |  |  |  | 98.2 | 91.5 | 85.4 | 79.6 | 73.8 | 67.7 |
| 15 |  |  |  |  |  |  |  | 95.1 | 89.1 | 83.5 | 78.2 | 72.8 |
| 16 |  |  |  |  |  |  |  | 98.4 | 92.5 | 87.1 | 82.1 | 77.1 |
| 17 |  |  |  |  |  |  |  |  | 95.6 | 91.3 | 85.4 | 81.6 |
| 18 |  |  |  |  |  |  |  |  | 98.6 | 93.3 | 88.4 | 83.9 |
| 19 |  |  |  |  |  |  |  |  |  | 96.1 | 91.3 | 86.8 |
| 21 |  |  |  |  |  |  |  |  |  | 98.7 | 94.1 | 89.5 |
| 21 |  |  |  |  |  |  |  |  |  |  | 96.5 | 92.1 |
| 22 |  |  |  |  |  |  |  |  |  |  | 98.9 | 94.5 |
| 23 |  |  |  |  |  |  |  |  |  |  |  | 96.8 |
| 24 |  |  |  |  |  |  |  |  |  |  |  | 99.9 |

If the maximum allowable number of ports is one and the sampling probe cannot cover all the measurement points, from among the points suggested in Figure 4-5 and Figure 4-6, those with the possibility to be measured are selected. This will increase the uncertainty level of flow velocity’s measurement. Thus, based on how much the results are rationally acceptable, such uncertainties can be reduced by repeating the measurement process (at least three times). Scientific reviews and analyses (e.g., fluid dynamics modeling) should also be performed to obtain the actual conditions of the flow as determined by the measurements.

###### 4-1-5-7 Measurement Steps

- The device should be calibrated before use according to the recommended instructions provided by the manufacturer [9, 10, and 11].

- Make sure the sensors are calibrated by measuring clean air (under such circumstances, calibrated the level of O2 and other gases would be 20.9% and zero, respectively).

- Leak test should be conducted after each round of measurement.

- If a leak is detected, the O2 level shown by the device would be equal to the O2 level reported in fresh air. Potential leaks are investigated by blocking the gas suction path and reading the amount of gas sucked by the device sucked gas (flow rate of the device's pump).

- If no leak is detected, the flow rate obtained through direct reading would be zero.

- Make sure all cables and connectors of sampling probes are securely attached to the measurement device.

- Before conducting the measurement, make sure the equipment is in normal operation conditions [9, 10, and 11].  Before inserting the sampling probe into the chimney, make sure the sampling port is of an appropriate size. If the port's opening is larger than the sampling probe’s diameter, the space between the two should be filled with materials such as asbestos.

- The device should be operated according to the instructions provided by the manufacturing company.

- In order to measure velocity, the probe should be placed in a direction perpendicular to the flow of gas in the chimney.

- The coefficient for pitot tube is selected based in the type of the probe used in velocity measurement.

- The results should be recorded after the readings become stable [9, 10, and 11].

- The probe should not be left inside the chimney for a long time.

- The measurement should be repeated if the recorded data do not make sense.

- After each reading, the device will be given a chance to cool down and reach the ambient temperature. After the readings become stable, the device will be used to get new readings.

- After each measurement, sensors are cleaned through clean air suction.

###### 4-1-5-8 Quality Assurance

- The results will be reviewed based on the points mentioned in Section 4-1-7-7.

- All data should be recorded on pre-prepared worksheets.

- At each step, the measurements should be repeated three times [9, 10, and 11].

- Device calibration should be performed at certain intervals and according to the instructions provided by the manufacturing company [9, 10, and 11].

- If any problem occurs in the measurement process, comments should be added to the worksheets.

- Two devices should be used to record readings from a single point to guarantee maximum accuracy.

###### 4-1-6 Safety

Adhering to the present guideline might involve handling hazardous substances or working under accident-prone conditions (e.g., working at heights). Thus, the necessary safety instructions should be followed before using the present guideline. The safety considerations mentioned in the measurement system’s (analyzer device) instruction can also be helpful.

### 5. References

- Standard Operating Procedure, eleven, General Air Sampling Guidelines, U.S Environmental Protection Agency, 2003.

- Colls, J. and A. Tiwary, Air Pollution: Measurement, Modeling and Mitigation, Third Edition. 2009: Taylor & Francis.

- Volatile Organic Compounds (Screening) 2549, National Institute for Occupational Safety and Health, 1996.

- Agency., U.S.E.P., Methods 1664, Revision B: n-Hexane Extractable Material (HEM; Oil and Grease ) and Silica Gel Treated n-Hexane Extractable Material (SGT-HEM; Non-polar Material) by Extraction and Gravimetry.2010.

5. Surface Water (flows) Quality Monitoring Instruction, Issue: 522, 2010

- Determination of Volatile Organic Compound Leaks, Method 21, Environmental Protection Agency.

- EPA. Method 1 - Sample and Velocity Traverses for Stationary Sources. 40 CFR Part 60, Appendix A,

- EPA. Method 2—Determination of Stack Gas Velocity and Volumetric Flow Rate (Type S Pitot Tube).

- EPA. Method 3A - Determination of Oxygen and Carbon Dioxide Concentrations in Emissions from Stationary Sources (Instrumental Analyzer Procedure)

- EPA. Method 6C- Determination of Sulfur Dioxide Emissions from Stationary Sources (Instrumental Analyzer Procedure).

- EPA. Method 7E- Determination of Nitrogen Oxides Emissions from Stationary Sources (Instrumental Analyzer Procedure).