National Iranian Oil Company Safety, Health and Environment Management Petroleum Sludge and Waste Transactions Instructions Reviewing Translation; Confirmed by; Publication Date of English Version. Davood Naderi/ Laleh H. Abbasi Training & Performance Measurement Department; Mani Abdollahzadeh Rad/ HSE Manager; September 2018. Responsibility; Prepared by; Corroborated by; Approved by; Publication date. Full Name; Farzad Nejadbahadori [Signed] Sludge and Waste Committee Secretariat; Siavash Derafshi [Signed] HSE Director; Reza Khalili [Signed] Non-industrial operations Deputy; Ali Kardor [Signed] CEO; December 2016. Position. Document Code: NIOC-HSE-EN-WI-025-00 Revision date: December 03, 2016 Revision No.: 0

Table of Contents

1. Introduction:	4

2. Objective:	4

3. Scope of Application:	5

4. Definitions and Terms:	5

	Sludge (Petroleum Sludge) and Waste:	5

	Sludge and Waste committee at NIOC and its Associate Companies:	5

	Transaction:	6

	Valuable Non-Hydrocarbon Substances:	6

	Crude Oil Price:	6

	Volume of 200 Barrels:	6

	Resolution:	7

	Salt Content:	7

Extraction and titration method: IP-77  (Extraction and Titration)	7

	Water and Sediment Weight Percent:	7

ASTM D4007, IP 359/82, API MPMS CH.10.3	7

	Total Sulfur Content:	7

ASTM D4294 and ASTM D2622	7

	Pour Point:	7

A.S.T.MD 5853	7

	Specific Weight (Density):	7

Hydrometer Method: Standards ASTM D1298-99 , API 2547 , IP160/87	7

(Suitable for field tests)	8

Digital Density Meter: Standard ASTM D4052	8

	Viscosity:	8

	Hydrogen Sulfide:	8

	Base Price Discovery Formula:	8

Ps= Pih-((ρe+ Ste+ Be+ Sre)/100)	8

5. Legal References and Requirements:	8

6. Roles and Responsibilities:	9

	Operating Companies:	9

	International Affairs Department:	11

	NIOC HSE Department:	11

	Oil and Gas Production Supervision Department:	11

	General Directorate for Petroleum Transactions and Export Control (Ministry of Petroleum):	12

7. Qualitative Criteria for Pricing	12

8. Reference Quantitative and Qualitative Tables	12

10. Depositing Transaction Revenue	14

11. Reviewing the Sludge and Waste Resolution and Documentation	14

12 . Appendices	14

Appendix 1 - The Workflow Diagram	15

Appendix 8- The Barter Price of Sludge and Related Services	26

# 1. Introduction:

In the oil and Gas industry, a considerably large amount of petroleum sludge is produced during different processes, i.e., crude oil exploration, production, transportation, storage and export. Given the materials used in these processes, the products, the technology, and the employee’s educational level and culture, proper management of the materials involved is of utmost importance as they should be handled according to the relevant environmental standards.

Solid particles found in extracted crude oil stored in oil storage tanks in combination with wax, water, and oil emulsions produce some sticky sludge substances. These sludge substances have always been problematic, reducing the storage tanks’ capacity, contributing to their corrosion, etc.

Accordingly, the production company operating the operational unit takes measures to clean the oil storage tanks and discharge the petroleum sludge. The accumulated sludge is treated, recycled, and sold/disposed along with other waste materials produced during the processes.

Thus, a legal framework should be developed to maximize petroleum sludge recycling and sale and to prevent their improper disposal into the environment.

To this end, a committee was formed, consisting of representatives of HSE Department, General Directorate for Petroleum Transactions and Export Control, Oil and Gas Production Coordination and Supervision Department, International Affairs Department, Legal Department, and Financial Department. After holding several expert meetings, the present instruction was prepared.

Hopefully, the instruction will facilitate the operational and repair processes, resolve NIOC’s and its subsidiaries’ environmental problems, and avoid further unnecessary costs.

# 2. Objective:

The present instruction was prepared to create uniformity of practice in the implementation of Article 2, Note 1 of the notified Resolution No. 1892-31482 dated February 2, 2015 concerning the sludge and waste produced by NIOC, to simplify operational and repair conditions, to facilitate the transaction process, to avoid unnecessary costs, and to accelerate the process aimed at addressing environmental challenges.

# 3. Scope of Application:

This document shall apply to the exploration, drilling, production, transport, and storage chain at NIOC and all its subsidiary and associate companies.

# 4. Definitions and Terms:

## Sludge (Petroleum Sludge) and Waste:

Industrial waste or hydrocarbon-impregnated sediments removed from the exploration, drilling, production, transfer, and storage chain under certain operational conditions and requirements technically and operationally unrecyclable by the operator through the processing systems available at NIOC.

- Note 1: If proved technically and economically feasible, hydrocarbon liquids found in operational basins containing petroleum fluids and water will be transfered to special reservoirs through certain operational measures. After separating water from the liquids, the sampling will be done to perform the relevant qualitative tests (as mentioned in Table 1).

Subsequently, the necessary planning will be done to generate a mixture known as the “crude oil for export” (if mixing is possible and under the prerequisite that the qualitative properties of the crude oil for export be maintained) or to return it into the system to be reprocessed. If the results of the relevant analyses show that the aforementioned are not possible, the relevant decision shall be made by the Sludge and Waste Committee of subsidiary and associate companies and upon the approval of the relevant company’s/department’s highest authority.

- Note 2: For solid and semi-solid petroleum waste, the relevant decisions shall be made by the Sludge and Waste Committee of subsidiary and associate companies.

## Sludge and Waste committee at NIOC and its Associate Companies:

According to Article 1 of the notified Board Resolution No. 1892-31482 Dated February 2, 2015, the Committee is comprised of the representatives of Operations, Repairs, Chemicals , and HSE Departments of NIOC and its associate companies, and General Directorate for Petroleum Transactions and Export control (Ministry of Petroleum). The Committee shall technically, qualitatively, and quantitatively assess the produced petroleum sludge and waste, and prepare the relevant value tables in a case-by-case basis.

Given the above Resolution and the previous relevant experience, the representatives of the following departments shall be on NIOC’s Sludge (Petroleum Sludge) and Waste Committee chaired by HSE Department:

- HSE Department (Committee Secretariat)

- General Directorate for Petroleum Transactions and Export Control (Ministry of Petroleum)

- Oil and Gas Production Coordination and Supervision Department

- Legal Affairs Department

- International Affairs Department

- Financial Affairs Department

- Operating companies (as applicable)

## Transaction:

Transaction refers to any sale/recycling and/or barter operation carried out according to the workflow diagram shown in Appendix 1.

## Valuable Non-Hydrocarbon Substances:

The valuable non-hydrocarbon substances include valuable minerals, metals, elements such as nickel, vanadium, titanium, and lithium, radioactive substances, and other valuable inorganic substances detectable at the laboratories operating within the oil and gas industry.

## Crude Oil Price:

In the sludge and waste sale formula, “crude oil price” refers to the average official price of Iran’s heavy crude oil destined for Asia (M-1)  in the previous month and is shown by Pih.

## Volume of 200 Barrels:

According to the note added to Article 3 of the resolution, each barrel is equal to 159 liters. In cases where the number of barrels is less than 200, the companies’ relevant committees can proceed individually. This applies to all containers, equipment and basin, separately.

## Resolution:

In the present instruction, the term “resolution” refers to Resolution No. 1892-31482 dated February 2, 2015 issued by NIOC Board of Directors (Appendix 2).

## Salt Content:

The salt (sodium chloride) content of oil (gr/m3) according to the following standards:

Conductometer Method: ASTM D 3230

## Extraction and titration method: IP-77  (Extraction and Titration)

## Water and Sediment Weight Percent:

The water and sediment content of the sample (weight percent) is calculated according to the following standards:

## ASTM D4007, IP 359/82, API MPMS CH.10.3

## Total Sulfur Content:

The total sulfur content (free/compound) of oil (weight percent) is calculated according to the following standards:

## ASTM D4294 and ASTM D2622

## Pour Point:

The pour point, the lowest temperature (°F) at which an oil product can maintain its fluidity is calculated according to the following standard:

## A.S.T.MD 5853

## Specific Weight (Density):

The ratio of the volumetric mass density of a substance at a specific temperature to the volumetric mass density of water at a specific temperature is calculated according to the following standards:

## Hydrometer Method: Standards ASTM D1298-99 , API 2547 , IP160/87

## (Suitable for field tests)

## Digital Density Meter: Standard ASTM D4052

(Suitable for laboratory)

## Viscosity:

The fluid’s resistance to flow (C.st).

## Hydrogen Sulfide:

The H2S content of the sample (parts per million)

Note: The above-mentioned standards are used to analyze sludge and waste in their liquid forms. Other relevant standards should be used to analyze sludge and waste in their solid and semi-solid forms as deemed appropriate by the committee.

## Base Price Discovery Formula:

The following formula is used to calculate the initial base price of sludge and waste produced during the relevant processes (the executable Excel file  - Appendix 3). The relevant explanations are presented in Appendix 4.

## Ps= Pih-((ρe+ Ste+ Be+ Sre)/100)

Note: The above-mentioned formula is used to calculate the price of sludge and waste in their liquid forms.The calculation of the price of sludge and waste in their solid and semi-solid forms shall be done as deemed appropriate by the committee and upon the approval of the relevant organization’s highest authority.

# 5. Legal References and Requirements:

- The notified resolution No. 18-9231482 dated February 2, 2015 by NIOC’s Board of Directors

- Letter No. H/316924 dated 28 September, 2015 by Legal affairs Department (Appendix 8)

# 6. Roles and Responsibilities:

## Operating Companies:

1- Establishing the sludge and waste committees assigned with the following tasks:

- Receiving reports on the traded sludge and waste in the exploration, drilling, production, transfer chain from relevant sources.

- Holding meetings with the presence of committee members

- Preparing the required quantitative and qualitative reports, confirming the minutes, and producing the feasibility study report

- Placing a base price inquiry from International Affairs Department pursuant to NIOC International Affairs Department’s Letter No. MB 18055/55 dated July 1, 2015 and Letter No. MBA 94-33245 dated December 23, 2015 (Appendices 5 and 6)

- Submitting the report on the measures taken by the General Directorate for Petroleum Transactions and Export Control, Oil and Gas Production Coordination and Supervision Department, and NIOC’s HSE Department

- Estimating the annual sludge and waste production and submitting the relevant report to the General Directorate for Petroleum Transactions and Export Control, Oil and Gas Production Coordination and Supervision Department, and NIOC’s HSE Department

2. Submitting the quantitative and qualitative tables approved by the committees to the General Directorate for Petroleum Transactions and Export Control, Oil and Gas Production Coordination and Supervision Department, and NIOC’s HSE Department

3. Defining and specifying the Transaction statement of Work

4. Employing new methods to reduce and prevent the production of sludge and waste during the relevant processes

5. Conducting transactions and depositing the transaction revenue (if available) in NIOC’s account after deducting the relevant operational costs

6. Complying with NIOC’s HSE requirements (Appendix 7)

7. Undertaking the remaining assigned tasks within the framework of the resolution

## International Affairs Department:

According to the provisions stipulated in the International Affair’s Department’s Letter No. MB 24006/55 dated September 8, 2015, transport costs (if necessary) should be taken into consideration when making transactions. At the time of the transaction, the relevant companies and departments shall communication in writing with the International Affairs Department and inquire for the updated prices, which will be communicated to the purchasing company within 3 working days.

2. Annual revision of the proposed base price estimation method

3. Undertaking the remaining assigned tasks within the framework of the resolution

## NIOC HSE Department:

1 - Committee Secretariat

2. Supervising over the proper implementation of the Resolution’s provisions

3. Exchanging of experience with the companies

4. Negotiating with international companies operating in the field of sludge and waste recycling

5. Contributing to the subsequent coordination among the companies to accelerate the transaction process (if necessary)

6. Effectively communicating with production companies

## Oil and Gas Production Supervision Department:

1. Establishing a database containing quantitative and qualitative data on the companies’ available sludge and waste following the submission of the report on the relevant quantitative and qualitative tables

2. Undertaking the remaining assigned tasks within the framework of the resolution

## General Directorate for Petroleum Transactions and Export Control (Ministry of Petroleum):

1. Given the provisions stipulated in the aforementioned Resolution requiring the presence of a representative from the General Directorate for Petroleum Transactions and Export Control in the Sludge and Waste Committee, the prepared quantitative and qualitative tables should be approved by the General Directorate’s representative.

2. In the event of any disagreement between the members of the Sludge and Waste Committee of NIOC’s associate and subsidiary companies, the Committee shall process as the representative of the General Directorate for Petroleum Transactions and Export Control deems appropriate.

3. Undertaking the remaining assigned tasks within the framework of the resolution

# 7. Qualitative Criteria for Pricing

Pursuant to the Enquiry from the International Affairs Department in the Letter No. MB 18055/55 dated July 1, 2015, the qualitative criteria for pricing petroleum sludge and waste are their specific weight (density), salt content, water weight percent, solid content, and sulfur content. These criteria and their impact on the pricing are specified in the relevant formula (Appendix 5).

# 8. Reference Quantitative and Qualitative Tables

The reference quantitative and qualitative tables were drawn up to determine and systemize the parameters and units required to estimate the initial price of sludge and waste, to establish a database to be used in similar situations, and to save time and money when deciding over future shipments.

The following table introduces the relevant criteria and standards, is based on the qualitative criteria proposed by NIOC’s International Affairs Department for petroleum sludge and waste pricing, is approved by the Oil and Gas Production Supervision Department and General Directorate for Petroleum Transactions and Export, and consequently, can be used to respond to price enquiries by the companies party to the transactions.

Liquid Petroleum Waste - Qualitative Table

| Test | SPECIFICATION | Unit | TEST METHOD | RESULT |
| --- | --- | --- | --- | --- |
| 1 | SPECIFIC GRAVITY @ 15.56 /15.56 °C or API | ‐ | ASTM D 5002 OR ASTM D 1298 |  |
| 2 | TOTAL SULPHUR CONTENT * | Wt.% | ASTM D 2622 / ASTM D 4294 |  |
| 3 | H2S CONTENT * | ppm | ‐ |  |
| 4 | SALT CONTENT * | P.T.B | ASTM D 3230 / IP 77 |  |
| 5.1 | KINEMATIC VISCOSITY @10 ° C | mm2 /Sec | ASTM D 445 |  |
| 5.2 | " " @ 20 ° C | mm2 /Sec | ASTM D 445 |  |
| 5.3 | " " @ 40 ° C | mm2 /Sec | ASTM D 445 |  |
| 6 | POUR POINT | ° C | ASTM D 5853 |  |
| 7 | ACIDITY , TOTAL * | MGk OH/gr | UOP 565 |  |
| 8 | BS&W | % | ASTM D 4007 |  |
| 9 | R.V.P |  |  |  |

9. Sludge and Waste Measurement Method

The initial estimate of the amount of sludge should be made based on volume of the reservoirs calculated using the current methods or the operational documents associated with the time when the pits were emptied and the operating company’s previous experience.

The precise estimate of the amount of sludge and waste should be made during the loading. The weight method will have priority and the truck scales should be approved by the General Directorate for Petroleum Transactions and Export.

In special cases, the volumetric method can be considered for substances in their liquid form.

All the measurements mentioned in the present instruction (quantitative and qualitative) should be supervised and approved by the General Directorate for Petroleum Transactions and Export.

# 10. Depositing Transaction Revenue

The transaction revenue (if available) should be deposited in NIOC’s account after deducting the relevant costs.

# 11. Reviewing the Sludge and Waste Resolution and Documentation

Given the Resolution’s emphasis on its experimental three-year implementation, all the relevant companies and departments shall be obliged to compile their own experience and technical and operational opinions to the General Directorate for Petroleum Transactions, Oil and Gas Production Coordination and Supervision Department, and NIOC’s HSE Department upon the completion of the transaction. This information will be used to prepare the new version of the Resolution.

# 12 . Appendices

## Appendix 1 - The Workflow Diagram

Appendix 2 - NIOC Board Resolution

National Iranian Oil Company

In the Name of God

Date: February 2, 2015

No.: 1892-31482                                                                                            Notification of the Board Resolution

“Amendment”

To: NIOC CEO

With reference to NIOC’s 1892nd Board Meeting dated September 28, 2014 (Sunday), Letter No. MBT 414850 dated November 26, 2014 by the Consolidated Planning Department, and Letter No. MNTH 395425 dated November 17, 2014 by Oil and Gas Production Supervision Department forwarded by NIOC CEO, “the request to amend Resolution No. 1892-31482 dated September 28, 2014 concerning the environmental problems posed by the sludge produced in the reservoirs, pipelines and facilities of production units” made in NIOC’s 1906th Board Meeting dated January 18, 2015 was reviewed and the Reservoir Sludge Instruction, part of the aforementioned Resolution was reviewed, amended, and replaced as follows:

Objective: Establishing a specific framework for sludge and waste transactions in order to facilitate operational conditions, avoid further unnecessary costs, and addressing the environmental challenges of NIOC and its associate companies.

Application Domain: Exploration, drilling, production, transport, and storage chain at NIOC and all its associate companies.

Sludge and Waste – Definition: Industrial waste or hydrocarbon-impregnated sediments removed from the exploration, drilling, production, transfer, and storage chain under certain operational conditions and requirements technically and operationally unrecyclable by the operator through the processing systems available at NIOC.

Article 1: The sludge and Waste Committee is comprised of the representatives of Operations, Repairs, Chemicals, and HSE Departments of NIOC and its associate companies, and General Directorate for Petroleum Transactions and Export control (Ministry of Petroleum). The Committee shall technically, qualitatively, and quantitatively assess the produced petroleum sludge and waste, and prepare the relevant value tables in a case-by-case basis.

Note 1: Any increase in the number of Committee members shall be decided by and approved by the board of directors and/or CEO of NIOC and its associate companies in a case-by-case basis.

Note 2: The heads of Sludge and Waste Committees at NIOC and its associate companies shall be appointed by the CEO of the relevant company.

Article 2: The Committee shall examine the sludge and waste produced by NIOC and its associate companies, perform the required samplings, estimate the volume of the produced sludge and waste, prepare relevant reports, submit the samples to the laboratory based in the relevant company or Research Institute of Petroleum Industry, and draw up the associated quantitative and qualitative tables.

Note 1:  The committees at NIOC and its associate companies should formulate the procedures to be used in the preparation of the quantitative and qualitative tables according to the criteria and standards governing the industry.

Note 2: To prevent any delay in the activities and as deemed appropriate by NIOC, the Committee should prepare some samples as the representative sample (with properties similar to those of the produced sludge and waste), and draw up the relevant mean value tables to be submitted to the pricing authority (International Affairs Department). The relevant Committee should determine the number of representative samples to be prepared.

Note 3: NIOC and all its associate companies should take the required measures to examine the possible presence of non-hydrocarbon elements in the produced sludge and waste. If the presence of non-hydrocarbon elements was confirmed, the relevant assessment committee should take the required measures to proceed as appropriate.

Article 3: NIOC and its associate companies should submit the tables containing the quantitative and qualitative data on the produced sludge and waste to Oil and Gas Production Supervision Department. Upon examination and approval, the tables will be communicated to International Affairs Department for base price estimation.

Note: In order to accelerate the completion of repair projects, facilitate operational processes, and avoid unnecessary costs, the aforementioned Committee shall decide whether or not the International Affairs Department should estimate the base price for sludge and waste shipments smaller than 200 barrels. It should be noted that the sludge and waste shipments should not be divided in shipments containing less than 200 barrels.

Article 4: Upon receiving the relevant quantitative and qualitative tables, the International Affairs Department should communicate the base price to Oil and Gas Production Supervision Department. NIOC and its associate companies will, in turn, be notified of the announced base price.

Article 5: NIOC and its associate companies shall take the environmental considerations into account and be obliged to make the appropriate decision over the aforementioned substances (sale/recycling). The decision shall be approved by the aforementioned committee and be in accordance with the relevant regulations. The acquired revenue should be deposited in NIOC’s account.

Note: In cases where there are no purchasers and the feasibility study report indicates that no decision can be made on how to handle the produced sludge and waste (sale/recycling), these materials should be disposed according to the relevant environmental considerations. The disposal should be approved by the assessment committee and carried out under its supervision.

Article 6: NIOC and its associate companies are obliged to use the latest methods and procedures to reduce/prevent sludge and waste production, and minimize relevant environmental and operational problems.

Article 7: The present instruction aims at addressing the environmental challenges and facilitating the operational processes. It is an experimental instruction to be implemented in a three-year period starting from its notification date. NIOC’s Oil and Gas Production Supervision Department is responsible for supervision over its proper implementation.

Bijan Zangeneh

Oil Minister

[Singed]

Ministry of Petroleum Headquarters

The General Administrative Automation System

Letter No.: 569164

Registration Date: February 21, 2015

Taleghani St., Tehran, Tehran, Iran – PO Box: 1863 – Phone: +98 21 61622929 – Fax: +98 21 61623886

Appendix 4 - The proposed Formula for Sludge Base Price Discovery

The proposed Formula for Sludge and Waste Base Price Discovery

()

Ps= Pih-(Pe+ Ste+ Be+ Sre100)

Pih: (M – 1): The average official price of Iran’s heavy crude oil destined for Asia in the previous month

(M – 1): Persian Gulf F.O.B of Iranian crude oil

P (gr/cm3): Density (gr/cm3)

Pe: The impact of density on base price

Pe = (P – 0.8756)*10000

St (mg/kg): Salt content (mg/kg)

Ste: The impact of salt content on base price

Ste = (St/300)*10

BS&W (wt%): Water and sediment weight percent

Be: The impact of water and sediment on base price

Be = (BS&W (wt%)/0.1)*10

Sr(wt%): Sulfur weight percent

Sre: The impact of sulfur on base price

Sre = (Sr(wt%)/0.1)*10

Appendix 5 - Base Price Enquiry

National Iranian Oil Company

International Affairs Department

In the Name of God

Date: July 1, 2015

No.: MB 18055/55

To: Mr. Jashnsaz, Director of Oil and Gas Production Supervision Department

With reference to Letter No. MNTM 136703 dated June 15, 2015 concerning price estimation of petroleum sludge and waste, please kindly be advised that as petroleum waste has many different qualities (from solid to fluid) and is sometimes of a rather poor quality, its price cannot be determined based on the technical properties as the oil price is. At the same time, petroleum waste is not considered as a trade item able to be uninterruptedly offered in international markets. Hence, given the low volume of petroleum waste and other relevant problems, such as their highly scattered distribution and challenging transportation, it is recommended that the auction framework be used to discover their price. However, with respect to the presented technical properties and in order to provide a theoretical strategy, the approximate price of petroleum waste should be calculated while taking the following into consideration:

- Calculating the price of petroleum waste as a discount on the price of the Iranian heavy crude oil (the latest official price of the Iranian crude oil (May) is $61.52 per barrel)

- For shipments with densities lower that 0.9600, a 10-cent discount per barrel for each additional 0.0010 density if exceeding 0.8756; For shipments with densities higher than 0.9600, a 20-cent discount per barrel for each additional 0.0010 density (due to their extra heavy weight and mixture of different sulfur, carbon, and harmful metal sediments)

- 10 cent per barrel for each 300 mg/kg salt content

- 10 cent per barrel for each 0.1% water and solid weight (max. 30%)

- 10 cent per barrel for each 0.1% sulfur weight (min. 2%)

- 20 cent per barrel for each temperature degree higher than the pour point -20 (crude oil pour point)

It should be noted that the above-mentioned method cannot be used to calculate the price of petroleum waste with densities higher that 1.01 or with water and sediment content greater that 35% (weight percent). Thus, for such cases, the best strategy would be to adopt an auction framework.

Seyyed Mohsen Ghamsari

Director of International Affairs Department

[Signed]

Ministry of Petroleum Headquarters – NIOC Headquarters

The General Administrative Automation System

Letter No.: 171965

Registration Date: July 4, 2015

Appendix 6 - Base Price Enquiry through the International Affairs Department Website

]

National Iranian Oil Company

International

In the Name of God

Date: December 23, 2015

No.: MBA / 94 – 3245

Attachment: -

To: Mr. Jashnsaz, Director of Oil and Gas Production Supervision Department

Subject: Investigating Sample Contracts and the Instruction for Implementation of Sludge and Waste Transaction Resolution

In reference to Letter No. MNTH – 431755 dated December 1, 2015, please be kindly advised that, currently, all of Iran’s crude oil and petroleum products sale contracts are signed according to the export terms and conditions passed by NIOC’s Board of Directors. Amendments and changes are only allowed in section such as payment method and loading.  These amendments and changes are made according to the market conditions and with the purchaser’s agreement. Thus, it is recommended that the extensive experience of the National Iranian Oil Products Distribution Company (NIPPDC) be used in in the sludge contracts signed with domestic purchasers.

In addition, with respect to the International Affairs Department’s responsibilities stipulated within the Instruction for Implementation of Sludge and Waste Transaction Resolution dated October 25, 2015, please be advised that the response time to the base price enquiries made in writing shall be changed from 3 working days to one month according to Resolution No. 1892/30482 dated February 2, 2015 passed by the NIOC’s Board of Directors. Finally, it should be noted that the price of Iranian heavy crude oil (Sludge transactions base) will be available at NIOC’s International Affairs Department, www.nioc-intl.ir on a monthly basis.

Mohammadali Keramati

[Signed]

Head of Domestic Reservoirs Construction Supervision Committee and

International Affairs, marketing and Crude Oil Operations Deputy Director

Ministry of Petroleum Headquarters – NIOC Headquarters

The General Administrative Automation System

Letter No.: 472895

Letter No.:

Registration Date: December 31, 2015

Registration Date:

Appendix 7 - HSE Requirements: Appendix to the Waste and Sludge Transactions Instruction

1. The purchaser shall comply with NIOC’s and the subsidiary’s current HSE requirements and instruction during the contract period.

2. Compliance with the Contractors HSE Instruction No. 527855 dated February 2 , 2016 notified by the Deputy Minister and NIOC CEO is mandatory.

3. The subsidiary’s HSE department is responsible for supervision over the waste and sludge produced during the whole process (from production to recycling and disposal) for the duration of the sale contract.

4. Before any attempt to sell sludge and waste, the subsidiaries should prepare an MSDS. The subsidiary’s HSE department is responsible for performing this task.

5. The subsidiary’s HSE department should investigate and confirm if the purchaser is able to use the given sludge and waste as declared. The subsidiary’s HSE department should investigate and ensure that the use of the aforementioned substances, especially sludge, does not violate the environmental requirements and will not harm the environment.

6. In case of any accident taking place after the delivery of sludge and waste to the purchaser, the purchaser is obliged to notify the subsidiary of the circumstances in writing. The accident report should be submitted by the relevant company’s HSE department  to NIOC’s HSE Department within a maximum of three days. The purchaser is responsible for cleaning the environment after the accident and should be committed in doing so.

7.  The subsidiary’s HSE Department is obliged to retain the relevant documents, including records of contracts, bills of lading, and accident reports, for at least five years after their submission.

8. The subsidiary shall be obliged to comply with the relevant HSE requirements and assess the customer's environmental competence and capability to undertake the contract.

## Appendix 8- The Barter Price of Sludge and Related Services

Legal Affairs

National Iranian Oil Company

In the Name of God

Date: September 28, 2015

No.: HM 316924

Attachment: Yes

To: Mr. Derafshi, HSE Director

Subject: Addressing the Environmental Challenges Posed by Petroleum Waste

With reference to Letter No. 94-305710 dated September 22, 2015 concerning the above-mentioned subject, please be kindly advised that according to Resolution No. 1892-31482 dated February 2, 2015 on “establishing a specific framework for petroleum sludge and waste transactions so as to facilitate operational processes, avoid unnecessary costs, and address environmental challenges faced by NIOC and its associate companies” (attached image), the produced waste shall technically and operationally recyclable using the processing systems of NIOC.

According to Article 1 of the aforementioned resolution, the sludge and Waste Committee is comprised of the representatives of Operations, Repairs, Chemicals, and HSE Departments of NIOC and its associate companies, and General Directorate for Petroleum Transactions and Export control (Ministry of Petroleum). The committee was established to technically, quantitatively, and qualitatively assess the produced sludge and waste, and prepare the relevant value tables. In addition, according to Article 3 of the aforementioned resolution, NIOC and its associate companies should submit the tables containing the quantitative and qualitative data on the produced sludge and waste to Oil and Gas Production Supervision Department. Upon examination and approval, the tables will be communicated to International Affairs Department for base price estimation. According to this resolution and the provisions stipulated in Article 5 of the aforementioned resolution (the revenue to be deposited by the operating company in NIOC’s account), the sludge can be bartered for the offered services.

Given the above, regardless of technical and financial aspects requiring the opinion of International Affairs, Consolidated Planning, and Financial Affairs Departments, in cases where the definitions provided for the term “Petroleum Waste” are in accordance with with the provisions stipulated  in the above-mentioned resolution, the methods mentioned in the resolution could be used to address the relevant environmental challenges.

Aliakbar Mahrokhzad

Director of Financial Affairs

[Signed]

Director of Consolidated Planning Office

No.: 316924

Date: September 29, 2015

Attachment:

Energy Consumption Optimization and Project Assessment Department

No.: 316924

Date: October 4, 2015

Attachment:

In the Name of God

Mr. Azizi,

Mr. Shadrokh,

FYI

[Signed]

10/03/2015

Mr. Khaksaran,

Mr. Naghibi,

[Signed]

10/01/2015

No. 17, Yaghma Alley, Jomhouri-e Eslami St. (btwn Hafez St. & Valiasr Ave.), Tehran, Tehran, Iran

Phone: +98 21 66726016, Fax: +98 21 6609723, SMS System: +98 10007615

HSEE@NIOC.IR	HTTP://HSE.NIOC.IR