Guideline for the classification and coding of wastes in the HSE management system

MOP-HSED-Gl-302 (0)

Revision table

| Revision No. | Date | The section/sections number | The page/pages number |
| --- | --- | --- | --- |

Contents

1. Purpose ……………………………………………………………………………………….………… 4

2. Scope …………………………………………………………………………………………………… 4

3. Responsibilities and enforcement processes …………………………………………………………… 4

4. Reference requirements and documents …………………………………………………………..…… 4

5. Definitions ……………………………………………………………………………………………… 5

5.1 Waste classes ……………………………………………………………..…………………………… 5

5.1.1 Hazardous wastes …………………………………………………………………………………… 5

5.1.2 Non-hazardous wastes ……………………………………………………………………………… 7

5.2 Waste code ………………………………………………….………………………………………… 7

6. Procedures ……………………………………………….……………………………….…………… 13

Appendix 1 - Waste Classification Checklist …………………………………………………………… 14

1. Hazardous waste determination ……………………………………………………….……………… 15

1.1 Possible Exclusions from Hazardous Classification ………………………………………………… 15

1.2 Listed Hazardous Waste Determination ………………………………..…………………………… 15

1.3 Needed information to make the determination ………………………...…………………………… 15

2. Nonhazardous Industrial Waste Classes 1 and 2 …………………………...………………………… 17

3. Nonhazardous class 3 wastes …………………………………………………………………….…… 19

3.1 Initial determination of class 3 wastes ……………….……………………………………………… 19

3.2 Final determination of class 3 wastes ……………………………..………………………………… 19

Appendix A- Ignitable solids ………………………………………………………………….………… 20

Appendix B- Hazardous Substances …………………………………………………………..………… 22

Appendix C- Class 1 Toxic Maximum Constituents’ Leachable Concentrations ….………………… 29

Appendix D- 7-Day Distilled Water Leachate Test’s Maximum Contaminant Levels ……….………… 31

Appendix E- Class 1 Toxic Constituents ……...………………………………………………………… 32

Appendix 2- Form codes ………………………………………………………………………………… 35

1. Introduction …………………………………………………………………………………………… 36

2. Definitions associated with the form codes ………………………………………………………… 36

1. Purpose

To establish and maintain the integrity and consistency of the HSE management system, this document is provided as a guide to achieve the objectives of waste management in this management system. This document provides some of the minimum requirements needed to meet the waste management system requirements and helps companies/organizations active in the oil industry to establish such processes. All units active in the oil industry in Iran and subsidiaries and all industrial/service units located in the special economic zones of the oil industry of Iran shall comply with the requirements of this guideline.

The document is prepared as a technical guide to complete the waste management system requirements (No. MOP-HSED-Gl-301). It shall be used to classify and codify wastes and to prepare reports on the waste management process.

2. Scope

The requirements specified in the present guideline apply to wastes produced in all operational and non-operational units, special zones, cultural/educational/tourism settings and their facilities, as well as those produced outside the unit/complex/area and transported into it for treatment, processing, recycling, storage, conversion, burning or disposal. It should be stated that this guideline should be applied for all kinds of wastes, including: industrial and non-industrial, urban, agricultural and medical wastes.

3. Responsibilities and enforcement processes

3.1 In each organization, the highest authority is responsible for the establishment and implementation of this guideline.

3.2 In all units/facilities and subsidiaries, HSE Management shall supervise the proper implementation of this guideline.

3.3 The proper implementation of the requirements of the present document in each organization is among the major criteria used to assess the HSE performance of that organization.

4. Reference requirements and documents

Considering the following legal requirements, the establishment of a waste management system, classification and coding of wastes and providing relevant performance reports is obligatory in all companies/organizations.

- The Waste Management Law, approved by the Islamic Consultative Assembly on 9/5/2004

- The Executive Code of the Waste Management Law, approved by the Iranian Board of Ministers on 27/7/2005

- The Tehran Protocol on the Control of Transboundary Marine Transmission of Hazardous Wastes and Other Wastes (Tehran, 1997)

- The Basel Convention on the Control of Transboundary Movements of Harmful Wastes and Their Disposal (Basel 1980)

- The London Convention on the Prevention of Marine Pollution by Dumping of Wastes and Other Matter (London 1972)

- The Rotterdam Convention on the Prior Informed Consent Procedure for Certain Hazardous Chemicals and Pesticides in International Trade (Rotterdam 2003)

- The Stockholm Convention on Persistent Organic Pollutants

- The Executive Code of the Transportation of Hazardous Materials, approved by the Iranian Board of Ministers on 13/3/2002

- The requirements and procedures of executive management of medical wastes and relevant wastes

- The requirements and procedures of executive management of electrical and electronic wastes

- The requirements and procedures of executive management of agricultural wastes

- Guideline for the establishment of waste management and meeting the relevant requirements (No. MOP-HSED-Gl-302 (0))

5. Definitions

5.1 Waste classes

Figure 1 illustrates the general classification of hazardous and non-hazardous wastes, regardless of their production sources or relevant activities. Due to the importance of hazardous wastes and the need for their proper management, their definition is applied as the main criterion in the classification process. It is worth noting that this classification does not negate other common classifications, such as Waste Management Law and common categories in industrial units (process and non-process wastes), etc.; however, it completes such classifications. Brief definitions of various expressions used in this figure are provided in the following paragraphs (for more information, see the waste classification checklist in Appendix 1).

5.1.1 Hazardous wastes

A waste is considered hazardous when:

1. It is listed as such by the U.S. Environmental Protection Agency (EPA) and designated as hazardous under the Resource Conservation Recovery Act (RCRA).

Or

2. Exhibits one or more of hazardous characteristics (ignitibility, reactivity, corrosivity, toxicity) (also specified by the EPA). Such wastes are called characteristically hazardous wastes.

Those wastes that do not comply with the above conditions are called non-hazardous wastes.

Figure 1: General classification of hazardous and nonhazardous wastes

A - Listed hazardous wastes

EPA has introduced about 422 wastes as hazardous and has listed them in four classes (for more information, see Appendix 1, section 3.1). EPA updates this list frequently and classification and coding professionals and authorities shall update their knowledge, continuously. This list is available on the EPA website:

http://www.epa.gov/epawaste/hazard/wastetypes/listed.htm

B- Characteristically hazardous wastes: These wastes display one or more of the following four hazardous characteristics:

- Ignitability (easily flammable wastes, such as solvents);

- Reactivity (capable of rapid chemical reaction, such as peroxides);

- Corrosivity (highly acidic or alkaline, able to dissolve metals or burn the skin, such as hydrochloric acid or sodium hydroxide); and

- Toxicity (wastes that release toxic constituents into the environment, such as lead-based paints).

For more information on hazardous characteristics, see Appendix 1, section 5.1.

5.1.2 Non-hazardous wastes

There are wastes that are not listed as hazardous and do not have hazardous characteristics.

In these wastes, class 1 of nonhazardous wastes can include certain levels of constituents and specified properties that, at higher levels, might otherwise render the waste hazardous. Due to their constituents and properties, if not properly managed, these types of wastes can threaten human health and the environment. An example is water contaminated with ethylene glycol. Examples of class 2 wastes include waste-activated sludge from biological wastewater treatment. Class 3 includes materials such as demolition debris. Examples include bricks that are insoluble, do not react with other materials and do not decompose. A Class 2 or 3 designation does not mean that the waste is incapable of causing harm in every management/mismanagement situation (for more details, see Appendix 1, sections 2 and 3).

5.2 Waste code

In the waste management system, waste code is a code assigned to wastes generated by the organization to facilitate the identification and reporting processes. In the Ministry of Petroleum of Iran, the waste code components include:

- Generator’s name

- Stream number

- Form code

- Classification code

- Hazard code

The common components of waste code in the Ministry of Petroleum of Iran are shown in Figure 2. These components are described in the following paragraphs.

A. The generator’s name

In the first part of the formula, the name of the waste generator is inserted in the form of a three-four letters acronym. The acronyms of all industrial and non-industrial complexes and units are presented in tables (2) to (5) and the HSE Department of the Ministry of Petroleum of Iran will determine and insert the acronym of future companies and industrial units, too.

In cases where the generator’s code is less than four letters; a dash (-) mark fills out the empty cells. For example, the Fajr Petrochemical Company that is known as FPC is displayed as (-FPC).

B. Stream number

This number is inserted as a consecutive number. Although called a “sequence number”; this part of the code may contain a mixture of numbers and letters and sometimes it may consist of letters alone. Various types of sequence numbers used in the waste code include:

- A 3-digit consecutive number that begins from 121 for each generator. Once assigned to a particular waste stream, a sequence number cannot be reassigned to another waste stream. The generators are recommended to assign consecutive numbers. Only one number is assigned to each organization for each of the urban, agricultural and medical waste groups.

- A 3-digit alphabetic number assigned by the organization’s HSE Department to wastes generated by generators outside the organization. These letters include:

- “SPL”: It can be assigned only by the Emergency Response Team for spill wastes regulated under the Emergency Response Plan.

- “OUTS”: It is assigned to wastes generated outside the organization and may be transported for disposal, incineration, processing, and recycling or temporary storage.

- “TSD” (treatment, storage and disposal facilities), to be used by facilities that receive and consolidate a waste stream with other like waste streams (thus not changing the form or composition of the waste); or (2) store a received waste without treating or changing its form or composition. This number does not apply to wastes that are treated or altered in some other way. The “TSDF” designation is used only by facilities that store and/or accumulate waste from more than one site for subsequent shipment to a treatment or disposal facility.

C. Form code

In the third part, a three-digit form code is used to show the qualitative characteristics and physical state of the waste under atmospheric conditions. Various form codes are presented in the Appendix 2 of the present guideline.

D. Classification code

In the next part, wastes are classified in accordance with the Waste Management Law. The classification code is a code used to cover all wastes, including industrial, urban, medical, etc. in the form of a coding system and in accordance with the Waste Management Law of Iran in order to facilitate the reporting system. In this section, wastes with different production sources are classified in the table 1.

Table 1: Source code of various wastes

| No. | Waste type | Waste code |
| --- | --- | --- |
| 1 | Ordinary (urban) wastes | U |
| 2 | Medical wastes | M |
| 3 | Agricultural wastes | A |
| 4 | Process and non-process industrial wastes | I |

The final code indicates the hazardness of various types of wastes. Descriptions on the level of hazardness of various wastes are presented in the definitions section. Appendix 1 can be used to determine the hazard code of various wastes. As shown in Figure 1, this part of the waste code is shown with “H”, “1” or “2” or “3”.

| No. | Company | Code |
| --- | --- | --- |
| 1 | Abadan Oil refining Company | ABRC |
| 2 | Imam Khomeini Shazand Oil Refining Company | IKRC |
| 3 | Esfahan Oil Refining Company | ESRC |
| 4 | Bandar Abbas Oil Refining Company | BNRC |
| 5 | Tabriz Oil Refining Company | TBRC |
| 6 | Tehran Oil Refining Company | THRC |
| 7 | Shiraz Oil Refining Company | SHRC |
| 8 | Kermanshah Oil Refining Company | KRRC |
| 9 | Lavan Oil Refining Company | LVRC |
| 10 | National Iranian Oil Products Distribution Company | PDC (1-37) |
| 11 | Iranian Oil Pipeline and Telecommunications Company | PTC (1-12) |
| 12 | National Iranian Oil Engineering and Construction Company | ECC |

Table 2: The codes allocated to various units by the National Iranian Oil Refining and Distribution Company (NIORDC)

Table 3: The codes allocated to various units by the National Petrochemical Company of Iran

| No. | Company | Code |
| --- | --- | --- |
| 1 | Amir kabir petrochemical company | AKPC |
| 2 | Bou Ali Sina Petrochemical Company | BAPC |
| 3 | Shahid Tondgooyan Petrochemical Company | STPC |
| 4 | Khuzestan Petrochemical Company | KZPC |
| 5 | Fajr Petrochemical Company | FPC |
| 6 | Arvand Petrochemical Company | APC |
| 7 | Marun Petrochemical Complex | MPC |
| 8 | Karoon Petrochemical Company | KRNPC |
| 9 | Fanavaran Petrochemical Company | FNPC |
| 10 | Rejal Petrochemical Company | REGAL |
| 11 | Ghadir Petrochemical Company | GPC |
| 12 | Shahid Rasouli Petrochemical Company | SRPC |
| 13 | Navid Zar Chimi | NZC |
| 14 | Shimibaft Petrochemical Company | SHBT |
| 15 | Laleh Petrochemical Company | LPC |
| 16 | Arya Sasol Polymer Company | ASPC |
| 17 | Pardis Petrochemical Company |  |
| 18 | Borzouyeh Petrochemical Company | BPC |
| 19 | Pars Petrochemical Company | PPC |
| 20 | Jam Petrochemical Company | JPC |
| 21 | Zagros Petrochemical Company | ZPC |
| 22 | Mobin Petrochemical Company | MOPC |
| 23 | Terminals and tanks Petrochemical Company | PPO |
| 24 | Pazargad Non Industrial Operation Services Company | PZSC |
| 25 | Uromia Petrochemical Company | UPC |
| 26 | Bandar ImamPetrochemical Complex | BIPC |
| 27 | Bistoon Petrochemical Company | BSPC |
| 28 | Tabriz Petrochemical Company | TPC |
| 29 | Khorasan Petrochemical Company | KHPC |
| 30 | Razi Petrochemical Company | RPC |
| 31 | Shiraz Petrochemical Company | SPC |

Table 4: The codes allocated to various units by the National Iranian Gas Company

| No. | Company | Code |
| --- | --- | --- |
| 1 | Iranian Gas Transmission Company | NIGTC |
| 2 | Iranian Gas Commercial Company | IGCC |
| 3 | Iranian Gas Engineering and Development Company | NIGCENG |
| 4 | Natural Gas Storage Company | NGSC |
| 5 | Ilam Gas Treating Company | IGTC |
| 6 | Bid Boland Gas Refining Company | BDBL |
| 7 | Parsian Gas Refining Company | PRSN |
| 8 | Sarkhoon & Qeshm Gas Refinery Company | SQGC |
| 9 | Khangiran Gas refinery Company | KHNG |
| 10 | Fajr-e-Jam Gas refinery Company | FJRJ |
| 11 | South Pars Gas Company | SPGC |
| 12 | Gas Company of East Azerbaijan Province | EAZAR |
| 13 | Gas Company of West Azerbaijan Province | WAZAR |
| 14 | Gas Company of Ardabil Province | AR |
| 15 | Gas Company of Isfahan Province | ISFN |
| 16 | Gas Company of Ilam Province | ILAM |
| 17 | Gas Company of Bushehr Province | BSHR |
| 18 | Gas Company of Tehran Province | TPGC |
| 19 | Gas Company of Chaharmahal and Bakhtiari Province | CHBGAS |
| 20 | Gas Company of South Khorasn Province | SKGC |
| 21 | Gas Company of Razavi Khorasn Province | KHRZ |
| 22 | Gas Company of North Khorasn Province | NKGC |
| 23 | Gas Company of Khuzestan Province | KHGC |
| 24 | Gas Company of Zanjan Province | ZNJN |
| 25 | Gas Company of Semnan Province | SMNN |
| 26 | Sistan and Baluchestan Province Gas Company | SBGC |
| 27 | Gas Company of Fars Province | Fars |
| 28 | Gas Company of Qazvin Province | QZVN |
| 29 | Gas Company of Qom Province | QPGC |
| 30 | Gas Company of Kurdistan Province | KD |
| 31 | Gas Company of Kerman Province | KRMN |
| 32 | Gas Company of Kermanshah Province | KSH |
| 33 | Gas Company of Kohgiluyeh and Boyer-Ahmad Province | KB |
| 34 | Gas Company of Golestan Province | GLSN |
| 35 | Gas Company of Ilam Province | GL |
| 36 | Gas Company of Lorestan Province | LRSN |
| 37 | Gas Company of Mazandaran Province | MZDN |
| 38 | Gas Company of Markazi Province | MPGC |
| 39 | Gas Company of Yazd Province | Yazd |
| 40 | Gas Company of Hormozgan Province | HPGC |
| 41 | Gas Company of Hamedan Province | HM |

Table 5: The codes allocated to various units by the National Iranian Oil Company

| No. | Company | Code |
| --- | --- | --- |
| 1 | Petroleum Engineering and Development Company | PEDEC |
| 2 | Pars Oil & Gas Company | POGC |
| 3 | Iran’s North Drilling Company | NDCO |
| 4 | Iranian Central Oil Fields Company | ICOFC |
| 5 | Iranian Offshore Oil Company | IOOC |
| 6 | National Iranian South Oil Company | NISOC |
| 7 | Arvandan Oil & Gas Company | AOGC |
| 8 | National Iranian Drilling Company | NIDC |
| 9 | Iranian Oil Terminals Company | IOTC |
| 10 | National Iranian Oil Company, Exploitation Management | EXP |
| 11 | Iran Fuel Conservation Company | IFCO |
| 12 | Pars Special Economic Energy Zone Company | PSEEZ |
| 13 | Manufacturing Support and Procurement Kala Naft Company of Tehran | KNM |
| 14 | Petroiran Development Company | IPDC |
| 15 | Petropars Company | PPC |
| 16 | Karoun Oil and Gas Exploitation Company | KOGPC |
| 17 | Marun Oil and Gas Production Company | MOGPC |
| 18 | Gachsaran Oil and Gas Production Company | GOGPC |
| 19 | Masjed Soleyman Oil and Gas Production Company | MSOGPC |
| 20 | Aghajari Oil and Gas Production Company | AOGPC |
| 21 | East Oil and Gas Production Company | EOGPC |
| 22 | West Oil and Gas Production Company | WOGPC |
| 23 | South Zagros Oil and Gas Production Company | SZOGPC |
| 24 | Iran Drilling Services Company | IDSC |
| 25 | Transportation of Oil and Support Services Company | TOSSC |
| 26 | South Turbine Engineering Services and Industrial Equipment Company | SESTIEC |
| 27 | Oil General Welfare Service Company | OGWSC |
| 28 | Mahmoodabad Sport Training Complex |  |
| 29 | Khazar Exploration and Production Company | KEPCO |
| 30 | National Iranian Gas Export Company | NIGEC |

6. Procedures

6.1 For each waste, a classification and coding method shall be determined and documented in accordance with the latest version of the HSE management system’s guideline.

6.2 Each organization shall identify its wastes and classify and codify them according to the requirements of the present guideline.

6.3 The classification process is performed in accordance with the regulations of EPA and RCRA and based on a checklist presented in the Appendix 1 of this guideline.

6.4. The coding process shall be performed in accordance with the present guideline.

6.5 The codes used for labeling all wastes should be determined on the basis of the coding system presented in this guideline.

6.6 The waste management process reports shall be provided using this guideline.

6-7 All relevant statistics and information shall be recorded by the HSE Department. The duration of information storage can be determined by national and local laws or in accordance with the ISO 14000.

6.8 The HSE management of the main companies shall take the necessary steps to implement the present guideline in all subsidiaries and shall supervise its proper implementation.

Appendix 1 - Waste Classification Checklist

1. Hazardous waste determination

In this section, it is determined whether a waste is hazardous because (a) it is listed as hazardous by EPA or (b) it displays characteristics that EPA says make it hazardous.

The first step in classifying a waste is called “hazardous waste determination” The definition of hazardous waste, based upon the Resource Conservation and Recovery Act (RCRA), is found in Title 40 of the Code of Federal Regulations (CFR), Part 261.

This guideline is provided based on the definition of hazardous waste (presented in the definition section). If that definition changes, the HSE Department of the Ministry of Petroleum of Iran will be responsible for providing new definitions and communicating it to the main companies. However, this does not negate the duty of each generator in identification and classification of wastes in accordance with the provisions of the Waste Management Law.

IF the answer to any of the questions in section is “Yes,”

THEN the waste is hazardous.

1.1 Possible Exclusions from Hazardous Classification

Under certain conditions, some types of wastes are excluded from being considered hazardous (subsection E of RCRA). Generators shall review these exclusions before working through section 1 of this checklist.

1.2 Listed Hazardous Waste Determination

The listed wastes are defined in 40CFR, subsection 26.21 of the RCRA. EPA has introduced about 400 wastes as hazardous and has listed them in four classes.

1.3 Needed information to make the determination

These wastes are often referred to as follows and descriptions of listed waste are found in CFR, Subpart D, sections 261.31–33:

▪ “F” listed waste (waste from nonspecific sources, section 261.31);

▪ “K” listed waste (wastes from specific sources, section 261.32);

▪ “P” listed waste (unused acutely hazardous off-specification materials as well as container residues and spill residues of these materials, section 261.33);

▪ “U” listed waste (unused toxic hazardous off-specification materials as well as container residues and spill residues of these materials, section 261.33).

Question: Is the waste a listed hazardous waste or is it mixed with or derived from one?

Yes □ No □

1.4 Characteristic Hazardous Waste Determination

Wastes may be hazardous if they display any of the following four characteristics:

1) Ignitability, 2) Corrosiveness, 3) Reactivity 4) Toxicity.

1.4.1Needed information to make the determination

A. Ignitability:

The following wastes are hazardous because they may ignite:

- Liquid wastes (except for those aqueous waste containing less than 24% alcohol by volume) that have a flash point less than 60OC (140OF). (The test method is the Pensky- Martens closed cup tester, using the test method specified in ASTM Standard D-93-79 or D-93-80, or a Seta flash closed cup tester, using the test method specified in ASTM Standard D-3278-78.)

- No-liquid wastes that, under standard temperature and pressure, are capable of causing fire through friction, absorption of moisture, or spontaneous chemical changes and, when ignited, burn so vigorously and persistently that they create a hazard.

- Wastes that are ignitable, according to the definition of compressed gases (see 49 CFR, section 173.300).

- Wastes that are oxidizer, according to the definition of oxidizers (see 49 CFR, section 173.151).

Question: Is the waste ignitable according to the 40 CFR, section 261.21? Yes □ No □

The list of some flammable solids is presented in the Appendix A of this checklist.

B. Corrosiveness

Corrosive wastes include:

- Aqueous wastes with a pH of 2 units or below or of 12.5 units or above

- Liquid wastes that corrode steel at a rate greater than 6.35 mm per year

Question: Is the waste corrosive according to the 40 CFR, section 261.22? Yes □ No □

C. Reactivity

A waste is considered reactive if it meets any of the following conditions:

- It is capable of detonation or explosive decomposition or reaction:

  - At standard temperature and pressure,

  - If subjected to a strong ignition source,

  - If heated under confinement.

- When mixed with water, it is:

  - Potentially explosive,

  - Reacts violently, or

  - Generates toxic gases or vapors.

- When a cyanide or sulfide-bearing waste is exposed to pH conditions between 2 and 12.5, it can generate enough toxic gases, vapors or fumes to present a danger to human health or the environment. Generally, if a waste generates 250 ppm or more of reactive cyanides or 500 ppm or more of reactive sulfides, it is considered a reactive waste. (It should be noted that these levels of reactive compounds are just guidance. Each waste must be tested for reactivity).

- A waste that is normally unstable and undergoes violent changes without detonating.

- It is a forbidden explosive (as defined in 49 CFR 173.51, or a class A explosive as defined in 49 CFR 173.53).

- Class B explosives (see 49 CFR, section 173.88).

Question: Is the waste reactive according to the 40 CFR, section 261.23? Yes □ No □

D. Toxicity

A waste is toxic if the toxicity characteristic leaching procedure (TCLP) shows that a representative sample from the waste contains one or more constituents at or above the levels listed in Table 1. The TCLP is described in EPA Method 1311.

Question: Is the waste toxic according to the 40 CFR, section 261.24? Yes □ No □

2. Nonhazardous Industrial Waste Classes 1 and 2

The following information applies only to nonhazardous industrial wastes.

IF the answer to any of the un-numbered questions in this section is “YES”

THEN the nonhazardous industrial waste is a class 1 waste.

IF all the answers to the un-numbered questions in this section are “No,”

THEN the industrial waste is a class 2 waste.

Generator’s classification

Question: Has the generator chosen to classify the nonhazardous waste as a class 1? Yes □ No □

Container waste

IF the waste is a container, greater than 20 liter in holding capacity, which has held

- A hazardous substance (as defined in 40 CFR Part 302 and listed in Appendix B of this guidance document),

- A hazardous waste (including acutely hazardous wastes),

- A class 1 waste, and/or

- A material that would be classified as a hazardous or class 1 waste if disposed of,

THEN answer questions 1 and 2. (Note: Containers that have held acutely hazardous wastes must be triple-rinsed before they can be classified as empty).

IF the waste does not match with these conditions;

THEN answer the next un-numbered question.

1. Is the container fully emptied? Yes □ No □

2. Has the container been rendered unusable? Yes □ No □

Question: Are any of the answers to questions (1) or (2) above “NO”? Yes □ No □

Regulated Asbestos-Containing Material (RACM)

Question: Does the waste contain asbestos material identified as RACM, as defined in 40 CFR, section 61? Yes □ No □

Polychlorinated Biphenyls (PCBs)

Question: Is the waste contaminated by a material that originally contained 50 or more ppm PCBs? Yes □ No □

Petroleum Substance Waste

1. Is the waste specifically identified as a petroleum substance (see Definitions) or contaminated with a material identified as a petroleum substance waste? Yes □ No □

2. Does the waste contain more than 1,500 ppm petroleum hydrocarbons? Yes □ No □

Question: Are the answers to both of the questions above “Yes”? (If one or both of the answers are “No,” enter “No” for this question). Yes □ No □

New Chemical Substance

Question: Is the waste from the production of a “new chemical substance,” as defined by the federal Toxic Substances Control Act, 15 U.S.C.A. Section 2602(9)? Yes □ No □

Out-of-the region Origin

Question: Is the waste generated outside the region? Yes □ No □

Constituent levels and specified properties for nonhazardous industrial class 1 wastes

Question: If the waste is a liquid, does it have a flash point of less than 65.6OC (150OF)?

Yes □ No □

Question: Is the waste a solid or semi-solid that under conditions normally encountered in storage, transportation, and disposal;

- Is liable to cause fires through friction or through retained heat from manufacturing or processing; or

- Can be ignited rapidly and when ignited burns so vigorously and persistently as to create a serious hazard? Yes □ No □

Question: Is the waste a semi-solid or solid that, when mixed with an equivalent weight of ASTM Type II laboratory distilled or deionized water, produces a solution with a pH of 2 or less or 12.5 or more? Yes □ No □

(Exception: There is an exception for solidified, stabilized, encapsulated, or otherwise chemically bound wastes)

Question: Does the waste leach toxic constituents at or above the levels mentioned in Appendix C, when submitted to the TCLP procedure? Yes □ No □

(Where matrix interferences of the waste cause the PQL of the specific analysis to be greater than the maximum concentration listed in Appendix C, then the achievable PQL becomes the maximum concentration).

Lack of class 2 or 3 information

Question: Is there adequate information to classify the waste in class 2 or 3? Yes □ No □

3. Nonhazardous class 3 wastes

This section applies only to nonhazardous wastes that do not meet the definition of class 1 or 2 wastes.

3.1 Initial determination of class 3 wastes

IF the answer to any of the following questions is “Yes,”

THEN the nonhazardous waste cannot be considered a class 3 waste.

Containers

Question: Is the waste an empty container? Yes □ No □

Medical wastes

Question: Is the waste a medical waste, according to available rules and regulations? Yes □ No □

Distilled water leaching test

Question: When subjected to a 7-day distilled water leaching test, does the waste leach constituents at or above the maximum contaminant levels listed in Appendix D? Yes □ No □

Toxicity characteristic leaching procedure

Question: When submitted to the toxicity characteristic leaching procedure, does the waste leach class 1 toxic constituents listed Appendix E? Yes □ No □

(The list of Class 1 toxic constituents is represented in Appendix E of this guidance document)

Exclusion: Excluded from this list of Class 1 toxic constituents are those addressed in the Appendix D.

Petroleum hydrocarbons

Question: Does the waste contain detectable levels of petroleum hydrocarbons (Using TNRCC Method 1005)? Yes □ No □

Polychlorinated Biphenyls (PCBs)

Question: Does the waste contain detectable levels of PCBs? Yes □ No □

Decomposition

Question: Is the waste readily decomposable? Yes □ No □

3.2 Final determination of class 3 wastes

Inertness

Question: Is the waste inert? (Inertness refers to chemical inactivity of an element, a compound, or a waste.) Yes □ No □

Insolubility

Question: Is the waste essentially insoluble? Yes □ No □

(Note: wastes that contain liquids are considered soluble)

Appendix A- Ignitable solids

Constituents listed from Department of Transportation Regulations, 49 CFR Part 173 Subpart E, October 1, 1993. Note: The presence of a constituent on this table in a nonhazardous waste does not identify that waste as a class 1 ignitable waste. The constituents on this table are examples of materials which could be considered class 1 ignitable wastes. The physical characteristics of the waste will be the determining factor as to whether or not a waste is ignitable.

Appendix A- Ignitable solids

Appendix B- Hazardous Substances

Application: Evaluation of Empty Container Class 2

The following is a list of materials identified as hazardous substances (40 CFR Table 302.4). This list must be obtained following the introduction of new lists by relevant authorities. Chemical Abstract Service (CAS) Registry Numbers of the materials are also provided.

Appendix B- Hazardous Substances

Appendix B- Hazardous Substances

Appendix B- Hazardous Substances

Appendix B- Hazardous Substances

Appendix B- Hazardous Substances

Appendix B- Hazardous Substances

Appendix C- Class 1 Toxic Maximum Constituents’ Leachable Concentrations

Application: Evaluation of class 1, 2, and 3 wastes

Values are based on information contained in the U.S. Federal Registers Vol. 56 / June 7, 1991; and Integrated Risk Information Systems, Environmental Protection Agency and 40 CFR 264 Appendix 9.

Appendix D- 7-Day Distilled Water Leachate Test’s Maximum Contaminant Levels

Application: Evaluation of class 3 wastes

Values obtained from 40 Code of Federal Regulations, section 141, subparts B and G, (maximum contaminant levels) and 40 Code of Federal Regulations, section 143, (total dissolved solids)

For a Class 3 waste classification, these constituents must also be evaluated using the test methods described in 40 CFR, section 261, Appendix 2.

Appendix E- Class 1 Toxic Constituents

(Other than those presented in Appendix C and their Estimated Quantitation Limits [EQLs])

Application: Evaluation of class 3 wastes

This table is to be utilized by the generatorin evaluating detection limits for the identified constituents. The EQLs in this table are defined as the lowest detectable levels that can be reliably achieved using the TCLP method. Applicable EPA method numbers are presented. Please note that more than one test method may be available for a particular constituent. (Synonyms are provided in brackets).

Appendix 2- Form codes

1. Introduction

Acquiring a proper knowledge of waste compounds is essential in order to make appropriate managerial decisions for the optimal waste recycling, sale, burning and burial. In this regard, to determine a strategy and policy for controlling and managing wastes by the subsidiaries of the Ministry of Petroleum, all wastes generated in this industry are classified into ten general classes (Figure 1), which include: laboratory packages, non-organic liquids, organic liquids, inorganic solids, organic solids, inorganic sludge, organic sludge, inorganic gases, organic gases and non-industrial wastes. Each group is classified into some subgroups and a code is allocated to each subgroup. It should be noted that accurate determination of the form code is very important and the generator is recommended to initially identify the main group of the stream, review all the definitions associated with those groups and select the best code or codes appropriate for the waste. The generator must then select the form code from this list.

The definition of all classes, subclasses and form codes are presented below.

2. Definitions associated with the form codes

Acidic: A material having a pH less than 7.0.

Alkaline: A material having a pH greater than 7.0.

Aqueous: A water solution containing organic and/or inorganic constituents dissolved in solution.

Caustic: A material which is corrosive or irritating to tissues and has a pH greater than 7.

Figure 1: Classification of wastes based on their form codes

Inorganic: Chemicals that are not organic (e.g. water, carbon dioxide, carbon disulfide, iron, zinc, steel). Generally, if a waste is composed of more than 50% inorganic materials, it is considered an inorganic waste.

Organic: Chemicals composed primarily of carbon and hydrogen and their derivatives. (e.g. methylene chloride, benzene, petroleum products). In general, if a waste is composed of 50% or more organic materials, it is considered an organic waste.

Nonindustrial waste: Includes the following 2 categories:

The first group: Class 2 and 3 wastes, which are produced as a result of plant production, manufacturing, laboratory, general office, restaurant, etc. and include: paper, cardboard, linings, wrappings, paper or wood packaging materials, food wastes, cafeteria wastes, glass, aluminum foil, aluminum cans, aluminum scrap, stainless steel, steel, iron scrap, plastics, styrofoam, rope, twine, uncontaminated rubber, uncontaminated wooden materials, equipment belts, wirings, uncontaminated cloth, metal bindings, empty containers with a holding capacity of less than 20 liters, contaminated floor sweepings, agricultural wastes and non-infectious medical wastes. Please note that hazardous waste and class 1 waste cannot be designated to this group. Nonindustrial waste shall not include oils, lubricants of any type, oil filters, contaminated soils, sludge or wastewaters. Examples of “nonindustrial waste” include food cans, lunch sacks, food scraps, envelopes, plastic binders, empty boxes, pallets, styrofoam shipping boxes, chemical container liners, shrink wrap, and broken glassware, etc.

As another example, used typing paper from the secretarial area could be considered “nonindustrial waste”; because it resulted from general office operations. (Please note that typing paper would normally be considered a class 2 waste unless it is contaminated with something to cause it to be considered a hazardous or class 1 waste. For example, if typing paper were used to clean up a spill of a F003 waste, it would be considered a hazardous waste).

As another example, an off-specification production chemical could not be considered “nonindustrial waste” because it does not meet the definition of a “nonindustrial waste”.

The second group: This group consists of wastes similar to the first group; however, they have properties that place them in the hazardous waste group or the H1 group. Wastes such as urban and medical wastes are not considered as industrial wastes; however, some of these wastes may have a hazardous nature; therefore, non-industrial wastes are divided into two groups.

The first group of these wastes involves codes 921 to 949, with the hazard code of 2 or 3. The second group of wastes involves codes 951 to 999, with the hazard code of 1. For example, an urban waste without hazardous compounds has a code of 921 and hazardous wastes isolated from urban wastes have a code of 951. In addition, the code of non-infectious and non-hazardous wastes generated by health centers is 922 and the code of infectious and hazardous wastes generated by health centers is 952.

Reactive: A material is reactive if it is capable of detonation or explosive decomposition:

1. At standard temperature and pressure, or

2. If subjected to a strong ignition source, or

3. Heated under confinement.

Solvent: A liquid used to dissolve another material.

A. Lab packs

Lab packs of mixed wastes, chemicals, lab wastes, etc.

| Form code | Waste |
| --- | --- |
| 001 | Lab packs of old chemicals |
| 002 | Lab packs of debris |
| 003 | Mixed lab packs |
| 004 | Lab packs containing acute hazardous wastes |
| 005 | Lab packs containing granular powder and polymer |
| 006 | Empty glass and plastic containers |
| 009 | Other lab packs |

B. Liquids

Inorganic Liquids: A waste that is primarily inorganic and highly fluid (e.g., aqueous), with low suspended inorganic solids and low organic content.

| Form code | Waste |
| --- | --- |
| 103 | Still bottoms without organic compounds |
| 104 | Aqueous waste with low solvents |
| 105 | Aqueous waste with low other toxic organics |
| 106 | Spent acid with metals |
| 107 | Spent acid without metals |
| 108 | Acidic aqueous waste |
| 109 | Caustic solution with metals but no cyanides |
| 110 | Caustic solution with metals and cyanides |
| 111 | Caustic solution with cyanides but no metals |
| 112 | Spent caustic |
| 114 | Caustic aqueous waste |
| 115 | Aqueous waste with reactive sulfides |
| 116 | Aqueous waste with other reactives (e.g., explosives) |
| 117 | Other aqueous waste with low dissolved solids |
| 118 | Scrubber water |
| 119 | Leachate |
| 120 | Waste liquid mercury |
| 121 | Non-hazardous non-organic photographic waste |
| 122 | Other inorganic liquids |

C. Organic liquids:

A waste that is primarily organic and is highly fluid, with low inorganic solids content and low-to-moderate water content

| Form code | Waste |
| --- | --- |
| 201 | Concentrated solvent-water solution |
| 202 | Halogenated (e.g., chlorinated) solvent |
| 203 | Non-halogenated solvent |
| 204 | Halogenated/non-halogenated solvent mixture |
| 205 | Oil-water emulsion or mixture |
| 206 | Waste oil |
| 207 | Concentrated aqueous solution of other organics |
| 208 | Concentrated phenolics |
| 209 | Organic paint, ink, lacquer, or vanish |
| 210 | Adhesives or epoxies |
| 211 | Reactive or polymerizable organic liquids |
| 212 | Ethylene glycol wastes |
| 213 | Ethanolamine or other Amine wastes |
| 214 | Acidic wastes |
| 215 | Catalytic wastes |
| 216 | Paint thinner or petroleum distillates |
| 217 | Still bottoms with organic compounds |
| 218 | Used polymer wastes |
| 219 | Used aniline wastes |
| 230 | Other organic liquids |
| 297 | Nonhazardous photographic wastes |
| 298 | Nonhazardous liquids containing greater than or equal to (>) 50 and less than (<) 500 ppm PCBs |
| 299 | Nonhazardous liquids containing greater than or equal to (>) 500 ppm PCBs |

D. Inorganic solids

(These codes do not apply to pumpable wastes)

Inorganic solids: A waste that is primarily inorganic and solid, with low organic content and low-to- moderate water content and not pumpable.

| Form code | Waste |
| --- | --- |
| 301 | Ash, slag, or other residue from incineration of wastes |
| 302 | insulation wastes, such as glass wool and rock wool |
| 303 | Graphite and metal tubes and gaskets |
| 304 | Gasket made of asbestos or other asbestos-containing wastes |
| 305 | Non-organic-based catalytic wastes |
| 306 | Waste from machining, grinding and shearing contaminated with oil and soap water |
| 307 | Waste from machining, grinding and shearing that are not contaminated with oil and soap water |
| 308 | Waste including: fiber-optic wastes, wire and cable pieces, fuses and switches, inflammatory filament lamps, fans, compressors and coolers and other electrical and electronic devices where the main part is metal |
| 310 | Molecular sieve, activated carbon and clay, zeolite, active alumina and mixtures of each of them |
| 311 | Coke derived from decoking of soot, coal, etc. |
| 312 | Soil and sand contaminated with organic matter (such as oil, etc.) |
| 313 | Soil and sand contaminated with inorganic matter |
| 314 | Glass bottles pieces contaminated with various chemicals |
| 315 | Sediments of converters, tubes and towers |
| 316 | Various packing made of metal, ceramics and other inorganic materials |
| 317 | Different parts, compartment and contents of batteries |
| 318 | Batteries or battery parts, casings, cores |
| 319 | Metal parts and pieces, shearing wastes, tubes, defective screw and nut, defective gauges, electrode, hot scars, etc. |
| 320 | Bottom stills without organic compounds |
| 321 | Counter and regulator parts |
| 322 | Fluorescent and LED lamps |
| 323 | Nonhazardous dewatered water treatment sludge |
| 325 | Automobile parts, such as break pad, clutch, etc. |
| 326 | Chemicals used for water or wastewater disinfection |
| 327 | Wastes of firefighting powder and foam |
| 328 | Various minerals used to make drilling mud |
| 329 | Construction debris |
| 332 | mineral dehumidifiers |
| 334 | Fireproof bricks and minerals |
| 335 | Copper slag |
| 336 | Sulfuric wastes |
| 380 | Various resins made of polymer materials |
| 381 | Nonhazardous dewatered water treatment sludge |
| 382 | Nonhazardous dewatered wastewater treatment sludge |
| 383 | Nonhazardous dewatered air pollution control device sludge |
| 384 | “Dry” lime or metal hydroxide solids chemically “fixed” |
| 385 | “Dry” lime or metal hydroxide solids not “fixed” |
| 386 | Nonhazardous sandblasting waste |
| 387 | Nonhazardous concrete/cement/construction debris |
| 388 | Metal-cyanide salts/chemicals |
| 389 | Reactive cyanide salts/chemicals |
| 390 | Reactive sulfide salts/chemicals |
| 391 | Other reactive salts/chemicals |
| 392 | Other metal salts/chemicals |
| 393 | Other waste inorganic solids |
| 394 | Nonhazardous soils containing greater than or equal to (>) 50 ppm and less than (<) 500 ppm PCBs |
| 395 | Nonhazardous soils containing greater than or equal to (>) 500 ppm PCBs |
| 396 | Nonhazardous electrical equipment/devices containing greater than or equal to (>) 50 ppm and less than (<) 500 ppm PCBs |
| 397 | Nonhazardous electrical equipment/devices containing greater than or equal to (>) 500 ppm PCBs |
| 398 | Nonhazardous solids containing greater than or equal to (>) 50 ppm and less than (<) 500 ppm PCBs |
| 399 | Nonhazardous solids containing greater than or equal to (>) 500 ppm PCBs |

E. Inorganic solids

Organic solids: A waste that is primarily organic and solid, with low-to-moderate inorganic content and water content and not pumpable.

| Form code | Waste |
| --- | --- |
| 401 | PVC lumps, granules and powder produced during the production process. |
| 402 | PET, PTA, CTA and TA lumps, granules and powder produced during the production process. |
| 403 | PP and PE lumps, granules and powder produced during the production process. |
| 404 | Various packing made of non-PVC polymeric materials |
| 405 | Various plastic pallets, geomembrane wastes, polyethylene coatings and pieces and parts and other plastic parts other than PVC |
| 406 | Plastic containers and barrels used to store various chemicals |
| 407 | Trash bags made of polymeric materials, rolls, bags, etc. other than PVC |
| 408 | Gaskets, rims, tires, belts and other polymeric parts other than PVC |
| 409 | Various polymeric resins other than PVC |
| 410 | Various air filters made of cellulose or polymeric materials other than PVC |
| 411 | Phenol and other aromatics lumps, granules and powder produced during the production process. |
| 412 | Various used packing made of PVC materials |
| 413 | Other pieces and parts made of PVC materials |
| 414 | Organic-based catalytic wastes |
| 415 | Oil and grease |
| 416 | Various types of waxes |
| 417 | Various wooden pallets, boards, etc. |
| 418 | Cleaning cloth, paper, clothing and other solid substances contaminated with chemical and petroleum products |
| 419 | Used tubes and tires |
| 420 | Various oil, gas or fuel filters or other materials made of cellulose or polymeric materials |
| 422 | Still bottoms containing organic compounds |
| 423 | Sediments due to dust or dripping of pipes, debris and cleaning septic tanks |
| 424 | Hazardous dewatered treatment sludge |
| 425 | Hazardous dewatered biological treatment sludge |
| 426 | Hazardous dewatered sewage or other untreated biological sludge |
| 427 | Nonhazardous dewatered treatment sludge |
| 428 | Nonhazardous dewatered biological treatment sludge |
| 429 | Nonhazardous dewatered sewage or other untreated biological sludge |
| 430 | Oligomer |
| 431 | Ethylene glycol wastes |
| 432 | Used X-ray films |
| 433 | Electrical and electronic wastes and components, such as computer parts, toner cartridges, telephones, chargers, faxes, etc., that are primarily made of polymeric materials |
| 434 | Fluorescent and LED lamps |
| 435 | Wire and cable covers |
| 436 | Disulfide Oil (DSO) |
| 488 | Halogenated pesticide solid |
| 489 | Non-halogenated pesticide solid |
| 490 | Reactive organic solid |
| 491 | Other halogenated organic solids |
| 492 | Other non-halogenated organic solids |
| 493 | Sand blasting wastes |
| 494 | Solids containing greater than or equal to (>) 50 ppm and less than (<) 500 ppm PCBs 495 |
| 495 | Solids containing greater than or equal to (>) 500 ppm PCBs 496 PCBs |
| 496 | Electrical equipment/devices containing greater than or equal to (>) 50 ppm and less than (<) 500 ppm |
| 497 | Electrical equipment/devices containing greater than or equal to (>) 500 ppm PCBs |
| 498 | Soil containing greater than or equal to (>) 50 ppm and less than (<) 500 ppm PCBs |
| 499 | Soils containing greater than or equal to (>) 500 ppm PCBs |

F. Sludge:

Inorganic Sludge: A waste that is primarily inorganic, with moderate-to-high water content and low organic content, and pumpable.

| Form code | Waste |
| --- | --- |
| 501 | Lime sludge without metals |
| 502 | Lime sludge with metals/metal hydroxide sludge |
| 503 | Wastewater treatment sludge with toxic organics |
| 504 | Other wastewater treatment sludge |
| 505 | Untreated plating sludge without cyanides |
| 506 | Untreated plating sludge with cyanides |
| 507 | Other sludge with cyanides |
| 508 | Sludge with reactive sulfides |
| 509 | Sludge with other reactive substances |
| 510 | Degreasing sludge with metal scale or filings |
| 511 | Air pollution control device sludge (e.g., fly ash, wet scrubber sludge) |
| 512 | Sediment or lagoon dragout contaminated with organics |
| 513 | Sediment or lagoon dragout contaminated with inorganics |
| 514 | Drilling mud |
| 515 | Asbestos slurry or sludge |
| 516 | Chloride or other brine sludge |
| 517 | Other inorganic sludge |
| 518 | Organic or oil-based drilling mud |
| 597 | Catalytic wastes |
| 598 | Nonhazardous sludges containing greater than or equal to (>) 50 ppm and less than (<) 500 ppm PCBs |
| 599 | Nonhazardous sludges containing greater than or equal to (>) 500 ppm PCBs |

G. Organic sludge

A waste that is primarily organic with low-to-moderate inorganic solids content and water content, and pumpable.

| Form code | Waste |
| --- | --- |
| 601 | Still bottoms of halogenated (e.g., chlorinated) solvents or other organic liquids |
| 602 | Still bottoms on non-halogenated solvents or other organic liquids |
| 603 | Oily sludge |
| 604 | Organic paint or ink sludge |
| 605 | Reactive or polymerizable organics |
| 606 | Resins, tars, or tarry sludge |
| 607 | Still bottoms containing organic compounds |
| 608 | Hazardous biological sewage treatment sludge |
| 609 | Other hazardous biological sewage treatment sludge |
| 610 | Nonhazardous biological sewage treatment sludge |
| 611 | Other Nonhazardous biological sewage treatment sludge |
| 612 | Sediments or other dredging material |
| 613 | Other organic sludge |
| 614 | Petroleum contaminated sludge other than still bottoms and oily sludge |
| 697 | Catalytic waste |
| 698 | Nonhazardous sludge containing greater than or equal to (>) 50 ppm and less than (<) 500 ppm PCBs |
| 699 | Nonhazardous sludge containing greater than or equal to (>) 500 ppm PCBs |

H. Gases

Inorganic gas: A waste that is primarily inorganic with a low organic content and is a gas at atmospheric pressure.

| 701 | Empty fire extinguisher or nonhazardous chemical container |
| --- | --- |

I. Organic gases: A waste that is primarily organic with low-to-moderate inorganic content and is a gas at atmospheric pressure

| 801 | Organic gases |
| --- | --- |

J. Nonindustrial waste:

In order to be considered a plant refuse designation, a waste must first meet the relevant definition. For more information on these terms, please refer to the terms listed in the “Definitions” section.

The first group: Nonindustrial wastes with a hazard code of 2:

| Form code | Waste |
| --- | --- |
| 901 | Urban wastes: All wastes generated from daily activities of individuals including kitchen, restaurant, pantry, cafeteria, etc. The hazard code of these wastes is only 2. |
| 902 | Medical wastes: All wastes generated in health centers, such as clinics, emergency rooms etc. The hazard code of these wastes is only 2. |
| 903 | Agricultural wastes: All wastes generated in agricultural and gardening activities, such as gardening wastes, lawns, waste resulting from pruning trees and shrubs, etc. The hazard code of these wastes is only 2. |
| 904 | Unique trash: Any 2 or 3 waste generated in office units, laboratories, production facilities or similar sites. These wastes include paper, paper and cardboard, all types of linings and wrappings, packaging equipment, paper and wood packaging, glass, aluminum foil, aluminum cans, aluminum scrap, iron scrap, plastics, Styrofoam, rope, twine, uncontaminated rubber, uncontaminated wooden materials, equipment belts, wirings, uncontaminated cloth, metal bindings, empty containers with a holding capacity of less than five gallons, uncontaminated floor sweepings, etc. |

The second group: Nonindustrial wastes with a hazard code of 1 (H):

| Form code | Waste |
| --- | --- |
| 951 | Urban wastes: All wastes generated from daily activities of individuals that are consistent with the definition of hazardous wastes (with a hazard code of H) or waste with a hazard code of 1. Examples of these wastes include: nickel, cadmium batteries, lithium and mercury, and fluorescent lamps. |
| 952 | Medical wastes: All wastes generated in health centers, such as clinics, emergency rooms etc., waste with a hazard code of 1. Infectious wastes are among these wastes. |
| 953 | Agricultural wastes: All wastes generated in agricultural and gardening activities, such as gardening wastes, etc. with a hazard code of 1. Agricultural pesticide residues are among these wastes. |
| 954 | Unique trash: Wastes described in code 924 generally have properties of wastes with a hazard code of 2; however, some of these wastes have a hazard code of 1 and thus, the code of 954 is assigned to them. |