National Iranian Oil Company Safety, Health and Environment Management Office Ergonomic Considerations Guideline Reviewing Translation; Confirmed by; Publication Date of English Version. Davood Naderi/ Laleh H. Abbasi Training & Performance Measurement Department; Mani Abdollahzadeh Rad/ HSE Manager; September 2018. Responsibility; Prepared by; Corroborated by; Approved by; Publication date. Full Name; Mostafa Sadeghpour Head of Occupational Health [Signed]; Siavash Derafshi [Signed] Head of Health, Safety, and Environment Department; Reza Khalili [Signed] Non-industrial operations Deputy; Ali Kardor [Signed] CEO; February 2017. Position. Document Code: NIOC-HSE-OH-GU-030-01 Revision date: February 12, 2017 Revision No.: 1

Table of Contents

1. Objective	3

2. Scope of Application	3

3. Approving and Implementing Authorities	3

3-2 Implementing Authority	3

4. Checklists	4

5. Implementation Phases	4

5-1-1-2 The required office space	4

D) Anthropometry (body dimensions)	5

5-1-1-3 A Guideline on how to determine the required workspace for individuals	5

5-1-2 Office Desks	5

5-1-3 Office Chairs	5

5-1-3-2 considerations to make when selecting an office chair	6

A) Important features of a good office chair:	6

B) Armrest	7

C) Adjusting Chairs for Different Heights	7

D) Necessary adjustments when the Workstation (Desk) is at a Fixed Height	7

E) An overview of an Appropriate Office Chair	7

5-1-4 Positioning the Monitor	7

5-1-4-1 Factors Affecting the Proper Position of the Computer Monitor	9

A) Setting Up an Appropriate Viewing Angle	9

Warnings	11

B) Setting Up a Proper Viewing Distance	13

5-1-5 Computer Monitors and Display Colors	15

A) Character Color	15

B) Display Polarity	15

C) Image Contrast and Resolution	17

5-1-6 Selecting and Purchasing Ergonomic Office Equipment	17

1. Individual Factors	17

2. Nature of Duties	17

3. Important Environmental Factors	17

Points to consider when selecting office equipment:	18

Important Chair Maintenance Tips	18

The next step after selecting a suitable product	18

5-2 Workplace Inspection	19

6. References	19

Appendix 1 - Office Chair Selection Tips	20

The following tips should be considered when selecting office chairs:	20

The following tips should be considered when selecting lab chairs:	20

Appendix 2 - Office Chair Adjustment Tips for People of Different Heights	21

For adjusting a chair for your own height, follow the instructions below:	21

Appendix 3 - Computer Workstation Ergonomic Considerations	23

1. Monitor positioning	23

2. Glare	23

3. Monitor screen reflection	23

4. Document positioning	23

5. Mouse and other input devices	23

4. Legs and feet	23

7. Head, neck, and shoulders	25

8- Adequate space for legs	25

9. Selecting the right chair	25

10. Adjusting office chair and desk	25

11. Office desk	26

Appendix 4 - Office Ergonomics Checklist	27

No. 17, Yaghma Alley, Jomhouri-e Eslami St. (btwn Hafez St. & Valiasr Ave.), Tehran, Tehran, Iran	32

### 1. Objective

The present guideline includes ergonomic considerations aimed at preventing and controlling musculoskeletal disorders at NIOC’s offices.

### 2. Scope of Application

The guideline applies to all NIOC’s subsidiaries and associate companies.

### 3. Approving and Implementing Authorities

3-1 Approving Authority

NIOC’s HSE Department shall be responsible for the approval and updating of the present guideline.

### 3-2 Implementing Authority

3-2-1 HSE Department

- Establishing the requirements needed for observing the ergonomics principles aimed at preventing musculoskeletal disorders in office personnel

- Inspecting offices and workstations using the standard checklists (Checklists provided in Appendixes 1 and 2 can be used as sample checklists)

- Providing control and corrective solutions aimed at eliminating the risk factors contributing to ergonomic disorders

- Supervising the proper and accurate implementation of such control and corrective solutions

- Inspecting office equipment in terms of ergonomic requirements, and issuing purchase approvals (the approved items are to be purchased through the Procurement and

3-2-2 Procurement and Supply Department

- Purchasing and supplying office equipment according to the requirements stipulated in the present guideline

- Acquiring the HSE Department’s approval for the purchase of the necessary items

### 4. Checklists

Standard ergonomics checklists should be used during workplace, workstation, equipment, and tools inspections (See Appendixes 4 & 5).

### 5. Implementation Phases

5-1 Office and Office Requirements

5-1-1 Workspace Requirements for Office Work

5-1-1-1 Workspace

When designing workspaces, all tools, equipment, and furniture needed to do the intended work should be taken into consideration. Office jobs entail a great variety of physical and mental activities.  Often, the core activities of any office job take place at the desk or at the workstation.

In addition to the physical dimensions of the workspace, other features should also be considered in any design, reorganization or relocation. The most important of such features are as follows:

- Workspace should provide employees with acoustical privacy. For example, people should be able to talk in privacy, according to the level of confidentiality required.

- Noises and conversations should not interfere with people’s concentration

- Noises and conversations should not make it difficult to hear or understand speech while in a phone conversation.

- The word space should provide people with visual privacy.

- All employees should have their own dedicated workspace.

- All employees should have access to both natural and artificial lighting.

- The workplace’s layout should be designed to facilitate interpersonal contact.

### 5-1-1-2 The required office space

The following aspects should be taken into consideration while determining the required office space:

A) Nature of work: If most of the work day is spent on field assignments, meetings, site visits and consultations, a smaller office space may be satisfactory.  However, for office workers who perform their job at their workstation most of the time, a small space may create discomfort due to feelings of confinement. Some job functions that, for example, include frequent meetings in their office space or require the use of multiple sources of material for consultation, research, writing, etc. may be assigned more work space.

B) Cultural aspects: Cultural aspects influence our perception of sufficient space.

C) Individual perception: The amount of space available can have profound psychological meaning. It is natural for people to strive to occupy more space. In the workplace, the amount of our personal space is often linked with our status within the organization.

### D) Anthropometry (body dimensions)

Actual office space requirements depend on the size and shape of employees because an office has to accommodate them, enable them

to move safely and unhindered in the workspace, and allow them to complete their jobs.   Large workspaces always allow for easy movement.

Office spaces should also be able to accommodate visitors and have sufficient storage space for different tools and equipment.

### 5-1-1-3 A Guideline on how to determine the required workspace for individuals

The allocation of the amount of working space for offices, and for workplaces in general, is rather complex. As a result, the available standards and guidelines, which are based on the average body dimensions, are taken as the minimum requirements.

For example, according to a number of occupational health codes of practice, the minimum office space required for each individual is 12m3, provided that the height is at least 3 m.

### 5-1-2 Office Desks

The necessary standards should be taken into consideration when selecting office desks. Accordingly, standards No. 9698-1 and 9698-2 established by Institute of Standards and Industrial Research of Iran and Occupational Health Rules and Regulation published by NIOC (Chapter 3, Articles 12-20) should be referred to when selecting the most appropriate office desk for any situation.

### 5-1-3 Office Chairs

5-1-3-1 Appropriate workstations:

The workstation should let the employee sit and carry out their duties in comfort while allowing for voluntary changes in the working position.

There are three contact areas in the work space that affect the worker's posture: the seat, the work surface (commonly it is a desk top or keyboard) and the floor.

To ensure the most comfortable posture possible, two of these factors have to be adjustable. If you can afford to do nothing else, a fully adjustable chair is a must.

The alternative option would be a fully adjustable desk. Another effective (and cheaper) option is to use an adjustable chair and footrest to secure postural comfort.

### 5-1-3-2 considerations to make when selecting an office chair

A basic rule of ergonomics is that there is no such thing as an "average" person. However, providing a chair specifically designed for each individual is not practical. The only solution is to provide workers with fully adjustable chairs that can accommodate a maximum range of people (around 90 percent of the population). The remaining 10 percent (the shortest and the tallest) will need custom-made chairs.

### A) Important features of a good office chair:

A number of points should be taken

into consideration when selecting office chairs (see Appendix 1). Standard No. 9697-1 established by Institute of Standards and Industrial Research of Iran should be referred to when selecting desks and chairs to be used in educational centers.

### B) Armrest

Armrests are generally not recommended because they can prevent the users from getting close enough to the desk.  However, armrests that extend 25 cm (10 inches), or less, allow users to move their chairs closer to their workstations.

The armrests give them a place to rest their arms which, in turn, takes some of the load off their shoulders and neck.

### C) Adjusting Chairs for Different Heights

For tips on adjusting the chair to your height, refer to Appendix 2.

### D) Necessary adjustments when the Workstation (Desk) is at a Fixed Height

- If you cannot fit your legs under the workstation or there is not enough space to move them freely, your workstation is too low for you and you should refrain from using such a workstation.

- If you can sit comfortably but need to elevate your arms in order to place them over the work surface, your workstation is too high. Adjust the chair height so your elbows are about the same height as the work surface.

- Use a footrest if you cannot place your feet flat on the floor.

- The footrest should be adjustable and support both feet.  Keep feet flat on the footrest.

### E) An overview of an Appropriate Office Chair

Refer to Occupational Health Rules and Regulation published by NIOC (Chapter 3, Articles 76-80).

### 5-1-4 Positioning the Monitor

The monitor is an integral part of a computer workstation. When placed in the wrong position, it can force the operator to work in a variety of awkward positions.

Working with one's chin tilted upwards, and the head and upper body bent forwards or sideways is common wherever the monitor is improperly situated. Such forced working body positions significantly contribute to the operator's discomfort, and can potentially lead to work related musculoskeletal injury (WMSD). Other adverse effects of a poorly located monitor are burning eyes and headaches, collectively called eyestrain.

Common complaints among computer operators include discomfort, aches and pains in the neck and shoulder, and eyestrain. Monitor placement and positioning is an important, though relatively neglected, part of the workstation design process.

### 5-1-4-1 Factors Affecting the Proper Position of the Computer Monitor

Two factors deciding the proper position of a computer monitor include viewing angle and viewing distance. Viewing angle refers to the degree above or below an imaginary horizontal line at the level of the viewer's eyes and the center of the object being looked at. In case of a computer work, it is the center of the screen (Figure 1).

Viewing distance refers to the space between the operator's eyes and the screen (Figure 1). These factors play an important role in correcting the position of a computer monitor.

A poor angle leads to postural (neck and shoulders) discomfort, while the wrong distance can contribute to eyestrain.

Figure 1

### A) Setting Up an Appropriate Viewing Angle

Researchers agree that at rest, the eyes naturally assume a straightforward and downward cast (Figure 1) line of sight.

How far downwards, however, is not clear. Experimental findings range from about 15 degrees to almost 30 degrees.

People limit their downward eye movements to about half of the whole available range of 60 degrees.

Therefore, for comfortable viewing of images on a computer screen, it is probably reasonable to place the monitor at about 15 degrees (or slightly lower) below the horizontal line. Such a location creates a preferable visual zone of 30 degrees (+ 15 degrees to -15 degrees from the normal line of sight).

Figure 2

Studies indicate that looking upwards (above the horizontal) is tiring for people doing intense visual work. In other words, looking downwards, lower than 15 degrees below the horizontal, was not reported as fatiguing. This finding allows one to extend the visual zone downward by another 15 degrees (an acceptable visual zone).

Figure 3

### Warnings

1. Occasionally monitors are placed on top of the hard case due to space limitations. A monitor located at a high level is a source of discomfort and, in the long run, can cause musculoskeletal problems in the neck and shoulder area.At a workstation where the desk and chair heights are properly adjusted, the monitor should be placed at the same level as the keyboard. The discomfort caused by a monitor which is too high (above the horizontal line) is worse than the one which is due to the monitor being placed too low (below an acceptable visual zone).

2. When using a larger monitor (17", 19" or larger) or one that is oriented to the "portrait" position, make sure that the top of the screen is not at a level higher than the operator's eye.

### B) Setting Up a Proper Viewing Distance

Looking at far distances does not cause eyestrain; it is the muscular effort required to focus on objects at close distances that strains the eyes.

The distinction between far distance and close distance is never razor sharp. It changes over time due to age.

Accommodation and convergence are the two main functions, which govern the viewing of objects at closer distances. The shorter the viewing distance, the greater the muscular effort required for accommodation and convergence, increasing the risk for eye discomfort. A greater viewing distance, on the other hand, eliminates the risk for eyestrain but can make the resolving of the finer images or characters displayed on the computer screen more difficult. The right viewing distance is the one at which the computer operator can easily read the letters on a screen without experiencing eyestrain.

The eyes have a default accommodation distance, called the Resting Point of Accommodation (RPA) and a default Resting Point of Vergence (RPV). In practical terms, at distances greater than the RPA and RPV, there is no need for either accommodation or convergence.

Under these conditions, individuals with perfect (20/20) vision or those with properly corrected vision are not likely to experience eyestrain.

Numerical values of RPA  (around 75 cm (about 30 in.)) and RPV (around 80 cm (about 32 in.)) are close to each other which helps setting up the right viewing distance.

The viewing range 60 cm to 80 cm (about 24 - 32 in.) provides visual comfort for majority of computer users.

In the situation where the recommended viewing distance is too great for the operator to see images clearly, it is better to increase the font size (images) than to force a shorter viewing distance.

Accommodation is the automatic adjustment of the eye for focusing on near objects. The eyes achieve this by changing the convexity (or shape) of the lenses. Convergence is the automatic inward (toward the nose) turn of the eyes when viewing close objects. Computer workstation ergonomic considerations can be found in Appendix 3.

### 5-1-5 Computer Monitors and Display Colors

The choice of color for characters on your computer monitor or Video Display Terminal (VDT) is largely a matter of personal preference. However, several aspects of the VDT should be considered before you select the colors. The most important points to consider are the clarity (legibility) of the characters and the contrast between the color of the characters and the background color.

### A) Character Color

The consensus is that the non-colors, white and black, and the colors yellow, green, and orange are generally most acceptable. These colors are in the middle of the visible spectrum and are the easiest for the eye to see.

Our eyes are not as sensitive to the colors at the extreme ends of the visible spectrum (e.g., blue, violet, and red). Thus, characters in colors situated at the extreme ends of the visible spectrum should be avoided unless displayed on a light or contrasting background.

Colors may create images following prolonged viewing. For example, after looking at a green image for a period, when you look away, white images may appear pink-tinged. This is a harmful, very short term effect that can be caused by any colored source (e.g., a painted surface), not just computer screens.

### B) Display Polarity

Positive polarity means that a VDT or monitor displays dark characters on a light background, while negative polarity means that light characters are displayed on a dark background (dark characters on a light background similar to what we are most used to in print materials).

The resolution of positive polarity screens tends to be less affected by glare (screen reflections). In other words,, it tends to make screen flicker more noticeably. . At the same time, negative polarity provides better character contrast and is less prone to flicker.  User preference should be the determining factor when setting (adjusting) display polarity.

### C) Image Contrast and Resolution

The contrast ratio should usually be 10:1 to make a display easier to read. What is acceptable to an individual will also depend on character size, viewing distance and the type of task being done. A monitor should provide the user with right

### 5-1-6 Selecting and Purchasing Ergonomic Office Equipment

The points to consider when purchasing office equipment are as follows:

### 1. Individual Factors

Knowing the characteristics of the individuals who are going to use the equipment is of utmost importance. Here the critical points to consider are the potential user’s  body size, height, gender, and right- or left-handedness.

### 2. Nature of Duties

Different office tasks require different equipment, different accessories and different layouts.  Understanding the basics of the work carried out helps one to understand more clearly what the worker needs in order to make his job better ergonomically.

Consider the nature of the tasks to be done:

- A lot of typing or very little

- Typing combined with other desk work (e.g., taking notes, using a phone, etc.)

- Using a mouse or other input devices (e.g., voice input).

### 3. Important Environmental Factors

Many aspects of the work environment must be considered. You should take into account the following:

- Available space and office dimensions Layout or arrangement of existing furniture

- Light sources, specifically lamps

- Type and size of computers to be used

- Table-mounted or floor-mounted computer cases

- Accessories such as standalone hard drives, storage devices, mice, etc.

- Interaction with colleagues

### Points to consider when selecting office equipment:

- Office equipment should have an adjustability range that can fit all users

- Some chairs have interchangeable cylinders to accommodate very tall or short people.

- A fully adjustable chair with height-adjustable armrests is recommended.

- An adjustable desk is preferable.

- Do not forget to get a footrest if you decide on a non-adjustable desk.

- Accessories, such as a copy-holder, mouse, task lamps, etc. (getting the personnel’ feedback is very important).

- Office equipment should not have a reflective surface (e.g., the color of the chair or desk should not be reflective).

### Important Chair Maintenance Tips

When buying chairs, consider any maintenance and repair costs. Some chair manufacturers will recommend an inspection and maintenance schedule.

Common problems may include:

- Bolts and screws loosening and falling out

- Hydraulic cylinders failing to hold or seize

- Chair failing to roll properly

### The next step after selecting a suitable product

Before you make a final decision, you should ask your staff for their opinion.

Having the staff actively involved in the decision-making process is very important for the selection of furniture and equipment that is suited to them and their work tasks. Training on how to use, adjust, and readjust new equipment is also crucial for the successful introduction of new equipment and furniture into the workplace.

Keep in mind that the supplier's claim that his products are ergonomically correct is no guarantee of comfort.

Arming yourself with valid information from reliable sources will help you make the right investment that will benefit an employer and employees equally. Consulting a specialist can also prove helpful.

### 5-2 Workplace Inspection

The occupational health inspector should annually inspect all working places and workstations based on the relevant standard checklists (checklists provided in appendixes 4 and 5 can be helpful) to ensure all basic ergonomic principles are observed. The inspection’s report will help the organization take the necessary measures to improve the personnel’s working conditions. In case of any further complaint by the staff, the inspection should be re-conducted.

5-3 Records Storage

The records containing ergonomic assessment reports of control rooms and offices should be stored in the specified locations for the duration mentioned in the table below.

| Storage Duration | Storage Location | Records |
| --- | --- | --- |
| 5 Years | HSE Department | Office Inspection Checklists and HSE’s Administrative Buildings Inspection Checklist |

### 6. References

1. Standards formulated by the Institute of Standards and Industrial Research of Iran

2.  Occupational Health Rules and Regulation published by NIOC

3. http://www.hse.gov.uk/comah/sragtech/techmeascontrol.htm

4. http://www.ccohs.ca/oshanswers/ergonomics/office/vdtcolor.html

- 5. http://www.ccohs.ca/oshanswers/ergonomics/office/monitor_positioning.html

- 6. http://www.ccohs.ca/oshanswers/ergonomics/office/working_space.html

- 7. http://www.ccohs.ca/oshanswers/ergonomics/office/purchase.html

# Appendix 1 - Office Chair Selection Tips

### The following tips should be considered when selecting office chairs:

- Controls that are easy to operate from sitting position

- A seat that adjusts for both height and tilt

- A seat that does not put pressure on the back of thighs or knees

- A seat with a front edge that bends (curves) towards the floor

- Breathable, non-slippery fabric on the seat

- A backrest shaped to support the lower back

- A stable five-point base

- Wheels suitable for the type of flooring

- A swivel mechanism

- Armrests that can be adjusted to the elbow height when your upper arms are hanging down and your forearms are at about a 90 degree angle to the upper arms

- Armrests that do not interfere with free movements within the workstation

### The following tips should be considered when selecting lab chairs:

- Ergonomic chairs equipped with adjustable backrests and seats and a footing ring should be used in laboratories.

- The chair’s size should be determined based on Standard ANSI/HFS 100-1988.

# Appendix 2 - Office Chair Adjustment Tips for People of Different Heights

### For adjusting a chair for your own height, follow the instructions below:

- Stand in front of the chair so the highest point of the seat, (when in the horizontal position) is just below the knee cap.

- Sit on the chair and keep your feet flat on the floor. Check that the clearance between the front edge of the seat and the lower part of the legs (your calves) fits a clenched fist (about 2 inches).

- Adjust the backrest so that it fits the hollow in your lower back.

- Sit upright with your arms hanging loosely by your sides. Bend your elbows at about a right angle and adjust the armrests height until they barely touch the undersides of the elbows.

- Remove the armrests from the chair if this level cannot be achieved or if armrests, in their lowest adjustment, elevate your elbows even slightly.

- Tilt the seat forwards or backwards if you prefer.

# Appendix 3 - Computer Workstation Ergonomic Considerations

### 1. Monitor positioning

The computer monitor should be placed at right angle to the window and be adjustable horizontally, vertically, and sideways. It should be placed directly in front of the keyboard.

Place the top line of the screen at user’s the eye level. The typical advice is to sit about an arm's length away from your monitor.

However, the ideal distance is approximately 26 cm.

### 2. Glare

The glare can be reduced by using blinds and dimming the overhead light. Use local lighting (a desk lamp) to provide lighting for your desk.

### 3. Monitor screen reflection

To reduce screen glare and reflection, tilt it upwards, downwards, to the left, or to the right or use a special filter.

### 4. Document positioning

If possible, use a paper holder to keep documents close to the monitor screen. The holder should be placed as a height and distance similar to those of the monitor screen.

### 5. Mouse and other input devices

Place the mouse and other input devices close to the keyboard to reduce the tension in your shoulders.

Use a keyboard with a wrist rest If you need to keep your hands up when typing. When your hand is on the mouse, your fingers, wrist and forearm should be placed in a straight line and your elbow should be close to your body.

### 6. Legs and feet

Thighs should be parallel to the floor and the legs should be positioned vertically (a right angle to the floor). The feet should be places on the floor or a footrest.

### 7. Head, neck, and shoulders

When seated, your head and neck should be straight (forward). Keep your shoulders loose and relaxed and your elbows as close to your body as possible.

### 8- Adequate space for legs

Allow enough space under the keyboard for your knees and thighs.

### 9. Selecting the right chair

A good chair should:

- Have a backrest serving as a support.

- Have a seat with an appropriate height that does not put pressure on the back of thighs or knees

- Have a seat with a front edge that slightly curves downward

- Has armrests preventing it getting too close to the desk.

### 10. Adjusting office chair and desk

Stand in front of the chair and adjust the seat's height so that its front edge is just below the knee cap. Sit on the chair so that the clearance between the front edge of the seat and the lower part of the legs fits a clenched fist. Adjust the backrest so that it fits the hollow in your lower back. The backrest should be firmly secured so that it will not move due to the user's body weight.

Adjust the height of the work surface to the elbow’s height with your arms hanging loosely by your sides.  If using a work surface with a fixed height (e.g., a desk), adjust the chair's height so that your arms and upper body are in the appropriate position.

In such situation, follow the steps below:

A) Adjust the chair height so your elbows are about the same height as the work surface

B) Use a footrest if you cannot place your feet flat on the floor and pressure is put on the back of your legs. The footrest should be adjustable and have enough area for support both feet simultaneously.

### 11. Office desk

The desk’s dimensions are determined by the chair’s dimensions. The desk surface should be adjusted to an appropriate height. There should be enough space on the desk surface where you can put all the tools and equipment needed to do your work. The desk surface is used for activities like writing and controlling the computer. You can also place objects such as books and papers on your office desk.

There should be sufficient space under the desk so that the operator is not forced to fold up his legs. Objects should not be left out and stored on the desk.

The desk should be designed so that the user is not forced to bend over the surface to do the job.

# Appendix 4 - Office Ergonomics Checklist

| Comments | No | Yes | Question | Number |
| --- | --- | --- | --- | --- |
|  |  |  | Does the building comply with the standards in terms of use, occupancy, and services? | 1 |
|  |  |  | Have appropriate materials been used for flooring? | 2 |
|  |  |  | Were colors selected based on the working conditions? | 3 |
|  |  |  | Were materials selected based on the working conditions? | 4 |
|  |  |  | Have lighting color, brightness, contrast, etc. been considered when conducting the office lighting design? | 5 |
|  |  |  | Is the workplace designed so the user is not forced to repeatedly bend downwards to do the job? | 6 |
|  |  |  | Were chairs and desk selected based on the working conditions? | 7 |
|  |  |  | Can the user get close enough to the workstation without coming across any barrier? | 8 |
|  |  |  | Are work surfaces (e.g., desk, shelf) adjusted to an appropriate height? | 9 |
|  |  |  | Are telephone devices, computers, and shelves accessible? | 10 |
|  |  |  | Are the working conditions so that the user is not forced to lift high weight? | 11 |
|  |  |  | Do chairs come with adjustable height, backrest, armrests, etc.? | 12 |
|  |  |  | Is a footrest available? | 13 |
|  |  |  | Is the task of a repetitive nature? | 14 |
|  |  |  | Do repetitive tasks follow a rotation pattern? | 15 |
|  |  |  | Is break time duration enough to prevent fatigue? | 16 |
|  |  |  | Do office personnel (working with VDTs) have sufficient break time (5 minutes per hour)? | 17 |
|  |  |  | Is there sufficient space under the desk so that the individual can place his legs at an appropriate angle( (thighs and legs forming a 90 degree angle)? | 18 |
|  |  |  | Do all personnel know how to adjust a chair and sort a workstation? | 19 |
|  |  |  | Are the available equipment and furniture suitable for the current working conditions? | 20 |
|  |  |  | Are the available equipment and furniture sufficient for th performing the job task? | 21 |
|  |  |  | Have additional and unused equipment been taken out of the workplace? | 22 |

| Comments | No | Yes | Question | Number |
| --- | --- | --- | --- | --- |
|  |  |  | Have the damaged, broken, and old equipment been taken out of the office? | 23 |
|  |  |  | Do damaged chairs and other equipment get fixed or replaced? | 24 |
|  |  |  | Does the furniture arrangement allow for easy access to exits? | 25 |
|  |  |  | Are emergency information boards installed in appropriate places? | 26 |
|  |  |  | Are entries and exits easily accessible and detectable? | 27 |
|  |  |  | I the number of exits sufficient for the number of people working at the office? | 28 |
|  |  |  | Are exit routs and doors clearly visible? | 29 |
|  |  |  | Are fire extinguishers and first-aid kits placed at appropriate places and easy to use? | 30 |

Checklist 5 - Office Workstation Ergonomics Checklist

|  | Personnel Code: | Full Name: |  |
| --- | --- | --- | --- |
|  | Job: | Organizational Unit: |  |
|  | Total years of professional experience: | Years of experience in current job: |  |
|  | Weight: | Birthday (year): |  |
| How many hours a day are you working at the following positions? Moving Standing Sitting |  |  |  |
|  |  | Are you right-handed or left-handed? | 1 |
|  |  | Do you wear prescription glasses? | 2 |
|  |  | If yes, what type of prescription glasses do you wear? | 3 |
|  |  | Does your chair come with an adjustable height? | 4 |
|  |  | Does your chair have armrests? | 5 |
|  |  | If yes, do the armrests interfere with your activities? | 6 |
|  |  | Does your chair have a swivel mechanism (rotating to the sides)? | 7 |
| Wheeled with 5 legs | Wheeled with 4 legs | Which one corresponds to your chair? | 8 |
|  |  | Does your desk come with an adjustable height? | 9 |
|  |  | Do you use a footrest while working behind a desk? | 10 |
|  |  | Are you a typist or computer operator? If yes. do you use paper holders? | 11 |
|  |  | Are the equipment placed on your desk within your easy reach? | 12 |
|  |  | Are you forced to bend your lower back forwards when working behind a desk? | 13 |
|  |  | Do you feel comfortable when working behind a desk? | 14 |
|  |  | Are your forced to turn to the either side to do your work behind the desk? | 15 |
|  |  | Do you use a computer to perform your tasks? | 16 |
|  |  | If operating a computer, how far are your eyes from the computer screen? | 17 |
|  |  | Is the computer monitor adjusted to a height that forces you to tilt your head too much upwards /downwards? | 18 |
|  |  | Does the type of activity you do allow you to use slope surfaces for writing and reading letters and documents? | 19 |
|  |  | Do you think your workplace has sufficient lighting? | 20 |
|  |  | Which sour of lighting is used at your workplace? | 21 |
|  |  | Do you do stretching and relaxation exercises behind your desk during the day? | 22 |
|  |  | Do you have sufficient knowledge of the above exercises and have received relevant training? | 23 |
|  |  | Have your received training on how to sit behind a desk and do other relevant stuff when working at an office? | 24 |

Health, Safety, and Environment (HSE) Department

## No. 17, Yaghma Alley, Jomhouri-e Eslami St. (btwn Hafez St. & Valiasr Ave.), Tehran, Tehran, Iran

Phone: +98 21 66726016, Fax: +98 21 6609723, SMS System: +98 10007615

HSEE@NIOC.IR	HTTP://HSE.NIOC.IR