HSE Requirements of Oil/Gas Equipment during the Drilling Stage at On-Shore Sites

| Reviewing Translation | Confirmed by | Publication Date of English Version |
| --- | --- | --- |
| Davood Naderi/ Laleh H. Abbasi Training & Performance Measurement Department | Mani Abdollahzadeh Rad/ HSE Manager | September 2018 |

| Responsibility/Subject | Prepared by | Confirmed by | Approved by | Publication date |
| --- | --- | --- | --- | --- |
| Name | Hossein Mirderikvand Senior Specialist of passive defense & crisis management | Siavash Derafshi/ HSE Manager | Ali Kardor CEO | Agust 2017 |
| Position |  |  |  |  |

Revision No.: 0  Code: NIOC-HSE-SF-PR-035-00  Revision Date: 08/07/2017

Contents

1. Introduction .............................................................................................................................................. 3

2. Purpose...................................................................................................................................................... 3

3. Scope ........................................................................................................................................................ 4

4. Roles and responsibilities ......................................................................................................................... 4

5. Definitions and terms ............................................................................................................................... 4

5.1 The main components of a drilling rig ................................................................................................... 4

5.2 Rigging up .............................................................................................................................................. 4

5.3 Mud circulating system .......................................................................................................................... 5

5.4 Drilling fluid (mud) ................................................................................................................................ 5

5.5 Rigging down ......................................................................................................................................... 5

5.6 Drilling ahead ......................................................................................................................................... 5

5.7 Drill stem ................................................................................................................................................ 5

5.8 Tripping out/in ....................................................................................................................................... 6

5.9 Casing and cementing operations .......................................................................................................... 6

6. HSE requirements of rigging up operation .............................................................................................. 6

7. HSE requirements of the drilling ahead stage .......................................................................................... 8

8. HSE requirements of tripping out/in operation ........................................................................................ 9

9. HSE requirements of casing and cementing operations ......................................................................... 10

10. HSE requirements of maintenance activities ....................................................................................... 10

11. HSE requirements of well control operations ...................................................................................... 11

12. Resources ............................................................................................................................................. 11

1. Introduction

After conducting various geological studies (e.g. seismography) and performing different tests on sedimentary stones, geologists can discover oil or gas resources in a specific location. The drilling operations will then begin taking into account the location of the site and all the factors effective in drilling. Drilling operations [using drilling equipment] in the oil and gas industry are among the most complicated, costly, cumbersome and high-risk occupations in the world. The oil and gas well drilling operations consist of various processes, including site preparation, drilling, well completion, servicing, and finally, well plugging and abandonment.

Drilling operations cause various environmental impacts that can contaminate the air, water and soil and may have some negative effects on human activity.

In the NIOC’s operational areas, drilling operations largely depend on the type of the drilling tools, machinery and equipment used, and, like other industrial environments, drilling sites are associated with various health, safety and environmental hazards. Therefore, if necessary measures are not taken to prevent potential hazards, there will be a high probability of damage to personnel, equipment and the environment. In this regard, the NIOC’s HSE management has established a working group composed of the authorities of the National Iranian South Oil Company, the National Iranian Drilling Company, the National Iranian Oil Company Exploration Directorate (NIOCEXP), the Iranian Offshore Oil Company and the North Drilling Company and has held expert meetings on preparation and compilation of HSE requirements/provisions for all drilling operations. This guideline presents the HSE requirements of oil/gas equipment during the drilling stage at on-shore sites.

Based on this document, all the NIOC’s subsidiaries and operational units, operating in the area of offshore drilling, shall prepare relevant working instructions [in accordance with the nature of their activities and their organizational structure] to comply with the provisions of this document (as minimum requirements) and shall implement it within a specified period.

2. Purpose

The purpose of this document is to identify HSE requirements of drilling operation and compliance with these requirements to prevent work-related accidents and to minimize relevant consequences and damages to the NIOC’s manpower, facilities and the environment.

- Rigging up

- Drilling ahead

- Tripping out/in

- Casing and cementing operations

- Maintenance activities

- Well control

3. Scope

This guideline is applicable to all the NIOC’s subsidiaries, associates and operational sites operating in the area of offshore drilling.

4. Roles and responsibilities

The NIOC’s HSE Management is responsible for updating this guideline and monitoring its proper implementation across relevant units.

The Managing Directors of the NIOC’s subsidiaries and associates are responsible for monitoring the proper implementation of this guideline through preparing relevant working instructions and HSE units shall implement the guideline across their units, in accordance with the nature of their activities and their organizational structure.

5. Definitions and terms

5.1 The main components of a drilling rig:

The following systems represent the main components of a drilling rig, each of which is divided into several sub-systems:

1) Power system: This system provides the driving force and electricity needs of drilling rigs.

2) Hoisting system: This system consists of the mast, draw-works, Crown blocks, Crown saver, drilling line, emergency brake system, emergency shutdown system, anti-collision systems and Crown-O-Matic.

3) Rotating system: This system consists of swivel, top drives, Kelly, rotary table, drill pipe, drill collar and drill bits.

4. Mud circulating system: This system consists of drilling mud, drilling mud tanks, mud pumps, mud treatment unit, Degasser, Desilter and Desander.

5. Well control system: This system consists of blowout preventers and accumulators.

6. Auxiliary equipment (e.g. generators, compressors, etc.).

5.2 Rigging up

Rigging up is the process of preparation and assemblage of various parts of equipment to make a drilling rig.

- Setting up the substructure

- Installing the power system

- Setting up the rig floor and mast or derrick

- Installing handrails, guardrails, stairs, walkways and ladders

- Rigging up the circulating system

- Installing auxiliary equipment

- installing the rotary system

- Setting up the blowout preventing system in repair wells

- Installing the waste management system equipment

5.3 Mud circulating system

In the mud circulation system, strong pipes circulate the drilling fluid (mud) through the stand pipe and the rotary hose. After passing through the drill nozzles, the mud enters into the well and finally, the drill mud will pass through the annulus and will re-enter into the mud tank.

5.4 Drilling fluid (mud)

Drilling fluid (mud) is a fluid circulating in the well during the drilling operations. This fluid collects drilling cuttings, serves as a cooling element for the drill bit, prevents the collapse of well walls, etc.

5.5 Rigging down

Taking down or dismantling the drilling rig that is basically the reverse of the rigging up process.

5.6 Drilling ahead

Following the site preparation and rigging up operations, the actual drilling of the well is initiated, which includes the following steps:

- Transporting tubulars and putting them in relevant pipe racks

- Preparing drilling fluid

- Starting the drilling operation by attaching a drill pipe to the drill stem

5.7 Drill stem

All components placed between the swivel and the drill bit that are used for rotating drilling and include the Kelly, drill pipes and sub, pipe collar, etc.

5.8 Tripping out/in

Tripping refers to the process of removing pipes (upper pipes) from the well or replacing them (lower pipes) when necessary.

5.9 Casing and cementing operations

Casing operation includes placing specific pipes in the well to prevent the collapse of walls and the injection of cement into the annulus (space) between the casing and the well wall is called cementing.

6. HSE requirements of rigging up operation

6.1 Interface permits shall be obtained in joint working areas before commencing the rigging up operation. In this regard, review the “Permit to Work System Guideline” (code: NIOC-HSE -SF-GU-010-01).

6.2 The rigging up operation permit shall be obtained before commencing the rigging up operation to ensure the safety and operability of the rig and other drilling equipment and working conditions.

6.3 The new location and duration of stay of personnel at the drilling site shall be immediately communicated to the nearest fire station and helicopter base to ask for help when needed.

6.4 Appropriate measures shall be taken to have an ambulance, a doctor and medical equipment at the drilling site.

6.5 The drilling rig shall be equipped with fire alarm and extinguishing systems.

6.6 All potential work-related risks and hazards shall be identified and relevant control measures shall be taken. In this regard, risk identification and control techniques can be employed (Job Risk Assessment, HAZID, etc.).

6.7 To inform personnel of various work-related hazards and to establish necessary coordination between participating teams, safety meetings shall be held before commencing the rigging up operation.

6.8 Ensure the competence of all machinery operators (cranes, lift trucks, Gin Poles, etc.) should be ensured. In this regard, review the “Guideline for Safe Operation of Lifting and Conveying Equipment” (code: NIOC-HSE-SF-GU-022-00).

6.9 Ensure the operability and proper function of electric brakes before commencing the rigging up operation.

6.10 Some activities require obtaining specific permits (hot work, cold work, working at height, etc.). In this regard, review the “Permit to Work System Guideline” (code: NIOC-HSE -SF-GU-010-01).

6.11 Ensure the operability and integrity of all components and equipment used for loading, lifting, etc. In this regard, review the “Guideline for Safe Operation of Lifting and Conveying Equipment” (code: NIOC-HSE-SF-GU-022-00).

6.12 To ensure safe load handling, comply with relevant lifting instructions and techniques. In this regard, review the “Guideline for Safe Operation of Lifting and Conveying Equipment” (code: NIOC-HSE-SF-GU-022-00).

6.13 Appropriate safety and warning signs should be provided and installed in different places to inform employees of possible hazards.

6.14 Emergency Escapes Devices must be installed in accordance with manufacturer’s standards.

6.15 All work areas, especially different floors should be clean, well-ordered and without any oil and spills.

6.16 Observe lock out/tag out instructions and electrical safety regulations when installing the power system. In this regard, review the “Permit to Work System Guideline” (code: NIOC-HSE -SF-GU-010-01).

6.17 Provide Material Safety Data Sheet (MSDS) forms for all chemicals and ensure that all personnel fully observe relevant HSE considerations.

6.18 The location and arrangement of all drilling equipment and devices (e.g. drilling pumps, mud tanks, etc.) should be confirmed by the relevant HSE unit.

6.19 All staff shall wear suitable PPE at work. Regarding the proper use of PPE, see the “Guideline for Selection, Preparation and Use of PPE and Safety Equipment” (code: NIOC-HSE-EN-GU-014-00).

6.20 Ensure the provision of adequate lighting overnight.

6.21 Ensure the installation and application of waste management equipment on drilling machines in Iran’s quadruplet areas under protection to prevent environmental pollutions. In this regard, review the “Guideline for Compliance with Environmental Considerations in Exploration and Drilling Operations” (Code: NIOC-HSE-EN-GU-015-00) and the “Waste Management Guideline” (Code: NIOC-HSE-EN-GU-006-00).

6.22 HSE requirements of the rigging down operation (similar to those of the rigging up operation) shall be observed.

6.23 Ensure the safety of the well before commencing the rigging down operation.

7. HSE requirements of the drilling ahead stage

7.1 Besides work-related risks and hazards, those associated with the drilling process should also be identified (using HAZOP study, etc.) and appropriate control measures should be taken.

7.2 To inform personnel of various work-related hazards and to establish necessary coordination between participating teams, safety meetings shall be held before commencing the drilling ahead stage

7.3 To ensure safe load handling, comply with relevant lifting instructions and techniques. In this regard, review the “Guideline for Safe Operation of Lifting and Conveying Equipment” (code: NIOC-HSE-SF-GU-022-00).

7.4 Suitable safety screens should be provided to prevent falling-object hazards.

7.5 The tubulars shall be securely fixed into relevant pipe racks.

7.6 Ensure the integrity and safety of all cables, chains, belts, shackles, molds, etc. by regular periodic inspections.

7.7 Provide MSDS forms for all chemicals (drilling fluid, etc.) and ensure the visibility of these forms at the drilling site and that all personnel fully observe the HSE requirements specified in these forms.

7.8 Regarding the proper use of PPE, see the “Guideline for Selection, Preparation and Use of PPE and Safety Equipment” (code: NIOC-HSE-EN-GU-014-00).

7.9 To review relevant environmental considerations, see the “Guideline for Compliance with Environmental Considerations in Exploration and Drilling Operations” (Code: NIOC-HSE-EN-GU-015-00).

7.10 Emergency shower and eyewash stations should be established in accordance with relevant MSDS forms.

7.11 Suitable fire extinguishing devices shall be provided at the drilling site.

7.12 Permits should be obtained before commencing any operation at confined spaces (cleaning, welding, etc.). In this regard, review the “Permit to Work System Guideline” (code: NIOC-HSE -SF-GU-010-01).

7.13 Ensure observance of all safety regulations during drilling operations on the drilling platform.

7.14 Ensure the provision of adequate lighting overnight.

8. HSE requirements of tripping out/in operation

8.1 All potential work-related risks and hazards shall be identified and relevant control measures shall be taken. In this regard, risk identification and control techniques can be employed (Job Risk Assessment, HAZID, etc.).

8.2 To inform personnel of various work-related hazards and to establish necessary coordination between participating teams, safety meetings shall be held before commencing the tripping out/in operation.

8.3 Ensure the integrity and safety of all equipment though conducting regular periodic inspections and by performing preventive maintenance operations.

8.4 Ensure that all staff properly use Fall Arrest Systems and safety belts when working at height. Regarding the proper use of PPE, see the “Guideline for Selection, Preparation and Use of PPE and Safety Equipment” (code: NIOC-HSE-EN-GU-014-00).

8.5 Ensure that all drilling rig staff are effectively trained to use emergency escape devices properly.

8.6 Ensure the proper function of all emergency power backup systems.

8.7 Ensure the provision of adequate lighting overnight.

9. HSE requirements of casing and cementing operations

9.1 All potential work-related risks and hazards shall be identified and relevant control measures shall be taken. In this regard, risk identification and control techniques can be employed (Job Risk Assessment, HAZID, etc.).

9.2 To inform personnel of various work-related hazards and to establish necessary coordination between participating teams, safety meetings shall be held before commencing the casing and cementing operations.

9.3 Regarding the proper use of PPE, see the “Guideline for Selection, Preparation and Use of PPE and Safety Equipment” (code: NIOC-HSE-EN-GU-014-00).

9.4 Ensure observance of all environmental considerations to prevent potential contaminations in cementing operation. In this regard, review the “Guideline for Compliance with Environmental Considerations in Exploration and Drilling Operations” (Code: NIOC-HSE-EN-GU-015-00) and the “Waste Management Guideline” (Code: NIOC-HSE-EN-GU-006-00).

9.5 Ensure the provision of adequate lighting overnight.

10. HSE requirements of maintenance activities

10.1 Ensure that all regular periodic inspections are conducted; preventive maintenance operations are performed and that all drilling equipment and tools are calibrated.

10.2 Some maintenance activities require obtaining specific permits. In this regard, review the “Permit to Work System Guideline” (code: NIOC-HSE -SF-GU-010-01).

10.3 Observe lock out/tag out regulations and instructions. In this regard, review the “Permit to Work System Guideline” (code: NIOC-HSE -SF-GU-010-01).

10.4 All staff shall wear suitable PPE during maintenance activities. Regarding the proper use of PPE, see the “Guideline for Selection, Preparation and Use of PPE and Safety Equipment” (code: NIOC-HSE-EN-GU-014-00).

10.5 Ensure the integrity and safety of all cables, chains, belts, shackles, molds, etc. by regular periodic inspections.

10.6 Relevant environmental considerations shall be observed regarding the production of wastes during maintenance activities. In this regard, review the “Guideline for Compliance with Environmental Considerations in Exploration and Drilling Operations” (Code: NIOC-HSE-EN-GU-015-00) and the “Waste Management Guideline” (Code: NIOC-HSE-EN-GU-006-00).

11. HSE requirements of well control operations

11.1 All staff shall successfully complete drilling and well control courses and must obtain relevant certificates approved by the NIOC’s HSE management before commencing their job.

11.2 The selection, arrangement, maintenance, performance testing, pressure testing of all blowout prevention equipment (blowout preventers, accumulators, choke manifolds, etc.) should be conducted in accordance with the API STD 53: Blowout Prevention Equipment Systems for Drilling Wells.

11.3 Regarding the proper use of PPE, see the “Guideline for Selection, Preparation and Use of PPE and Safety Equipment” (code: NIOC-HSE-EN-GU-014-00).

11.4 Ensure the proper installation and function all fire alarm and extinguishing systems.

11.5 Ensure the proper installation and function of detectors, portable and fixed gas meters and visual and auditory alarms, especially around the wells.

11.6 Stop the drilling operation and secure the well immediately, in the event of release of hydrocarbon substances in the vicinity of the wells.

11.7 Ensure the availability and accuracy of the emergency response plan and the crisis management system. In this regard, review the “Crisis Management Plan Guideline” (code: NIOC-HSE-PC-GU-017-00) and the “Incident Command System Guideline” (code: NIOC-HSE-PC-GU-011-00).

11.8 Ensure the adequacy of materials needed, including barite, cement, water, etc. In this regard, review the “Guideline for Compliance with Environmental Considerations in Exploration and Drilling Operations” (Code: NIOC-HSE-EN-GU-015-00) and the “Waste Management Guideline” (Code: NIOC-HSE-EN-GU-006-00).

12. Resources

- Occupational Safety & Health (OSHA): Oil and Gas Well Drilling and Servicing eTool.

- API STD 53: Blowout Prevention Equipment Systems for Drilling Wells.

- Guideline for Selection, Preparation and Use of PPE and Safety Equipment (code: NIOC-HSE-EN-GU-014-00).

- Permit to Work System Guideline (code: NIOC-HSE -SF-GU-010-01).

- Crisis Management Plan Guideline (code: NIOC-HSE-PC-GU-017-00).

- Incident Command System Guideline (code: NIOC-HSE-PC-GU-011-00).

- Guideline for Safe Operation of Lifting and Conveying Equipment (code: NIOC-HSE-SF-GU-022-00).

- Waste Management Guideline (Code: NIOC-HSE-EN-GU-006-00).

- Guideline for Compliance with Environmental Considerations in Exploration and Drilling Operations (Code: NIOC-HSE-EN-GU-015-00).

No. 17, Yaghma Alley, Between Hafez and Valiasr St., Jomhuri Ave, Tehran

Phone: (+98) 2166726016; Fax: (+98) 2166709723; SMS: 10007615;

HSEE@NIOC.IR                                                                         HTTP://HSE.NIOC.IR