HSE Requirements of Oil/Gas Equipment during the Servicing Stage at On-Shore Sites

| Reviewing Translation | Confirmed by | Publication Date of English Version |
| --- | --- | --- |
| Davood Naderi/ Laleh H. Abbasi Training & Performance Measurement Department | Mani Abdollahzadeh Rad/ HSE Manager | September 2018 |

| Responsibility/Subject | Prepared by | Confirmed by | Approved by | Publication date |
| --- | --- | --- | --- | --- |
| Name | Hossein Mirderikvand Senior Specialist of passive defense & crisis management | Siavash Derafshi/ HSE Manager | Ali Kardor CEO | Agust 2017 |
| Position |  |  |  |  |

Revision No.: 0  Code: NIOC-HSE-OO-GU-040-00  Revision Date: 07/08/2017

Contents

1. Introduction .............................................................................................................................................. 3

2. Purpose...................................................................................................................................................... 3

3. Scope ........................................................................................................................................................ 4

4. Roles and responsibilities ......................................................................................................................... 4

5. Definitions and terms ............................................................................................................................... 4

5.1 Transporting the rig and rigging up ....................................................................................................... 4

5.2 General technical services ...................................................................................................................... 4

5.3 Specific technical services ..................................................................................................................... 4

5.4 Work-over .............................................................................................................................................. 5

6. HSE requirements of transporting the rig and rigging up ........................................................................ 5

7. HSE requirements of general technical services ...................................................................................... 6

8. HSE requirements of special technical services ...................................................................................... 6

9. HSE requirements of the work-over operations ....................................................................................... 8

10. Resources ............................................................................................................................................... 9

1. Introduction

After conducting various geological studies (e.g. seismography) and performing different tests on sedimentary stones, geologists can discover oil or gas resources in a specific location. The drilling operations will then begin taking into account the location of the site and all the factors effective in drilling. Drilling operations [using drilling equipment] in the oil and gas industry are among the most complicated, costly, cumbersome and high-risk occupations in the world. The oil and gas well drilling operations consist of various processes, including site preparation, drilling, well completion, servicing, and finally, well plugging and abandonment.

Drilling operations cause various environmental impacts that can contaminate the air, water and soil and may have some negative effects on human activity.

In the NIOC’s operational areas, drilling operations largely depend on the type of the drilling tools, machinery and equipment used, and, like other industrial environments, drilling sites are associated with various health, safety and environmental hazards. Therefore, if necessary measures are not taken to prevent potential hazards, there will be a high probability of damage to personnel, equipment and the environment. In this regard, the NIOC’s HSE management has established a working group composed of the authorities of the National Iranian South Oil Company, the National Iranian Drilling Company, the National Iranian Oil Company Exploration Directorate (NIOCEXP), the Iranian Offshore Oil Company and the North Drilling Company and has held expert meetings on preparation and compilation of HSE requirements/provisions for all drilling operations. This guideline presents the HSE requirements of oil/gas equipment during the servicing stage at on-shore sites.

Based on this document, all the NIOC’s subsidiaries and operational units, offering or receiving offshore services, shall prepare relevant working instructions [in accordance with the nature of their activities and their organizational structure] to comply with the provisions of this document (as minimum requirements) and shall implement it within a specified period.

2. Purpose

The purpose of this document is to identify HSE requirements of servicing operations and compliance with these requirements to prevent work-related accidents and to minimize relevant consequences and damages to the NIOC’s manpower, facilities and the environment.

- Transporting the rig and rigging up

- General technical services

- Special technical services

- Work-over

3. Scope

This guideline is applicable to all the NIOC’s subsidiaries, associates and operational sites that offer or receive offshore services.

4. Roles and responsibilities

The NIOC’s HSE Management is responsible for updating this guideline and monitoring its proper implementation across relevant units.

The Managing Directors of the NIOC’s subsidiaries and associates are responsible for monitoring the proper implementation of this guideline through preparing relevant working instructions and HSE units shall implement the guideline across their units, in accordance with the nature of their activities and their organizational structure.

5. Definitions and terms

5.1 Transporting the rig and rigging up

Transporting and rigging up all equipment required for the provision of technical services (e.g. general, special and maintenance services) in oil and gas wells.

5.2 General technical services

Services such as removing the wellhead and well bottom equipment, pulling and running the rod, pulling and running the tubing, installing liner hangers, etc. in oil and gas wells.

5.3 Specific technical services

Services such as well logging, cement injection, perforating, snubbing, acidizing, swabbing, underbalanced drilling (UBD), etc., provided in oil and gas wells using special equipment and tools:

- Snubbing:

In this process, special tools are used to insert drill pipe, tubing and specialized equipment into pressurized oil and gas wells, while ensuring that there is no need for killing the well.

- Swabbing:

This method (extraction method) is used when the reservoir rock does not provide sufficient pressure to push the fluid to the surface.

- Underbalanced drilling (UBD):

In this process, the hydrostatic pressure of the drilling fluid inside the well is kept lower than the reservoir pressure through injection of a fluid (nitrogen, etc.). As the well is being drilled, the reservoir fluid (oil/gas) flows into the wellbore and increases the wellhead pressure.

5.4 Work-over

Services such plug-back, bending and repairing the casing and liners, etc. in oil and gas wells with the aim of increasing production.

6. HSE requirements of transporting the rig and rigging up

6.1 Ensure the technical integrity and operability of all relevant transportation and handling machinery and equipment.

6.2 Obtain a rigging up permit to ensure the safety of the drilling rig and its components, before commencing the rigging up operation.

6.3 Interface permits shall be obtained in joint working areas before commencing the rigging up operation. In this regard, review the “Permit to Work System Guideline” (code: NIOC-HSE -SF-GU-010-01).

6.4 All potential work-related risks and hazards shall be identified and relevant control measures shall be taken.

6.5 To inform personnel of various work-related hazards and to establish necessary coordination between participating teams, safety meetings shall be held before commencing the work.

6.6 Regarding the proper use of PPE, see the “Guideline for Selection, Preparation and Use of PPE and Safety Equipment” (code: NIOC-HSE-EN-GU-014-00).

6.7 Ensure the competence of all machinery operators (cranes, lift trucks, Gin Poles, etc.). In this regard, review the “Guideline for Safe Operation of Lifting and Conveying Equipment” (code: NIOC-HSE-SF-GU-022-00).

6.8 Ensure the integrity of all cables, components and equipment, before commencing the loading operation. In this regard, review the “Guideline for Safe Operation of Lifting and Conveying Equipment” (code: NIOC-HSE-SF-GU-022-00).

6.9 To ensure safe load handling, comply with relevant lifting instructions and techniques. In this regard, review the “Guideline for Safe Operation of Lifting and Conveying Equipment” (code: NIOC-HSE-SF-GU-022-00).

6.10 Appropriate safety and warning signs should be provided and installed in different places to inform employees of potential hazards.

6.11 Provide Material Safety Data Sheet (MSDS) forms for all chemicals and ensure that all personnel fully observe relevant HSE considerations.

6.12 Ensure the safety and security of transportation, loading and unloading operations. In this regard, review the “Guideline of Hazardous Materials Road Transport” (No. H.22029/ T 44870, enacted on 18/03/2002 by the Iranian Board of Ministers).

6.13 Regarding other relevant HSE requirements, review the “HSE Requirements of Oil/Gas Equipment during the Drilling Stage at On-Shore Sites” (Code: NIOC-HSE-SF-PR-035-00) and the “HSE Requirements of Oil/Gas Equipment during the Site Preparation Stage at On-Shore Sites” (Code: NIOC-HSE-SF-PR-034-00).

7. HSE requirements of general technical services

7.1 Some activities require obtaining specific permits. In this regard, review the “Permit to Work System Guideline” (code: NIOC-HSE -SF-GU-010-01).

7.2 All process risks and hazards and potential work-related hazards shall be identified (using HAZOP study, etc.) and relevant control measures shall be taken.

7.3 To inform personnel of various work-related hazards and to establish necessary coordination between participating teams, safety meetings shall be held before commencing the work.

7.4 To ensure safe load handling, comply with relevant lifting instructions and techniques. In this regard, review the “Guideline for Safe Operation of Lifting and Conveying Equipment” (code: NIOC-HSE-SF-GU-022-00).

7.5 Ensure observance of all relevant environmental considerations to prevent potential contaminations. In this regard, review the “Guideline for Compliance with Environmental Considerations in Exploration and Drilling Operations” (Code: NIOC-HSE-EN-GU-015-00) and the “Waste Management Guideline” (Code: NIOC-HSE-EN-GU-006-00).

7.6 Ensure observance of all safety regulations on the drilling platform by all technical service staff. Regarding the proper use of PPE, see the “Guideline for Selection, Preparation and Use of PPE and Safety Equipment” (code: NIOC-HSE-EN-GU-014-00).

7.7 Ensure the provision of adequate lighting overnight.

8. HSE requirements of special technical services

8.1 All process risks and hazards and potential work-related hazards shall be identified (using HAZOP study, etc.) and relevant control measures shall be taken.

8.2 To inform personnel of various work-related hazards and to establish necessary coordination between participating teams, safety meetings shall be held before commencing the work.

8.3 Ensure observance of all safety regulations on the drilling platform by all technical service staff.

8.4 Regarding the proper use of PPE, see the “Guideline for Selection, Preparation and Use of PPE and Safety Equipment” (code: NIOC-HSE-EN-GU-014-00).

8.5 Keep unauthorized persons away, when providing pressurized services (e.g. cementing, acidizing, nitrogen injection, UBD, etc.).

8.6 Secure all lines before testing lines and prior to pressurized operations (e.g. cementing, acidizing, nitrogen injection, UBD, etc.).

8.7 Ensure observance of all relevant environmental considerations to prevent potential contaminations during cementing, acidizing, etc. In this regard, review the “Guideline for Compliance with Environmental Considerations in Exploration and Drilling Operations” (Code: NIOC-HSE-EN-GU-015-00) and the “Waste Management Guideline” (Code: NIOC-HSE-EN-GU-006-00).

8.8 Before installing the casing and liners, ensure proper installation and safety of the stabbing board.

8.9 Avoid performing pressurized acidizing overnight.

8.10 Ensure proper disposal of used acids, liquids and solids following the acidizing operation.   In this regard, review the “Guideline for Compliance with Environmental Considerations in Exploration and Drilling Operations” (Code: NIOC-HSE-EN-GU-015-00) and the “Waste Management Guideline” (Code: NIOC-HSE-EN-GU-006-00).

8.11 Emergency shower and eyewash stations should be established in accordance with relevant MSDS forms.

8.12 Ensure the safe transportation and handling of explosives, when performing perforating operation. In this regard, review the API RP 54, the perforating section.

8.13 Assemble the perforating gun in a safe place (away from electrical sources and equipment, etc.). In addition, safety and warning signs should be installed around this place.

8.14 The perforating gun shall be used (assembled and disassembled) during the day. All unauthorized personnel shall stay away from the perforating gun, explosives and hazardous devices. In this regard, review the API RP 67.

8.15 To prevent potential hazards (explosions, etc.) associated with radio frequencies, radars and communication devices, ensure that all radio devices are turned off during the perforating operation.

8.16 Keep unauthorized persons away, when performing perforating operations.

8.17 Ensure safe transportation and handling of radioactive materials during line-wire operations (well logging, etc.).

8.18 Keep unauthorized persons away, when performing line-wire operations.

8.19 Secure radioactive sources remaining from the well logging operations. In this regard, review regulations specified by the Atomic Energy Organization of Iran.

8.20 Avoid performing swabbing activities overnight.

8.21 Install the swabbing unit in the wind direction.

8.22 Ensure the proper function of fire alarm and extinguishing systems.

8.23 Ensure the proper installation and function of detectors, portable and fixed gas meters and visual and auditory alarms, especially around the wells.

8.24 Stop the operation and secure the well immediately, in the event of release of hydrocarbon substances in the vicinity of the wells.

8.25 In all pressurized operations (snubbing, coild tubing, etc.), all tubing strings shall be equipped with check valves, high pressure switches, etc. installed on the bottom of the strings.

8.26 Create emergency escape paths for those working on the top of hydraulic snubbing equipment.

9. HSE requirements of the work-over operations

9.1 Interface permits shall be obtained in joint working areas before commencing the rigging up operation. In this regard, review the “Permit to Work System Guideline” (code: NIOC-HSE -SF-GU-010-01).

9.2 All process risks and hazards and potential work-related hazards shall be identified (using HAZOP study, etc.) and relevant control measures shall be taken.

9.3 To inform personnel of various work-related hazards and to establish necessary coordination between participating teams, safety meetings shall be held before commencing work-over operations.

9.4 Ensure the competence of all machinery operators (cranes, lift trucks, Gin Poles, etc.). In this regard, review the “Guideline for Safe Operation of Lifting and Conveying Equipment” (code: NIOC-HSE-SF-GU-022-00).

9.5 Ensure the integrity of all cables, components and equipment, before commencing the loading operation. In this regard, review the “Guideline for Safe Operation of Lifting and Conveying Equipment” (code: NIOC-HSE-SF-GU-022-00).

9.6 To ensure safe load handling, comply with relevant lifting instructions and techniques. In this regard, review the “Guideline for Safe Operation of Lifting and Conveying Equipment” (code: NIOC-HSE-SF-GU-022-00).

9.7 All personnel shall wear suitable PPE at work. Regarding the proper use of PPE, see the “Guideline for Selection, Preparation and Use of PPE and Safety Equipment” (code: NIOC-HSE-EN-GU-014-00).

9.8 Appropriate safety and warning signs should be provided and installed in different places to inform employees of potential hazards.

9.9 Provide Material Safety Data Sheet (MSDS) forms for all chemicals and ensure that all personnel fully observe relevant HSE considerations.

9.10 Relevant environmental considerations shall be observed regarding the production of wastes at this stage. In this regard, review the “Guideline for Compliance with Environmental Considerations in Exploration and Drilling Operations” (Code: NIOC-HSE-EN-GU-015-00) and the “Waste Management Guideline” (Code: NIOC-HSE-EN-GU-006-00).

9.11 Ensure the provision of adequate lighting overnight.

10. Resources

- Occupational Safety & Health (OSHA): Oil and Gas Well Drilling and Servicing eTool.

- API RP 54: Recommended Practice for Occupational Safety for Oil and Gas Well Drilling and Servicing Operations.

- API RP 67: Recommended Practice for Oilfield Explosives Safety.

- Permit to Work System Guideline (code: NIOC-HSE -SF-GU-010-01).

- Guideline for Selection, Preparation and Use of PPE and Safety Equipment (code: NIOC-HSE-EN-GU-014-00).

- Guideline for Safe Operation of Lifting and Conveying Equipment (code: NIOC-HSE-SF-GU-022-00).

- Waste Management Guideline (Code: NIOC-HSE-EN-GU-006-00).

- Guideline for Compliance with Environmental Considerations in Exploration and Drilling Operations (Code: NIOC-HSE-EN-GU-015-00).

- HSE Requirements of Oil/Gas Equipment during the Site Preparation Stage at On-Shore Sites (Code: NIOC-HSE-SF-PR-034-00).

- HSE Requirements of Oil/Gas Equipment during the Drilling Stage at On-Shore Sites (Code: NIOC-HSE-SF-PR-035-00).

No. 17, Yaghma Alley, Between Hafez and Valiasr St., Jomhuri Ave, Tehran

Phone: (+98) 2166726016; Fax: (+98) 2166709723; SMS: 10007615;

HSEE@NIOC.IR                                                                         HTTP://HSE.NIOC.IR