Guideline for Performing Safe Pigging Operations

| Reviewing Translation | Confirmed by | Publication Date of English Version |
| --- | --- | --- |
| Davood Naderi/ Laleh H. Abbasi Training & Performance Measurement Department | Mani Abdollahzadeh Rad/ HSE Manager | September 2018 |

| Responsibility/Subject | Prepared by | Confirmed by | Approved by | Publication date |
| --- | --- | --- | --- | --- |
| Name | Mani Abdolahzadeh Head of Safety & Firefighting Department | Siavash Derafshi/ HSE Manager | Ali Kardor CEO | Agust 2017 |
| Position |  |  |  |  |

Revision No.: 0  Code: NIOC-HSE-SF-GU-039-00  Revision Date: 02/08/2017

Contents

1. Introduction .............................................................................................................................................. 3

2. Purpose...................................................................................................................................................... 3

3. Scope ........................................................................................................................................................ 3

4. Definitions and terms ........................................................................................................................... 3

5. Roles and responsibilities .................................................................................................................... 5

5.1 Managing directors …......................................................................................................................... 5

5.2 The Operation and R&M Departments ............................................................................................ 5

5.3 Technical Inspection Department ..................................................................................................... 6

5.4 HSE Department ................................................................................................................................. 7

5.5 Security Department ........................................................................................................................... 7

6. Specific safety requirements of pigging operations.......................................................................... 7

7. Resources ............................................................................................................................................. 12

1. Introduction

Pigging operations are among the major sources of threat to national capital and the main causes of major pipeline accidents, fires and oil spillages across operational units and process facilities, in cases where safety requirements are ignored. Due to the corrosive nature of oil and the depreciation of the NIOC’s facilities and pipelines, a significant increase has been observed in the number of pipeline accidents in recent years. In this regard, the explosion and fire occurred at the slug catcher of Binak’s 24-inch pipelines is an example.

Due to the high sensitivity of pigging operations, as one of the most important, high-risk operations in the oil and gas industry, and due to the lack of an integrated policy on pigging procedures across the NIOC (subsidiary and associate companies adopt different pigging procedures), necessary measures shall be taken and special rules should be observed to properly implement these operations.

Therefore, to protect the health of the manpower and equipment and to maintain the integrity of the integrated asset and HSE management systems, this document entitled “Guidelines for Performing Safe Pigging Operations” has been developed in accordance with the standards and rules of the petroleum industry, by the NIOC’s HSE Management. The NIOC’s subsidiaries and operating companies shall develop “Safety Instructions for Pigging Operations” in accordance with this guideline to institutionalize the requirements of the implementation of this document across their company. Companies shall compile this document with regard to the type and the nature of their activities.

It should be noted that the requirements mentioned in this document are only minimum requirements employed to maintain the safety of all personnel, facilities, properties, neighbors and stakeholders, and all the NIOC’s subsidiaries and contracting companies shall comply with its provisions.

2. Purpose

This guideline has been developed with the aim of outlining safety requirements of pigging operations across the NIOC, coordinating relevant units and predicting necessary measures to ensure proper and safe operation, reducing HSE risks and preventing accidents and damages to individuals, facilities and property.

3. Scope

This guideline is applicable to all subsidiaries, associates, managements and operational areas of the NIOC.

4. Definitions and terms

- Company: The National Iranian Oil Company

- Subsidiaries and associates: Companies operating directly under the NIOC

- Employer: As defined in the relevant contract

- Contractor: All legal/real persons who have signed contracts with the subsidiaries of the NIOC

- Hazard: An agent with the potential to cause harm or damage

- Pigging operations: A process used to remove solids and liquids from pipelines or to examine them in terms of corrosion. In this guideline, the term “operation” refers to pigging operation.

- Direct presence: Being aware of the operation and being present in sensitive conditions and operations such as opening the launcher’s cap, installing the pig and closing the cap, preparing the system for starting the operation and air removal, opening the receiver’s cap, taking it out of the pig trap at the end of the operation, closing the receiver’s cap and complete evacuation of air from the system and being present to perform relevant assigned tasks.

- Permanent presence: Being present from the beginning to the end of the operation, no permission to leave the operational area and to engage in unrelated operations, performing relevant tasks in line with the pigging operation.

- Person in charge of the work: The supervisor of employees (official, contracting and the third-party employees) who are involved in the operation. This person must be present at the site throughout the operation and must monitor and supervise their behavior in the work area. This person will be allowed to issue work permits after undergoing relevant training courses held by the HSE Unit. Their responsibilities and working scope is specified by relevant competent authorities following the approval of the Head of the HSE Department and the Pigging Operation Committee. Persons in charge of the work must have a direct, effective and permanent presence.

- Site manager/managers: Person/persons who manage the operational area and supervise the undergoing process. In operational facilities, heads of units or their representatives are often considered as site managers. This person will be allowed to issue work permits after undergoing relevant training courses held by the HSE Unit. Their responsibilities and working scope is specified by relevant competent authorities following the approval of the Head of the HSE Department and the Pigging Operation Committee. Site managers must have a direct, effective and permanent presence.

- HSE Representative: This person is directly appointed by the HSE Unit to supervise the operation and to assess compliance with the provisions of this guideline and other relevant HSE requirements.

- Blow-down valve: A valve used to discharge gases into the atmosphere.

- Pig receiver: This device is installed on pipelines to receive pipeline pigs.

- Pig launcher: This device is installed on pipelines to launch pipeline pigs.

- Pig: This device is launched into pipelines for various purposes.

- Cups: Rubber pieces placed on both sides of the pig to seal the pipeline.

- Pig signaler: This mechanical/mechanical-electrical unit is used to indicate the passage of pigging tools or spheres.

- Disc: This tool is used for cleaning the pipeline in both directions.

- Brush: This tool is used for removing hard deposits such as rust.

- Springs: Springs are used to push scarper blades and brushes against the pipe.

- Pig body

- Blades: Tools used for removing bulky and soft materials, as well as, clinging and depositing materials.

- Spontaneous combustible materials: These materials react immediately when come into contact with oxidizing materials (O2). This may lead to self-burning, inflammation and explosion (e.g. iron sulfide).

5. Roles and responsibilities

To identify all safety hazards associated with pipeline operations and to safely perform pigging operations, all relevant subsidiaries and associates of the NIOC shall interact with each other in accordance with their roles and responsibilities specified in the following paragraphs.

5.1 Managing directors

1. Ensuring the provision and allocation of required financial resources and provision of necessary facilities and resources in line with safety hazards in pipeline operations and in accordance with available standards.

2. Ensuring the proper implementation of the requirements of this guideline.

5.2 The Operation and R&M Departments

1. Establishing a visible and understandable framework, in which the roles and responsibilities of all personnel is clearly defined.

2. Performing pigging operations in accordance with the proposed and approved method.

3. Taking all preventive measures provided by the HSE Unit and examining the competence of all persons involved in the operation with regard to their records, training courses and physical health.

4. Holding toolbox meetings for at least half an hour, before commencing the operations at the presence of the HSE representative, and reviewing tasks, duties and work hazards.

5. Obtaining necessary permits, such as pigging permit, hot work permit, confined space permit, and other permits required in accordance with relevant instructions.

6. Ensuring the establishment of needed coordination between operational areas and relevant authorities. It should be noted that such arrangements must be clearly made before, during and after the pigging operation.

7. Informing all subsidiary companies working in the area to establish their repair teams at specific locations (stations) and introducing the site manager to sign relevant permits, if launcher and receiver are installed in their working area.

8. Ensuring the establishment of a suitable communication network (e.g. wireless network) throughout the pigging rout and taking necessary actions to use a temporary local telecommunications network, in the absence of such networks and conducting risk assessments during pigging operations and monitoring proper implementation of all safety procedures by all personnel involved in the operations.

9. Introducing all relevant individuals to the HSE Unit to examine their eligibility and competence.

10. Records of the most recent repairs and inspections (e.g. receiver/launcher valves and other valves illustrated in the station’s map) should be completed and approved by repair centers of all operational areas.

11. Holding a meeting at the presence of all people involved in the operation, including repairers, drivers, etc. one day before the operation to discuss all critical matters.

12. Checking communication systems and ensuring their proper function.

13. Taking all preventive measures provided by the HSE Unit on the basis of risk assessments and in accordance with specified job descriptions.

14. The type of operations, locations and potential risks should be communicated to the HSE Departments of Operating Companies, before commencing the operation.

5.3 Technical Inspection Department

1. Controlling and supervising pigging operations in accordance with the proposed and approved method.

2. Checking and confirming technical certifications of all machinery and examining their technical operability.

3. Ensuring the technical operability of all control systems (e.g. automatic valves, instrumentation, etc.).

5.4 HSE Department

1. Supervising the overall pigging process.

2. Predicting and taking necessary measures in the event of emergencies (e.g. providing emergency relief supplies, installing warning signs and informing relevant departments, including the fire and emergency departments about the time and location of the operation). In addition, adjacent residential buildings should be informed of the time and place of hazardous operations, as well as all operational risks.

3. Ensuring the availability of necessary protective devices and equipment (e.g. gas cartridge powder extinguishers, warning tapes, suitable spotlights, etc.).

4. Planning and performing emergency drills. The scenario will be either pipeline rupture and the spillage of oil into the river (if possible) or fire in the vicinity of a residential local area.

5.5 Security Department

1. Coordination and interaction with all departments involved in the pigging operation.

2. Reporting suspicious cases to the HSE Department

5.6 Clinic/Healthcare Department

1. Interacting with all departments involved in the pigging operation and planning to deploy the required relief and rescue equipment.

2. Interacting with the nearest treatment centers at the vicinity of pipeline facilities in emergency situations.

6. Specific safety requirements of pigging operations

1. Forming the Pigging Operation Committee and determining key personnel in each department (Operation, Technical Services, HSE, R&M and Security Departments, etc.) to undertake various tasks and holding coordination and briefing meetings before commencing the operation and completing pigging minutes and reports approved by all members.

2. Monitoring and controlling the approvals of the Pigging Operation Committee: When the Pigging Operation Committee informs relevant units of the implementation of pigging operations, a group, entitled “Pigging Committee”, composed of the person in charge of the work, the site manager, automated valve manager and the HSE representative will be formed at least two weeks before the operation. This committee will inspect the launcher and receiver facilities, their fittings and their access routes and will control various items (e.g. the status of the launcher/the status of the receiver/the drain and kicker valves, the main valve between the transfer line and the trap, the bypass valve between the transfer line and the trap and bleeding valves/the status of valve position indicators/controlling the maintenance of valves on the basis of relevant reports/the position of valves inside the ponds/the status of valves and valve position indicators/the blow-down valve/the drain pit/all pressure gauges/the pig signaler/the presence or absence of Device Lock/the status of LBV within the scope of the operation, etc.) on the basis of a technical inspection checklist. Within this process, they will complete a checklist provided by the Pigging Operation Committee. Following these inspections, the committee will prepare the list of deficiencies. The relevant department will submit the list to the person in charge of the work and after solving all problems, the pigging permit will be issued.

3. Examining the competence of all persons involved in the operation and the relevance between their skills and their assigned tasks (guidelines should be developed for examining the competence of all executive and management posts on the basis of education, training courses, experience, performance evaluation, etc.).

4. Allocating appropriate manpower, material resources and equipment to the pigging operation.

5. Determining a mechanism for the interaction of different units (e.g. Technical Services and HSE Units), and for inter-organizational relationships.

6. Identification and assessment of operational risks (safety risks, corrosion risks, etc.) based on the existing Risk Assessment and Management Procedures (all the risks that can potentially damage individuals, the environment and property should be systematically identified and assessed, and control measures should be taken before commencing the operation to protect the health of the manpower and to preserve available resources and the environment).

7. Providing instructions for various tasks associated with pigging operations. Examples include: “HSE considerations in pigging operations”, “instructions for entering into scrubbers”, “instructions for the scrubber’s header”, “instructions for the launcher and receiver”, “instructions for discharging the air and injecting gas into the launcher and receiver tank to place or remove the pig”, “instructions for the re-commissioning of pipelines”, “instructions for inspecting pipeline equipment and fittings” (before commencing the work, periodically, etc.) and “instructions for performing periodic inspections”.

8. Preparing and implementing pipeline pre-commissioning instructions in accordance with the PSSR Guideline (code: NIOC-HSE-SF-GU-002-01) provided by the NIOC’s HSE Management and Passive Defense Department before any start-up operation.

9. Carrying out periodic technical inspections and establishment of a regular preventive maintenance system to ensure the health of the pipeline facilities.

10. Proper establishment and implementation of a permit to work system, in accordance with the NIOC’s Permit to Work System Guideline (code: NIOC-HSE -SF-GU-010-00) and the need for effective supervision over pigging permits issued by various units and departments.

11. Preparing technical ID cards on the specifications of pipelines and related facilities to determine the status of corrosions, the amount of used inhibitors, determining locations for sampling and for monitoring internal corrosions, the location of cathodic protection stations, etc. These technical ID cards must at least contain the following items:

(Pigging timetable/main pipeline information on the basis of relevant forms/operating conditions and the range of changes in fluid flow conditions in the pipeline during the past two weeks and those of the previous year within the same period/as built drawings of the launcher and receiver/instructions provided by manufacturers on launcher and receiver and other pigging equipment and devices/technical ID cards of launchers and receivers/documents of the last inspection conducted on launcher and receiver facilities and previous approvals in order to consider all potential hazards/documents of the last periodic inspection conducted on check valves/documents of previous pigging operations/documents of periodic lubrications/detailed specifications of the used pig/verification of the Technical Inspection Department regarding the functionality of fixed and portable lifting equipment/verification of the Technical Inspection Department regarding the functionality of the trap caps and other launcher and receiver equipment, such as the trap body, fittings, pressure gauges, pig markers, holders and all the information of instruments used in the operation for opening and closing the traps and valves and for other activities).

12. Performing appropriate training need assessments (to promote occupational skills of relevant personnel) and planning for evaluating the knowledge and technical competence of relevant personnel before performing assigned tasks.

13. Implementing the Crisis Management Plan (CMP) and the Emergency Response Plan (ERP) and reviewing them by taking into account the joint work area and conducting relevant drills and exercises (possible pipeline emergencies include: pipeline leakage or rupture and environmental pollution/H2S leakage and fire and explosion due to pipeline leakage or rupture).

14. Any changes in pipeline equipment (e.g. eliminating the slug catcher, etc.) should be made in accordance with the Management of Change (MOC) Guideline provided by the NIOC’s for the HSE Management and must be approved by the HSE Department of the respective company.

15. Sending the contractor’s HSE personnel to high-risk areas for strict supervision.

16. Installing warning signs in accordance with the type of hazards of each activity.

17. Ensuring the safety and operability of all equipment before commencing the pigging operation (e.g. operability of the emergency shut-off valves, etc.).

18. Deployment of fire fighting vehicles, fire extinguishing equipment and other necessary safety equipment during the pigging operation (e.g. explosion-proof lights for night work, suitable gas-meters to measure LEL percentage, flammable gases and toxic gases such as H2S in ppm and portable or fixed vanes).

19. Availability of suitable communication devices, such as explosion-proof wireless LAN.

20. Supplying and ensuring the availability of necessary relief and rescue equipment (such as ambulances and respiratory equipment for entering into launcher and receiver boxes. Wheeled respiratory systems, equipped with Air Line, should be used and air-supplying devices should be located outside the box. Backpack respiratory equipment can be used only following the approval of the HSE Department).

21. Preparing instructions for the transportation of injured persons to nearby hospitals during the pigging operation (arrangements should be made for permanent presence of a medical assistant or a trained person, when launching or receiving the pig).

22. Supplying necessary PPE (e.g. safety boots, flame-retardant overalls, earplugs/earmuffs, hard hats, face shields and safety goggles) for all persons involved in the pigging operation.

23. All fixed and portable lifting equipment must have up-to-date technical certificates (in accordance with the guideline provided by the NIOC’s HSE Management).

24. Industrial medicine examinations should be periodically conducted to confirm the health of all personnel involved in the pigging operation. Such examinations and tests should be confirmed by the HSE Unit.

25. Following the reconstruction of damaged parts of the pipeline, air bleeding and pressure boosting operations should be performed in accordance with relevant safety instructions and by taking all safety precautions.

26. Before commencing the operation, the pig vane shall be installed at a suitable place and the prevailing wind direction shall be specified at various places (e.g. at launcher and receiver stations) during the operation, and emergency escape routes shall be determined and communicated accordingly.

27. Special safety measures shall be taken to prevent spontaneous fires and explosions, when handling flammable hydrocarbon substances. Therefore, after discharging pipelines, they should be kept wet to prevent the explosion of flammable deposits (e.g. iron sulfide).

28. Wipe out all material remained on pigs, before moving them.

29. The static electricity is a major source of ignition; therefore, the person in charge of the work should take necessary actions (e.g. establishment of an earthing system) to prevent the accumulation of static electricity, especially at high altitudes and cloudy weather.

30. Pigging and air bleeding operations must be performed during the day and under desirable weather conditions (non-stormy weather conditions with no lightning).

31. Generated wastes should be handled in accordance with relevant regulations and instructions.

32. Avoid unnecessary stops within a 50-meter distance from the launcher/receiver and the drain pit.

33. Never discharge gas into the drain pit to move the pig from far distances toward the receiver station.

34. The drain system is only designed to discharge fluids into the path. It is prohibited to use the drain valve for other purposes.

35. Use only the vent valve for discharging gas or for reducing launcher’s/receiver’s pressure.

36. The vent path should not lead to the drain pit.

37. Take samples from the contents of the receiver for future analyses (if necessary).

38. Automatic control valves must be checked by an operator, when launching or receiving a pig.

39. All valves, especially the receiver’s valves, should be lubricated at the end of each pigging operation.

40. At the cleaning stage, if the pipeline contains sour gas, then, the use of respiratory equipment and personal gas meters will be essential for all personnel.

41. A pig may get stuck in the pipe; thus, increasing the fluid pressure is the best way to drive the pig forward. Before starting the operation, the maximum permissible pipeline pressure must be specified. Increasing the pressure above this value is strictly prohibited.

42. In cases where increasing pressure does not help, other options should be chosen. Consult with the Operation Unit and the Head of the HSE Department before making any new decision.

43. Any unexpected action must be taken after obtaining the approval of the Head of the HSE Department.

7. Resources

1. IPS-M-PI-70: Material and Equipment standard for pigs

No. 17, Yaghma Alley, Between Hafez and Valiasr St., Jomhuri Ave, Tehran

Phone: (+98) 2166726016; Fax: (+98) 2166709723; SMS: 10007615;

HSEE@NIOC.IR                                                                         HTTP://HSE.NIOC.IR