National Iranian Oil Company Safety, Health and Environment Management Personnel Protection Equipment (PPE) and Safety Guideline Reviewing Translation; Confirmed by; Publication Date of English Version. Davood Naderi/ Laleh H. Abbasi Training & Performance Measurement Department; Mani Abdollahzadeh Rad/ HSE Manager; September 2018. Responsibility/Subject; Prepared by; Confirmed by; Approved by; Publication date; Descriptions. Name; Mani Abdollahzadeh/ Fereshte Miraj Head of Safety and Firefighting Department; Siavash Derafshi/ HSE Manager; Ali Kardor/ CEO; June 2016. Position. Document Code NIOC-HSE-SF-GU-014-00 Revision date: June 21, 2016, Revision No.: 0

Table of Contents

1 Introduction	3

2 Goal	4

3 Scope	4

4 Definitions	4

5 Legal Requirements	7

6 Roles and Responsibilities	7

6.1	Chief Executives	7

6.2	Departments of Commerce and Logistics	7

6.3	Department Directors and Unit Supervisors	8

6.4	HSE Department	8

6.5	Security Office	9

6.6	Employees	9

7 Legal Requirements	9

8 Personal Protective Equipment Replacement Interval	11

9 Personal Protective Equipment Use and Maintenance Training	13

10 Personal Protective Equipment Usage Supervision	13

11 Personal Protective Equipment Evaluation	14

12	Special Requirements	14

12.1	Head Protective Equipment	14

12.2	Eye and Face Protective Equipment	18

12.3	Hearing Protective Equipment	23

12.4	Respiratory Protective Equipment	25

12.5 Hand and Arm Protective Equipment	36

12.6 Leg Protective Equipment	40

12.7 Body Protective Equipment (Workwear)	43

12.8 Personal Protective Equipment for Working at Height (Harness)	50

12.9 Special Protective Equipment	52

# 1 Introduction

Given the numerous harmful factors in workplaces in the oil industry, the use of Personal Protective Equipment (PPE), as well as engineering controls (including the adoption of low-danger equipment and procedures, using low-danger materials, isolating the hazardous procures, etc.) and management controls (reducing the number of employees exposed to hazards, changing the working hours, etc.), as a supplement and the last solution for creating a safe work environment, are inevitable.  Hence, it must be borne in mind that PPE is not a replacement for engineering and management controls, nor preventive measures. The equipment (including protection for eye and face, head, respiratory system, ears, body legs, etc.) only protects the users against hazards.

Therefore, to protect the personnel, as the most important resource of the National Iranian Oil Company (NIOC), the management of Health, Safety, and Environment (HSE) Department of the NIOC prepared the “Guidelines for Selecting, Procuring, Distributing, Using, and Maintaining Personal Protection and Safety Equipment”.

The subsidiary executives and operating companies of the NIOC are obligated to codify the institutionalization procedure and the requirements for the implementation of the said document in their respective companies in the form of "Instructions for Selecting, Procuring, Distributing, Using, and Maintaining Personal Protective and Safety Equipment" according to these guidelines.

All contractors and subsidiaries of the NIOC are hereby notified that the implementation of the requirements of these guidelines is necessary.

# 2 Goal

The aim of preparing these guidelines was to explain the requirements regarding the selection, procurement, distribution, usage, and maintenance of the PPE in order to reduce the exposure of employees to workplace hazards.

# 3 Scope

This guideline is irrevocable in all subsidiary companies, departments, and the operation fields of the NIOC.

# 4 Definitions

- Company: In this document, Company refers to the National Iranian Oil Company (NIOC).

- Subsidiary Companies (Subsidiaries): Includes all subsidiaries of the National Iranian Oil Company (NIOC).

- Employer: In this document, the Employer of each contractor is the one specified in the corresponding contract.

- Contractor: Any contractor that has a contract with any subsidiary of the National Iranian Oil Company (NIOC).

- Contractor's Personnel: All personnel working under the supervision of a contractor employed by the National Iranian Oil Company (NIOC) or any of its subsidiaries.

- Personal Protective Equipment (PPE): Equipment used by a person to prevent direct exposure to workplace hazards and adverse conditions and to reduce their impact.

- Safety Helmet: A device protecting the wearer’s head against damages.

- Eye Protection: A device protecting the wearer's eyes against physical impact, illumination (of a wavelength between 0.1 to 100 micrometers), molten metals, hot objects, liquid droplets, splash, dust particles, and gases.

- Sunglasses: Eyewear for protection against harmful solar radiations (Ultra Violate, Infra-Red, etc.). In addition to eliminating 100% of the UV radiations, sunglasses are also expected to protect the eyes against intense illumination and glare.

- Optical Filter: A type of lens used to reduce the intensity of the illumination in a certain range of wavelengths.

- Grading: The transmission properties of optical filters are specified by their grades. The grade is a combination of a numerical code and the shade number that are separated by a hyphen. The numerical code represents the type of Filter.

- Welding Masks: are specifically designed to protect wearer’s eyes and face against the dangerous radiations produced by the arc welding and to reduce the UV and IR radiations.

- Safety Face Shields: Face shields cover the wearer's face fully, offering a high degree of safety. This type of shield must be designed to withstand the impact of a 0.86 gr, 6 mm steel ball at 190 m/s.

- Respirator Masks): these masks filter out the pollutants present in the workplace, preventing them to enter the wearer’s lungs.

Different types of respirator masks include:

1. Full-face mask, which covers the whole face including the eyes, the nose, the mouth, and the chin.

2. Half-face mask, which covers the mouth, the nose, and possibly the chin.

- Filter: Filters are parts of the respirator masks that can be changed and are intended for filtering the ambient air. Various types of filters are available depending on the type of pollutants,

- Hearing Protector: A device used to protect the wearer's ear against noises above the allowable limit. Various types of hearing protection equipment include:

1. Earmuffs: hearing protection devices that cover the auricle, preventing the exposure of the wearer’s ears to noises above the allowable limit.

2. Earplugs: hearing protection devices that are placed inside the wearer's ear, preventing the exposure of the eardrums and the inner ear to noises above the allowable limit by blocking the ear canals.

- Workwear: Suitable items of clothing worn by workers to prevent exposure to workplace hazards.

- Apron: A type of cover that is worn over the workwear and is selected appropriately for the workplace hazards.

- Safety Gloves and Arm Protection: These devices are available in different types that protect the hands and arms from fingers up to shoulders against workplace hazards.

- Safety Shoes: Shoes worn to protect the legs against damaging agents. 

- Gaiter: An item of clothing used to cover the unprotected area between the lower pants and over the boots. Gaiters are available in different sizes and materials based on the usage.

- Personal Protective Equipment for Working at Height: These pieces of equipment are used for fall-arrest purposes and help reduce the damages associated with the fall of workers and their accessories from a height which include full-body harness, carabiner, and different lanyards (operational, dynamic, static, and support).

# Legal Requirements

5.1 Safety Regulations of the National Iranian Oil Company, 1975.

5.2 Occupational health and safety regulations of the National Iranian Oil Company.

5.3 The PPE code no. 19456 approved by the Supreme Council for Occupational Safety and the Minister of Labor and Social Affairs on June 11, 2011.

5.4 All guidelines, instructions, and procedures communicated by the National Iranian Oil Company.

# Roles and Responsibilities

## 6.1	Chief Executives

1. Ensuring that sufficient financial resources are allocated to the procurement of standard personal protective and safety equipment commensurate with the workplace hazards.

2. Ensuring that the requirements of this guideline are implemented.

## 6.2	Departments of Commerce and Logistics

1. Buying all personal protective and safety equipment according to the specifications of this guideline approved by the HSE department.

2. Filling the PPE purchase request forms by mentioning the standards of the said items in written according to the requirements of this guideline.

3. Note: Request forms that do not specify the required standards shall be returned to the requester and the Department of Procurement is not permitted to proceed to purchase based on such requests.

4. Providing the head of HSE with the list of personal protective and safety equipment according to a periodic, specifically-defined schedule and the company’s inventory methods.

## 6.3	Department Directors and Unit Supervisors

1. Informing the HSE department about the quality of the procured personal protective and safety equipment after they were used by the workers for the purpose of continuous improvement.

2. Ensuring that the workers do not use the personal protective and safety equipment belonging to another person.

3. Supervising the correct use of the new equipment by the workers (the workers must use new equipment and they are forbidden from re-using second-hand equipment).

4. Ensuring that the workers are using the necessary personal protective and safety equipment during the working hours.

## 6.4	HSE Department

1. Preparing operating instructions for the personal protective and safety equipment according to these guidelines.

2. Supervising the good implementation of the requirements of this guideline by the respective units through periodic inspections.

3. Reporting the results of evaluating and analyzing periodic inspections about the quality and state of the purchased personal protective and safety equipment used by the workers, for the purpose of continuous improvement.

4. Supervising the purchase, use, and maintenance of the personal protective and safety equipment and training the workers (about using the equipment, the workplace hazards, and the requirements of this guideline).

5. Maintaining an updated list of the PPE inventory.

6. Offering the necessary instructions to the directors on how to choose the right PPE items for their respective employees.

7. The HSE department is in charge of deciding on special PPE (such as sunglasses, raincoats, warm clothes, etc.) for special occupations (such as surveyors, etc.).

## 6.5	Security Office

Cooperating with the HSE department about the workers’ use of PPE.

## 6.6	Employees

1. The employees are required to use and maintain the personal protective and safety equipment provided to them according to this guideline and the respective instructions inside the company.

2. The employees must inform their supervisors of any defects in their equipment and provide them with feedback about the quality of the PPE.

# Legal Requirements

Clause 1. All NIOC subsidiaries are obligated to observe all the items listed in this guideline for selection, procurement, usage, and distribution of PPE.

Clause 2. In addition to protecting the users from workplace hazards, the purchased PPE shall not expose the workers, nor the environment, to additional hazards.

Clause 3. The procedure for the verification and purchase of PPE must be carried out according to the contents of this guideline as well as the list of the approved producers and suppliers of NIOC (Vendor List).

Clause 4. In addition to the employer’s requirements, the labor laws must be complied with for the workers to whom the law applies (contract employees, and limited-term employees). The employers and respective authorities are obligated to procure and provide their employees with the PPE necessary for their health and safety in the workplace, instruct them on how to use the said equipment, and supervise the correct use and maintenance of the equipment.

Clause 5. The employer is obligated to take the necessary measures about providing the project interns and drafted employees with the PPE necessary for the job assigned to them with the approval of the HSE department.

The said employees are also required to use and maintain the PPE they are provided with according to this guideline and the respective instructions inside the company.

Clause 6. The contractors employed by the NIOC or its subsidiaries are obligated to comply with the requirements of this guideline and implement them in their respective workplace according to HSE instructions.

Clause 7. The employees who are not able to use PPE must be introduced to the department of occupational medicine for examination in coordination with the HSE department and through the Department of Human Resource Development. Necessary decisions shall be made by the Human Resource Management, under the supervision of the HSE department, about changing the said employee’s job or their working conditions based on the examination results and occupational medicine’s advice.

Clause 8. Occasional and first-time visitors shall use disposable equipment. Re-washable and disinfectable equipment must be cleaned and disinfected after every single use.

Clause 9. The workers shall not take the new provided PPE out of the workplace, or give them to someone else.

Clause 10. All subsidiaries of the company are obligated to adhere to the requirements of this guideline regarding the choice of helmet and workwear color.

Clause 11. The HSE department is responsible for deciding on revising the distribution schedule and adjusting the replacement time in case of new requests for PPE out of the scheduled time.

Clause 12. Non-conventional use of personal protective and safety equipment is against the contents and requirements of this guideline.

Clause 13. The distributor is expected to take a receipt containing the name, surname, staff ID number, and signature of the user, and the date of issue upon delivery of the PPE.

Clause 14. The use of personal protective and safety equipment is mandatory for every person in the site (including the members of senior management, managers, workers, visitors, interns, etc.).

Clause 15. The personal protective and safety equipment must be procured and used in the company and its subsidiaries, in compliance with the standards listed in this guideline.

Clause 16. Personal protective and safety equipment must be procured and distributed by the HSE department in cooperation with the related directors of the departments and units according to the schedule authorized by the respective company.

Clause 17. Given the nature of some occupations that deteriorate the personal protective and safety equipment (such as gloves) more quickly, the equipment may be provided according to Section 8 of this guideline.

Clause 18. The replacement of a PPE item with a new one earlier than its due time will require returning the worn out and unusable items and the verification of the HSE unit.

# Personal Protective Equipment Replacement Interval

The employees will be provided with a certain number of PPE items at specific intervals, or under special conditions at the discretion of the HSE department, as follows:

Clause 19. The operating employees shall receive two sets of workwear suitable for their job every year.  At the discretion of the HSE department, the operating units directly involved in annual turnaround maintenance (overhaul) are entitled to an additional set of workwear.

Note: The staff and operational workers whose job requires their regular visit to the work sites may receive suitable workwear for their job based on the request made by their supervisor and approved by the HSE department.

Clause 20. Safety boots are distributed among the operating staff annually.

- Note: The staff and operational workers whose job requires their regular visit to the work site may receive suitable boots for their job based on the request made by their supervisor and approved by the HSE department.

Clause 21. All employees working in operating sectors are entitled to receive a pair of gloves suitable for their job every three months. This interval may be reduced in turnaround maintenance (overhaul) sections, emergency repairs, or special working conditions at the discretion of the HSE department.

Clause 22. The personnel are entitled to receive new safety helmets every five years. In case of expiration (according to the credentials on the helmet) or breaking, the helmet shall be replaced in exchange for the defective one based on the request made by the respective supervisor and approved by the HSE department.

Clause 23. The personnel are entitled to receive new safety goggles every three years. In case of inefficiency or breaking, the goggles shall be replaced in exchange for the defective ones based on the request made by the respective supervisor and approved by the HSE department.

Clause 24. The personnel whose work demands their continual exposure to sunlight and UV radiations are entitled to receive new sunglasses every five years. In case of inefficiency or breaking, the sunglasses shall be replaced in exchange for the defective ones based on the request made by the respective supervisor and approved by the HSE department.

Clause 25. At any time and as needed, the personnel are entitled to receive fall-arrest equipment based on the request made by the respective supervisor and approved by the HSE department.

Clause 26. The personnel whose work demands their exposure to noises above the standard limit are entitled to receive new earmuffs every three years. In case of inefficiency, expiration, or breaking, the earmuffs shall be replaced in exchange for the defective ones based on the request made by the respective supervisor and approved by the HSE department.

Clause 27. At any time and as needed, the personnel are entitled to receive masks and breathing apparatus. In addition, a few breathing apparatus must be kept in every unit for emergencies at the discretion of the HSE department.

Clause 28. The personnel are entitled to receive a new raincoat every two years based on the request made by the respective supervisor and approved by the HSE department.

Clause 29. All items of clothing that the personnel are entitled to, including warm winter clothes, overcoats, etc. must be procured in accordance with the regulations formulated by the Department of Employment Regulations and with the technical verification of the HSE department and delivered to all operating and headquarters staff.

- Note 1: The contractor is obligated to provide their employees with PPE according to the requirements of this guideline.

- Note 2: If the contractor fails to procure PPE for their workers, the employer will provide the contractor with the required PPE to distribute and will reduce the resulting costs, in addition to the forfeits imposed according to the HSE instructions for contractors, from the general contractor statement.

# Personal Protective Equipment Use and Maintenance Training

Clause 30. HSE departments are obligated to train all employees about the maintenance and the use of PPE provided to them to make their job safer. In addition, the respective directors must supervise the training to ensure their effectiveness. The PPE training must be inclusive and cover the necessity of using the PPE, explaining the correct way of using the PPE (both theoretical and practical), explaining the capacities and limitations of the PPE, explaining the proper equipment upkeep (inspection, cleaning, and maintenance).

# Personal Protective Equipment Usage Supervision

Clause 31. a) All directors and project executives must pay attention to the use of PPE in the workplace by the workers under their supervision.

b) The contractor is responsible for the use of PPE by their employees.

c) In any project, the director of the unit or the project manager is responsible for supervising and controlling the use of PPE by the project workers and contractors' employees and providing them with safety training.

d) In case of witnessing or getting reports from the HSE department about a worker's violation of regulations regarding the use of PPE, the respective director must warn the violator, and proceed to take the following disciplinary measures if the violations are repeated.

- First Step: verbal warning/Second Step: Filling the forms regarding the violation (not using the personal protective and safety equipment) and giving it to the violator and the respective administrator or director/Third Step: Presenting the violation to the disciplinary committee of the respective company,

# Personal Protective Equipment Evaluation

Clause 32. The employees are responsible for the initial inspection and evaluation of the PPE provided to them.

In this regard, the employees are required to inspect their pieces of PPE as they were trained to do on a daily basis and report any defects in their equipment to their supervisor.

The defects may include rupture in the clothes, gloves, aprons, gaiters, or malfunction of the respirators, chemical protective gloves, safety glasses and face guards, etc. particularly in cases where the worker is handling chemical or radioactive materials.

# Special Requirements

## 12.1	Head Protective Equipment

Clause 33. In places where hazards such as falling objects, flying objects that may hit the head, stationary objects against which one may knock their head, electrocution, hazardous liquid spill over the head, or any head-related danger are present, it is necessary to use a proper helmet that is appropriate for protection against related hazards.

Clause 34. All employees are required to use a safety helmet when working in or walking around the operational units (operation, maintenance, units and facilities under construction, etc.).

Clause 35. Employees with long hair must cover their hair fully in the workplace using a headband or some other protectors.

Clause 36. The employees working close to fire, sparks, or molten materials, etc. must use fire-retardant headbands that can be washed and disinfected.

Clause 37. The headbands must be easy to clean.

Clause 38. For sanitary issues, the use of one another’s headbands or helmets is forbidden for the workers.

Clause 39. The color of the safety helmets must be selected according to the following table in all subsidiary companies.

| Table 1: Helmet color code based on the type of occupation |  |  |
| --- | --- | --- |
| Helmet Color | Unit |  |
| Gray | Senior executives and region administrators |  |
| White | Operation |  |
| Blue | Maintenance and communications |  |
| Green | Technical services (technical inspections, etc.) |  |
| Yellow | Health, Safety, and Environment units | HSE department |
| Red | Firefighting department |  |
| Orange | Visitors |  |
| According to the discretion of the HSE department of the respective company. | Other units |  |

- Note 1: In companies with organizational structures different from the table above, the HSE department determines the color of the safety helmet.

- Note 2: All contractors’ personnel must be provided with safety helmets in colors specified in Table 1 while bearing the logo of their respective company.

- Note 3: The name and logo of the respective company must be stamped on the helmet.

Clause 40. Depending on the application, technical specifications of the safety helmet are as follows:

1. The helmet must comply with the ANSIZ89.1 standard or national standards.

2. The weight of the helmet, together with the accessories, must not exceed 400 gr.

3. The distance between the top of the suspending element (the star-like component) and the top of the external surface of the helmet must be at least 3 cm. This design absorbs shock upon impact while providing ventilation during normal use (absorbs impact).

4. The safety helmet material must be selected from among fire-resistant materials in Type A or B electrical insulating materials.

5. The helmet must be resistant to penetration and water and burn slowly.

6. The helmet must have a brim all around to shield the head, face, and back of the neck.

7. The helmets used in confined spaces must be short and, if needed, without a brim.

8. Helmets used in humid environments must be impermeable to moisture.

9. Helmets must come with clear instructions on how to adjust and replace the connection and the harness.

10. The bands and the leather inside the helmets must be easy to replace.

11. Helmets must be resistant to impact.

12. Helmets must withstand the impact of a 2 kg weight falling directly from a 1 m height above it and show no dents.

13. All helmets must come with a chin strap.

Clause 41. The helmets used in night-time shifts must be made of materials that reflect light for visibility (i.e. Fluorescent materials).

Clause 42. The safety helmet must be cleaned once a month (or more if needed) according to the instructions.

## 12.2	Eye and Face Protective Equipment

Clause 43. In workplaces where hazards to the eyes may exist, the employees are required to use eye protective equipment.

Clause 44. For employees who require vision correction, the eye-protective equipment must comply with the following requirements:

- The protective glasses with a vision correction function and provide clear vision and protection for the workers.

- The protective glasses that are worn over prescription glasses provided that they do not change the way the prescription glasses were placed.

- The Protective glasses that have prescription lenses under the protective glasses.

Clause 45. The eye-protective equipment used by the workers working with corrosive liquids (e.g. acids and bases) must cover the eyes fully and prevent the penetration of the liquid from the perforations.

Clause 46. In cases where the eyes may be hurt by the entrance of the agent from the sides of the glasses, an eye-protection with side shields must be used.

Clause 47. Protective glasses and the welding masks used by oxy-acetylene or arc welding workers or those exposed to the glare of a furnace must be equipped with color lenses (filters) to absorb the said radiations.

Clause 48. Protective glasses must have straps.

Clause 49. After every use, the workers must clean the face and eye protective equipment and place them in their special case.

Clause 50. Technical specifications for safety glasses:

1. The glass or transparent plastic used to make safety glasses must:

2. The glass or transparent plastic used to make safety glasses must:

- Be resistant enough for the job it is going to be used for.

- Be uniform and free of air bubbles, corrugation, cracks, or any defects.

- Apart from the prescription lenses, the inner and outer surfaces of the protective glass must be parallel with no curvature.

- The lens of the safety glass must be 44.5 mm wide and 38 mm long. In addition, non-prescription glasses with round lenses must have a minimum diameter of 50 mm.

3. Lenses used for protection against impact must allow a minimum light transmission of 80% off the workpiece surface.

4. The frame must be light and strong and fit the face perfectly. In addition, the glasses must be equipped with side shields if needed. Safety glasses used for cutting, riveting, grinding, polishing, sanding, etc. must have lenses strong enough for the job.

5. Safety glasses must come with flexible frames, matching the size of the wearer’s face.

6. The choice of eye and face protective equipment must be approved by the HSE department according to Table 2 and the type of occupation.

| Table 2. Eye and face protective equipment suitable for each type of occupation. |  |  |
| --- | --- | --- |
| Recommended Eye and Face Protection Equipment | Hazards | Type of Occupation |
| Welding mask and safety glasses or special goggles Welding, with a suitable lens for the job | Sparks/radiations/molten metals /flying objects | Oxy/acetylene welding cutting/welding |
| Chemical protective goggles or splash guards | Chemical splash/acid burns/ Vapors | Handling of chemicals |
| Impact-resistant safety glasses or goggles | Flying objects | Demolition |
| Welding mask and safety glasses or special goggles Welding, with a suitable lens for the job | Sparks and harmful radiations/molten metal | Arc-welding |
| Face shields or special safety glasses or goggles equipped with the suitable lens for the job | Radiations /heat/molten metal | Furnace operation |
| Impact resistant safety glasses or goggles or protective masks | Flying objects | Grinding and light polishing |
| Face shields or special safety glasses or goggles equipped with the suitable lens for the job | Flying objects | Grinding and heavy polishing |
| Chemical protective goggles or splash guards and safety glasses | Chemical splash/glass shattering | Chemical laboratory |
| Impact resistant safety glasses or goggles or protective masks | flying objects/radiations/heat | Physical and mechanical laboratory |
| Impact resistant safety glasses with side shields or protective masks | Flying objects | Machining |
| Face shields or special safety glasses or goggles equipped with the suitable lens for the job | Heat/glare Sparks and splashes | Melting of metals |
| Impact resistant safety glasses with side shields or protective masks | Flying objects/sparks | Spot welding |

Clause 51. The eye and face protective equipment must be stored according to the manufacturer's instructions and the instructions prepared by the respective company.

Clause 52. In the case of a foreign object entering the eye, a specialist is required to remove the object.

Clause 53. The face shield is not intended to resist dust and shall not be used in a dusty environment.

Under such circumstances, it is recommended to use full-face masks equipped with dust filters.

Clause 54. To provide full protection against the penetration of liquids, acid-resistant helmets and full-face masks equipped with suitable filters for acid vapors are recommended for sampling, loading/discharging or displacing corrosive liquids such as acids, sodium hydroxide, etc. Acid-resistant safety glasses and special masks may also be attached to the helmet. In cases with a lower risk of hazards, acid-resistant safety glasses may suffice.

Clause 55. During maintenance or cleaning of the vessels and machinery that contain - or have contained - acids or bases, and also during opening/closing the pipeline valves or starting the pumping of such chemicals, the workers are required to use acid-resistant helmets, safety glasses, face shields, or full-face masks equipped with filters suitable for acid vapor.

Clause 56. Arc welders are required to use welding helmets or shields (masks) with a correct, and approved, lens shade.

- Note: The shade numbers required for industrial-grade safety glasses in different types of welding processes are presented in Table 3 based on the electrode size, amperage, and the thickness of the workpiece.

| Table 3. Selecting the correct safety glasses shade number. |  |  |  |
| --- | --- | --- | --- |
| Minimum shade number | Amperage | Electrode Size (1/2 in). | Type of Occupation |
| 7 | Below 60 | Below 3 | Arc Welding |
| 8 | 60-160 | 3-5 |  |
| 10 | 160-250 | 5-8 |  |
| 11 | 250-550 | Above 8 |  |
| 7 | Below 60 |  | Flux Core Arc Welding |
| 10 | 60-160 |  |  |
| 10 | 160-250 |  |  |
| 10 | 250-550 |  |  |
| 8 | Below 50 |  | TIG Welding |
| 8 | 50-150 |  |  |
| 10 | 250-500 |  |  |
| 10 | Below 500 | Light | Oxy-Fuel Cutting |
| 11 | 500-1000 | Heavy |  |
| 6 | Below 20 |  | Plasma Arc Welding |
| 8 | 20-100 |  |  |
| 10 | 100-400 |  |  |
| 11 | 400-800 |  |  |
| 8 | Below 300 | Light | Plasma Cutting |
| 9 | 300-400 | Medium |  |
| 10 | 400-800 | Heavy |  |
| 3 |  |  | Riveting, Soldering, MIG welding |
| 2 |  |  |  |
| 14 |  |  |  |

Clause 57. The protective lens of the welding helmet must be protected against hot flying debris by a second lens that is placed over it. When cleaning the weld, the welder must use the clear lens embedded in the helmet. If the welding helmet is not equipped with a clear lens, the welder is expected to proceed to use a clear safety glass to protect their eyes against flying debris.

Clause 58. Glassworkers are required to use glass-working safety glasses.

Clause 59. Special, ventilated workwear equipped with eye protective equipment must be used for sandblasting.

It is recommended to use hose supplied-air respirators or full-face helmets or masks to protect the respiration system and the face and eyes simultaneously.

Clause 60. Seamless safety glasses are required to be used when applying or removing fiberglass or mineral wool insulations or cutting the sheets (the use of asbestos is prohibited because of its pathogenic/carcinogenic properties).

Clause 61. When lighting, inspecting or adjusting the furnace, special safety glasses, helmet-attached face shields, or both are required to be used.

Clause 62. When working with fumes and vapors that are harmful to the eye, the use of acid-resistant or full-face masks with appropriate filters is mandatory.

Clause 63. When sampling chemicals, a face shield must be attached to and used over the safety helmet.

## 12.3	Hearing Protective Equipment

Clause 64. The use of hearing protective equipment is mandatory for workers who are exposed to noises above the standard limit (continuous or pulsating noises).

Clause 65. The noise in the workplace must be measured using a standard, calibrated sound meter and the type of hearing protection must be determined based on the frequency analysis results at the discretion of the HSE department.

Clause 66. The hearing protector must fit the worker’s physique and the working conditions.

Clause 67. The workers are not allowed to use one another's hearing protectors.

Clause 68. Hearing protective equipment must be resistant to sparks, metal particles, and other foreign bodies.

Clause 69. Hearing protectors must be cleaned every day and disinfected before use.

Clause 70. The hearing protection devices used to protect ears against sparks, metal particles, and other foreign objects must be equipped with a strong and light stainless mesh with leather cushions that fit ears easily with the help of an adjustable spiral spring behind the head.

Clause 71. Make sure the hearing protector fits the ear perfectly and stays in place.

| Table 4. Comparing different hearing protectors and their applications |  |
| --- | --- |
| Sponge earplug |  |
| Disadvantages | Advantages |
| Infects the ear when it is re-used | small and light-weight |
| Not an economical choice in the long run | Comfortable to use in hot environments |
|  | Fits ear cavities perfectly |
|  | Disposable, free of maintenance |
|  | Prepared to provide a high level of protection |
| Plastic earplug |  |
| Disadvantages | Advantages |
| Takes time to clean | Small and light-weight, easily inserted into the ear cavity |
| Being pre-shaped, this type of earplug may fail to completely fill the ear cavity, making it ineffective. | Comfortable to use in hot environments |
|  | Easily used with other safety equipment |
|  | An economical choice compared to sponge earplugs |
| Earplugs that are fixed on the ear by a ring |  |
| Disadvantages | Advantages |
| Failure to protect the ear fully | Small and light-weight, comfortable to use in hot environments |
| Take time to clean | Easy to put on and remove from the ear |

Clause 72. It is necessary to do as instructed to keep the equipment functional, for example:

1. When the hearing protection device is not being used, it must be kept in a special case to protect it from grease or any harmful agent.

2. The hearing protectors must be cleaned every day and disinfected before use according to the manufacturer's instructions.

## 12.4	Respiratory Protective Equipment

Clause 73. All operating and construction units where there is toxic gas or smoke hazards are required to procure and install Self-Contained Breathing Apparatuses (SCBA).

Note 1: The HSE department is responsible for specifying the number and location of the breathing apparatus.

Clause 74: Respiratory protective systems must be standard and appropriate for the conditions they will be used in and require the approval of the HSE department before use.

Clause 75: Respiratory protective equipment must be selected after undertaking the following tasks:

- Measuring the concentration of the chemical pollutants in the environment

- Evaluating the respiratory-related dangers in the workspace

- Realizing the type and concentration of the chemical pollutants

- Realizing the potential limitations of the workers when using the equipment

Clause 76: When it is recommended to use the respiratory masks for the first time in a place, it is essential to undertake a Fit Test before procuring and using the masks.

Clause 77: Using filtered breathing apparatus to protect the respiratory system against corrosive chemical vapors, solvents, harmful gases, and the deoxygenated air is prohibited.

Clause 78. In case of breathing difficulty, or sensing an odd taste or smell when using filtered masks, the filter must be changed even if it is not expired yet.

Clause 79. The breathing masks and filters must be purchased with the verification of the HSE department.

Clause 80. Using filtered masks is prohibited in partially-ventilated places or places where the oxygen concentration level of the air is below 19.5 %vol.

Clause 81. Unpacked filters must be replaced after six months (or any duration of time recommended by the manufacturer) even if not used.

Clause 82. Unpacked filters must be replaced after the expiration date.

Clause 83. Air purifiers and masks equipped with flexible hoses are designed to be used:

a) in hazardous locations and places where the equipment is guaranteed to be able to supply air.

b) for tasks that are not a top priority and must be undertaken in situations with smoke or harmful gases; and it is not possible to use other types of filtered, and chemical protective devices.

Clause 84: Breathing masks and apparatuses may be supplied using compressed air only if:

a) The air has been cleaned and dried in advance by the filters placed in its path.

B) It is best that the compressed air is pumped into the masks and respiratory devices by a ventilator.

In this case, it is suggested not to use a compressor if possible.

Clause 85. Supplying respiratory masks with the air coming from instrumentation devices is strongly prohibited.

Clause 86. The distance between the masks equipped with flexible hoses and the air supply must be in accordance with the manufacturer's instructions and approved by the HSE department.

Clause 87. Modification, adjustment, or maintenance of the air supply equipment that is not recommended by the manufacturer is prohibited.

Clause 88. The employees exposed to toxic gas hazards must be trained for using the breathing apparatus in advance.

Clause 89. The vulnerable parts of the respiratory protective equipment, the parts that are more exposed to damage and wear, and also parts serving for supplying fresh air or oxygen, must be inspected by a competent person at least monthly.

Clause 90. Considering the dangers of oil, grease, etc. when coming in contact with oxygen, such materials are not allowed near oxygen tanks and fasteners.

Clause 91. The employees must readily report any defects in the breathing apparatuses to their supervisor.

Clause 92. The workers are not allowed to have a beard or a mustache that prevent the correct function of the breathing apparatus.

Clause 93. A used breathing apparatus can only be used by another person if it has been completely washed with soap and lukewarm water and disinfected by a trained person.

Clause 94. Respiratory protective devices must fit the users' faces and rest on their faces without leaving any open gaps.

Clause 95. The belts and straps that are used to attach the device to the body shall not restrict the user’s freedom of movement.

Clause 96. Requirements, technical specifications, and the maintenance conditions of the breathing apparatus.

1. The pressure of the air entering breathing masks and apparatus shall not exceed 1.75 kg/cm2.

2. Whenever the air pressure exceeds the said limit, the air supply device must be equipped with a relief valve that is installed at the connection of the flexible hose to the air supply.

3. To be more cautious and to prevent the irregular function of the relief valve, the device must be also equipped with a safety valve that is adjusted at a higher pressure than the relief valve.

4. The compressors and ventilators must be regularly and carefully maintained and the air inlet must be located somewhere that ensures a supply of clean air of the device.

5. The inner diameter of the flexible hose of the mask may not be smaller than 5.2 cm, and the hose material must be tear- and tangle-resistant, not to block the airway.

6. The distance of the masks that are equipped with flexible hoses and the connection to the main pipe shall not exceed 5.7 m.

7. Belts and fasteners used to attach the breathing apparatus to the body must have a minimum tensile strength of 115 kg.

8. The breathing apparatus tanks must be equipped with manometers displaying their maximum pressure.

9. The relief valve of the oxygen breathing apparatus must be adjusted to deliver at least 2 liters of oxygen per minute.

10. The automatic safety valves of the oxygen breathing apparatuses require monthly inspections.

11. To ensure the correct function of the control equipment, the oxygen breathing apparatus must be carefully inspected by an eligible person at least once every six months.

12. The air supplied by the breathing apparatus must be standard and free from contamination.

13. People using breathing apparatuses must be trained for:

1. Quick and correct placement of the mask on the face

2. Correct use of device in emergencies

14. The employees must readily report any defects in the breathing apparatuses to their supervisor or the HSE department.

15. The following conditions must be fulfilled for the maintenance of breathing apparatuses:

- They must be stored under the direct supervision of an eligible expert who is responsible to keep them in a good condition.

- Must be kept in a clean, dry, and cool place inside special cases while ensuring they can be easily accessed.

- The breathing apparatuses that are equipped with filtered cases and frames must be cleaned with soap and water, then rinsed with clean water, and dried before being put back into their place.

- These apparatuses are designed to be initially worn in a safe environment in advance and function if an emergency (toxic air conditions) arose later.

Clause 97. Different types of breathing apparatuses and their specific requirements:

1. Air-Purifying Respirators (APR): These devices are used in low-risk situations (standard oxygen level) where more protection is required compared to normal conditions and include disposable masks, half-face masks, full-face masks, and Powered APRs (PAPR).

5. Gas Masks: These masks are designed to protect the lungs against a special type of gas or a group of gases.

2. Air-Supplying Devices: These devices include:

2.1 Supplied Air Respirator (SAR): These respirators are used in toxic or IDLH (Immediately Dangerous to Life or Health) conditions or in oxygen-deficient atmospheres.

SAR fittings must fulfill the following requirements:

1. The maximum allowable hose length is 90 m. 2. The hose must be long enough to allow the user to move freely.

3. The air supplied by the compressor is dry, therefore, the workers are required to rest and breath in clean air after using the device for 2-4 hours.

2.2 Self-Contained Breathing Apparatus (SCBA): These devices are only intended for rescue from an emergency as their accompanying compressed air tank supplies air for 30 to 60 minutes.

Requirements: 1. The device must be equipped with an alarm system to warn the user when the tank is depleted to 25% so they leave the contaminated place. 2. The device must be composed of a full-face mask that has embedded belts to tighten the mask and a compressed air tank that can be carried with the help of shoulder straps and buckles. 3. The user must be trained in advance to correctly use the equipment. Meanwhile, the devices shall not impair the user's ability to speak.

2.3 Supplied Air Breathing Apparatus (SABA):

This equipment is used in cases where the workers are exposed to toxic gases for a long time. It supplies air through an air tube and incorporates a tank (the tank supplies air when the air tube fails to do so).

The air may be supplied by a compressor and a compressed air tank, a compressor and a hose, or a combination of both, to allow the user to breathe effortlessly.

Requirements: 1. The compressed air tank must be able to supply air for at least 5 to 15 minutes (must supply air for effortless respiration for at least 5 minutes). 2. When a compressor is used, the maximum allowable hose length is 90 m. 3. The user must have received proper training in advance to use the equipment correctly.

| Table 5. Comparing Air-Purifying Respirators and Supplied-Air Respirators |  |
| --- | --- |
| Supplied-Air Respirators | Air-Purifying Respirators |
| Air is supplied by a tank or a compressor | For using in situations where there is sufficient oxygen in the air |
| For using in situations where there are hazardous substances or in oxygen-deficient atmospheres | For using in situations with negligible amounts of toxins in the atmosphere (dust, smog, spray, vapors, gases, etc.). |
|  | Purify the air to a limited extent |
|  | Never to be used in oxygen-deficient atmospheres |
|  | The user must be aware of the type, and extent of the contamination in the area |
|  | Are equipped with replaceable filters and cartridges depending on the ambient air |

Clause 98. Selecting the correct air-purifying respirators: APRs must be selected based on the following measures:

1. Determining the type of contamination

2. Specifying the form of contamination (vapor, fog, smoke, etc.)

3. The specified concentration of the contaminants; 4. The specified oxygen level in dangerous areas; 5. The time of exposure to contamination, as well as personal allergies; 6. The requirements of the other pieces of PPE that are used together with the respirator (Is it necessary to wear safety glasses? Is any other type of PPE needed?).

Note: In some places (such as IDLH atmospheres) the above-mentioned parameters cannot be specified.

The following table helps employers and employees to choose the right respiratory PPE for any activity:

| Table 6. Selecting the right respirator for different activities. |  |
| --- | --- |
| Type of Respirator | Hazards |
| Masks that remove particulates or air-purifying respirators with half-face masks | Solid and liquid particulates (dust, fog, thick smoke) |
| Full-face respirator | skin and eye irritants |
| Respirators with half- or full-face masks or electrical air-purifying respirators | Petroleum-based chemicals |
| PAPR | Glass wool and lead |
| PAPR | High toxicity welding metals (lead, cadmium, etc.) |
| SAR | IDLH substances |
| SAR | Oxygen deficiency (below 19.5%) |

Clause 99. It is necessary to select the right filter for the air purifying respirator according to Table 5.

| Table 7. Selecting the right filter and cartridge for the air-purifying respirator. |  |
| --- | --- |
| Not resistant to petroleum-based chemicals | N |
| Resistant to petroleum-based chemicals For using for up to 8 hours in the vicinity of petroleum-based chemicals | R |
| Resistant to petroleum-based chemicals and can be exposed to them for longer than 8 hours; therefore, they can be used in more than a single work shift. | P |
| High-resistance particulates can be exposed to petroleum-based chemicals Use for PAPR | High-efficiency particulates |

Clause 100. Color coding for the chemical cartridges: All manufacturers of chemicals use the same color code for gas/vapor that is white for acid gases, black for organic vapors, green for ammonia gases, yellow for acid gases and organic vapors, olive green for gaseous compounds (mixes of different gases and vapors), dark purple for high-efficiency particulates.

Clause 101. The masks must be used according to the manufacturer's instructions.

Note: Make sure the mask is in good condition and fits your face properly before being exposed to contaminants.

Clause 102. The masks must be tested periodically as follows:

1.1 Checking Disposable Masks:

1. Place both your hands on the mask without applying excess pressure on it, then breath.

2. Check whether the mask is drawn in.

3. If the mask is drawn in and no air has entered, the mask is fixed on the face properly. Otherwise, re-adjust the mask and its belts (elastics) on the face until it fits perfectly.

1.2 Checking half- and full-face masks:

1.2.1 The positive pressure check: Using your hand, block the mask outlet and exhale into the mask, then, do not breathe for ten seconds and check whether the mask is bulging.

If the mask remains bulging, it fits properly; otherwise, readjust the straps to fix it.

1.2.2 The negative pressure check: Hold the inlet of the mask (filter/cartridge) with hands, inhale and hold your breath for 10 seconds.

Check whether the mask is caving in.

If it remains caved-in without letting air in, the mask fits the face; otherwise, re-adjust the mask and its straps to fit it properly.

Note: Consider that the mask must be inspected in terms of quality and quantity before the above checks.

1.3 Respirator performance check:

1.3.1 Do the quality check to evaluate the performance of the respirator. This check involves exposing the wearer to released pollutants (vapors, smoke, and aerosol). In this test, the physical criteria of the wearer and their response to the test items are evaluated. It must be noted that if the wearer can sense the pollutants through smelling, tasting, and allergic rhinitis, the respirator is not fixed on their face properly.

Note: The fundamental theory of quality check is to effectively protect the wearer from pollutants and workplace hazards.

1.3.2 Quantity check: This check is aimed to evaluate the effectiveness of the respirator for a specific user by measuring the real level of the invasive factors in an out of the respirator mask.

This check must be undertaken at least once a year or more frequently depending on the shape of the users’ faces.

The user must do the positive and negative pressure checks before entering the hazard area.

Clause 103. Masks and respirators maintenance conditions: 1. Always inspect the equipment before use.

2. Make sure the equipment is not damaged, ruptured, embrittled, dirty, degraded, etc.  3. Do not use the equipment if you notice any defects and immediately proceed to repair or replace them.

4. Never use the equipment that is not fully functional.

Clause 104. The respirator is not applicable in the following cases: 1. When you can smell or taste the pollutants.

2. When you find it difficult to breathe. 3. When you feel dizzy or light-headed.

4. After using, if the equipment is disposable, put it aside and never use it again; otherwise, clean and re-check it. If you are not planning to use the equipment store it in appropriate conditions.

5. Keep and protect the equipment in a tightly-closed plastic bag”.

6. Make sure the equipment is stored in appropriate conditions, protected from dust, sunlight, and extreme temperature, moisture, and harmful chemicals.

## 12.5 Hand and Arm Protective Equipment

Clause 105. Appropriate protective gloves must be selected for the job according to the potential hazards and shall not impose any restriction on wearer's ability to move.

Clause 106. When working with machinery that has moving parts, in which the hand or glove may be caught, the workers shall not wear gloves.

Clause 107. The workers that carry sharp-edged or barbed objects must use proper, cut-resistant safety gloves.

Clause 108. Workers who are responsible for cutting meat (chicken, fish, etc.) or separating bones from meat in restaurants must use steel chainmail gloves.

Clause 109. Workers who handle hot objects are required to use standard, refractory gloves that are properly insulated and heat-resistant.

Clause 110. Workers who are exposed to electrical hazards must use insulating and resistant gloves that are appropriate for the voltages involved.

Clause 111. Workers handling corrosive substances such as acids and bases, irritating toxins or infectious waste must use appropriate safety gloves.

Clause 112. Standard lead gloves (according to the Atomic Energy Organization of Iran) must be used for protection against ionizing radiations. The gloves must protect the hands and completely cover them up to above the elbow.

Clause 113. Technical specifications of the gloves:

1. Must be long enough to fully cover the arms.

2. All parts of this type of safety gloves must be resistant to the said substances.

3. The gloves shall not have any holes or fissures.

4. The gloves must be quickly replaced if ruptured during work.

5. Lead gloves must be used for X-ray protection. These gloves must protect the hands fully and cover the arms up to above the elbow. To provide enough protection, the lead sheet must be at least 0.55 mm thick.

6. Despite the weight of lead (due to its high density), the gloves must be light and flexible and provide full protection for hands against ionizing radiations.

Clause 114. Criteria for selecting gloves:

1. The gloves must fit hands properly.

2. All gloves cannot protect hands against different surfaces and substances. Gloves are available with three levels of protection (high, medium, low) against chemicals.

3. The gloves used for handling chemicals must block the chemicals from entering into the glove or spilling over the elbow.

4. Do not use a metal glove when working with electrical devices.

Clause 115. Different types of gloves: Gloves are classified into thirteen groups in terms of their materials, namely:

1. Based on the type of resistance:

- Impact-Protection Gloves, including: Leather, heavy rubber, wool, split-leather, canvas-back gloves (canvas gloves are the most common impact-resistant gloves). Material of gloves and their thickness are selected based on the mobility of the fingers and also the potential hazards.

- Shielded gloves incorporate a steel mesh that protects the hands against sharp edges, such as knives or industrial cutters.

2. Chemical-Protection Gloves: These gloves are made of natural rubber, or polymer compounds such as Neoprene, Viton, Nitrile, PVA, PVC, or Butyl.

3. Thermal-Safety Gloves: These gloves may be either heat- or cold-resistant. Heat-resistant gloves are often made of leather, aluminum-coated fabrics, glass wool, asbestos, aramid, split-leather, etc. Meanwhile, cold-resistant gloves are often made of glass wool or aramid.

4. Electrical-Safety Gloves: These gloves are often made of rubber or special polymers and are available in a range of electrical resistance based on the working voltage.

5. Anti-Vibration Gloves: Based on the frequency of the vibrations produced by hand-held devices, anti-vibration gloves may be made of different materials, such as heavy leather and rubber (to absorb high-frequency vibrations), Sorbothane (to absorb frequencies in the range 5-1400 Hz), or Poron (to absorb frequencies in the range 5-5000 Hz).

6. Radiation-Safety Gloves: This type of gloves is made with fabrics incorporating a lead film.

These gloves must cover the arm at least half-way up to the elbow and have a lead equivalence of no less than 0.55 mm.

Moreover, given the high density of lead, the gloves must be designed to be light-weight and comfortable in addition to protecting the hands.

7. Protective Sleeves: Protective sleeves are used in cases where the arms and forearms are also exposed to hazards.

The sleeves may be used together with or without a glove, and are often made of the same material as the glove.

8. Metal Mesh Gloves: these gloves are resistant to sharp edges and prevent the tools from cutting or hurting the user’s hands. They are made of strong and light synthetic fibers and are used as protection against cuts and bruises.

9. Vinyl and Synthetic Rubber Gloves: For protection against toxic chemicals.

10. Impermeable: Resistant to hazardous chemicals.

11. Butyl Gloves: Impermeable to gases and water vapor and often used for chemicals such as Ketones and Esters.

12. High-Resistance Gloves: Resistant to aromatic and chlorine solvents.

13. Nitrile Gloves:

Nitrile gloves provide excellent protection against different solvents including chemicals, greases, proteins, in addition to being resistant to cut, piercing, abrasion, and impact.

Clause 116. Types of gloves based on application:

1. Plastic Gloves: these gloves protect hands when working with electrical devices.

2. Lead Gloves: these gloves are used for protection against ionizing radiation and its source.

3. Disposable Gloves: Gloves made of rubber to protect hands against microbes and bacteria.

4. Fiber Gloves: these gloves protect hands against sharp edges, shavings, dust, and vibration.

5. Leather Gloves: Leather gloves provide protection against rough surfaces and frost.

6. Heat- and Flame-Resistant Gloves: these gloves are used in casting, welding, and handling furnaces.

Clause 117. The use and maintenance of gloves:

1. Before putting the gloves on, remove rings, watches, bracelets, or any other object that may tear the glove.

2. Wash hands before and after using the gloves.

3. Inspect the gloves before using them and check for holes or slits that can pass chemicals into the glove and expose the skin.

4. After working with chemicals, wash the gloves with abundant water to clear off any trace of the contaminants left on them.

5. Some gloves have an expiration date after which using the gloves is not advised.

6. If needed, wash the cotton gloves.

7. As a piece of PPE, gloves shall not be transferred to others.

8. Keep the gloves in a cool, dry, and ventilated place.

Clause 118. Never use gloves when working with rotating machines such as drills and lathes.

## 12.6 Leg Protective Equipment

When working or passing through facilities, it is necessary for the workers to put on safety boots.

Clause 120. Workers handling corrosive chemicals (acids, bases, etc.) must wear suitable resistant shoes.

Clause 121. Electrical safety shoes shall not incorporate metal parts and must be insulated to electrical current.

Clause 122. To protect the wearer’s feet, safety boots must incorporate a steel reinforcement in the toe area to provide adequate protection against impact and pressure.

Clause 123. In places where sparks are considered a hazard, the workers must use anti-static shoes that do not incorporate metal parts.

Clause 124. Safety boot technical specifications:

1. The safety boot must wrap over the ankle.

2. The front side of the safety boot must be reinforced by a steel toe while its sole is made of flexible materials such as Polyurethane.

Depending on the occupation, the toe area can be made of leather (electrical insulation) or PVC (chemical protection).

3. Safety boots must incorporate a steel toe or other reinforcements to protect the wearer’s foot.

4. Electrical-safety and anti-static boots shall not incorporate any metal parts and must be insulated.

5. In activities where contact with sharp and piercing objects is possible, suitable, resistant safety boots must be used.

6. The workers who handle oils and petroleum products must use shoes with non-slip soles.

7. Workers who move heavy objects must wear protective boots or other footwear with toe reinforcement.

8. Workers handling corrosive chemicals such as acids or bases must wear rubber or leather safety boots that are resistant to the said chemicals.

9. The workers handling hot and corrosive materials must wear boots that:

a) fit the ankles tightly to block the said materials out of the shoe;

b) are seamless (without lace) to block the said materials out of the boot.

10. To protect the wearer’s feet, safety boots must incorporate a steel reinforcement in the toe area to provide adequate protection against impact and pressure.

11. Protective gaiters must be able to be quickly removed in case of an emergency.

12. Workers handling molten metals must use over-the-knee gaiters that are made of asbestos, or other special, heat-resistant materials, and fit the legs tightly to prevent the molten material from contacting the wearer’s legs.

13. The gaiters used by workers who are exposed to small or strong sparks, or work with sharp and rough objects, must be made of leather or other resistant material.

14. Workers who use axes or sledgehammers must wear suitable ankle braces.

15. Toe protectors: Covers that protect toes under impact energies of over 200 J.

Clause 125. Different types of safety boots and their application:

1. Steel-Toe Safety Boots: For protecting the wearer’s toes against impact and pressure.

2. Safety Boots with Special Soles: Special protection for the ankle and toes and the entire foot during running.

3. Safety Boots with Hard Soles: Incorporating a metal piece in the sole to prevent puncture.

4. Plastic or Rubber Safety Boots: Resistant to chemicals and prevent slip by increasing friction.

5. PVC Safety Boots:

Protection against moisture and preventing slip over the surface by increasing friction.

6. Butyl Safety Boots: Protecting feet against Ketones, Aldehydes, Alcohols, Acids, Salts, and Bases.

7. Vinyl Safety Boots: Resistant to solvents, acids, bases, salts, water, greases, and blood.

8. Nitrile Safety Boots: Resistant to animal fats, oils, and chemicals.

9. Electrostatic Discharge Safety Boots: These boots are designed to discharge static electricity to the ground.

10. Anti-Static Safety Boots: Considering that these boots are made of hard rubber, they act as insulators and prevent electrical shocks and foot burn.

Clause 126. Selecting the Right Safety Boot:

1. Never use leather or fabric boots in places where they can be exposed to acids or bases since these chemicals can deteriorate the boot quickly and injure the wearer.

2. Pick a safety boot that fits your feet.

3. Select safety boots that are appropriate for the type of the occupation you are planning to use them for.

Clause 127. Taking care of the safety shoes:

1. Inspect the shoes before using them and check for holes and fissures that can pass chemicals.

2. If damaged or worn-out, change the shoes.

3. After handling chemicals and before leaving the place, wash the shoes with abundant water to remove any trace of contaminants.

4. As a personal item, a pair of safety boots may not be used by multiple people for sanitary reasons.

Keep the boots in a dry, cool, clean, and ventilated place.

## 12.7 Body Protective Equipment (Workwear)

Clause 128. Due to the workplace conditions and the exposure of the workers to hazards, using workwear is necessary for the personnel.

Note 1: According to HSE requirements, to prevent the transmission of contaminants, the workwear used by the personnel in public places, including the restaurants, prayer rooms, etc. must be clean (therefore, it is best to change clothes before entering these places).

Clause 129. The workwear must be selected according to the type and nature of the occupation and with the approval of the HSE department.

Clause 130. The workwear and the number of items provided to the personnel in a given period of time are specified based on the nature of the occupation, according to the table prepared by the HSE department.

Clause 131. Washing and cleaning the workwear out of the workplace (especially at home) is prohibited.

Clause 132. Washing and cleaning the workwear with chemical and flammable solvents is prohibited.

Clause 133. Making any changes to the workwear, including replacing buttons with zippers, shortening the sleeves, etc. is prohibited.

Clause 134. For full-body protection, the workers wearing two-piece apparel must fasten all buttons and zippers.

Clause 135. Workers operating or working near rotating machines shall not wear loose and free-flowing apparel and must use shirts with tight cuffs. Aprons shall not be worn for such works either.

Clause 136. Welders and workers who are exposed to dangerous dust (such as silica) or toxins shall not wear apparels with pockets or turn-ups.

Clause 137. The technical specifications of the apparel worn by the workers who handle radioactive materials must be approved by the Atomic Energy Organization of Iran.

Clause 138. The workwear must be selected considering the season and geographical conditions and provided at least in the form of summer and winter clothes.

Clause 139. The workwear must fit the wearer and the textile used to make it should not cause skin irritations.

Clause 140. Workwear color: To maintain uniformity in the workwear worn by the personnel, the color and the form of the workwear must be selected based on the type and nature of the occupation, the operating unit/department where the employees work, and according to Table 8.

| Table 8: Workwear color code based on the type of occupation |  |  |  |
| --- | --- | --- | --- |
| Workwear Color | Department/Unit |  | no. |
| Blue | Maintenance and communications |  | 1 |
| Light gray | Operating units |  | 2 |
| White (overalls) | Laboratory |  | 3 |
| Navy blue | Warehouse |  | 4 |
| White | Inspection units |  | 5 |
| White | Health, Safety, and Environment units | HSE department | 6 |
| Red | Fire department |  |  |
| Garments | Offices |  | 7 |

- Note 1: The color of the workwear worn by the contractor’s personnel must be selected according to Table 8 and the clothes must have the contractor’s clear and distinguishable logo.

Clause 141. Workwear requirements and specifications:

1. The type of the workwear the employees receive is different depending on their occupation.

2. The workwear must be selected based on the workplace conditions and must minimize the exposure of the workwear to hazards. Moreover, the pants and shirts must have illuminated bands over the knees and on the arms.

3. The workwear must fit the body and shall not have any loose parts. It is better to come with a small number of small pockets, and the belt must be always fastened.

4. The workwear must be easy to take off; and therefore, it must have metal snap-fasteners.

5. Metal zippers are prohibited for being electrostatic hazards.

6. Workers operating or working near rotating machines, shall not wear open or torn clothes. The workwear must be tight-fitting and have short sleeves so it does not get stuck between the machine moving parts.

6. Celluloid sun-protection collars and eyeglasses frames are prohibited in places with a high risk of fire and explosion.

7. If the working conditions require the workers to roll up their sleeves, they must use short-sleeve workwear.

8. Workers who are exposed to dangerous, flammable, or toxic dust (such as silica) shall not wear apparels that have pockets or turn-ups as they can catch and retain the dust.

9. The workers handling corrosive or harmful materials must wear clothes that are impermeable to water and gas and resistant to the said materials.

10. Flame-resistant overalls for protection against possible exposure to flame must come with attached hoods, gloves, and boots.

11. In welding, grinding, and similar workplaces where the workers are exposed to sparks and flying hot debris, the collar of the workwear must fit tightly.

12. The workers who handle acids and radioactive materials must wear one-piece, seamless workwear with special and impermeable hood, boots, and gloves.

13. Aprons shall not be used when operating or working near rotating machines.

14. If it is necessary to wear an apron near a rotating machine, the worker shall wear a two-piece apron with separate lower and upper parts that can be easily and quickly removed in case of getting stuck in the machine to keep the worker safe from any harm.

15. The aprons worn by the workers who work near an open flame or molten metals must fully cover the wearer’s chest and be made of fire-retardant, heat-resistant materials.

16. The special aprons worn by workers handling corrosive fluids, such as acids and bases, must fully cover the wearer's chest and be made of natural or synthetic rubber or any material that is resistant to the said fluids.

17. Lead aprons used for X-ray protection must cover the whole chest and 30-40 cm below the waist.

18. Lead aprons must have a minimum lead equivalence of 1 mm.

19. Workers who operate or work near rotating machines must wear tight-fitting overalls.

20. Wearing chains, watches, bandanas, scarves, etc. over the workwear is prohibited.

21. The workers whose apparel is smeared with dangerous petroleum products or chemicals must wash or change their clothes and wash the chemicals off their bodies using appropriate detergents.

22. The employees are prohibited from getting close to a flame or heat sources or lighting matches, etc. with oil-, or chemical-soaked clothes.

23. The workwear must be turned over to the respective supervisor if they need to be replaced due to being damaged, torn, or impossible to clean.

24. Oil- and grease-soaked clothes must be taken off quickly before causing skin irritation.

25. Forklift and crane operators, road workers, and emergency-response workers must wear high-visibility clothing conforming to the Australian AS4602 standard.

- Class D: Designed only for daytime, outdoor use, these apparels incorporate fluorescent or other non-retro-reflective materials.

- Class N: Designed for night-time use only, this class of garment incorporates retro-reflective materials on a typical fabric.

- D/N Class: This class of garment is designed for both day- and night-time use and made up of both fluorescent and retro-reflective materials.

26. Similar to other pieces of PPE, these apparels must be stored according to the manufacturer's instructions and require periodic inspections to ensure they are in good condition.

Clause 142. Workwear material requirements:

1. Considering the working conditions, appearance requirements, wrinkle resistance, and preventing the creation of static electricity, a70% cotton (320 gr/m2) 30% polyester (420 gr/m2) fabric must be used.

2. In cases where the garments may catch fire as a result of hot shavings or high temperature (such as in case of welding workwear), the workwear must be 90% cotton.

3. When ordering the workwear, note that a twill-weave, high density (with three warps for each weft) textile must be used to provide sufficient strength.

Note: A durable workwear can be made by picking a high-quality fabric, strong threads, and re-sewing the seams.

Clause 143. The workwear worn by all permanent employees and contractor’s personnel must have a logo with following specifications:

- The NIOC logo on top and the name of the respective company below;

- Suitable background color for specifying the unit;

- The logo must be at least 10 cm large,

- The logo must be printed by high-quality screen printing in the specified colors;

- The logo must be printed on the breast pocket on the left side of the workwear;

- The workwear must be illuminated by fluorescent bands on the arms and the breast and back, and below the knees for night visibility.

- Note: The workwear of all contractors must also have their printed logo on it.

Clause 144. Selecting the Right Workwear:

The table below compares different types of workwear that protect the wearer against each class of toxins to help select the right one based on the type of occupation.

| Table 9. Selecting the workwear. |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Welding Workwear | Disposable | Breathable |  | Splash Safety | Chemical Safety |  | Fluid-Resistant | Particulate Safety |  | Fire Safety | Material |  |
| X |  |  |  |  |  |  |  |  |  | X | NomexIII® |  |
| X |  | X |  | X |  |  |  |  |  | X | PR97 ™ |  |
| X |  |  |  | X | X |  |  |  |  | X | Kermel® |  |
|  |  | X |  |  |  |  | X |  |  | X | Emphatex |  |
| X |  |  |  |  |  |  |  |  |  |  | Natural fibers (cotton, wool, leather) |  |
|  | X | X |  |  |  |  |  | IV |  |  | Tempro® |  |
|  | X | X |  | I |  |  |  | I |  |  | Pro/Shield2®(ALL STYLES) |  |
|  | X | X |  | I | X |  |  | I |  |  | Tyvek®(white &blue) |  |
|  | X | X |  | I |  |  |  | I |  |  | Kleenguard LP |  |
|  | X | X |  | X |  |  |  | X |  |  | Kimberly ClarkRepel® |  |
|  |  |  |  | I | X |  |  | I |  |  | Tyvek®QC Sewn and Sewn &Taped Seams |  |
|  |  |  |  | I | X |  | X | I |  |  | Tychem® SL Sealed Seams |  |
|  |  |  |  | I | X |  | X | I |  |  | PVCC overalls and Waterproof Suits |  |
| Nontoxic |  |  | Partial |  |  | Medium |  |  | High |  |  | Toxicity |
| IV |  |  | III |  |  | II |  |  | I |  |  | Degree of Toxicity |

Clause 145. In some workspaces, the workers are exposed to multiple types of hazards (for example, fire, heat, acids, and contaminants), thus requiring appropriate, multi-purpose protective clothing.

Clause 146. Types of workwear and their application:

- Apron;

- Overalls;

- Full-body, chemical-resistant workwear;

- Retro-reflective workwear: For night-time, or road-side operations;

- Fire-safety workwear:  This type of workwear is designed to protect the wearer against fire;

- Particulate-safety workwear: This type of workwear is designed for protection against solid particulates, such as dust, small grains, pellets, balls, etc.;

- Fluid-resistant workwear: This type of workwear is designed to block fluids and prevent their contact with the wearer’s body;

- Chemical-resistant workwear: This type of workwear is designed for protection against chemical fluids.

- Splash/spray-resistant workwear: This type of workwear is designed to protect the wearer against potential sprays and splashes. This type of workwear is neither resistant to fluids nor chemicals and should not be used where there is a risk of fluid or chemical discharge that requires appropriate workwear.

- Disposable workwear: This type of workwear is designed to be disposed of after use;

- Welding workwear: This type of workwear is designed to protect the wearer against sparks and splashes of molten metal during welding.

Clause 147. The use of workwear made of synthetic fibers, such as PVC, or fabrics made of petroleum-based products, polyester, nylon, vinyl, plastic, and other polymers under the workwear is prohibited. These materials are flammable and easily catch fire and melt, thus, sticking to the skin and intensifying the burn.

Clause 148. Taking care of and washing the workwear:

1. Fire-safety workwear must be free of grease, oil, and any other type of flammable material. The workwear should not be shrunk or degraded as a result of cleaning in a washing machine or dryer.

2. When using a washing machine or a dryer to clean fire-resistant clothing:

- Never to wash them together with other clothes;

- Wash them with warm water (below 35 °C);

- Use abundant water, and never use soap;

- Use oxidant-free, natural detergents (pH = 6-8);

- Dry the clothes at around 80 °C;

- Set the dryer/washing machine at mild wash;

- Never use paint and stain remover solvents;

- Never use starch to clean fire-safety clothes for it reduces their quality;

- Particulate-, fluid-, and chemical-resistant apparels must be kept clean to prevent any type of reaction taking place on them;

- The fabrics used to make these clothes is generally capable of drying with no special requirement;

- Before moving contaminated workwear (particulate-, fluid-, or chemical-resistant clothes), wash them with abundant water to remove any trace of hazardous chemicals and fluids.

- In general, follow manufacturer's instructions to keep the workwear clean.

## 12.8 Personal Protective Equipment for Working at Height (Harness)

Clause 149. Using harness is necessary when working at a height of more than 2 meters.

Clause 150. The safety harness and its fastenings must be selected based on the nature of the occupation and with the approval of the HSE department.

Clause 151. The safety harness must be made of high-strength leather, canvas, or hemp.

Clause 152. The minimum harness belt width and thickness are 12 cm and 6 mm, respectively, and the harness is expected to withstand at least 1150 kgf.

Clause 153. The lanyards must be made of high-quality hemp or similar materials and be capable of withstanding a tensile loading of 1150 kgf.

Clause 154. The safety harness and all its parts and accessories require periodic inspection and testing to identify and replace the defective or worn-out parts according to the manufacturer's instructions.

Clause 155. The harness rivets must be carefully and individually inspected to ensure they are free of defects.

Clause 156. The safety harnesses that are found to be incapable of withstanding the loads must be immediately put out of service and labeled accordingly so others do not use them.

Clause 157. The manufacturer recommends changing safety harnesses and their fastenings at least every five years.

Clause 158. Technical specifications for fall protection equipment (with reference to the BS EN 361 standard for full-body harness, the national Iranian standards, and the BS EN 363 standard for fall-arrest systems).

Clause 159. The minimum lanyard diameter is 0.5-inch and it is required to be made of Nylon or similar materials.

Clause 160. The maximum lanyard length must be adjusted to limit the free fall to 1.8 m and must have a minimum breaking strength of 2450 kgf.

Clause 161. The minimum breaking strength for the lifeline is 2450 kgf.

Clause 162. The fall-arrest equipment must be inspected, tested, and certified by the technical inspection department according to the timetable provided by the manufacturer, or at least once every six months.

Clause 163. Never tie lifelines together for it will reduce their performance significantly.

Clause 164. All lifeline parts and accessories must have the same strength as the lifeline.

Clause 165. Using the harness:

1. Inspect the safety harness and lifeline fastenings, especially the D-rings and clip-locks to ensure they are opened and closed easily.

2. The lifeline clip-lock must be attached to a securely stable point, preferably above the head.

Clause 166. Maintenance:

1. Safety harnesses and lifelines must be washed regularly with a natural detergent.

2. The safety harnesses and lifelines must be left to dry naturally after being washed. Make sure the equipment is dried before use.

Never dry the equipment by heating for it will deteriorate the equipment and reduce its strength.

3. Store the safety harnesses and lifelines in a natural position (ready to use) in a dry place, out of direct sunlight.

## 12.9 Special Protective Equipment

Clause 167. PPE requirements for visitor:

- Considering the hazards in the area, visitors are provided with personal protective and safety equipment with the discretion of the HSE department. The visitors are obligated to use the said equipment.

Clause 168. Requirements for emergency showers and eye-wash stations:

1. In cases where there is the risk of irritating chemical splashes on the skin, the workplace must be equipped with emergency shower and eye-wash station.

2. These equipment are typically supplied with tap water, but if that is not available, sanitary water tanks with sufficient capacity for supplying water for a minimum of 15 minutes can be used.

3. The water used for this purpose must be potable and pH-neutral.

4. The emergency shower and eye-wash station must be installed no further than 33 m (100 ft) away from the potential hazard.

Clause 169. Marine safety and flotation devices:

Requirements for life jackets:

1. Wearing a life-jacket is necessary for all occupations where there is a risk of falling into deep water (such as offshore platforms or oil tankers).

2. Putting on a life-jacket, in addition to keeping the wearer afloat and saving them from fatigue, reduces the loss of body heat, thus preventing hypothermia.

3. Life jackets must be selected based on the type of operation and must fit the wearer comfortably.

4. Life jackets are often orange since this color is rarely found in nature, thus improving the visibility of the wearer and facilitating their rescue.

Clause 170. Lifebuoy requirements:

1. Lifebuoys are pieces of PPE designed to provide buoyancy for the person fallen in water.

2. Lifebuoys are necessary on decks, platforms, and facilities adjacent to water that is more than 5 ft deep.

3. Lifebuoys must be installed in sufficient numbers at accessible locations.

4. Lifebuoys cannot be used for those who are unconscious or unresponsive.

5. Lifebuoys are not recommended for non-swimmers.

6. Lifebuoys are, in fact, a type of life-jackets that are not worn but installed at appropriate places to be thrown at casualties.

It is obvious that another person is needed to carry this out.

7. Lifebuoys must be maintained to provide at least 75% of the required nominal buoyancy.

8. The personnel must be trained for using lifebuoys.

Clause 171. Requirements for special protective equipment:

1. In occupations where the feet are at risk, aluminum shin guards must be used in addition to the safety boots.

When the job requires a lot of kneeling, leather knee pads with comfortable linings must be used.

2. To prevent the risks of lifting sheet metal or plate glass with sharp edges, suction cups must be used.

3. When moving sheets of iron with sharp edges or placing them in or retrieving them from the press or cutting equipment, magnet pliers must be used.

4. The use of leather aprons, gaiters, sleeves, hoods, and face guards is necessary in occupations that involve heating, such as welding or melting.

5. The use of skin-care products and lotions is necessary for workers whose occupations involve exposure to skin-irritating chemicals or agents, including oils, organic paints, acids, bases, alcohols, salts, or even radiation.

No. 17, Yaghma Alley, Jomhuri Eslami Avenue, Tehran.

Tel. 66726016	Fax 66709723	SMS 10007615

HSEE@NIOC.IR	HTTP://HSE.NIOC.IR