National Iranian Oil Company Safety, Health and Environment Management Safety Manual for Lifting and Handling Equipment Reviewing Translation; Confirmed by; Publication Date of English Version. Davood Naderi/ Laleh H. Abbasi Training & Performance Measurement Department; Mani Abdollahzadeh Rad/ HSE Manager; September 2018. Responsibility/Subject; Prepared by; Confirmed by; Approved by; Publication date; Descriptions. Name; Mani Abdollahzade/ Fereshte Miraj Head of Safety and Firefighting Department /Project Supervosor of Safety and Firefighting Department; Siavash Derafshi/ HSE Manager; Roknodin Javadi/ CEO; February 2016. Position. Document Code NIOC-HSE-SF-GU-008-00 Revision date: February 21, 2016, Revision No.: 1

Contents

2	Goal	4

3	Scope	5

4	Terms and Definitions	5

Common Terms in the Transport of Loads	5

5	Roles and Responsibilities	6

6	Crane Classes According to the ASME Standard	7

7	Crane Accessories	8

7.1	The Hydraulic System, which consists of:	8

7.2	Rotating Parts, Including:	9

7.3	Hook	9

7.4	Electrical Systems	9

8	General Requirements of Working with Cranes	9

11	References	21

12	Appendices	21

12.2	Appendix 2. Crane Safety Checklist (Periodic Inspection)	25

1	Introduction

Much of activities across the National Iranian Oil Company (NIOC) involve the transportation of materials and equipment. Given the history of fatal and disabling accidents in crane and lift operations in the oil industry, it is essential to observe the HSE requirements and regulations to reduce and control the incidents related to lifting and handling equipment. Considering the requirements of this guideline, the HSE departments of the companies are obligated to develop and implement operating procedures for working with cranes and lifting equipment in cooperation with relevant departments and to specify the responsibilities of the individuals involved in the operation (driver, rigger, maintenance personnel, technical inspector, safety personnel, operational manager, and the owners of cranes) according to the requirements of this guideline and to ensure that the lifting and handling equipment are operated safely and the workers involved in these operations are trained adequately.

Therefore, to protect the health and safety of the human resources and the equipment, the “Safety Manual for Lifting and Handling Equipment” was prepared by the HSE management of NIOC in compliance with current standards and regulations in the oil industry. The subsidiary executive and operating companies of NIOC are obligated to develop a document for the work institutionalization procedure and requirements for the implementation of the said document in their respective companies and contractors in the form of "Safety Manual for Lifting and Handling Equipment" based on these guidelines.

It must be noted that all NIOC’s subsidiaries and contractors are obliged to meet the requirements of this guideline.

# 2	Goal

This manual was developed with the goal of establishing the safety requirements of lifting and handling equipment to reduce the exposure of the workers to the workspace hazards and to enhance the safety of the workers, equipment, and activities.

# 3	Scope

This guideline applies to all subsidiaries and associates, departments, and operational sites of NIOC.

# 4	Terms and Definitions

- Company: In this document, Company refers to the National Iranian Oil Company (NIOC).

- Subsidiary Companies (Subsidiaries): Includes all subsidiaries of the National Iranian Oil Company (NIOC).

- Employer: In this document, the Employer of each contractor is the one specified in the corresponding contract.

- Contractor: Any contractor that has a contract with any subsidiary of the National Iranian Oil Company (NIOC).

- Contractor's Personnel: All personnel working under the supervision of a contractor employed by the National Iranian Oil Company (NIOC) or any of its subsidiaries.

## Common Terms in the Transport of Loads

- Crane:  A lift that can move a load both vertically and horizontally, while hanging.

- Mobile Crane (wheel- or track-mounted cranes): A crane that can move over one or multiple tracks.

# 5	Roles and Responsibilities

5.1	Executive Directors

1. Ensuring that sufficient financial resources are allocated to the procurement of standard lifting and handling equipment, in accordance with the workplace (work site) hazards and the required standards

2. Ensuring that the requirements of this guideline are met.

5.2	Department Directors and Unit Supervisors (technical service, operating, maintenance, support, training, etc.)

1. Ensuring safe working conditions when operating lifting and handling equipment.

2. Implementing the provisions of this guideline and cooperation with the HSE department.

5.3	Departments of Commerce and Logistics

1. The purchase of the lifting and handling equipment in accordance with the specifications of this guideline, with the approval of the HSE and other relevant departments.

5.4	HSE Department

1. Preparing equipment safety manuals/instructions for working with the lifting and handling equipment based on this guideline and cooperating with the relevant units.

2. Supervising the correct implementation of the requirements of this guideline by the respective units through periodic inspections.

3. Training the administrators/workers about safe lifting and handling of equipment.

5.5	Security Office

Cooperating with the HSE department in conjunction with the control of safety requirements of lifting and handling equipment

5.6	Workers (riggers, drivers, etc.)

1. Workers are obligated to abide by the safety requirements before starting the work according to this guideline and other instructions provided by the company.

2. Workers must inform their supervisors of any defects in the lifting and handling equipment and provide them with feedback about the quality of the PPE.

Note: The responsibilities of all units/departments involved in the operation of lifting and handling equipment must be specified clearly in the corresponding operating manual/instructions based on the organizational structure.

# 6	Crane Classes According to the ASME Standard

1. Industrial Cranes

- Over-head cranes;

- Gantry cranes;

- Jib cranes;

2. Construction cranes

- Mobile cranes (wheel- or track-mounted);

- Tower cranes;

- Derricks;

Figure 1. Various cranes.

# 7	Crane Accessories

A crane is composed of the following main parts:

## 7.1	The Hydraulic System, which consists of:

1. The main flow control levers; 2. Telescopic extension cylinders;

3. Boom lift cylinder;

4. The system of crane’s 360° rotation around the centerline (Swing);     5. The hydraulic pump.

## 7.2	Rotating Parts, Including:

1. The sheaves on which cables move; 2. The slewing axis around which the cabin and boom can rotate 360 degrees.

3. The winding drum.

## 7.3	Hook

The hook has a latch that allows the attachment of wire ropes and belts.

## 7.4	Electrical Systems

1. The cut-off switch that stops the hook at a specific distance from the boom and prevents the cables from being torn;

2. Alarm and illumination lights;

3. Horn etc.

7.5	Outriggers

Structures used to stabilize the device during operation.

# 8	General Requirements of Working with Cranes

- Before setting up a new lift, all components (the frame and accessories) must be inspected and tested by an eligible person.

- The mobile machinery and lifting equipment shall not travel in operating units, workshops, and other restricted areas without a valid inspection certificate and operating permit.

- The operators are required to prepare a lifting plan by assessing the risks, identifying the hazards, and making the necessary arrangements before handling or lifting loads.

Note 1: Lifting plans shall be prepared at the discretion of the relevant departments (HSE, maintenance, technical services, transport, etc.) and are essential for the handling of high-risk (critical) loads. The important factors defining a critical load include physical criteria (lifting height, travel distance, how to supply electricity, etc.), environmental criteria (ambient temperature, humidity, elevation, presence of toxic or combustible gases, etc.), and operational criteria (high-risk loads, the minimum and maximum required speeds, the type, the size, the volume, and the weight of the load), etc.

Note 2: The lifting plan must address the following: descriptions of the equipment/loads to be handled, the method of communication with other task groups, the personnel involved in the handling, specifying the physical considerations and the hazards in the area (electrical facilities, gas lines, pressurized pipes, etc.), secondary and special precautions, map of the operation site, and obtaining the related work permits.

- The technical integrity of all cranes must be certified by the trusted authorities for a specific period of time after carrying out the inspection of every component.

- Note 3: The certificate must specify: the testing method and the reference standard, the date of issue and expiry, the inspector’s name and signature, the maximum allowed load, the test load, the equipment specifications in full details, the owner, and the installation location.

- All crane components must be inspected periodically at the intervals specified in the following and the records must be preserved in a suitable place:

1. Daily by the driver before starting the operation;

2. Weekly and monthly by the (mechanical and electrical) maintenance department based on the manufacturer’s checklist)

3. Annually (by an authorized inspector/company, for certification of technical integrity) with the records preserved in a suitable place.

4. Crane inspection includes: making sure that there are no oil leaks, the wire ropes are in good condition, and the cut-off switch and brakes are functional before starting the operation, testing the hook and the levers, engine, the power transmission, the frame, the hydraulic fluid tank, the hydraulic pump, the slewing system, the winch, the boom, the outriggers, and the slings (belts, wire ropes, chains) for wear, tear or crushing, inspecting the hook, the safety equipment, the chain and mediator belts (example checklists can be found in Appendices).

- The crane driver and the rigger are required to have license from Iran Technical and Vocational Training Organization for their ability to operate the cranes. Moreover, they are required to take annual occupational medical examinations in coordination with the HSE department to verify their physical and mental fitness.

- Note 1: The important load identification factors must be examined by the driver before the start of operation, this includes: The weight of load (in kg, lb, tonne, etc.), the shape of load (symmetrical, asymmetrical, square, circular, etc.), the volume (small, voluminous, etc.), contents (solid, liquid, gas), the location of fasteners (placement of hooks, chains, etc.), the center of mass (is it specified or not?), level of hazardousness (chemical, toxic, etc.).

- Note 2: It is essential that the driver’s skills are tested in the following areas: 1. Full attention to and control over the handling; 2. Keeping the personnel and the equipment safe; 3. Halting the operation if unsafe conditions arise; 4. Making sure about the integrity of the equipment and machinery; 5. Paying attention to the rigger’s/operation manager’s signals; 6. Complying with the instructions.

- In cases with more than one riggers for guiding and hoisting the load, only one of them who will remain in the driver’s field of view shall signal him. Anyhow, the driver shall mark the agreed signals from any of the said auxiliary workers.

- Note 1: The rigger is required to be skilled in the following areas:

- Safely strapping and opening the load;

- Guiding the crane driver;

- Placing the load at the intended spot;

- Ensuring the load descends at a secure spot;

- Selecting the correct route for handling the load in cooperation with the driver;

- Knowing how to use fire suppression equipment.

- A fire extinguisher must be installed in the driver’s cabin and the rigger is required to know how to use it.

- The driver must remain seated in the cabin during unloading.

- Vehicles or machinery are prohibited from traveling in the load handling area and operating units.

- Listening to music players, cell phones, etc. while working is forbidden for drivers and riggers.

- The personnel must stay clear of the lift path at all times and shall never remain between the load and another object.

- Before setting up the outriggers, the area should be inspected and suitable timbers must be placed under the outriggers.

- Make sure there are no powerlines above the operating area.

- The minimum safe distance from a powerline is six meters (this must be revised by the HSE department for humid environments).

- Before hoisting the load, the driver must inform others by sounding its horn.

- The S.W.L (Safe Working Load) must be inscribed on both sides of all lifting equipment.

- When handling loads that reach the working load limit, the operation must be controlled and supervised by the safety manager.

- The operation must be suspended in high wind or storm conditions.

- Heavy loads must be installed under the supervision of the safety manager in the morning so there is enough time to resolve potential installation issues.

- The personnel working at height must be moved by man baskets and never suspended from the hook.

- The Safe Working Limit (SWL) of the man basket must be specified.

- The crane operating personnel must be trained for safety in working at heights, working with the crane, etc.

- Using personal protective equipment (helmet, boots, gloves, harnesses, etc.) is mandatory.

- All loading and unloading must be carried out at a slow pace.

- Moving loads over personnel, or important machinery or equipment is forbidden.

- A load shall not remain hanging for a long time.

- Strapping and hoisting a load with an unknown weight or loads heavier than the capacity of the crane is forbidden.

- After the job is done, the crane boom must be repressed and the cabin door must be locked.

- For non-telescopic booms, if the boom is long, it must be laid horizontal on the ground.

- All loads and man baskets must be tethered by a tagline while moving.

- The lifting area must be confined using barricade tapes and warning signs (crush hazard, etc.).

- All slings must have their specifications labeled on them.

- Wire ropes must be inspected according to the ISIRI9296 standard.

- Wire ropes that have more than one strand dented, rusted, or broken in every twenty strands (more than 5%) shall no longer be used.

- All crane components that are under tension or compression and may be subject to an impact must be made of steel or an equivalent alloy.

- Natural or synthetic fiber ropes shall not be soaked with chemicals (acids, bases, etc.).

- The crane hook shall comply with the safety requirements of the ASME B30 standard.

- The hook must come with a latch.

- The full specifications of manufacturer including the material, production process (cast or wrought), and the SWL must be inscribed on the hook with a large and bold font.

- The lifting equipment shall not be welded, cut, hammered, or hot-worked.

- The use of hand-made, non-standard, defective equipment is prohibited.

- In hoisting, lowering, and handling loads by lifts, guiding must be carried out using comprehensible and established signals as instructed by an eligible authority with a signal designated to every move. It is best if hand and arm signals are used.

9	Over-Head Crane Safety Requirements

- The SWL must be inscribed on the frame.

- The lift path must be clearly defined.

- Loads shall not be moved over the personnel or important equipment.

- End stops and buffers must be placed at the end of the tracks to prevent the crane from getting off track.

- A movement warning alarm must be considered for the device, so that its sound can be clearly heard and understood by the workers.

- The crane must be equipped with a red or yellow movement warning light.

- Electrical equipment must be grounded.

- The bridge girders and tracks must be tested and their integrity should be evaluated.

- An anti-shock [rubber] pad must be installed at the end of the tracks to prevent damage to the equipment.

- All devices and equipment must be inspected before starting.

- The lift path must be marked on the workshop floor with yellow paint.

- A safe access way must be planned for the driver.

- If needed, the necessary radio communication equipment must be provided to the driver and the rigger.

- The driver and the rigger and other involved personnel must be trained before starting their job.

- Only authorized personnel may have access to the crane controller; and in order to prevent potential incidents, the controller must have a lock the keys of which shall remain in possession of the person in charge of the crane.

- Motion guide symbols (left, right, up, down) must be installed on the controller.

10	Elevated Work Platform (EWP) Safety Requirements

- In some cases, it is required to elevate the personnel to work at height using a crane. This approach is only acceptable in cases where other methods such as hydraulic lifts, scaffolding, etc. are not feasible or there are more risks to individuals; and all safety requirements are observed.

- To use a lift basket, a comprehensive risk assessment is necessary that considers the operating conditions.

- The personnel must be acquainted with the potential hazards and know how to control them.

- The crane shall not be used for multiple purposes at once. For example, it is forbidden to lift a man basket and a load simultaneously.

- Lifting personnel near powerlines is extremely dangerous and requires the powerline to be cut off and grounded in addition to obtaining the necessary permits.

- Cranes carrying persons must have technical inspection certificates issued by the trusted authorities.

- The necessary permits must be issued by the HSE department before starting the operation.

- The following must be specified in the said permit: the objective of using the man basket, the project location, the working hours, the number of individuals working in the man baskets, crane specifications, the driver’s name, the crane SWL, the basket specifications, the number of persons allowed into the basket, the basket SWL, the empty and total weight of the basket, the test validity date, the inspector’s name, the standard inspection code (SWL), the name of the person verifying the results, the method of communication between the personnel in the basket and on the ground (by transceiver or other means of communication). Find an example form in Appendix 4.

- The crane driver must make sure that the control devices are all functional.

- The location and position of the crane shall not lead to a situation where the load exceeds 50% of the working load limit at that angle.

- In every work shift, before lifting any person, the basket must be loaded up to the required weight and test-lifted by the crane up to the point where it is needed. Then, the related workers can be lifted by it. This must be repeated every time the crane is relocated, the lift path is changed, or another crane is used.

- The workers are required to hook their safety lanyards to a secure spot before getting off the basket.

- One of the workers in the man basket is required to be familiar with rigging standard hand signals.

- The workers in the man basket must be equipped with helmets and full-body harnesses. The harness must be secured to the crane hook directly or using a synthetic belt. In case it is not possible, the harness must be secured onto another safe spot (compliance with safety requirements of working at height is necessary).

- The responsibilities of the driver while raising the man basket include:

1. He must remain in the crane during the entire time and monitor the operation.

2. Listening to the radio or music players or using cell phones during operation is forbidden.

3. If the crane driver is in doubt about something, he shall consult with the operation manager at once.

4. The man basket must be moved slowly and abrupt motions must be avoided.

5. The maximum speed when raising the man basket shall not exceed 100 ft/min.

# 11	References

1. General Safety Regulations of the National Iranian Oil Company.

2. Materials Lifting and Handling Equipment Safety Manual, Ministry of Labor and Social Affairs Research Center.

3. Safety manual for “Operating Lifting and Handling Equipment”, the Health, Safety, and Environmental Department of the Ministry of Petroleum.

- 4. NFPA70, ARTICLE610

- 5. ASME/ANSI  B30, 10…

# 12	Appendices

12.1	Appendix 1. Crane Safety Checklist (pre-operational safety check)

| Description | No | Yes | Item | no. |
| --- | --- | --- | --- | --- |
|  |  |  | Has the equipment been checked for oil leaks, and are the wire ropes, the cut-off switch, and brakes in good condition? | 1 |
|  |  |  | Have the hook and the levers been tested? | 2 |
|  |  |  | Was the location inspected before setting up the outrigger and have timbers been placed under the outriggers? | 3 |
|  |  |  | Has it been verified that there are no powerlines above the crane? | 4 |
|  |  |  | In case there is a powerline, was the crane set up at a safe distance? | 5 |
|  |  |  | Is the rigger familiar with the hand signals and the regulations and is he technically informed about the job? | 6 |
|  |  |  | Is the driver familiar with the chart load? | 7 |
|  |  |  | Are the drivers licensed and are their medical records available? | 8 |
|  |  |  | Is the safety unit informed if the load reaches the working load limit? | 9 |
|  |  |  | Are heavy objects loaded under the supervision of safety authorities and early in the morning (so there is time to fix potential issues)? | 10 |
|  |  |  | Is there a man basket for raising the persons working at height? | 11 |
|  |  |  | Is the S.W.L of the man basket specified? | 12 |
|  |  |  | Are there harnesses available for the personnel working in the man basket? | 13 |
|  |  |  | Are there safety grips inside the man basket? | 14 |
|  |  |  | Have safety arrangements been made for preventing the fall of objects? | 15 |
|  |  |  | Is the man basket secured to the tagline? | 16 |
|  |  |  | Is it forbidden for the driver to listen to music-players, use cell phones, etc. while working? | 17 |
|  |  |  | For non-telescopic booms, if the boom is long, is it placed horizontally on the ground? | 18 |
|  |  |  | Will the boom repressed and the cabin door locked after the job is done? | 19 |
|  |  |  | Is it forbidden to use the release lever (that causes the wire rope to move at a fast pace) when using the techwise? | 20 |
|  |  |  | Are all loads and man baskets tethered by a tagline while moving? | 21 |
|  |  |  | Are different-sized loads handled separately? | 22 |
|  |  |  | Is it forbidden to remain under or on top of a suspended load? | 23 |
|  |  |  | Have standard belts, shackles, wires, hooks, etc. been incorporated? | 24 |
|  |  |  | Is the S.W.L of the equipment specified on them? | 25 |

| Description | No | Yes | Item | no. |
| --- | --- | --- | --- | --- |
|  |  |  | Is the rotating crane set up at a safe distance (600 mm) from stationary objects? | 26 |
|  |  |  | Was the belt inspected for kinks or tear? | 27 |
|  |  |  | Are paddings or softeners placed over the sharp edges of the load to prevent damage to the belt or the chain? | 28 |
|  |  |  | Are loads such as bricks or rocks secured from falling using strong and tall baskets? | 29 |
|  |  |  | Is the lifting area contained using barricade tapes and warning signs (crush hazard, etc.)? | 30 |
|  |  |  | Are two- and three-leg slings attached to shackle using rings? | 31 |
|  |  |  | Are the legs 90° apart? | 32 |
|  |  |  | Are the natural and synthetic ropes kept free of chemicals (acids, bases, etc.)? | 33 |
|  |  |  | Is it forbidden to use crushed wire ropes and cables? | 34 |
|  |  |  | Is it forbidden to extend chains using bolts and screws? | 35 |
|  |  |  | Is it forbidden to tie chains to reduce their length? | 36 |
|  |  |  | Is the lifting area open and clear to give the driver enough space for the operations? | 37 |
|  |  |  | Is the lifting area illuminated adequately? | 38 |
|  |  |  | Are all crane lights functional (boom, front, rear, line, brake, etc.)? | 39 |
|  |  |  | Is the lift path kept clear of obstacles? | 40 |
|  |  |  | Was the unloading and descend area inspected before starting the work? | 41 |
|  |  |  | When working on slopes, have arrangements been made to protect the crane and prevent the load from falling? | 42 |
|  |  |  | Is it forbidden to use endless wire ropes? | 43 |
|  |  |  | Are all slings straight and without kinks? | 44 |

## 12.2	Appendix 2. Crane Safety Checklist (Periodic Inspection)

| Description | No | Yes | Item | no. |
| --- | --- | --- | --- | --- |
|  |  |  | Is the crane S.W.L inscribed on it? | 1 |
|  |  |  | Are lift paths specified clearly? | 2 |
|  |  |  | Is moving over the personnel or important equipment avoided? | 3 |
|  |  |  | Are the driver and the rigger and other personnel involved in the operation trained? Provide documents. | 4 |
|  |  |  | Are the required radio communication equipment provided to the driver and the rigger? | 5 |
|  |  |  | Is a safe pathway planned for the driver? | 6 |
|  |  |  | Is the lift path marked on the workshop floor with yellow paint? | 7 |
|  |  |  | Have all devices and equipment been inspected before starting? | 8 |
|  |  |  | Have [rubber] paddings been installed at the end of the tracks to prevent damages to the equipment? | 9 |
|  |  |  | Are all electrical devices equipped with the earthing system? | 10 |
|  |  |  | Is the sound alarm clearly heard and understood by the workers? | 11 |
|  |  |  | Is the crane equipped with yellow or red warning lights? | 12 |
|  |  |  | Is the crane equipped with end stops at the end of its tracks to prevent it from getting off track? | 13 |
|  |  |  | Is the controller equipped with a lock to prevent potential incidents? Is the key in the possession of the person in charge of the crane? | 14 |
|  |  |  | Is access to the controller restricted to authorized personnel? | 15 |
|  |  |  | Have direction arrows been installed on the controller? | 16 |
|  |  |  | Is there a fire extinguisher in the driver’s cabin? and do the driver and the rigger know how to use it? | 17 |
|  |  |  | Are the belts, wires, chains, and other accessories kept in a dry and suitable place? | 18 |
|  |  |  | Is it forbidden to attach the tagline to the hook, wire rope, or shackles? | 19 |
|  |  |  | Do the drivers have good sight and hearing? | 20 |
|  |  |  | Are the wire ropes decommissioned if they are crushed, rusted, or have more than one strand broken in twenty (more than 5%)? | 21 |
|  |  |  | Are wire ropes lubricated according to the manufacturer’s instructions and are they kept from contact with acids, hot metals, and corrosive materials? | 22 |
|  |  |  | Are outriggers used when the crane is parked? | 23 |
|  |  |  | Are the outriggers fully extended and are they parallel? | 24 |

| Description | No | Yes | Item | no. |
| --- | --- | --- | --- | --- |
|  |  |  | Are hydraulic equipment, such as outriggers and lifting arms, inspected for oil leaks? | 25 |
|  |  |  | Is the safety latch positioned properly to secure the load from falling? | 26 |
|  |  |  | Has the equipment been checked for oil leaks? Are the wire ropes, the cut-off switch, and brakes in good condition? | 27 |
|  |  |  | Have the hook and the levers been tested? | 28 |
|  |  |  | Was the location inspected before setting up the outrigger and have timbers been placed under the outriggers? | 29 |
|  |  |  | Has it been verified that there are no powerlines above the crane? | 30 |
|  |  |  | In case there is a powerline, has the crane been set up at a safe distance? | 31 |
|  |  |  | Is the rigger familiar with the hand signals and the regulations and is he technically informed about the job? (Provide documents) | 32 |
|  |  |  | Is the driver familiar with the chart load? | 33 |
|  |  |  | Are the drivers licensed and are their medical records available? | 34 |
|  |  |  | Is the safety unit informed if the load reaches the working load limit? | 35 |
|  |  |  | Are heavy objects loaded under the supervision of safety authorities and early in the morning (so there is time to fix potential issues)? | 36 |
|  |  |  | Is there a man basket for raising the persons working at height? | 37 |
|  |  |  | Is the S.W.L of the man basket specified? | 38 |
|  |  |  | Are there harnesses available for the personnel working in the man basket? | 39 |
|  |  |  | Are there safety grips inside the man basket? | 40 |
|  |  |  | Have safety arrangements been made for preventing the fall of objects? | 41 |
|  |  |  | Is the man basket secured to the tagline? | 42 |
|  |  |  | Is it forbidden for the driver to listen to music-players, use cell phones, etc. while working? | 43 |
|  |  |  | For non-telescopic booms, if the boom is long, is it placed horizontally on the ground? | 44 |
|  |  |  | Are the boom repressed and the cabin door locked after the job is done? | 45 |
|  |  |  | Is it forbidden to use the release lever (that causes the wire rope to move at a fast pace) when using the techwise? | 46 |
|  |  |  | Are all loads and man baskets tethered by a tagline while moving? | 47 |
|  |  |  | Are different-sized loads handled separately? | 48 |
|  |  |  | Is it forbidden to remain under or on top of a suspended load? | 49 |

| Description | No | Yes | Item | no. |
| --- | --- | --- | --- | --- |
|  |  |  | Have standard belts, shackles, wires, hooks, etc. been incorporated? | 50 |
|  |  |  | Is the S.W.L of the equipment specified on them? | 51 |
|  |  |  | Is the rotating crane set up at a safe distance (600 mm) from stationary objects? | 52 |
|  |  |  | Has the belt been inspected for kinks or tear? | 53 |
|  |  |  | Are paddings or softeners placed over the sharp edges of the load to prevent damage to the belt or the chain? | 54 |
|  |  |  | Are loads such as bricks or rocks secured from falling using strong and tall baskets? | 55 |
|  |  |  | Is the lifting area contained using barricade tapes and warning signs (crush hazard, etc.)? | 56 |
|  |  |  | Are two- and three-leg slings attached to shackle using rings? | 57 |
|  |  |  | Are the legs 90° apart? | 58 |
|  |  |  | Are the natural and synthetic ropes kept free of chemicals (acids, bases, etc.)? | 59 |
|  |  |  | Is it forbidden to use crushed wire ropes and cables? | 60 |
|  |  |  | Is it forbidden to extend chains using bolts and screws? | 61 |
|  |  |  | Is it forbidden to tie chains to reduce their length? | 62 |
|  |  |  | Is the lifting area open and clear to give the driver enough space for the operations? | 63 |
|  |  |  | Is the lifting area illuminated adequately? | 64 |
|  |  |  | Are all crane lights functional (boom, front, rear, line, brake, etc.)? | 65 |
|  |  |  | Is the lift path kept clear of obstacles? | 66 |
|  |  |  | Has the unloading and descend area been inspected before starting the work? | 67 |
|  |  |  | When working on slopes, have arrangements been made to protect the crane and prevent the load from falling? | 68 |
|  |  |  | Is it forbidden to use endless wire ropes? | 69 |
|  |  |  | Are all slings straight and without kinks? | 70 |

| 12.3 Appendix 3. Crane Safety Checklist (Technical Inspection and Over-Head Cranes) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Crane Capacity: Specifications: Location: |  |  |  |  |
| Considerations | No | Yes | Inspected item | Name |
|  |  |  | The crane base is in good condition | Pendant Push Button Station |
|  |  |  | Controller buttons are functional |  |
|  |  |  | The emergency stop switch is functional |  |
|  |  |  | The pendant cable is in good condition and moves freely |  |
|  |  |  | The sheaves guiding the controller are in good condition |  |
|  |  |  | The performance of operator is acceptable |  |
|  |  |  | The operator uses personal protective equipment |  |
|  |  |  | Crane bridge screws are fastened tightly | Bridge Girders |
|  |  |  | The normality of the crane commutators |  |
|  |  |  | The main power switch on the crane bridge is functional |  |
|  |  |  | The electrical enclosure on the crane bridge is secure |  |
|  |  |  | The crane bridges are in good condition |  |
|  |  |  | The stoppages at the end of the crane tracks are in good condition |  |
|  |  |  | Surfaces are clean and there is no sign of wear |  |
|  |  |  | The tracks are in good condition and parallel |  |
|  |  |  | The crane hook grip is standard | Hook |
|  |  |  | The hook is equipped with a safety latch and it is in good condition |  |
|  |  |  | The hook can rotate easily around its vertical axis |  |
|  |  |  | The hook pulleys are in good condition and not loose |  |
|  |  |  | The pulley casings are in good condition |  |
|  |  |  | The sheaves, tracks, and electrical cables are in good condition | Trolley |
|  |  |  | Microswitch functions properly |  |
|  |  |  | The main switch on the undercarriage and the power cables are in good condition |  |
|  |  |  | The ring and grooves on the drum are in good condition |  |
|  |  |  | The lights showing the voltage are on |  |
|  |  |  | The trolley bumpers and end stops are in good condition |  |
|  |  |  | Brake pads function properly in different maneuvers |  |
|  |  |  | No abnormal noises or oil leaks are in the gearbox |  |

| Considerations | No | Yes | Inspected item | Name |
| --- | --- | --- | --- | --- |
|  |  |  | The wire rope appears to be in good condition | Wire rope |
|  |  |  | The wire rope is lubricated |  |
|  |  |  | The apparent diameter of the wire rope matches the standard |  |
| Auditor’s Name and Signature Date of Audit Archive: |  |  |  |  |

12.4	Appendix 4. Man Basket Safety Checklist

| Requested by Company Date of Submission Project Location: Operation Date: The number of workers in the man basket: | Name: Objective: Start time: Finish time: |
| --- | --- |
| Crane Specifications Driver’s Name: Crane Model: Serial No. Safe Working Load (S.W.L): |  |
| Man Basket Specifications Safe Working Load (S.W.L): The total weight of the man basket (the basket + contents): Test Results Expiry Date: Inspection Standard Code: | Capacity (number of personnel): Man basket empty weight: Auditor: |
| Verified by Signature | Name: |
| Description |  |

Health, Safety, and Environmental Management

No. 17, Yaghma Alley, Jomhuri Eslami Avenue, Tehran.

Tel. 66726016	Fax 66709723	SMS 10007615

HSEE@NIOC.IR	HTTP://HSE.NIOC.IR