National Iranian Oil Company Safety, Health and Environment Management Scaffolding Use and Installation Guideline Reviewing Translation; Confirmed by; Publication Date of English Version. Davood Naderi/ Laleh H. Abbasi Training & Performance Measurement Department; Mani Abdollahzadeh Rad/ HSE Manager; September 2018. Responsibility; Prepared by; Corroborated by; Approved by; Publication date. Full Name; Mani Abdollahzadeh [Signed] Elham Jahani Nik Head of Safety and Firefighting Department Safety and Firefighting Specialist; Siavash Derafshi [Signed] Head of Health, Safety, and Environment Department; Reza Khalili [Signed] Non-industrial operations Deputy; Ali Kardor [Signed] CEO; December 2016. Position. Document Code: NIOC-HSE-SF-GU-028-00 Revision date: December 10, 2016 Revision No.: 0

Table of Contents

1. Introduction	3

2. Objective	5

3. Scope of Application	5

4. Definitions and Terms	5

5. Legal References and Requirements	9

6. Roles and Responsibilities	9

6-1 Employer	9

2. Employer’s Supervisor	9

6-3 Employer’s HSE Department	11

6-4 Security Department	11

6-5 Contractor’s Safety Expert	11

6-6 Scaffolding Installation Officer	13

6-7 Scaffolding Installation Group and Contractor’s Workforce	13

7. Scaffolding Use and Installation Proceedings	14

7-1 Issuance of the Installation Permit	14

7-2 Receipt of the Installation Permit	15

7-3 Start of the Scaffolding Installation Operation	15

7-4 Completion of the scaffolding Installation Operation	15

7-5 Scaffolding Dismantling and Removal	16

7-6 Inspection and Control	16

7-6-2 Employer’s HSE Department	19

7-6-3 Employer’s Supervisor	20

8. The need to Install and Use Scaffoldings	21

8-1 Scaffolding Components	21

8-2 Types of scaffolding	27

8-3 Types of Scaffolding Hazard	29

Falling from heights caused by slipping/tripping, unsafe access, and failure to use personal fall arrest equipment:	29

Head colliding with objects, injury from falling tools, equipment, and waste:	29

Electric shocks caused by collision or contact with power transmission lines:	29

Scaffolding's collapse, damage, and malfunction caused by imbalance or overloading:	30

Unsafe board installation:	31

9. General Safety Requirements to Consider When in Scaffolding Selection, Installation, and Use	31

10. Special Safety Requirements to Consider When in Scaffolding Selection, Installation, and Use	35

10-1 Scaffolding Components:	35

10-2 Vertical Poles or Standards:	37

10-3 Tubes and Couplers:	37

10-4 Toe Board:	39

10-5 Staircase:	40

10-6 Ladder:	41

10-7 Ground Conditions	43

10-8 Sole plate:	43

10-9  Board:	43

10-10 Working platforms	46

10-11 Ropes:	49

10-12 Instruments and Equipment Storage:	49

10-13 Lift:	51

11. Personnel Training	51

12. Preparing for Emergency Response	53

13. Appendices	54

# 1. Introduction

Scaffolding is used in a wide range of industries and activities as a safe and reliable structure helping workers do their job at heights. This solution helps reduce the risks of working at heights. In the petroleum industry, many activities are impossible without the use of scaffolding. Thus, given the scaffolding's wide range of applications in this industry, the occupational accidents caused by use of unsafe scaffolding when working at heights, and the high number of fall from height accidents, and their irreparable consequences, especially during the last 6 years (with an occupational fatality rate of 16%), the necessary measures should be taken to provide safe scaffoldings for different projects. The safety requirements should be taken into consideration in the selection, installation, and use of these structures. The relevant personnel should also receive the necessary trainings and the regular inspection and control visits should be given high priority.

The safe and appropriate use of scaffoldings requires that the best scaffolding system be selected for the project at hand. In cases where the basic features of the selected scaffolding are not appropriate for the job or all the necessary components or parts are not available, occupational accident is likely to happen. The appropriate selection of scaffolding and its components requires a thorough understanding of both the project and the site. In this regard, the relevant considerations include:

- The weight of workers, tools, materials, and equipment to be supported by the scaffolding

- Working conditions

- Height(s) at which the scaffolding is going to be installed

- Type of activity to be carried out n the scaffolding

- Project Duration

- The supervisor's or personnel’s experience and the different types of scaffolding available

- Weather conditions

- Ladder or other equipment used to access the working platforms

- Barriers

- The structure of the building or construction the workers are working on

- Installation and dismantling problems (Preventing scaffolders from falling from heights, use if mechanical equipment in the installation process to help scaffolders, etc. )

- Conditions of walkways under the scaffoldings

NIOC’s associate companies and subsidiaries should prepare and implement their own “Scaffolding Use and Installation safety Instruction” based on the present guideline. This personalized instruction should also be implemented by the companies’ contractors. The instruction will guide them on how to institutionalize the working procedure and will help them understand the requirements for the implementation of the present document.

It should be noted that the implementation of the provisions stipulated in the present guideline (serving as the minimum requirements) is mandatory for all NIOC’s affiliate companies and their contractors.

# 2. Objective

The present guideline aimed at explaining the requirements associated with the installation and use of scaffoldings in projects, repair activities, etc. at NIOC’s level.

# 3. Scope of Application

The guideline applies to all NIOC’s subsidiaries, associate companies, departments, and operational areas as well as those contractors working under project or volume contracts.

The present guideline should be followed in all the processes associated with the building's and production unit’s repair and maintenance, workplace management, and installation and construction projects at NIOC's level.

# 4. Definitions and Terms

-  Company: Hereafter in this document, the word “Company” shall mean “National Iranian Oil Company (NIOC)”.

- Affiliate Companies: Affiliate companies shall include all NIOC’s subsidiary and associate companies.

-  Employer: In this document, for each contractor, “Employer” shall be the employer referred to in the relevant contract.

-  Employer’s Supervisor: Employer’s supervisors shall be the employer’s representative who has completed the necessary technical training associated with scaffolding use and installation

It should be noted that, in each company,  this representative is selected from the relevant department through some predetermined organizational processes.

-  Contractor: All the contractors who have signed a contract with one of NIOC’s affiliate companies.

- Contractor's Workforce: Contractor’s workforce are all the personnel working under the supervision of a contractor party to the contract with NIOC or any of its affiliate companies.

-  Scaffolding: Scaffolding is a temporary metal structure consisting of at least one working platform, couplers, fittings, and supports. They are installed in industrial or non-industrial sites and used in repair activities, construction operation, etc. to get access to the buildings, support work crew and materials at heights.

-  Scaffolding Installation Group: The scaffolding installation group is a group of workers assigned to install the scaffolding. In other words, the members of this group are assigned to install, put up, and dismantles the scaffolding whenever necessary.

-  scaffolder: The scaffolder is the worker responsible for installing the scaffolding and dismantling it after the activity has been announced as completed.

-  Scaffolding Installation Officer: Within the scaffolding installation group, there is a scaffolding installation officer. This officer is an experienced technician and has successfully received the credible training certificates in the field of scaffolding installation.

-  Contractor’s Safety Expert: All of the contractor’s safety personnel having the relevant practical and theoretical experience qualifying them to identify hazardous conditions. These experts are responsible for identifying hazardous conditions and taking the appropriate measures to avoid hazards.

-  Hazardous Conditions: Hazardous conditions are conditions that are likely to contribute to accidents (e.g., use of broken planks/boards and/or failing to safely secure the scaffolding)

- Scaffolding’s Safety Card: The safety card contains a set of scaffolding safety regulations (back) and a periodic inspection and approval section (front). The relevant personnel inspect and control the quality of the structure and its components based on the information found on this card.

# 5. Legal References and Requirements

- 5-1 NIOC General Safety Regulations - 2013

- 5-1 NIOC General Safety Regulations - 2013

- IPS Standard No. IP-E-SF-400 (latest edition: 2015)

- Contractors’ HSE Management Guideline - Document No. NIOC-HSE-SF-GU-004-01

- Personnel Protection Equipment (PPE) and Safety Guideline - Document No. NIOC-HSE-SF-GU-014-00

- Permit to Work System Guideline - Document No. NIOC-HSE-SF-GU-010-00

- Occupational Health and protection Codes of Practice - Ministry of Labor

- Working at Heights Code of Safety - Ministry of Cooperatives, Labor, and Social Welfare - Labor Relations Department - General Directorate of Labor Inspections

# 6. Roles and Responsibilities

## 6-1 Employer

-  Ensuring the fulfillment of the guideline’s requirements

## 2. Employer’s Supervisor

-  Investigating the technical qualification of contractor and scaffolders

-  Supervising the proper implementation of the guideline's requirements by contractors or relevant construction companies throughout the scaffolding installation process and submitting reports on the operation's hazardous and unsafe conditions to the employer’s HSE Department

-  Suspending the scaffolding installation operation until the unsafe conditions are improved

## 6-3 Employer’s HSE Department

 Preparing the scaffolding Use and Installation Instruction according to the present guideline

 Supervising the proper implementation of the guideline's requirements by contractors or relevant construction companies through periodic inspections or case-by-case investigations according to the relevant checklists (attached is a sample checklist)

 Providing supervisors with guidance and training on the installation and use of scaffoldings so that they can offer training to their own scaffolding installation personnel

 Interacting and communicating with the employer’s supervisor to ensure the whole scaffolding installation process is being carried out under safe working conditions

## 6-4 Security Department

-  Interacting with HSE Department regarding the safe working conditions throughout the installation process and use of scaffolding

## 6-5 Contractor’s Safety Expert

-  The contractor’s safety expert should be obliged to take the following measures:

-  Reporting the unsafe working conditions, proposing corrective solutions to the scaffolding instillation officer, suspending the project until the unsafe working conditions are improved, and communicating the case to the employer’s supervisor

-  Commenting on the changes to be made to the installed scaffolding’s

- structure and equipment and technically approving the changes before they are implemented by the scaffolding installation group (in such cases, the scaffolding’s safety card should be refilled and re-approved)

-  Holing training programs for scaffolders and scaffolding installation officer regarding the present guideline’s requirements, methods, and procedures

-  Coordinating the contractors with the employer’s HSE Department and cooperating with them to fulfill the requirements stipulated in the prepared instruction throughout the scaffolding use and installation process

-  Deploying the Scaff Tag System

-  Developing a codified system to register the installation date, the inspection results, and the permits obtained

## 6-6 Scaffolding Installation Officer

-  The scaffolding installation officer is responsible for all operations regarding the installation and dismantling of the scaffolding.

-  The scaffolding installation officer is responsible for keeping and distributing scaff tags and safety cards and submitting the completed safety cards to the Safety Department.

## 6-7 Scaffolding Installation Group and Contractor’s Workforce

-  All scaffolders and contractor’s workforce involved in the scaffolding use and installation shall be obliged to follow all the requirements stipulated in the present guideline and other relevant instructions prepared by the companies.

-  All scaffolders and contractor’s workforce involved in the scaffolding use and installation shall be obliged to report any defect, unsafe working conditions, and unsafe conduct to the scaffolding installation officer and contractor’s safety expert.

# 7. Scaffolding Use and Installation Proceedings

## 7-1 Issuance of the Installation Permit

First, the construction company or its Repairs Department determines if there is a need to install and use scaffolding in its production unit, and/or non-operational sites. This decision is made while taking into consideration the definitions of the term “working at heights”. Next, the additional scaffolding installation permit will be issued base on the Permit to Work System Guideline - Document No. NIOC-HSE-SF-GU-010-00.

## 7-2 Receipt of the Installation Permit

The scaffolding installation group gets notified of the issuance of the installation permit and the scaffolding installation officer visits the site to check the working conditions and make sure they are completely safe, before signing and receiving the work permit.

## 7-3 Start of the Scaffolding Installation Operation

After receiving the work permit and making sure the working conditions are safe, the scaffolding installation officer starts the scaffolding installation operation. The operation is carried out in coordination with the contractor’s safety expert and the scaffolding installation group. No one, except the aforementioned mentioned individuals is allowed to use, climb or dismantle the scaffolding.

## 7-4 Completion of the scaffolding Installation Operation

After the completion of the scaffolding installation operation, the scaffolding installation officer visits the site to complete the relevant checklist. The visit serves to ensure that the scaffolding is completely safe and all the safety principles and regulations are met throughout the process, and to prepare the site for the contractor’s safety expert’s visit who is going to approve the operations’ completion. The officer approves the completion of the scaffolding instillation operation and fills the relevant parts

on the scaffolding’s safety card and permit papers.

## 7-5 Scaffolding Dismantling and Removal

After it was confirmed that there is no further need for the scaffolding (notified by the operator), the scaffolding installation group is obliged to dismantle the structure while taking the relevant safety regulations into consideration, and to transfer its components and scaff tags to the specified storage (or a special place prepared for the storage of scaffolding components). Next, the scaffolding's safety card should be returned to the Safety Department.

## 7-6 Inspection and Control

7-6-1 Contractor’s Safety Expert

-  The contractor’s safety expert is notified of the completion of the scaffolding installation operation and pays a visit to the site to inspect tp closely inspect the structure and carefully run through the safety check list's items and answer them by taking the real conditions into consideration.

- The scaffolding safety expert approves the scaffolding and completes the relevant parts on the safety card, before putting it into the scaff tag.

Note: If the contractor’s safety expert determines that the scaffolding is unsafe, the scaffolding installation officer is obliged to immediately take the necessary measures needed to make it safe. Next, the officer should coordinate with the contractor’s safety expert for another visit. If all goes well, the safety expert approves the scaffolding and signs the relevant safety card.

The contractor’s safety expert should regularly inspect the scaffolding to make sure it fully complies with the safe work conditions requirements and record the inspection results. The expert should take the following points into consideration:

A) The scaffolding should be in a stable condition.

B) The materials used in the scaffolding’s construction are intact and without defect.

C) The scaffolding is suitable for the project in question.

D) The necessary safety equipment are available.

E) The working platform should always be open and empty.

F) The working platform's passageway should be of a sufficient width.

G) Making any modifications to the scaffolding without notifying the scaffolding installation officer is prohibited.

H) The scaffolding should only be used for its intended application.

I) Any accident and damage to the scaffolding’s structure (e.g., falling, tilting, vehicle collision, and loose channels under the standards) should be communicated to the scaffolding installation officer.

- During the period when workers are working on the scaffolding, the contractor’s safety expert should pay daily visits to the site to make sure the scaffolding is safe and to inspect the potential changes made to its structure. These periodic visits end with the expert completing the safety card as a sign of approval. If modifications are made to the structure and/or the working conditions are unsafe, the expert removes the safety card from the scaff tag and records the defects on the card, while notifying the scaffolding installation officer of the problems needing to be addressed.

During this daily visits, the expert should make sure that all the members of the scaffolding installation group use the protective gears helping them avoid falling when working at heights. Theses inspections should follow the requirements stipulated in the Personal Protection Equipment (PPE) Guideline, Document No. NIOC.HSE.SF.GU.014.00. In addition, after extreme weather conditions, long-term suspension of operation, accidents resulting in damages to the scaffolding (e.g., vehicle collisions), and repair and maintenance of the Scaff Tag System, the scaffolding should be inspected.

Note: Scaff tags are used to notify users whether the scaffolding’s is safe. They are resistant to heat, sunlight, humidity, and rain. Special markers are used to write notes on the markers. The green scaff tag means the scaffolding is approved to be safe for use. This scaff tag is hung from the scaffolding by the scaffolding installation officer. The scaff tags should be easily visible to and identifiable by individuals entering the scaffolding. The red scaff tag means the scaffolding is not approved to be safe for use. Recommendations, safety tips, inspection points, defects, and inspection dates are written on the back of the green scaff tag. The back of the green scaff tag is yellow.

## 7-6-2 Employer’s HSE Department

The employer’s HSE representative should be obliged to conduct periodic or case-by-case inspections (based on the reports by the employer’s supervisor or other individuals referring to the unsafe working conditions at the site in question) to supervise the proper implementation of requirements stipulated in the present guideline by contractors or construction companies.

Note:  The periodic inspections should be conducted at specific time intervals and according to the relevant c checklist (attached is a sample checklist).

## 7-6-3 Employer’s Supervisor

Throughout the scaffolding use and installation process, the employer’s supervisor supervises the proper implementation of the requirements stipulated in the present guideline by contractors or relevant construction companies.

Note: All the measures taken by the scaffolding installation officer (issuance of installation permit, receipt of the installation permit, starting of the installation operation, completion of the installation operation,  and dismantling and removal) should be approved by the employer’s supervisor.

# 8. The need to Install and Use Scaffoldings

Scaffolds are used when ladders and other similar equipment cannot be used to provide safe working conditions for workers working at heights.

## 8-1 Scaffolding Components

The scaffolding components are plates made of wood/metal placed under the scaffolding. These components include:

-  Base plate: Base plates are metal plates used under standards (between ground and standards) to distribute load.

Plank: Planks are boards used in scaffoldings.

- Toe board clip: The toe board clip is a clip securing the toe board.

Brace: Braces are tubes attached transversely to the scaffolding. They are used to reinforce the scaffolding and support its structure.

Bay length: Bay length is the space between two adjacent standards in the length and along the face of a scaffolding.

Board: Board is made of a soft fire-resistant wood. It is used to access various parts including the scaffolding's working platforms.

-  Butlers: Butlers are tubes securely attached to face braces, reinforcing the scaffolding’s structure.

-  Castor: Castor is a revolving wheel with locking brakes. It is attached to a standard, enabling the easy rolling movement of the scaffolding.

-  Coupler: Couplers are links connecting and tying two tubes together. They are available in two types: load beating couplers and non-load bearing couplers.

-  Guard rail: Guard rail is made of tubes fixed to the interior part of the scaffolding to afford protection to the individuals on the scaffolding and prevent them from falling down.

-  Hoop iron: Hoop iron is metal belt attached to one end of the board, protecting it from breaking or cracking.

-  Ledger: Ledger is a horizontal beam attached to a standard (by load-bearing couplers), preventing it from being bent.

-  Access platform: Access platform is a platform made of wood, providing access to the ladder.

-  Lift: A lift is an assembly of ledgers, transoms, and load-bearing boards, forming each horizontal level of the scaffolding.

- Need transom: Need transoms are transoms sticking outwards of the scaffolding.

-     Node point: Node points are where transoms, ledgers and standards meet and intersect.

- Puncheon: Puncheon is a vertical tube coupled to the horizontal tube at its lowest point. It does not directly transfer the load to the base plate or the ground.

-  Raker: Raker is an inclined load-bearing tune used to reinforce the scaffolding.

-  Scaff tag: Scaff tag is a special tag hung at all entries to the scaffolding. It shows that the scaffolding has been inspected. The scaff tag is either green (safe scaffolding) or red (unsafe scaffolding).

-  Sleeve coupler: Sleeve coupler is a coupler used to join one tube to another. The sleeve coupler’s core is made of metals such as cast iron.

-  Sole Plate: Sole plate is a short board installed under the base plate, distributing the pressure. Is is widely used in situations where the ground is loose. Sole plates are placed between ground and base plates.

-    Spigot join pin: Spigot join pins are used to join one tube to another. They can be found at the end of the scaffolding.

-     Standard: Standards are vertical tubes used to transfer load to the ground's surface.

-  Stop end: Stop end is a guard rail used at the end of the scaffold to mark the parts without broads and prevent individuals from falling down.

-     Tie: Ties are tubes used to attach the scaffolding to an anchor.

-    Toe board: Toe board is board used on the edge along the platform to prevent materials and tools from falling down.

-     Transom: Transoms are tubes connecting face standards to internal standards.

- 	Putlog coupler: Putlog couplers are couplers joining transoms to ledgers.

-     Joint pin: Joint pin is a component used to join two longitudinal tubes to one another.

-     Spigot coupler: Spigot coupler is a tube used to join two longitudinal tubes to one another.

## 8-2 Types of scaffolding

The type of scaffolding to be installed at the site is determined and approved by the employer’s supervisor based on the type of work and the needs assessment report prepared by the requesting unit.

Some types of scaffolding include:

1. Single scaffoldings: Single scaffoldings are transversely installed at the site. Their lower end is placed on the ground level while their body and upper end are attached to the walls or inserted in the holes made into the wall.

2. Independent Scaffoldings: Independent scaffoldings consist of two rows of standards and ledgers. They are not supported by any wall or building.

3. Suspended Scaffoldings:

Suspended scaffoldings are suspended by steel harnesses (at least two harnesses) from an overhead support system. Their upper ends are fixed to hangers and/or concrete parts of the buildings or their metal frame. Pulleys are used to raise or lower the working platforms.

4. Cantilever scaffoldings: Cantilever scaffoldings are hung from two steel harnesses and heir working platform can move in all directions.

5. Prefabricated scaffoldings: Prefabricated scaffoldings are secured and fixed into the inside of the building using tubes and ledgers and protrude over its facade.

6. Tube and coupler scaffoldings: Tube and coupler scaffoldings are the most widely used scaffolding. They consists of two rows of tubes parallel to the building. A transom coupler is used to join these two rows of tubes.

Tube and coupler scaffoldings should be attached to a building.

7. Putlog Scaffolding: Putlog scaffoldings are made up of tubes and couplers to accommodate working platforms close to the wall/building. They consist of one row of standards parallel to the wall. The standards are connected to one another and supported by putlogs. The standard’s free end in fixed into the wall.

## 8-3 Types of Scaffolding Hazard

Throughout the risk assessment process, the construction company should take into consideration the following scaffolding hazards and provide appropriate solutions to the.

## Falling from heights caused by slipping/tripping, unsafe access, and failure to use personal fall arrest equipment:

Personal fall arrest systems should be used to prevent workers from falling from heights when climbing/descending the scaffoldings or working at places without proper fencing. Fences and safety barriers (guard rails/mid rails/top rails), grips, top fences, and mid fences should be installed. Guard rails should be installed at both sides and both ends of the scaffolding.

If the space between the platform and the working site is more than 35 cm, guard rails are not sufficient and the worker should wear the special safety belt.

Top rails are installed 1 m from the platform.

Mid rails are installed between the platform and top rails. Toe boards are installed on the edge of the scaffolding and has a height of 15 cm.

## Head colliding with objects, injury from falling tools, equipment, and waste:

To avoid such hazards, workers should wear safety helmets equipped with chin straps. A safety net should be installed on top of the working site.

Toe boards should be used as edges of the working platform.

## Electric shocks caused by collision or contact with power transmission lines:

When working at heights or on top of the scaffolding, be careful around the power transmission lines and equipment and keep your safe distance.

## Scaffolding's collapse, damage, and malfunction caused by imbalance or overloading:

Appropriate materials should be used to avoid scaffolding’s collapse and falling. Harness clamps should be used and tubes should be securely fastened and fixed.

The ground on which the scaffolding is installed should be hard and solid and the scaffolding should be fixed to the main structure (e.g., building, structure).

## Unsafe board installation:

The surface of the working platforms should be covered with appropriate boards.

# 9. General Safety Requirements to Consider When in Scaffolding Selection, Installation, and Use

The common mistakes contributing to unsafe scaffolding installing and use are as follows:

1. Scaffolding’s unsafe installation and dismantling

2. Failing to wear personal protection equipment while climbing and descending scaffolding

3. Breaking or slipping of working platform’s boards

4. Inappropriate loading/overloading

5. Using platforms without guard rails

6. Failing to install all the necessary components

7. Making contact with overhead electrical wires

8. Moving the scaffolding with workers on the working platform

9. Failing to fully adjust the platform during scaffolding’s installation, use, and dismantling

The following safety tips should be taken into consideration by the relevant individuals during the scaffolding installation process:

-  It is mandatory to observe all the safety regulations associated with working at heights throughout the whole process.

-  The scaffolding should be installed by skilled and experienced scaffolders and be subject to periodic inspections.

-  Appropriate materials in adequate quantities should be supplied for and used in the scaffolding installation operation.

-  Couplers, tubes, and joints should be handled with utmost care.

-  Use of broken, damaged, and bent parts is strictly prohibited.

-  Unauthorized personnel should not interfere with the scaffold installation operation.

-  It is prohibited to throw objects, tools, and ... from the top of the scaffolding to the ground.

-  Safety belts and foot grips should be used to ensure the safety of personnel working on the scaffolding.

-  All scaffoldings (except for independent ones) should be horizontally and vertically secured to the building at appropriate spacings.

-  In hazardous weather conditions (storms, fierce winds), scaffolding installation operation should be suspended and no one should be allowed to work on the scaffolding.

-  All the accessories and components of the scaffolding along with the purchased materials to be used in the installation process should be tagged with special color codes. These color codes will make any defect easily identifiable at sight.

- IT is prohibited to use any defective, broken, bent, and damaged component in the scaffolding installation operation. All such components should be removed from the site.

-  All scaffoldings should be properly and adequately secured.

-  According to the requirements stipulated in n the Personal Protection Equipment (PPE) Guideline, Document No. NIOC.HSE.SF.GU.014.00, all the individuals working on the scaffolding should use the necessary personal protection equipment.

-  Before starting the operation, all the personnel working on the scaffolding (scaffolders, etc.) should attend special training programs. After successfully completing their training and achieving the required score in both theoretical and practical tests, the personnel are allowed to start working on the scaffolding.

-  A special card will be issued for all scaffolders, indicating that the holder has successfully completed the relevant training program and is allowed to work on the scaffolding while following all the required safety regulations.

- Individuals suffering from acrophobia and those with insufficient experience or knowledge of working on scaffoldings should not be employed.

-  Putting up, installing, and making fundamental changes to scaffoldings should only be done under the control and supervision of a qualified and responsible individual. So far as possible, all these operations should be performed by workers with sufficient experience in the field.

-  The number and type of the components should be recorded and kept in a special book.

-  Any defective part of the scaffolding needing repair should be fixed before letting workers climb the structure and start working on it.

-  The scaffolding should be fully dismantled so that no one can work on the remaining parts, unless working on the remaining parts is allowed based on the relevant regulations.

-  The scaffolding should have a safety factor of four times the maximum loading allowed.

-  So fas as possible, the load born by the scaffolding should be distributed evenly. Uneven distribution of load is dangerous as it might disrupt the scaffolding’s balance.

# 10. Special Safety Requirements to Consider When in Scaffolding Selection, Installation, and Use

All the personnel involved in the installation and use of the scaffolding are obliged to follow the safety requirements stipulated in IPS Standard (IP-E-SF-400; latest edition). Some of the special safety requirements are as follows:

## 10-1 Scaffolding Components:

-  The scaffolding should be maintained and kept in good condition. All its components should be safely fastened, fit in, or secured so that they do not move out of place due to the continuous use of the scaffolding.

-  The scaffolding’s metal parts should not be cracked, rusted, etc. as these defective parts are likely to compromise the scaffolding’s strength.

-  The scaffolding components and all the relevant accessories should be made of the finest materials and be designed, manufactured, and prepared so that they comply with the safety working standards and be able to bear the associated loadings and pressures.

-  The tools and equipment used in the scaffolding installation operation should be kept and stored in good conditions. Use of inappropriate tools and equipment should be avoided.

-  During temporary storage, the scaffolding components should not block the roads.

## 10-2 Vertical Poles or Standards:

-  A good scaffolding is a scaffolding whose standards are all lined in a straight line.

-  The standards should be safely secured to avoid potential swinging, moving, and shaking of the scaffolding.

-  Standards play a significant role in contributing to the scaffolding’s strength.

## 10-3 Tubes and Couplers:

Scaffoldings are made by connecting tubes vertically and horizontally. The vertical and horizontal spacings will vary with the type and application of the scaffolding.

-  Never use tubes to climb up/down the scaffolding.

-  The scaffoldings tubes are made of metals and should be strong and rigid.

-  Use of bent and thermally cut tubes is prohibited.

-  The tube ends should not have any fissures or cuts.

-  Sleeve couplers should be used to join two adjacent tubes. These sleeve couplers must have a joint pin and be fully secured by bolts and nuts.

-  Tubes, boards, and planks should not be painted. Galvanized coatings or liners are only allowed to be applied to tubes, boards, and planks.

-  Tubes should be stored longitudinally.

-  Sleeve couplers should be used to join one ledger to another. Use of spigot join pins to join ledgers together is prohibited.

-  Face braces should be secured at appropriate spacings determined based on the relevant standards.

-  Internal braces should be coupled to ledgers at appropriate spacings.

-  When dismantling the scaffolding, tubes and couplers should not be thrown on the ground.

-  After use, all couplers, tubes, and components should be inspected.

-  Defective tubes and couplers should be removed from the site.

-  All couplers and clips should be regularly lubricated and kept in a mixture of diesel fuel and oil to improve their durability.

-  90 degree angle couplers are suitable for load-bearing applications and are used to vertically join the scaffolding’s tubes.

-  Rolling couplers are not suitable for load-bearing applications and are used in a wide range of angles.

-  Couplers should be sorted by their type.

Note: The following should be determined according to the latest edition of the relevant standard (IPS-E-SF-400):

Standard spacings, vertical floors spacings, tube spacings along the platform’s width, maximum protrusion of ledgers from couplers, ledger vertical spacings, face brace spacings, acceptable load for coupler’, and the minimum safe load for couplers.

## 10-4 Toe Board:

Toe boards should be installed on the edge of the platform to prevent objects and materials from falling down on the ground.

Note: The toe board’s width and minimum installation height are determined according to the latest edition of the relevant standard (IPS Standard No. IP-E-SF-400).

## 10-5 Staircase:

-  Staircase is the most suitable equipment used for safe access to the platform.

-  In higher elevations, a landing should be used to allow the user a rest.

-  Staircase is the point of entry into the scaffolding. It should be identifiable at sight with the help of the warning signs.

-  In gentle slopes and as required, an inclined plane can replace the staircase.

-  In permanent structures, staircases are of a vertical type and should be have backings.

-  Boards used in the staircase to facilitate movement should be secured with wires.

Note: The landing’s and backing’s minimum installation heights, the step’s interval, and the width of the staircase entry should be determined according to the latest edition of the relevant standard (IPS Standard No. IP-E-SF-400).

## 10-6 Ladder:

Ladder is the most dangerous part of the scaffolding and has different forms and applications.

-  Make sure all parts of the ladder are inspected before starting the work

- Use insulated ladders for electrical work.

-  The ladder used in the scaffolding should be made of metal. Use of short and wooden ladders is prohibited.

-  The ladder’s feet should form an acceptable angle with the ground as stipulated in the relevant standard.

-  The upper and lower parts of the ladder should be safely secured to a surface.

-  The individuals should face the ladder while climbing/descending it.

-  Users should use both their hands while climbing the ladder.

-  It is prohibited to carry tools in one’s hands while climbing/descending the ladder.

-  The ladder should be safely secure both at the upper and lower ends to prevent it from slipping.

-  Tall ladders should be secured in the middle by a harness.

-  The ladders should be intact and inspected for potential damage after during each visit.

Note: The ladder’s maximum vertical interval, and the angle between the ladder’s feet and the ground should be determined according to the latest edition of the relevant standard (IPS Standard No. IP-E-SF-400).

## 10-7 Ground Conditions

-  The ground where the scaffolding is going to be installed should be strong and rigid and be composed of rammed earth.

-  Sole plates and base plats should be place under the standards.

-  Slopping grounds should be dug like steps. Use of sole plates and base plates in mandatory.

Note: The minimum area under each standard, the minimum diameter of standards, the minimum diameter of planks used as sole plates, and the minimum area of sole plates used for different ground types should all be determined according to the latest edition of the relevant standard (IPS Standard No. IP-E-SF-400).

## 10-8 Sole plate:

-  The sole plate’s dimensions should be determined based on the ground type.

-  Blocks, broken planks, and barrels should not be used under the sole plates.

-  The sole plates should be placed under all standards.

-  The sole soles should be placed under all metal standards unless the iron/concrete surface is rigid.

-  Sole plates should be placed in parts of the cable or rope where cuttings and erosion is likely to protect them against such damages.

## 10-9  Board:

-  Boards should be square/rectangular. Metal belts should be used to close the boards’ ends.

-  The wooden parts used in scaffoldings should be of the finest quality, have long fibers, and not be contaminated with paint.

- They should also be plain, free from any potentially dangerous defect, have no knots and flakes, and be protected against woodworm.

-  The necessary measures should be taken to protect the scaffolding’s boards and planks against cracking.

-  Boards with cracks and cuttings should not be used.

-  Boards should not be cut, burnt, contaminated with oil/diesel fuel and contain nails.

-  Boards should not be painted.

-  All boards considered as part of the working platform should not stick out more than four times their thickness from the end of the support.

-  The boards should not be placed on top of on another unless the necessary precautions are taken. For instance, the boards or planks whose ends have been steam treated minimize the risk of falling and facilitate movement of trolleys.

-  Walking boards used in the platform should have identical thickness.

-  Boards considered as part of the working platform or used as the platform’s toe boards should be of adequate width and thickness to ensure safety at the site. The thickness and width of such boards should be determined based on the space between the platform’s beams.

Note: The following should be determined according to the latest edition of the relevant standard (IPS-E-SF-400):

The knots visible on the wood, their width, the spacings between groups of knots, the standard boards’ thickness and width, the maximum number of knots on the scaffolding’s edge, maximum protrusion of boards (as working platforms) from the support’s end, the board's (as working platform) width and thickness.

## 10-10 Working platforms

-  All scaffoldings used to support work crews should have adequate working platforms.

-  No part of the platform should be placed on unsteady bricks, water pipes, chimneys, and other unsafe and inappropriate materials.

-  Platforms should not be used unless the scaffolding’s construction is fully completed and the necessary protection equipment have been installed.

-  The platform’s width should correspond with the type of the work. Each part, regardless of the presence of fixed barriers and accumulated materials, should have an open passageway with appropriate width.

-  Under no circumstances, the width of the working platform should not be less that the specified standard width.

-  If possible, the boards should face one another to prevent workers tripping on them while walking along the platforms.

-  A good platform should be able to bear a certain weight per square meter.

-  Working platforms used for transferring materials and moving along the scaffold are categorized by their application. The platform’s width is determined by the type of work.

-  Working platforms should be constructed so that their boards would not move during normal operations.

- All structures and frameworks used as the platform’s support should be constructed according to the relevant technical standards, have a strong foundation, and be safely secured with sag rods.

-  In independent scaffoldings, at least one third of the beams holding the working platform should remain in their place or be securely coupled to the standards/ledgers (on a per-case basis) until the scaffolding has been fully dismantled.

-  Unsteady bricks, sewer pipes, detached clay blocks, barrels, boxes, or unsafe materials  should neither be used as the scaffolding’s support nor employed in its construction.

-  An empty overhead space should be provided on top of the working platform.

-  All platforms installed at height should have boards placed at close distances so as to prevent tools, equipment, and materials from falling down through the spaces between them.

-  The scaffolding’s boards should cover the platforms and landings and have metal belts or eaves boards at both ends.

-  Both ends and the center of boards should be placed on the scaffolding’s transoms.

Note: The following should be determined according to the latest edition of the relevant standard (IPS-E-SF-400):

Width of the working platform and its open passageway, the boards’ spacings, platform’s minimum weigh bearing capacity per square meter, number of support beams as determined by their length, maximum width of the platform held by putlogs, minimum height of the overhead space on top of the working platform, boards’ length given their thickness, boards’ width, boards’ thickness, maximum number of piled boards, and maximum protrusion of working platform.

## 10-11 Ropes:

-  Fiber ropes should never be used in scaffoldings as they are highly likely to easily get damaged.

-  Defective ropes or ropes having been in contact with acids and other corrosive substances should not be used.

## 10-12 Instruments and Equipment Storage:

-  After the working day is over, all tools and materials should be removed from the scaffolding.

-  Nails used to join different components of the wooden scaffolding should be of the right size and quantity. The nails should be fully driven into the wood and subsequently bent. The nails should not be left unbent and or half-driven.

- Cast iron nails should not be used in scaffoldings.

-  During the dismantling process, all nails should be pulled out from scaffoldings components.

-  Moving, working, and transporting and storing construction materials on the scaffolding  should be done with utmost caution. Sudden impacts caused by actions such as jumping and kicking are dangerous.

-  The scaffolding should be constantly supervised to prevent unnecessary loads and construction materials being piled on it.

-  The scaffolding should not be used for storing construction materials unless they are needed for urgent operations.

-  Workers should not use a platform covered with ice or snow. Soft sand should be poured on the platform to melt down the ice or snow covering the working platforms.

## 10-13 Lift:

If necessary, lifts will be installed on the scaffoldings. The following conditions should be met before installing lifts on scaffoldings: A) The scaffolding’s components should be carefully inspected. If required, the scaffoldings should be further reinforced.

B) Putlogs should not be moved. C) If possible, standards should be firmly secured to the building's rigid parts and the section where the lift is going to be installed.

If the lift fails to move between the rails or the load being carried by the lift is likely to forcibly come into contact with the scaffolding’s structure, vertical fences should be used to prevent the load from getting stuck into the scaffold’s parts.

# 11. Personnel Training

The employer’s supervisor should be fully familiar with the legal references and requirements mentioned in the present guideline. He should also attend and pass the necessary training programs and obtain the relevant certificates from the associated authorities.

The contractor shall be obliged to be fully familiar with the legal references and requirements mentioned in the present guideline. He should also attend and pass all the necessary training programs and obtain the relevant certificates from associated authorities.

The contractor shall also be obliged to organize continuous training programs for his employees and submit the employees’ certificates to the employer’s supervisor to obtain the necessary approval.

All members of the scaffolding installation group should be trained in scaffolding installation, dismantling, transfer, use, re-installation, maintenance, and inspection. They should also be fully familiar with the associated hazards and safety requirements of working at height and obtain the necessary certificated from the associated qualified authorities.

The following are among the subjects taught in the aforementioned training programs:

-  Electricity, falling from height, risk of falling objects, electric shock protection, protection against falling from height, etc.

-  Work-at-height machinery, personal fall arrest equipment, safety signs, personal protection equipment, descending methods and equipment, procedures used to minimize damage caused by falling from height, methods and equipment used to prevent falling objects, etc.

-  Emergency Response

# 12. Preparing for

# Emergency Response

Given the estimated risks of working at height, the following measures should be taken by the contractor and be coordinated with the employer’s supervisor to respond to emergency situations:

- A medical team with all the necessary equipment including an ambulance should always be present at the site.

- Relief vehicles and a team of trained paramedics should always be present at the site to transfer the injured to hospitals and medical centers and offer first aid services to the individuals working at the operation’s site

# 13. Appendices

13-1  Appendix 1 - A Sample Checklist for Scaffolding Inspection

| Scaffolding Installation Site: Scaffolding Installation Officer: Construction Company/Contractor: Scaffolding Installation Date: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Comment | No | Yes | Items to be inspected and checked |  | Number |
|  |  |  | Has the scaffolding installation permit been issued? |  | 1 |
|  |  |  | Have the scaffolding been installed by skilled and experienced workers? |  | 2 |
|  |  |  | Do the scaffolders use the appropriate safety and personal protection equipment? |  | 3 |
|  |  |  | Are the scaffolding’s components used under safe and reliable conditions? |  | 4 |
|  |  |  | Has the tagging system been properly implemented? |  | 5 |
|  |  |  | Have screw jacks (instead of unsafe and unstable objects such as blocks and unsteady bricks) been used to adjust the scaffolding both vertically and horizontally? |  | 6 |
|  |  |  | Are base boards and screw jacks firmly coupled to the standards and frames? |  | 7 |
|  |  |  | Have all the standards been safely secured with harnesses? |  | 8 |
|  |  |  | Has a ladder been supplied for access and entry to and exit from the scaffolding? |  | 9 |
|  |  |  | Are ladders safe and intact? Haven’t they been bent or broken? |  | 10 |
|  |  |  | Have the toe boards been installed on the edges of the lower parts of the platform? |  | 11 |
|  |  |  | Has a safe distance been kept from the power transmission lines? |  | 12 |
|  |  |  | Has the scaffolding been interfering with the vehicular traffic? |  | 13 |
|  |  |  | Have handrails and mid rails been installed in appropriate locations (to provide the necessary protection against falling)? |  | 14 |
|  |  |  | Have damaged and broken materials been used in the scaffolding? |  | 15 |
|  |  |  | IS the ground strong enough to bear the scaffolding’s weight? |  | 16 |
|  |  |  | Has the scaffolding been designed to bear the load in question? |  | 17 |
|  |  |  | Has the scaffolding been inspected and approved according to the schedule (as indicated in the relevant instruction)? |  | 18 |
| Comment: |  |  |  |  |  |
| Employer’s HSE Department: Date and Signature: |  |  |  | Contractor’s Safety Expert: Date and Signature: |  |

Health, Safety, and Environment (HSE) Department

No. 17, Yaghma Alley, Jomhouri-e Eslami St. (btwn Hafez St. & Valiasr Ave.), Tehran, Tehran, Iran

Phone: +98 21 66726016, Fax: +98 21 6609723, SMS System: +98 10007615

HSEE@NIOC.IR	HTTP://HSE.NIOC.IR