National Iranian Oil Company

Safety, Health and Environment Management

The NIOC’s Accident/Incident Management Procedure

| Reviewing Translation | Confirmed by | Publication Date of English Version |
| --- | --- | --- |
| Davood Naderi/ Laleh H. Abbasi Training & Performance Measurement Department | Mani Abdollahzadeh Rad/ HSE Manager | September 2018 |

| Responsibility/Subject | Prepared by | Confirmed by |  | Approved by | Publication date |
| --- | --- | --- | --- | --- | --- |
| Name | Mani Abdollahzadeh/ Fereshteh Miraj Head of Safety and Firefighting Department /project supervisor of Safety and Firefighting Department | Reza Khalili Non-industrial operations Deputy | Siavash Derafshi/ HSE Manager | Ali Kardor CEO | November 2015 |
| Position |  |  |  |  |  |

Revision No.: 0  Code: NIOC-HSE-SF-PR-033-00  Revision Date: 14/06/2017

Contents

1. Introduction .............................................................................................................................................. 4

2. Purpose...................................................................................................................................................... 4

3. Scope ........................................................................................................................................................ 4

4. Definitions and terms ............................................................................................................................... 4

5. Accident/incident assessment .................................................................................................................. 8

5.1 Accident consequence level table .......................................................................................................... 9

5.2 Control level Table ................................................................................................................................. 9

5.3 Accident assessment matrix ................................................................................................................. 10

5.3.1 Level 3 accidents ............................................................................................................................... 10

5.3.2 Level 2 accidents ............................................................................................................................... 10

5.3.3 Level 1 accidents ............................................................................................................................... 10

6. Accident/incident reports ....................................................................................................................... 10

6.1 Accident news reports .......................................................................................................................... 10

6.2 Immediate accident reporting ............................................................................................................... 11

6.3 Continuous accident reporting ............................................................................................................. 11

6.4 Analytical reports ................................................................................................................................. 11

6.5 Final accident/incident inspection and investigation report ................................................................. 12

7. Accident investigation and analysis (root cause analysis) ..................................................................... 12

7.1 Level 3 accidents in which ................................................................................................................... 12

7.2 In the case of major accidents .............................................................................................................. 13

7.3 The head of the HSE Department ........................................................................................................ 13

7.4 Before initiating investigations ............................................................................................................ 13

7.5 Collecting data and evidence ............................................................................................................... 14

7.5.1 Necessary data ................................................................................................................................... 14

7.5.2 Collecting evidence and data ............................................................................................................ 15

7.5.3 Preparation of the preliminary data/evidence collection plan ........................................................... 15

7.5.4 Data/evidence collection requirements and techniques .................................................................... 15

7.6 Data/evidence assessment and analysis ............................................................................................... 16

7.7 Identification of root causes using common root causes identified in the NIOC ............................ 17

7.8 Solutions (corrective and preventive actions) ...................................................................................... 18

7.9 Preparing the final report ..................................................................................................................... 19

8. Investigation and analysis of systemic chronic causes .......................................................................... 19

8.1 Collecting data and information at all levels ....................................................................................... 19

8.2 The quantitative assessment of accidents ............................................................................................. 19

8.3 Statistical analysis ................................................................................................................................ 20

8.4 HSE performance report ...................................................................................................................... 20

8.5 The root analysis of collected data ....................................................................................................... 20

9. Corrective actions and preventive actions (CAPA) management .......................................................... 21

9.1 Management of corrective and preventive actions at all levels ........................................................... 21

9.2 Management of corrective and preventive actions associated with root causes .................................. 21

10. Lessons learned from accidents ........................................................................................................... 22

11. References ............................................................................................................................................ 22

12. Appendices ........................................................................................................................................... 23

Appendix 1: Immediate Accident Reporting ............................................................................................. 23

Appendix 2: Data Entry Table ................................................................................................................... 28

Appendix 3: Accident/Incident Investigation Logbook ............................................................................. 28

Appendix 4: The NIOC’s Root Cause Map ........................................................................................... 46

Appendix 5: Supplementary report of major accidents ………................................................................. 47

Appendix 6: Quantitative Risk Assessment Matrix ................................................................................... 48

Appendix 7: The CAPA (Corrective Action and Preventive Action) Form .............................................. 49

Appendix 8: The CAPA Follow-up Form .................................................................................................. 49

Appendix 9: Lessons Learned Form .......................................................................................................... 50

1. Introduction

A major part of industrial accidents are not only preventable, but their severity is also predictable, and as a result, their damage can be controlled, too. However, this requires analyzing accidents with a preventive approach and taking necessary corrective actions and safety measures, based on the findings of the root cause analyses. In addition, the knowledge management process and lessons learned from accidents are very effective in preventing recurrence of similar accidents. The present document, entitled “The NIOC’s Accident/Incident Management Procedure” describes processes including accident recording, reporting, investigation and analysis. It should be noted that the main purpose of this procedure is not to find someone to blame, but to find a systematic approach to implement the processes of reporting, recording, informing, investigation and analysis of various accidents, including major accidents and even those with lower human, capital, production and environmental consequences and to determine the scope of responsibilities and authorities of relevant personnel in order to reduce the frequency of accidents and related damages and to prevent the recurrence of similar major accidents in the future.

2. Purpose

- Recording, reporting, investigation and analysis of various accidents in order to learn lessons and to prevent the recurrence of similar accidents in the future.

- Creating a database of causal factors involved in an accident, identifying hazardous inconsistencies and unsafe items (input of risk assessment studies in a preventive approach)

3. Scope

This procedure is applicable to all departments, subsidiaries, managements, operational areas and contractors and subcontractors of the NIOC.

4. Definitions and terms

- Company: The National Iranian Oil Company

- Subsidiaries: Companies operating directly under the NIOC

- Employer: As defined in the relevant contract

- Contractor: All legal/real persons who have signed contracts with the subsidiaries of the NIOC

- Contractor’s personnel: All personnel working under the supervision of the contractors or subsidiaries of the NIOC

- Staff: All persons who work in the company under the following conditions:

A. Corporate employees, including formal and contractual employees (fixed-term, temporary-term) and third-party employees (who are paid by the contractor, but the company provides them with other services such as training courses, etc.)

B. Contract employees, including those working in EPC contracts.

- Accident: An unplanned (unwanted) event that leads to injuries, deaths, illnesses and damages to equipment, capital, production and the environment, etc.

- Incidents: A large group of events that disturbs the work. In fact, an accident is a part of such incidents.

- Near miss: Events that do not harm people or damage equipment; however, they can potentially lead to death, serious injury or may create equipment or environmental damages. Such events can be used as a tool for learning lessons from accidents in order to prevent recurrence of accidents and to select appropriate risk control measures. Examples include fractured crane hook, torn ropes, etc.

- Statistical work-related injuries: Injuries incurred by employees when performing their duties or due to their duties, during their working hours (with regard to the Iranian Labor Law).

- Non-statistical work-related injuries: Injuries incurred by employees when commuting to and from work, during lunch hours, etc. These injuries are related to work, but they are not recorded in accident reports.

- Non-work-related injuries: All injuries that do not meet the definition of statistical and non-statistical work-related injuries (the person is not involved in a work-related task) such as injuries occurred by accidents outside working hours or on holidays.

- Deliberate self-harm: In these cases, the person deliberately injures himself/herself during working hours. These cases are not considered work-related injuries.

- Various types of harms:

- Fatal cases

- Disabling cases

- Restrictive cases

- Medical Treatment Cases (MTC)

- First aid cases

- Fatality: A death case caused by an accident, regardless of the time interval between the occurrence of accident and death.

- Natural deaths at the workplace: According to the Petroleum Industry Health Organization, natural death is a death that has not resulted from work-related illnesses or occupational diseases and the relevant work has not exacerbated the death-related illness. These cases are not recorded in relevant occupational reports.

- Disabling injuries: Injuries, other than death, that permanently or temporarily disable a person or result in loss of limb function or limb amputation. These accidents lead to permanent or temporary disability (for one or more days) and result in lost workdays. Examples include damages to the eyes, hands, feet, etc.

- Lost time injuries: Work-related injuries (e.g. disabling injuries and death) that result in lost working days.

- Restrictive injuries: Injuries (other than disabling injuries and deaths) that restrict the injured person’s activities and as a result, he/she will not be able to fully perform his/her specified tasks for a specified time (according to the diagnosis of an industrial medicine physician).

- First aid: Initial measures taken to save the life of a person/patient, before the physician arrives.

- First aid cases: Minor injuries that are not recorded and reported as medical treatment cases, yet they necessitate some first aids. These cases are not recorded in accident reports. Examples include removing a splinter from one’s finger, superficial cuts, etc.

- Medical treatment cases: These cases are not that severe to result in lost workdays; on the other hand, they cannot be treated only by using first aids. In these cases, patient management and care go beyond simple first aids. Fainting in the workplace is an example.

- Minor injuries: These injuries are treated in health care centers and the injured persons can return to work immediately or the day after the accident.

Note: Minor injuries do not result in any lost workday. Only those accidents with medical treatment cases are recorded in the accident reporting forms.

- Hazard: Factors that can potentially injure people or may damage equipment, capital, production and the environment.

- Unsafe cases: Unsafe conditions or acts that can potentially cause accidents or near misses, including:

- Unsafe conditions or work

- Violating instructions due to lack of proper instructions or incomplete instructions

- Unsafe equipment/ design or lack of suitable equipment

- Emergency condition/situation: An emergency situation is the result of a major accident. Its extent depends on the type and extent of the consequence as well as the number of organizations involved.

- Vehicles: Land, air and marine vehicles used to transport objects, equipment or manpower. Mechanical machines are also considered as vehicles when crossing public roads and streets.

- Mechanical equipment: Mechanical equipment is usually manufactured for off-road use. Examples include: mechanical grab, various kinds of cranes, tractors, mechanical shovels, etc.

- Vehicle accidents: Accidents occurring during the transportation or operation of this equipment, which result in financial losses, physical damages or casualties.

- Major fire: Fires that cause significant damages, injuries, financial losses or may stop activities/operations. These fires require the engagement of firefighting teams and the use of portable fire fighting vehicles.

- Minor fire: These fires can be extinguished using handheld and portable fire extinguishers and do not cause significant losses.

- Accidents with financial losses: These accidents impose financial losses to the corporate property or may disturb the routine course of corporate operations. Examples include: earthquake, storm, flood, explosion, fire, water or power outage in facilities and other unexpected events that, according to relevant authorities, their damage is unpredictable and uncontrollable.

Note: Damages resulting from negligence, inadvertence and those caused by accidents are not considered among these accidents.

- Direct costs: These costs are directly related to the accident and affect various groups of people, equipment and the environment, directly. These are usually covered by insurance (e.g. therapeutic costs, replacement of destroyed equipment, etc.).

- Indirect costs: Costs imposed on an organization or company indirectly, due to an accident and its consequences. These costs may endanger the company/organization’s reputation and are usually not covered by insurance (production cessation, damage to the factory, legal costs, costs of emergencies, site cleanup, lost working hours, accident assessment costs, fines, etc.).

- Direct causes: Conditions occurred right before the accident, which directly affect the accident (e.g. falling from height, electric shock, etc.).

- Indirect causes: Indirect or superficial causes are categorized into two categories of unsafe conditions and unsafe acts. These are also known as causal factors of accidents.

- Unsafe conditions

- Unsafe act

- Causal factors: These are some indirect causes of accidents that directly affect the accident and, if eliminated, the accident will not occur.

- Root/basic causes: These are the origin of superficial or indirect causes of accidents or may result from poor management policies and decisions. These causes may also result from environmental and individual factors. In fact, root causes are multidimensional factors (human conditions or factors) that play a significant role in creating a direct cause and the consequent accident. These are the roots of many problems and their identification and elimination can have a significant impact on reducing the number of accidents.

- Accident control: Controlling and managing an accident to prevent subsequent consequences and outcomes.

- Inhibition of an accident: The consequences of a completely controlled accident.

- The main event: The first event in the chain of events that disrupts the sequence of activities and may lead to subsequent damages and consequences.

- Loss event: An undesirable outcome and condition in the chain of events.

- Correction: An immediate action taken following the occurrence of an error (problem, inconsistency, event, accident, etc.) to eliminate it immediately. Such actions will not prevent the repetition of errors.

- Corrective actions: Actions taken to eliminate the causes of an incident, adverse conditions or actual errors (occurred, happened) to prevent their recurrence.

- Preventive actions: Actions taken to eliminate the causes of an incident, adverse conditions or potential errors (not happened yet) to prevent their recurrence.

- High-consequence areas (HCA): Areas with a high frequency of consequences, based on the consequence-level assessment table.

- High-potential areas (HPA): Areas with a high frequency of potential risk, based on the consequence-level assessment table.

- HSE Management’s Internal Accident and Incident Committee (IAIC): This committee serves as the main lever of the NIOC’s HSE management. The Head of the HSE Department is the head of this committee and the Head of Safety and Fire Department is the secretary of the committee and has specific tasks. Relevant job descriptions are fully defined and presented in the NIOC’s Accident/Incident Management Procedure

5. Accident/incident assessment

To assess and determine the level of accidents and events across the NIOC, two factors shall be taken into account: 1- the consequences of the accident/incident and 2- the level of control over the incident.  The above factors, along with the accident assessment matrix are used to assess the level of various accidents. The following table displays three levels of different accidents/incidents.

| Accident level | Classification |
| --- | --- |
| One | Minor |
| Two | Mediocre |
| Three | Major |

5.1 Accident consequence level table

Table 1 presents the level of consequences of accidents in terms of damage to manpower, facilities, equipment, production and the environment.

Table 1: Accident Consequence Level

| Level | Consequences and outcomes |
| --- | --- |
| 4 | ☐ Death of personnel or serious injury, limb amputation |
|  | ☐ Stopping production for over 6 hours or major damage to equipment and infrastructures resulting in complete failure of the unit or severe damage to it |
|  | ☐ Accidents resulting in the leakage of liquids, chemicals and gases in large quantities (and even beyond the site) with the potential for general impact |
| 3 | ☐ Major injuries and permanent disabilities in a way to prevent proper function of an organ (e.g. knuckle amputation) or exposing people to excessive amounts of hazardous and toxic substances |
|  | ☐ The failure of equipment or vital infrastructures in a way to shut down a major sector of the unit |
|  | ☐ The leakage of liquids, chemicals and gases in large quantities inside the site with the potential for affecting personnel and equipment |
| 2 | ☐ Minor injuries and temporary disabilities or exposure to insignificant amounts of hazardous and toxic substances, resulting in absenteeism and some reversible effects on one’s health |
|  | ☐ Damage to equipment/partial process cessation, where the process may require re-started |
|  | ☐ Environmental incidents such as leakage of liquids, chemicals and gases in large quantities inside the site, without a serious threat to personnel and equipment |
| 1 | ☐ Minor injuries with a small impact on one’s health and the possibility of returning to work on the same day |
|  | ☐ Partial damage to equipment, with the possibility of short-term cessation of processes for repairing equipment |
|  | ☐ The leakage of liquids, chemicals and gases in low quantities without no adverse impact on personnel and equipment |

5.2 Control level Table

In Table 2, the level of control over consequences of an accident/incident is defined in four levels.

| Level | Levels of control over accidents |
| --- | --- |
| 4 | ☐ The accident has not been controlled and apparently, it cannot be controlled it in less than three hours |
| 3 | ☐ The accident is a severe one; however, it may be controlled in less than three hours. |
| 2 | ☐ The severity of the accident is reduced and it is under control |
| 1 | ☐ The accident has been controlled; however, further monitoring and supervision are required. |

5.3 Accident assessment matrix

Considering the consequence level and control level tables, the accident assessment matrix is presented as follows.

|  |  | Consequence level |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Control level |  | 4 | 3 | 2 | 1 |
|  | 4 | Level 3 | Level 3 | Level 3 | Level 1 |
|  | 3 | Level 3 | Level 3 | Level 2 | Level 1 |
|  | 2 | Level 3 | Level 2 | Level 2 | Level 1 |
|  | 1 | Level 3 | Level 2 | Level 2 | Level 1 |

5.3.1 Level 3 accidents

The following accidents are considered among level 3 accidents:

- All accidents with level 4 consequences

- All accidents/ incidents with level 2 and 3 consequences that have not been controlled three hours after their occurrence and apparently, cannot be controlled it in less than three hours

- All accidents/ incidents with level 3 consequences that have not been controlled three hours after their occurrence; however, they may be controlled in the short term

5.3.2 Level 2 accidents

- All accidents/ incidents with level 3 consequences that have been largely/completely controlled three hours after their occurrence

- All accidents/ incidents with level 2 consequences that have not been controlled three hours after their occurrence and apparently, cannot be controlled in the short term

5.3.3 Level 1 accidents

- All accidents with level 1 consequences

6. Accident/incident reports

6.1 Accident news reports

In the events of occurrence of level 3 accidents (based on the accident matrix) the Company’s Head of HSE Department shall inform the N.I.O.C’s HSE Manager (using relevant phone numbers).

6.2 Immediate accident reporting

For all accidents with level 4 consequences, the Company’s Head of HSE Department shall inform the N.I.O.C’s HSE Manager and the HSE Department of the Ministry of Petroleum of Iran (using relevant phone numbers) within 3 hours (in accordance with the circular issued by the HSE Department (No. 327355; approved on 4/10/2014 see Appendix 1).

Note: Accidents with levels 2 and 3 consequences, which due to lack of control and/or ambiguity of consequences or extension of effects turn to level 4 accidents over time also require immediate reporting (e.g. death of the injured person several days after hospitalization).

6.3 Continuous accident reporting

For other accidents with a control level of 2 that apparently cannot be controlled in less than three hours and their consequence may also be expanded, initial accident report and continuous monitoring and reporting to the IAIC are required. An immediate report should also be prepared and submitted to the IAIC, in cases where the consequence turns into level 4.

- Accidents with level 3 and 4 consequences require providing reports on an hourly basis.

- Accidents with level 2 consequence require providing reports every 3 hours.

- Accidents with level 1 consequence require providing reports every 24 hours.

6.4 Analytical reports

- All accidents and incidents should be recorded by the HSE Department of the subsidiaries in the N.I.O.C’s accident and incident table in the form of an Excel file (see clause 8 Appendix 2).

- The head of the HSE headquarters of the subsidiaries shall monitor this process across all associate and subsidiary companies on a monthly basis.

- The head of the HSE headquarters of the subsidiaries shall receive and summarize all accident and incident reports and submit quarterly reports to the N.I.O.C’s HSE Management.

- The HSE departments of the subsidiaries shall analyze all accident and incident reports (statistical analysis/root cause analysis) on a quarterly basis, in accordance with the requirements of this procedure. They shall also provide CAPA (corrective actions and preventive actions) forms.

- The HSE departments of the subsidiaries shall review their analytical reports and submit relevant quarterly reports (analytical reports and reports on corrective actions) to the N.I.O.C’s HSE Management.

6.5 Final accident/incident inspection and investigation report

Various types of final analytical reports:

- The investigation, review and analysis of accidents with level 1 consequence: The N.I.O.C’s inspection team, appointed by the N.I.O.C’s HSE Management, prepares accident analysis and investigation reports and provides them to the IAIC.

- The investigation and review of highly frequent accidents/incidents: The HSE departments of the subsidiaries shall prepare analytical reports of highly frequent accidents/incidents, in accordance with the requirements of this procedure, and shall provide them to the N.I.O.C’s HSE Management.

- The review and analysis of the causes of accidents with level 2 and 3 consequences, in which the control time has exceeded 12 hours.

The head of the HSE Department of the affected company shall prepare such reports and submit them to the N.I.O.C’s HSE Management.

- The review and analysis of the root causes of accidents, which have the potential to turn to accidents with level 4 consequences.

To prevent major accidents, the head of the HSE Departments of the subsidiaries shall assess and analyze accidents, which have the potential to turn to accidents with level 4 consequences [under different conditions] and provide necessary reports to relevant authorities in accordance with the requirements of this procedure.

7. Accident investigation and analysis (root cause analysis)

The process of root cause analysis is applied in the following cases:

- Accidents with level 4 consequence (by the Expeditionary Team and  the head of the HSE Department of the affected company)

- Highly frequent accidents/incidents

- Accidents at other levels with the potential of becoming accidents with level 4 consequence

7.1 Level 3 accidents in which the control time has exceeded 12 hours. The N.I.O.C’s HSE Management establishes an investigation and analysis team and dispatches the team to the site. Before the arrival of the accident inspection team, the head of the HSE Department of the affected company is responsible for collecting and protecting relevant evidence.

7.2 In the case of major accidents (level 3 accidents) the head of the HSE Department of the affected company shall collect relevant evidence and documents and submit them to the manager of the inspection team:

- Cooperation with the Security Department for enclosing the site

- Collecting consequent-related information (e.g. manpower, production cessation and damage to the environment, facilities and equipment)

- Collecting objective evidence and data (e.g. the list of personnel involved in the accident, the list of damaged equipment, cessation hours)

- The list of rescue and relief measures and restoration measures

- Filming and taking pictures of the accident site and reporting measures taken to manage the accident and to cope with emergencies

- Conducting initial interviews with the people involved in the accident

7.3 The head of the HSE Department of the affected company shall fully cooperate with the manager and other team members during the investigation and analysis processes and must coordinate with operational units and other relevant agencies under the supervision of the team manager.

7.4 Before initiating investigations, the head of the affected company/region shall control and prepare the following items.

- Ensuring the safety of the accident/incident scene

- Risk assessment of operations research at the accident/incident scene

- Determining the status of systems and equipment available at the scene

- Identification and assessment of the possibility of recurrence of the accident during the investigation

- Identification of changes in the production process, systems, personnel, repairs, design, suppliers, etc.

- Holding coordination and briefing meetings in order to observe safety regulations at the scene

- Allocating a specific office, called “the investigators’ office” for conducting their meetings

- Equipping the investigators’ office to phone, fax machine, photocopier and whiteboard.

- Allocating another office for conducting interviews

- Creating a list of investigators with their phone numbers

- Planning for a short visit of the accident/incident scene

- Ensuring the availability of all resources required for investigation (including all initial resources and tools and investigation forms) by the team manager

The required tools are presented in Appendix 3.

7.5 Collecting data and evidence

Relevant units/departments of the affected company shall perform the following tasks:

- To ensure the accuracy of recorded data and to protect the true status of accident scene, it is better to collect data immediately following the occurrence of accident. These data include pre-occurrence, during-occurrence and post-occurrence conditions, personnel involved in the accident, measures taken, environmental factors, etc.

- In some cases, photos taken from different directions may facilitate data analysis. Relevant investigators shall try to protect all physical evidence, such as broken parts, faulty gaskets, burned caps, burned fuses, spilled liquids and incomplete instructions.

- All conditions and people involved in the accident should be identified. Once data are collected, they must be assessed to ensure their accuracy.

- The site must be quarantined and damaged materials and equipment must be labeled and separated from other equipment.

7.5.1 Necessary data

- Documents and reports

- Instructions, working procedures and other relevant reports, including logbooks and safety reports, correspondences, minutes, issued permits etc.

- Software information (e.g. computer logs) at least 24 hours before the occurrence of accident/incident

- Reports provided by other bodies such as the Iranian Legal Medicine Organization, hospitals, traffic police, technical specialists, etc.

- Personnel (should be available for interview):

- The list of personnel involved in the accident

- The list of personnel involved in operational and production processes

- The list of Emergency Response Team members

- Systematic information:

- Process Flow Diagrams (PFDs), Piping and Instrumentation Diagrams (P&ID) and Heat and Mass Balance Diagrams (H&MB)

- Pipelines, processes and equipment drawings

- Information on alarm systems, detectors and safety and process controllers

- Information on fluids, temperatures, pressures and other process parameters

- Information on power management of various equipment and devices

- Repair and maintenance:

- Requests for repair works

- Issued permits

- Records of technical inspection reports and operational and repair reports

- Records of repair and maintenance program

- Materials and spare parts:

- Product specifications

- Product samples

- Samples of raw materials

- Damaged parts and pieces

- Laboratory reports on products, raw materials, etc.

- Quality control reports

- Sampling analysis results

- Photos and videos:

- Accident scene photos and videos

- Photos and videos of damaged parts and equipment

- Photos and videos of spills and leakages

- Photos and videos of signs of failure and defects

7.5.2 Collecting evidence and data

Data collection is the basis of investigation, analysis and subsequent conclusions, which in turn help authorities, prevent the recurrence of similar accidents in the future.

The collection of data and evidence on the accident scene is directly related to the ability, knowledge and experience of data collectors. On the other hand, the effect of planning on the accuracy of data and evidence should not be ignored.

7.5.3 Preparation of the preliminary data/evidence collection plan

A data collection plan reduces the overall data collection costs through increasing accuracy and reducing investigation time. The following issues are crucial in this process:

- The inspection team members should be familiar with the data collection process.

- Data should be collected on the basis of a data collection plan.

- Needed tools and devices should be identified and prepared.

The preliminary data collection plan is prepared by the team manager. It outlines the responsibilities and job descriptions of the inspection team members. This preliminary plan may be changed (following some reviews) in order to ensure the collection of all data and evidence and to guarantee the safety and health of all investigators.

7.5.4 Data/evidence collection requirements and techniques

Time and accuracy are significantly important in the data collection process. Therefore, all members of the inspection team shall consider the following tips:

- Conducting interviews and recording the statements of all witnesses in order to collect human data

- Collection, review and detailed analysis of physical data (physical data, including a wide range of control systems, measurement and monitoring equipment, safety and security protectors and controls, before/during and after accidents, personal protective equipment (PPE) and damaged equipment and devices)

- Identification, collection and analysis of recorded data and relevant documents (analyzing all relevant instructions, working procedures, checklists, shift logbooks, startup tests and inspections, repair records, records of previous accidents and logistics records can play an important role in understanding the mechanism of accidents/incidents and even in identification of their root causes)

- Identification, collection and analysis of relevant paper and digital records: Digital data are as important as paper data. Examples include data on alarms, sensors, digital drawings, etc.

- Recording spatial data associated with personnel and objects: The location of personnel, power switches and control devices, control valves, panels and signs, gas cylinders and, in general, everywhere at the site of the incident are considered as spatial data. Besides those spatial data available at the scene during site inspection, available CCTV cameras should be watched and interviews and even previous records should be reviewed to collect more spatial data.

7.6 Data/evidence assessment and analysis

- Data/evidence analysis is the most important stage in an accident/incident investigation process. This stage is used to identify root causes. Several root causes are identified following the identification of a causal factor.

- The inspection team should put data and evidence together in order to analyze and reconstruct major accidents. In this regard, the team must use the following four approaches simultaneously to ensure the accuracy and adequacy of the data and evidence. In cases where more data is needed to reconstruct the accident, the data analysis process reveals this need (Fig. 2).

- Causal factor charting

- Cause analysis

- Timeline

- The cause and effect tree

The change analysis method can also be used as an auxiliary method to help specialists analyze accidents.

No

Yes

Before using various accident investigation and analysis methods, the following three measures must be taken:

- Separating assumptions from evidence and data

- Constructing accident scenario using available evidence and data

- Ensuring the integrity of the scenario

7.7 Identification of root causes using common root causes identified in the NIOC

The next step is to identify the root causes. In this regard, data mining and clustering techniques are used to identify and validate root causes.

The root cause analysis map is the best technique used to identify the root or systemic causes.

Causal factors are based of “a root cause analysis map”, which are usually divided into three main categories:

- Human Factors

- Equipment and software

- External factors

Each major category is also divided into smaller groups as follows:

- Human factors (employer/contractor):

- Formal employees

- Contractual employees

- A third-party (contractors)

- Equipment and software:

- Process equipment/machinery

- Software

- Materials/products

- Auxiliary and backup equipment

- Other equipment

- External factors:

- Natural disasters

- External accidents/incidents

- Internal/external sabotage actions

And finally, the root causes are categorized in 11 categories and the type of defect and problem is rooted:

- Design

- Equipment reliability

- Documents and records

- Raw materials/components/products

- Failure mode and effects analysis (FMEA)

- Executive procedures

- Human factors

- Staff training/competence

- Monitoring and supervision

- Informal verbal and written communication

- Staff performance

Thus, each of these categories is extended to six more detailed levels and the root causes are finally identified.

Appendix 4 presents the NIOC’s root cause analysis map, which is used by the inspection team to identify root causes.

“Root cause analysis maps” vary from system to system; they can also be modified and reviewed over time.

When using the “root cause analysis map”, there may be a need for recollection of data.

7.8 Solutions (corrective and preventive actions)

To prevent the recurrence of similar accidents in the future, at this stage, the inspection team identifies some preventive solutions for each of the root causes [identified in the previous stage using the root cause analysis map] and records them in relevant reports. It should be noted that proposed corrective and preventive actions should be relevant to causal factors and root causes.

The inspection team members will then identify and propose appropriate corrective and preventive actions for each accident.

7.9 Preparing the final report

- Upon completion of the investigation process, the manager of the inspection team shall prepare the final accident report in the form of Appendix 5 of this procedure and submit it to the NIOC’s HSE management.

- This report includes all activities recorded by the inspection team, data and evidence, investigation checklists and forms, results, the list of identified causal factors, root causes and proposed solutions.

- The IAIC receives and reviews the report and defines and implements corrective measures in the form of CAPA (corrective actions and preventive actions) forms.

- The HSE management submits root cause analysis reports and proposed corrective and preventive actions to relevant authorities.

- The Managing Director of the affected company shall ensure the proper implementation of all corrective measures.

- The HSE Department of the affected company monitors the implementation of corrective and preventive actions in accordance with the CAPA form and submits the respective report to the NIOC’s HSE management.

8. Investigation and analysis of systemic chronic causes

Another major objective of this procedure is to organize the investigation of other accidents with lower consequences and those that have the potential to turn into major events. This can help authorities promote preventive approaches toward risk assessment and management, identify causal factors and unsafe conditions and prevent the recurrence of similar major accidents.

8.1 Collecting data and information at all levels

The HSE Department of the subsidiaries shall record all accidents and incidents at different levels in the form of Excel files (Appendix 2). Events are categorized in the accident/incident table, as defined in section 8.2.

8.2 The quantitative assessment of accidents

The level of damage of each accident is determined according to the table entitled “the quantitative assessment of accidents” (Appendix 6).

8.3 Statistical analysis

- The HSE Departments of the subsidiaries/operational areas shall statistically analyze collected data and insert them into the accident/incident record table. These analyses include:

- Determining the frequency and type of consequences in various operational areas or units, processes, activities and locations

- Determining high-consequence areas (HCA)

- Determining high- potential areas (HPA)

- Determining the time control of various accidents in various operational areas or units, processes, activities and locations based on the level and type of accident consequences

- Using the findings of statistical analyses, the HSE Departments of the subsidiaries shall organize inspection programs and determine the requirements of specific inspections, in cases where there is a high potential for equipment damages in operational areas and units.

- To identify systemic chronic problems and organizational weaknesses, the head of the HSE headquarters of the subsidiaries shall collect and analyze all data and information on accidents and incidents [that have occurred in their subsidiaries/operating areas] and submit them to the HSE management in the form of quarterly reports.

8.4 HSE performance report

The HSE Departments of the subsidiaries shall submit their quarterly and biannual HSE performance reports to the NIOC’s HSE management, in accordance with available formats and indicators. The relevant format will be communicated by the NIOC’s HSE management.

- Fatal Accident Rate (FAR): The number of deaths due to work-related accidents in a specified period of time per one million working hours

- Fatal Incident Rate (FIR): the number of accidents resulting in one death or more deaths due to work-related activities in a specified period of time per one million working hours

- Accident Severity Rate (ASR): The number of lost workdays due to disabling injuries.

8.5 The root analysis of collected data

According to the requirements of this procedure, the HSE Departments of the subsidiaries shall analyze the root causes of the following items and submit their findings to the HSE management in the form of quarterly reports.

- Highly frequent accidents

- Highly frequent causal factors

- Highly frequent root causes

- HPAs and HCAs

9. Corrective actions and preventive actions (CAPA) management

Corrective and preventive actions should be managed for the following items:

- High cost/consequence accidents

- Low cost/consequence accidents, with a potential for unacceptable consequences

- Near misses with a potential for unacceptable consequences

- Reports on unsafe cases and inconsistencies in inspection and audit findings

- Dissatisfactions and complaints

- Changes and reforms

- Hazard identification and risk assessment results

Proper corrective and preventive actions contain the following items:

- The type of accident

- Persons/companies in charge of implementation

- Start date and end date

- The effectiveness of actions

With regard to the above items, the CAPA form is presented in Appendix 7.

9.1 Management of corrective and preventive actions at all levels

The HSE departments of the subsidiaries/operational units shall complete CAPA forms for level 1, 2 and 3 accidents, determine deadlines for corrective and preventive actions and submit these forms to relevant authorities in order to prevent the recurrence of accidents/incidents.

- In each subsidiary company, the highest authority shall ensure the proper implementation of measures and allocation of necessary resources.

- The HSE Department of the affected Company shall monitor the proper implementation of corrective actions on specified deadlines. The causes of all delays and postponements shall be determined and recorded and actions should be taken to address relevant problems. A sample of CAPA form is presented in Appendix 8.

- The head of the HSE headquarters of the subsidiaries shall receive reports on corrective actions from their subsidiary companies, analyze them and submit biannual analytical reports to the NIOC’s HSE management.

9.2 Management of corrective and preventive actions associated with root causes

The IAIC receives and reviews relevant reports and submits corrective and preventive actions associated with root causes to relevant authorities.

10. Lessons learned from accidents

The NIOC’s HSE management prepares “lessons learned forms” for all accidents that have been rooted by the inspection team in order to prevent the recurrence of similar accidents. This form is also available at the HSE management site (Appendix 9).

Some accidents are investigated and analyzed by the subsidiary companies. In these cases, the head of the HSE headquarters of the subsidiary company will prepare “lessons learned forms” for all accidents and will submit them to its subsidiaries and contractors.

11. References

- The Core of Problem Solving and Corrective Action, Duke Okes, Hardcover, 200pages, published 2009, ISBN: 978-0-87389-764-8, Item Number: H1363.

- Guidelines for Technical Planning for On-site Emergencies; Center for Chemical Planning for On-Site Emergencies; Center for Chemical Process Safety (CCPS); New York; 1995; ISBN-10: 081690653X.

- Guidelines for the Management of Change for Process Safety; Center for Chemical Process Safety (CCPS); New York; 2008; ISBN-10:0470043091.

- Human Error; James Reason; Cambridge University Process Safety: Center for Chemical Process Safety (CCPS); New York; 2008; ISBN-10:047122048.

- http://www.ogj.Com/articles/print/volume-97/issue-37/in-thisissue/petrochemicals/Petchem producer-gets-help-from-root-cause- analysis.html.

- http://www.abs-group.com/what-we-Do/Safety-Risk-and-compliance/incidentInvestigation-and Root-Cause-Analysis/

- http://www.maritimelogic.com/marine- incident-investigation.html.

- https://www.env.nm.Gov/aqb/Proposed_Regs/Part_7Excess_Emissions/NMED_Exhibit_18Root_Cause_Analysis_for_Beginners.pdf.

- https://www.cms.gov/medicare/provider-enrollment-and certification/qapi/downloads/guidanceforrca.pdf

- Incident Investigation and Root Cause Analysis Services, ABS consulting, (865)671-5848.

12. Appendices

Appendix 1: Immediate Accident Reporting

Appendix 2: Data Entry Table

Appendix 3: Accident/Incident Investigation Logbook

Appendix 4: Root Causes Analysis Map

Appendix 5: Complementary Reports of Major Accidents

Appendix 6: Quantitative Assessment of Accidents Table

Appendix 7: CAPA Form

Appendix 8: CAPA Follow-up Form

Appendix 9: Lessons Learned Form

Appendix 1: Immediate Accident Reporting

Date: 06/10/2014

Number: ABAM/327355

Appendix:

Mr. Razmi – Esteemed N.I.O.C’s HSE Manager

Mr. Yousefi Pour – Esteemed N.I.G.C’s HSE Manager

Mr. Ramezani– Esteemed N.I.O.R.D.C.’s HSE Manager

Mr. Nasiri– Esteemed N.P.C.I.’s HSE Manager

Subject: Accident Reporting System

Hello

Regarding the recent emphasis of the esteemed minister on the need to pay more attention to safety issues in the petroleum industry and the need to improve safety culture, fundamental reviews in current procedures of all relevant areas and sectors are required and new measures must be taken at various organizational levels.

In this regard and in view of the importance of the reporting system, as the first step in analyzing accidents in the petroleum industry, the recipients of this letter are required to oblige their subsidiaries and associates to submit immediate, supplementary and detailed reports of accidents.

1) Submitting immediate reports within 3 hours

2) Submitting supplementary reports (including relevant details and possible causes of accidents) within 24 hours

3) Submitting detailed reports (including detailed review and analysis of accidents, causes and blamable personnel) within 20 days

Obviously, the interaction and cooperation of relevant authorities in the implementation of the above method will play an effective role in identifying causes of various accidents and in preventing the occurrence of similar incidents in workplaces.

Mohammad Hossein Ardeshiri

The N.I.O.C’s General HSE Manager

Copy

- Mr. Vaziri Hamaneh, the Esteemed Deputy Minister of Petroleum

In the name of God

ABAM/595424

Date: 07/03/2015

Department of Health, Safety and Environment (HSE)                                      Appendix: Yes

Appendix:

"The Year of Economy and Culture with National Determination and Jihadi Management"

Mr. Razmi – Esteemed N.I.O.C’s HSE Manager

Mr. Ramezani– Esteemed N.I.O.R.D.C.’s HSE Manager

Mr. Nasiri– Esteemed N.P.C.I.’s HSE Manager

Mr. Yousefi Pour – Esteemed N.I.G.C’s HSE Manager

Subject: Immediate Accident Reporting Form and Instructions for Completing the Relevant Form

Hello

Considering the deficiencies identified in the above mentioned report form and the need for immediate revision; the issue was discussed at the Petroleum Ministry’s Heads of Industrial Health Committee and necessary amendments [e.g. industrial health considerations in major accidents] were made to reform and complete the form. The revised form and instructions required for its implementation across all subsidiaries, associates and operational and contracting companies are attached to this letter. Obviously, upon the communication of this letter, the former form and instructions will be considered invalid and all subsidiaries, associates and operational and contracting companies will be required to submit relevant reports to authorities using the revised form and instructions.

Process accidents generally result in all health, safety and environmental consequences and the integrity of the HSE management system necessitates analyzing all health, safety and environmental aspects, when investigating accidents. In this regard, all HSE experts and professionals shall closely interact and cooperate to calculate and assess financial losses of accidents and to take preventive measures to prevent their reoccurrence. Therefore, all subsidiaries and operational units shall submit their accident reports to HSE Units and shall consider all health, safety and environmental aspects, when preparing supplementary reports.

Ali Asghar Rajabi

Vice General Manager

Immediate Accident Reporting Form

This form should be sent to the Office of the esteemed Minister of Petroleum (telefax: + (98) 88939304) and the Office of the esteemed N.I.O.C’s General HSE Manager (telefax: + (98) 88808856(within three hours.

1- General information: Main Company: Subsidiary Company: Area: Occurrence date: Occurrence time: Completion time: Full name of the person in charge of the work at the time of accident: Full name and phone number of the person who completes the form:

2- The type of accident: Fire □ Traffic collision □ Pipeline fractures/ruptures □ Explosion □ Gas release/hydrocarbon leakage □ Epidemic diseases □ Fainting □ Acute food poisoning □ Acute chemical poisoning □ Falling from height □ Tanker overturn □ Other cases □

3- Consequences: Primary consequence: Human resources □ Facilities/Capital □ Environment □

4- Secondary consequences: Death □ Number: … persons Formal employees □ Contractual employees □ Contracting employees □ Operation Cessation □ Injury □ Number: … persons Formal employees □ Contractual employees □ Contracting employees □ Pollution: Air □ Soil □ Surface water □ Groundwater □

5. Information on gas/hydrocarbon/chemical substance spillage and leakage: The type of material: Leakage/spillage time (duration): Pollution radius: Location: Pathways □ Inside facilities □ On soil □ Residential area □ Protected area □ Agricultural land □ River Dam/lake □ Volume of discharge/ spillage/leakage (Description: chemical products/fluids (in liters), oil (in barrels), contaminated wastewater (in cubic meter), gas (in cubic meter)) The source of leakage/spillage: Pipeline □ Barrel □ Tank □ Cylinder □ Oil tanker □ Others □

6. Possible cause of the accident: □ Problems in Mechanical □ Electrical □ Rotary machines □ Instrumentation □ Construction □ designs □ Operational problems (e.g. inappropriate equipment maintenance, timing failures, etc.) □ Facility weariness (e.g. weariness of pipelines, reservoirs and internal and external corrosion) □ Human error (e.g. failing to understand the procedure, forgetting the procedure due to lack of training, decentralization, fatigue, etc.) □ Unsafe act (e.g. leaving equipment in a dangerous position, using no PPE, etc.) □ Unsafe conditions (e.g. unsafe roads, collisions, lack of adequate lighting, sound, vibration, etc.) □ Problems with the procedure (e.g. lack of monitoring and inspection, lack of permit, non-implementation of HSE procedures, etc.) □ Natural factors (natural disasters such as earthquakes, floods, storms, etc.)

7- Brief accident description:

8- Immediate action taken to control the accident:

9- The status of injured person/persons: Name: Age: Working experience: Organizational post/activity: Brief description of the type of injury:

10- Verification of accident form: The accuracy of the above information is approved and the supplementary report will be sent to the General HSE Department within 10 days. The Head of the HSE Department (Signature) The Managing Director/Head of the Company (Signature) Telefax: Phone number:

The Head of the HSE Department shall prepare reports of the following accidents within the first 3 hours and following the approval of the CEO or the head of the company, copies shall be sent to the Minister’s Office as well as the N.I.O.C’s General HSE Manager.

- Fires in oil industry facilities or relevant facilities, resulting in injuries (permanent disabilities) and financial losses.

- Accidents and explosions resulting in serious damages, failure of one or more units or production cessation.

- Leakage of toxic and flammable gases and rupture/fracture of oil and gas pipelines resulting in serious damages to personnel (permanent disabilities), facilities or the environment (e.g. large scale air, soil or water pollution).

- All urban or suburban accidents associated with the Ministry of Petroleum resulting in physical and financial damages to non-corporate individuals [with a negative reflection in the society]. Examples include: fire, explosion or leakage of oil, petroleum products, gases, chemicals or flammable materials through pipelines, installations, traffic collisions, etc.

- All disasters such as earthquakes, landslides, thunderbolt, floods, tornados or severe storms resulting in total or partial destruction of oil and gas facilities, administrative buildings, warehouses and workshops.

- All accidents leading to death, amputations or acute chemical and food poisoning of all employees (formal, contractual or contracting employees) working within the facilities of the Ministry of Petroleum of Iran.

Note: Acute chemical poisoning refers to severe poisoning caused by exposure to gases and vapors emitted from chemicals in the workplace and acute food poisoning refers to food poisoning cases in which a significant number of personnel is affected.

- Epidemic diseases: A group of contagious and non-contagious diseases that afflict a large number of staff over a short period of time and disrupt routine activities of the respective unit.

- Fainting: A person may faint for many reasons (e.g. physical collision, sudden changes in environmental conditions such as pressure, hypoxia, high temperatures and exposure to chemicals.

- All accidents, such as traffic collisions, explosions, fires, leakage of oil, petroleum products, gases, chemicals or flammable materials occurring outside the scope of petroleum facilities that may expand and endanger facilities and personnel.

In the event of failure to send reports on time, offenders will be treated in accordance with available regulations.

An instruction for sending immediate reports of fire and major HSE accidents

Appendix 2: Data Entry Table

This table will be subsequently presented in the form of an Excel file.

Appendix 3:

Accident/Incident Investigation Logbook

Resources and tools required for investigating the accident/incident

Each inspection team needs some basic resources and tools to conduct inspections. The IAIC provides inspection teams with these resources.

| No. | Resources | √ |
| --- | --- | --- |
| 1 | Measurement instruments: Ruler and measuring tape |  |
| 2 | Marking pencil and pen |  |
| 3 | Inspection notebook for the NIOC’s inspectors (including checklists, forms, emergency telephone numbers, etc.) |  |
| 4 | Inspection gloves |  |
| 5 | Plastic bags for collecting evidence |  |
| 6 | Digital camera with all accessories |  |
| 7 | Inspection flashlight |  |
| 8 | PPE |  |
| 9 | Uniforms |  |
| 10 | Laptops and Flash Memories for data analysis |  |
| 11 | Sticky notes |  |
| 12 | Plastic tags |  |
| 13 | Zip keep |  |

Form No. S-101: Checklist for general and specialized inspection of accident scene

N.I.O.C’s HSE Management

Number: --------------- Inspector: ------------ Date: -------------- ----- Summary of the accident: -------------------------------------------------------------------------------------------

| No. | Preliminary investigation (general inspection) | √ |
| --- | --- | --- |
| 1 | Preliminary general inspection in a short time |  |
| 2 | Taking necessary safety measures and obtaining needed inspection permits |  |
| 3 | Coordination with relevant individuals and units for inspection |  |
| 4 | Observing all safety and health precautions before inspection |  |
| 5 | Visiting all relevant sites and facilities (area) |  |
| 6 | Moving no object (if necessary, take photos before moving objects) |  |
| 7 | Taking notes and recording: the location of objects and equipment, their specifications, including their dimensions, manufacturers, etc., taking photos, filming |  |
| 8 | Completing the information and evidence form |  |
| 9 | Preparing the data/evidence collection plan |  |
| 10 | Providing the list of actions required for supplementary inspection |  |
| No. | Supplementary investigation (specialized inspection) | √ |
| 11 | Preparation and development of inspection plan |  |
| 12 | Ensuring the availability of required evidence and information |  |
| 13 | Gathering experts and professionals to complete the inspection team |  |
| 14 | Determining the duties of inspectors in collecting evidence and data during a pre-inspection meeting |  |
| 15 | Initiating inspection and collecting relevant evidence and data |  |
| 16 | Putting data and evidence together to reconstruct the accident scenario |  |
| 17 | Analyzing evidence and data and performing re-inspections to finalize evidence and information |  |
| 18 | Analyzing root causes through using identified causal factors and performing re-inspections and re-analyses |  |
| 19 | Reconstructing the accident/incident scenario |  |
| 20 | Preparing the final inspection report |  |
| 21 | Coordination with the IAIC to announce the termination of the inspection process |  |
| 22 | The termination of the inspection process |  |

Full name and signature of the manager of the inspection team:                             Date:

Form No. S-102: Recording accident inspection and investigation activities

N.I.O.C’s HSE Management

Number: --------------- Inspector: ------------ Date: -------------- ----- Summary of the accident: -------------------------------------------------------------------------------------------

| Inspector | Activity | End time | Start time | Date |
| --- | --- | --- | --- | --- |

The inspector’s full name and signature:                             Date:

Form No. S-103: Recording necessary data and evidence

N.I.O.C’s HSE Management

Number: --------------- Date: -------------- ----- Summary of the accident: ------------------------------------------------------------------------------------------- Inspector: ------------

| Meetings | Information provided by witnesses and relevant people | Spatial evidence and information | physical evidence and information (equipment, chemicals) | Documents, records, computerized and digital information, etc. |
| --- | --- | --- | --- | --- |

Full name and signature of the manager of the inspection team:                             Date:

Number: --------------- Full name of inspectors: -----------   Date: --------------

Summary of the accident: -------------------------------------------------------------------------------------

| Inspectors | Tasks | Comments |
| --- | --- | --- |

Form No. S-104: Accident inspection and investigation plan

N.I.O.C’s HSE Management

Full name and signature of the manager of the inspection team:                             Date:

Number: ---------------                                                      Full name of inspectors: ------------------

Summary of the accident: --------------------------- Accident date: ------- Inspection Date: -------

| Name and position of the interviewee | Inspector/inspectors | Needed information | Interview priority | Time and date | Interview is |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Not completed | Completed |

Full name and signature of the manager of the inspection team:                             Date:

Form No. S-105: Planning and scheduling interviews

N.I.O.C’s HSE Management

Witnesses offer invaluable information about accidents. These important data and evidence should be collected immediately following the occurrence of accidents (within 42 hours). Such information can help authorities improve workplace safety through identification of root causes and implementation of preventive strategies. The N.I.O.C’s HSE unit believes that all accidents occur due to systematic causes and it never considers its personnel as the root causes of accidents. In this unit, only those who commit theft, drug abuse and vandalism should be punished and personnel is never considered as the main causes of accidents in this company.

Form No. S-106: Witness’s self-declaration form

N.I.O.C’s HSE Management

Full name: ------------------------------ Organizational position: ---------- -------------------- Workplace:----------------- Phone number: -------------------------: Occurrence date and time: -------Write the names of people you have seen on the scene: --------------------------- --------------------------------- ---------------- ----------------------------------------------------------------------------------- ---------------- ----------------------------------------------------------------------------------------------------------------------------------------------------- -------------------------- -------------------------------------------------- -----------------------  ------------------------------------------------------------------------------------Describe the weather conditions at the time of occurrence: -------------------------------------- ----------------------- -------------------------- ------------------------------------------------------------------------- ---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------- -------------------------- -------------------------------------------------------------------------------------------------Has anything (equipment, machine, tool, object, etc.) been displaced following the occurrence of the accident? If yes, please describe: ------------ -------------------------------------------------- ------------------------- ------------------------ -------------------------------------------------- ------------------------- ------------------------ ------------------------ -------------------------- -------------------------------------------------- ----------------------- ------------------------------------------------- -------------------------------Do you think our personnel shall receive special training to prevent such accidents? If the answer is yes, please explain: ---------------------------------------- ---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------- Describe the accident /incident in details. Your statements will be archived and we assure you about the confidentiality of your statements. -------------------------------------------------- -------------------------------------------------- -------------------------------------------------- -------------------------------------------------- -------------------------------------------------- -------------------------------------------------- ------------------------------------------------------------------------------------------------------------------------------------------------------ -------------------------------------------------- -------------------------------------------------- -------------------------------------------------- -------------------------------------------------- ---------------------------------------------------------------------------------------------------- -------------------------------------------------- -------------------------------------------------- -------------------------------------------------- -------------------------------------------------- ---------------------------------------------------------------------------------------------------- -------------------------------------------------- -------------------------------------------------- -------------------------------------------------- -------------------------------------------------- ---------------------------------------------------------------------------------------------------- -------------------------------------------------- -------------------------------------------------- -------------------------------------------------- -------------------------------------------------- ---------------------------------------------------------------------------------------------------- -------------------------------------------------- -------------------------------------------------- -------------------------------------------------- -------------------------------------------------- ---------------------------------------------------------------------------------------------------- -------------------------------------------------- -------------------------------------------------- -------------------------------------------------- -------------------------------------------------- ---------------------------------------------------------------------------------------------------- -------------------------------------------------- -------------------------------------------------- -------------------------------------------------- -------------------------------------------------- ---------------------------------------------------------------------------------------------------- -------------------------------------------------- -------------------------------------------------- -------------------------------------------------- -------------------------------------------------- ---------------------------------------------------------------------------------------------------- -------------------------------------------------- -------------------------------------------------- -------------------------------------------------- -------------------------------------------------- --------------------------------------------------

The inspector’s full name and signature:                             Date and time:

Form No. S-107: Contact information of people involved in the accident:

N.I.O.C’s HSE Management

Number: ---------------                                                    Full name of inspectors: -----------------

Summary of the accident: --------------------------- Accident date: ------- Inspection Date:

| Full name | Company | Position (organizational rank) | Phone number and Email address |
| --- | --- | --- | --- |

Full name and signature of the manager of the inspection team:                             Date:

Form No. S-108: Interview

N.I.O.C’s HSE Management

Number: ---------------                                                   Full name of inspectors: ------------------

Full name of interviewee (witness): … Date: … Full name of interviewer (inspector):

Witness’s organizational position:     Company: …      Witness’s phone number: …

Summary of the accident: --------------------------- Accident date: ------- Inspection Date: -------

Please let us know what you know about the accident/incident. (Important Note: Write down the order of events (in terms of date and time), places being mentioned, actions taken by other people, actions taken by the crisis management team as well as Fire Department): -------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------Name all people who have been somehow involved in the accident or those you have seen at the scene: ------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

Describe the weather conditions at the time of occurrence: -------------------------------------- ----------------------- -------------------------- ------------------------------------------------------------------------- -------------------------- -------------------------------------------------- ----------------------- ------------------------------------------------------------------------------------------------------------------------------- Has anything (equipment, machine, tool, object, etc.) been displaced following the occurrence of the accident? ------------ -------------------------------------------------- ------------------------- -----------

------------- -------------------------------------------------- ------------------------- ------------------------ ------------------------ -------------------------- -------------------------------------------------- ----------------------- --------------------------------------------------------------------------------

The inspector’s full name and signature:                             Date:

Form No. S-109: Illustrating the accident scene using spatial data

N.I.O.C’s HSE Management

Number: ---------------                                               Full name of inspectors: ----------------------

Summary of the accident: --------------------------- Accident date: ------- Inspection Date: ----

| Object/equipment | Comments | Measurement reference | Distance from the measurement reference and direction | The cause of displacement | Unit |
| --- | --- | --- | --- | --- | --- |

The inspector’s full name and signature:                             Date:

Form No. S-110: The sketch of an accident scene

N.I.O.C’s HSE Management

Number: -------------------------------------------------------- Date: ------------------------------- Summary of the accident: ----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------

The inspector’s full name and signature:                             Date:

Form No. S-111: Recording visual data and evidence (videos and photos)

N.I.O.C’s HSE Management

Number: ---------------                                                  Full name of inspectors: -----------------------

Summary of the accident: --------------------------- Accident date: ------- Inspection date: -----------

| No. | Video/photo number | Equipment/system and location | Comments | Date and time |
| --- | --- | --- | --- | --- |

Full name and signature of the manager of the inspection team:                             Date:

Form No. S-112: Recording physical data and evidence (equipment, materials, components, etc.) and performed tests and analyses

N.I.O.C’s HSE Management

Number: ---------------                                                   Full name of inspectors: ----------------------

Summary of the accident: --------------------------- Accident date: ------- Inspection date: -----------

| No. | Physical data | Required performance test | Required laboratory test | Testing/sampling time and date | Comments |
| --- | --- | --- | --- | --- | --- |
| No. | Storage place (where spatial data and samples are stored) |  | Recipient of the sample (to conduct needed tests) | Delivery date and time | Comments |

Full name and signature of the manager of the inspection team:                             Date:

Form No. S-113: Recording written and electronic data and evidence (records, guidelines, logbooks, etc.)

N.I.O.C’s HSE Management

Number: ---------------                                                   Full name of inspectors: ----------------------

Summary of the accident: --------------------------- Accident date: ------- Inspection date: -----------

| Document | Unit | Comments | Storage place (where data are collected and analyzed) |
| --- | --- | --- | --- |

Full name and signature of the manager of the inspection team:                             Date:

Form No. S-114: The NIOC’s Root Cause Map

| 2) Number: |  | Summary of the accident |  |  |  |  |  | 1) B. Near miss □ |  |  |  | 1) A. Accident □ |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7) The final report |  | 6) Commencing investigations: |  |  | 5) The initial report |  | 4) Consequences |  |  |  | 3) Occurrence date and time |  |
| Date: |  | Date: |  |  | Date: |  |  |  |  |  | Date: |  |
| Time: |  | Time: |  |  | Time: |  |  |  |  |  | Time: |  |
| 10) Portable equipment (related to the accident) |  |  |  |  | 9) Fixed equipment (related to the accident) |  |  |  |  |  | 8) Units (related to the accident) |  |
| 14) Working shift (time) |  | 13) Job |  |  | 12) Company |  |  | 11) Full names of injured people |  |  |  |  |
| 15) Working shift (time) |  | 14) Job |  |  | 13) Company |  |  | 15) Full names of people related to the accident/incident |  |  |  |  |
| 17) Accident/incident Description |  |  |  |  |  |  |  |  |  |  |  |  |
| 17) Relevant conditions | Causal factor |  |  | 18) Chain of events |  |  |  |  |  |  |  |  |
|  | □ |  |  | Event 9: |  |  |  |  |  |  |  |  |
|  | □ |  |  | Event 8: |  |  |  |  |  |  |  |  |
|  | □ |  |  | Event 7: |  |  |  |  |  |  |  |  |
|  | □ |  |  | Event 6: |  |  |  |  |  |  |  |  |
|  | □ |  |  | Event 5: |  |  |  |  |  |  |  |  |
|  | □ |  |  | Event 4: |  |  |  |  |  |  |  |  |
|  | □ |  |  | Event 3: |  |  |  |  |  |  |  |  |
|  | □ |  |  | Event 2: |  |  |  |  |  |  |  |  |
|  | □ |  |  | Event 1: |  |  |  |  |  |  |  |  |
|  | □ |  |  | The event leading to the final consequence: |  |  |  |  |  |  |  |  |
| 20) Full name of the team members: |  |  |  |  |  |  |  |  | 20) Full name of the team manager: |  |  |  |
| 25) Proposed solutions |  |  | 24) Code of the root cause (based on the root cause map) |  |  | 23) Root causes |  |  |  | 22) Causal factor |  |  |
| 26) Comments (of inspection team members): |  |  |  |  |  |  |  |  |  |  |  |  |
| 27) Full name and signature of team members: |  |  |  |  |  |  |  |  |  |  |  |  |
| 26) Comments (of IAIC members): |  |  |  |  |  |  |  |  |  |  |  |  |
| 27) Full name and signature of IAIC members : |  |  |  |  |  |  |  |  |  |  |  |  |

Form No. S-115: The final accident/incident inspection and investigation report

N.I.O.C’s HSE Management

Form No. S-116: Collected data and evidence

N.I.O.C’s HSE Management

Number: ---------------                                                   Date: ----------------------

Summary of the accident: --------------------------- Full name of inspectors: -----------

| Information provided by witnesses and relevant people | Spatial evidence and information | physical evidence and information (equipment, chemicals) | Documents, records, computerized and digital information, etc. |
| --- | --- | --- | --- |

Full name and signature of the manager of the inspection team:                             Date:

Appendix 4: The NIOC’s Root Cause Map

Appendix 5: Supplementary report of major accidents

| No. | Title | Description |
| --- | --- | --- |
| 1 | Accident title | Includes the following items: - The type and severity of the accident and the type of damage (vehicle, personnel or fire) - The name of subsidiaries, associates and contractors, etc. - Occurrence date |
| 2 | Introduction | Includes descriptions of the following items: - The importance of the accident - Statistics available of similar accidents |
| 3 | Description | This comprehensive section must include the following items: - Full names of injured/deceased persons - The name of subsidiaries, associates and contractors, etc. - Damaged vehicles / installations - Measures taken following the occurrence of the accident |
| 4 | Possible scenarios | Possible scenarios (with respect to the available hypotheses and observations) |
| 5 | The investigation and inspection team | Includes the names of the manager and other members and their organizational positions |
| 6 | Evidence and findings | Includes the following items: - The results of the inspections carried out - Investigation documents and records - Job descriptions and available checklists, contracts, requirements, instructions and regulations, etc. - Photos - Relevant minutes and documents |
| 7 | Causes | According to relevant documents, causes of accidents can be presented at three levels: direct causes, indirect causes and root causes: 1) Direct causes: The main causes of an accident 2) Indirect causes: Unsafe acts and unsafe conditions should be enumerated in details. These causes include items such as undesirable design, processes, etc. 3) Root causes: Including strategies/human resources (structural and organizational problems)/basic technical and maintenance services/management of spare parts and consumables/executive processes (engineering design problems, principles governing executive processes and interactions)/ software systems/equipment performance indicators/change-based analyses/teamwork/performance assessment |
| 8 | Corrective and preventive actions | Includes all actions taken to recover a system from undesirable conditions caused by the accident. |
| 9 | Suggestions | Includes all suggestions and corrective actions provided to prevent similar accidents. |
| 10 | Appendices | Includes all items (forms, checklist, etc.) mentioned in the report, especially those mentioned in the findings section. |
| 11 | References | Includes all references used to compose the report. |

Appendix 6: Quantitative Risk Assessment Matrix

| Level | Scope | Accident consequence level |
| --- | --- | --- |
| 4: Catastrophic impact | Human resource | Death of personnel or limb amputation (one hand, one eye, etc.) (permanent disability) |
|  | Environment | The release of more than one ton of liquid or gas chemicals in the environment/the spillage of more than 10 barrels of oil into the sea for more than 12 hours/the spillage of more than 20 barrels of oil on land in 24 hours/severe damage to the natural environment (vegetation and animals) (outside the scope of the company/in stark conflict with environmental rules and regulations/significant , irreversible and long-term negative impacts on the environment/ flaring with black smoke for more than thirty minutes, resulting in public dissatisfaction ad complaint/the accumulation of more than 10 tons of hazardous liquid or solid wastes inside the factory (or more than one ton outside the factory) |
|  | Capital (equipment/production) | Significant damage to equipment and infrastructures or cessation of operations for more than 6 hours resulting in a complete failure of the unit or severe financial losses (an estimated repair cost of over $ 1 million at offshore facilities and $ 500,000 at land facilities) |
| 3: Serious impact | Human resource | Serious injuries (permanent disability) and negative effects on the health of individuals, resulting in long-term absenteeism and irreversible side effects. |
|  | Environment | The release of 0.3-1 ton of liquid or gas chemicals in the environment/ the spillage of less than 10 barrels of oil into the sea/ the spillage of less than 20 barrels of oil on land per shift/any complaints from people and neighbors (stakeholders) resulting from environmental pollution/considerable damage to the natural environment (vegetation and animals) (within the scope of the company)/receiving a verbal warning from the government with the possibility of legal obligation/flaring with black smoke for less than thirty minutes (no public complaint)/the accumulation of 3-10 tons of hazardous liquid or solid wastes inside the factory. |
|  | Capital (equipment/production) | Significant damage to vital equipment or infrastructures resulting in the failure of a part of the unit for approximately two weeks (an estimated repair cost of 100,000-1,000,000 dollar at offshore facilities and 50-500,000 dollar at land facilities) |
| 2: Slight impact | Human resource | Injuries causing temporary disability or exposure to hazardous and toxic substances and subsequently absenteeism and reversible effects on the health of personnel and the possibility of returning to work with full capacity. |
|  | Environment | The release of less than 0.3 ton of liquid or gas chemicals in the environment/the accumulation of less than 3 tons of hazardous liquid or solid wastes inside the factory. |
|  | Capital (equipment/production) | Damage to equipment/cessation of a part of production/there may be a need to re-start the process (an estimated repair cost of 10,000-100,000 dollar at offshore facilities and 5-50,000 dollar at land facilities) |
| 1: Insignificant impact | Human resource | Partial injuries with slight impacts on the health of personnel and the possibility of returning to work with full capacity. |
|  | Environment | The spillage of oil into the sea or on the land with no negative impact on the environment and no public complaint/faring operation that merely illuminates the local area overnight. |
|  | Capital (equipment/production) | Partial damage to equipment with a low probability of process cessation for repair works (an estimated repair cost of less than 10,000 dollar at offshore facilities and less than 5,000 dollar at land facilities) |
| o: No impact |  | No human injury/no process cessation/minimal repairs/no negative impact on the environment/no social and environmental cost |

Appendix 7: The CAPA (Corrective Action and Preventive Action) Form

| No. | Environmental incident □ | Inspection inconsistencies □ |  | Proposed solutions □ | Complaints/Warnings □ |  | Inspection and audit □ |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CAPA001 | Document and record control □ | Equipment and system failures □ |  | Wrong recognitions/orders □ | Training □ | Contractual problems □ | Operator (human) error □ |
| Summary: |  | Unit: ................... |  | Date of failure: ............................... |  | Person who reports the failure: |  |
| Description of the failure or accident): ................................................................................. |  |  |  |  |  |  |  |
| Description of corrective actions taken: .................................................................. |  |  |  |  |  |  |  |
| Further Details: .......................................................................... |  |  |  | Date of completion of corrective actions: ..................................... |  | Person in charge of corrective actions: ......................................... |  |
| Preventive actions |  |  |  |  |  |  |  |
| Causal factors/root causes: ............................................................................................ .............. |  |  |  |  |  |  |  |
| Further Details: ...................................................... |  |  | Date: ............................... |  |  | Person who has identified the root causes: ………………………… |  |
| Long-term corrective actions/preventive actions taken and the reasons for the prolongation of the implementation process: .................................... |  |  |  |  |  |  |  |
| Further Details: ...................................................... |  |  | Completion date: ............................... |  |  | Person in charge of implementation of long-term actions: ............................................ ........ |  |
| Important tips about the effectiveness of corrective and preventive actions taken: ........................................................... |  |  |  |  |  |  |  |
| Further Details: ...................................................... |  |  | Completion date: .................... |  | Person in charge of completing long-term actions: ............................................ ........ |  |  |

Appendix 8: The CAPA Follow-up Form will be subsequently communicated in the form of a software program.

Appendix 9: Lessons Learned Form

| Lessons Learned Form |  | Lessons Learned Form |
| --- | --- | --- |
| Accident: | Occurrence time: | Location: |
| Introduction: |  |  |
| Description: |  |  |
| A photo of the location Analysis of causes: 1. Direct causes 2. Indirect/superficial causes: - Unsafe acts - Unsafe conditions 3. Root causes |  |  |
| Proposed solutions to prevent the recurrence of similar major accidents: |  |  |

No. 17, Yaghma Alley, Between Hafez and Valiasr St., Jomhuri Ave, Tehran

Phone: (+98) 2166726016; Fax: (+98) 2166709723; SMS: 10007615;

HSEE@NIOC.IR                                                                         HTTP://HSE.NIOC.IR