National Iranian Oil Company Safety, Health and Environment Management Welding and Cutting Operation’s Safety Guideline Reviewing Translation; Confirmed by; Publication Date of English Version. Davood Naderi/ Laleh H. Abbasi Training & Performance Measurement Department; Mani Abdollahzadeh Rad/ HSE Manager; September 2018. Responsibility; Prepared by; Confirmed by; Approved by; Publication date. Full Name; Mani Abdollahzadeh/ Fatemeh Salimi [Signed] Head of Safety and Firefighting Department Safety and Firefighting Specialist; Siavash Derafshi [Signed] HSE Director; Reza Khalili [Signed] Non-industrial operations Deputy; Ali Kardor [Signed] CEO; November 2015. Position. Document Code: NIOC-HSE-SF-GU-024-00 Revision date: November 22, 2015Revision No.: 0

Table of Contents

1. Introduction	4

2. Scope of application:	6

3. Objectives:	6

4. Definitions	6

5. Legal Requirements	8

6. Responsibilities/Duties	8

6-1 CEOs	8

6-2 Managers and Supervisors (operations and repairs/employer's representative)	8

6-3 Technical Operational Heads	10

6-4 HSE Department	10

5-6 Contractor’s Personnel	10

7. General Requirements	12

8. Technical Notes on Welding	24

9. Technical Notes on Band Saw Cutting	32

10. Plasma Cutting	36

11. Cutting with Guillotine Machines	38

12. Cutting with Hanheld Grinders	40

13. Cutting with Wirecut Machines	42

14. Welding and Cutting In-Service Pipelines and Equipment (HOT TAP)	44

15 . Appendices	57

Appendix A - A Sample Safety Checklist for Electric Welding operations	58

Appendix B - A Sample Safety Checklist for Acetylene Welding operations	62

Appendix C - A Sample Safety Checklist for In-Service Pipeline Welding and Cutting Operations	65

# 1. Introduction

In all facilities under construction or already operating, welding and cutting operations are considered inseparable parts of the construction and repair processes. Inattention to the safety procedures and standards associated with these operations can be dangerous and sometimes disastrous.

The facilities, plants, and operational areas within the oil industry are not exceptions. Unfortunately, inattention to and noncompliance with safety procedures in the repair process of pipelines and other equipment have occasionally inflicted irreparable damage to the Company’s major capital, i.e., its workforce.

Thus, the present document was prepared in an attempt to protect workforce and equipment health and safety, and to promote the integrity of the Company’s HSE system. The document serves as a guideline to safe welding and cutting operations. It is drawn up by the Health, Safety and Environment (HSE) Department of National Iranian Oil Company (NIOC) and complies with the industry’s governing standards and regulations.

NIOC’s associate construction and production companies should prepare and implement the instructions needed to institutionalize the relevant implementation mechanisms and requirements of the document at the company level. These instructions, which are known as “Welding and Cutting Operations Safety Instructions” and are formulated by the companies and all their associated contractors should be complied with respect to conditions and the type of activity.

It should be noted that the provisions set out in the present guideline include the minimum requirements for the fulfillment of the relevant needs and should be implemented in all NIOC’s affiliate companies and their associated contractors.

# 2. Scope of application:

The guideline applies to all NIOC’s subsidiary and associate companies, departments, and operational areas.

# 3. Objectives:

Explaining the requirements for safe welding and cutting operations.

# 4. Definitions

- Company: Hereafter in this document, the word “Company” shall mean “National Iranian Oil Company (NIOC)”.

- Affiliate Companies: Affiliate companies shall include all NIOC’s subsidiary and associate companies.

- Employer: In this document, for each contractor, “Employer” shall be the employer referred to in the relevant contract.

- Contractor: All the contractors who have signed a contract with one of NIOC’s affiliate companies.

- Contractor's Workforce: All the personnel working under the supervision of a contractor party to the contract with NIOC or any of its affiliate companies.

- Metal Welding: An operation aimed at fusing metal and/or metals by applying heat, pressure, and/or electricity.

- Electric Arc: When the positive and negative poles of an electric current are brought closer to each other, a brief spark is produced just before the contact takes place.The flame ignited by this brief spark is called an electric arc.

- Arc Welding: In this type of welding, electric arc is used to produce enough heat to melt metals and, consequently, join and fuse metal to metal.

- Electric resistance welding (ERW): It is a pressure-assisted welding, where the necessary heat is produced by a resistance resulting from an electric current passing through the weld spot between two metals.

- Gas Welding: It is a type of welding, where the source of heat used to melt metals (base metal or filler) is supplied by burning a gas in the vicinity of oxygen.

- HOT TAP: It involves attaching a branch connection to a main pipe in an operating pipeline.

- Hot cutting: In hot cutting, a flame is used to heath a metal up to its melting point. Next, the molten material is removed by applying oxygen pressure, resulting in a cutting groove.

# 5. Legal Requirements

5-1 NIOC Technical Safety Regulations - 1974

5-2 NIOC Occupational Health and Safety Laws and Regulations

5-3 Ministry of Labor and Social Affairs Code of Safety for Hot Work - 2008

# 6. Responsibilities/Duties

## 6-1 CEOs

- Ensuring the provision and allocation of financial resources, and providing the necessary facilities needed to implement the guideline, including the required welding and cutting equipment and tools and standard safety equipment dependent on the workplace hazards

- Ensuring the fulfillment of the guideline’s requirements

## 6-2 Managers and Supervisors (operations and repairs/employer's representative)

- Supervising the recruitment, hiring, and job-assignment of experienced and skilled welding and cutting personnel

- Interacting with the Education and Training Department and developing training programs for welding and cutting personnel

- Supervising the provision of an appropriate, standard, and safe environment to carry out welding and cutting operations

## 6-3 Technical Operational Heads

- Providing the necessary equipment to ensure safe welding and cutting

- Ensuring that all the personnel have received the necessary training concerning the safe performance of operations and inspection of welding and cutting equipment

- Ensuring the correct and safe performance of operations

## 6-4 HSE Department

- Preparing a personal protection and safety instruction based on the present guideline and interacting with the relevant departments

- Supervising the fulfillment of the requirements referred to in the guideline by the relevant department through periodic inspections

- Ensuring the correct implementation of the method by the contractor

- Ensuring the health and reliability of the equipment used in welding and cutting operations

## 5-6 Contractor’s Personnel

- Obliged to follow the requirements to safe welding and cutting as mentioned in the present guideline and all other relevant instructions

- Communicating the potential defects to the supervisors and providing their recommendation to the relevant department’s supervisors

# 7. General Requirements

7-1 Performance of welding and cutting operations requires that a permit-to-work document be issued according to the permit-to-work system guideline (Document No. NIOC.HSE.SF.GU).

7-2 The welding and cutting operations permit should at least include process type, occupational hazards, control measures, and job duration.

7-3 In venues where welding operations are carried out and personnel rather than the welders work and/or commute, fixed or movable curtains (barriers) with a minimum height of 2 m should be installed.

7-4 Curtains (movable barriers) should be painted black or dark gray (matt) to absorb the harmful light and prevent its reflection. The curtains are used as permanent or temporary protection against the hazards resulting from welding or cutting operations.

7-5 In welding or cutting operations, small or medium-sized workpieces should be placed on a work table or counter whose surface is covered with a metal plate.

7-6 All the electricity-carrying components of welding and cutting machines being fed from rectifier generators and/or transformers should be protected against the risk of contact with bare components and conductors connected to voltage supplies.

7-7 The air vents on the transformer cover should be designed so that no object could be slided therein.

7-8 The bodies of electric welding machines should be effectively equipped with an earthing system.

7-9 The spots where power cables are connected to the machines should be insulated.

7-10 The outer surface of electrode clamps and holder jaws should be insulated as far as possible.

7-11 Electrode clamps should be equipped with protective plates or shields to prevent the operator’s hand from being burnt by the heat from the electric arc.

7-12 In electric resistance welding machines, all electricity-carrying components except the weld spot should be fully covered and protected.

7-13 Electric resistance welding machines should be equipped with circuit breakers. These circuit breakers are either installed on the machine or located near it.

7-14 Bolts and nuts should be used to securely connect the power-supply conductors to the welding machine. Connectors are only allowed in the circuits supplying power to the welding spot.

7-15 Automatic and semi-automatic electric resistance welding machines should be equipped with protective barriers. The barriers should be designed so that the operator can easily operate the machine with both hands and prevent him from reaching into the danger area.

7-16 CO2 cylinders should be available at the welding and cutting site to extinguish potential transformer fires. Dry chemical powder cylinders should also be available to extinguish fires (Class A and B) originating from flammable materials. If necessary, on-site firefighters can also be employed after coordinating with HSE Department.

7-17 In workshops or welding sites, local exhaust ventilation and general exhaust ventilation systems should be used to effectively reduce the contaminants produced by welding operations.

Fixed or portable local exhaust ventilation systems have proved more effective.

7-18 You should never look at an electric arc from a distance less than 6 m as it can lead to corneal flash burns.

7-19 Before starting the welding operation, always make sure that the machines’ amperage is correctly set. The machine’s earthing system and resistance should also be checked.

7-20 The Welding machine should be equipped with fuses corresponding to the rate of the electrical current running through it.

7-21 The main input terminal should be placed inside an enclosure within the machine to avoid possible contact with hand or other tools and objects.

7-22 The oils and moisture levels in oil-filled transformers should be regularly checked.

7-23 Before operating the machine, always check the insulation status of cables and work clamps and the heat generated by the transformer. In case of any defect, refrain from using the machine until the problem is fixed.

7-24 Before starting the welding operation, always make sure that all the fittings are fully secured and cleaned.

7-25 Check the following in the electrode clamps:

- Electrode holder’s bolts being firmly secured

- Potential burns or fractures in electrode holder's insulation

- Inappropriate cable fitting or excess heat production

7-26 The machine's main switch should always be accessible so that the operator can turn it off in case of emergency.

7-27 Make sure that the welding cables correspond to the maximum output electrical current (generated by the machine).

7-28 To prevent accidental contact with high-voltage current, make sure that welding wires are placed away from the main wires and cables.

7-29 The electrode clamps should be placed in a safe place to avoid contact with individuals, flammable materials or leaked gases.

7-30 Used electrodes are lower than 38-55 mm in length after use should not be reused.

If the electrode’s length is reduced to less than 38-55 mm, it will sustain damage and might generate a short circuit.

7-31 Place the used electrode in a special compartment to prevent its contact with the operator.

7-32 When welding tanks containing flammable liquids, they should be fully emptied. The welding operation is performed only after we have made sure that the tank is fully emptied of its content and is safe and cleaned by water, steam, etc.

7-33 Never cool down the electrodes by putting them into water.

7-34 Turn off the welding machine, before switching the poles.

7-35 When raining or snowing, do not perform welding and cutting operations outdoors.

7-36 No welding or cutting activity should be performed in the vicinity of flammable and explosive materials or in places where dust and flammable and explosive vapors and gases are found. While coordinating with HSE Department, the supervisor or operator should make sure that the minimum safety distance is maintained from flammable and explosive sources as stipulated in the relevant permits.

7-37 Before performing any welding and cutting operation, the workplace should be cleaned of any flammable materials.

7-38 Workers involved in the welding operations should use a welding shield and wear a special mask, a safety helmet, long leather gloves and a leather napkin.

7-39 Aluminum foils, canvases, or fireproof blankets should be used underneath the work and especially in the areas where welding operation are being performed.

7-40 Use of defective or abraded cables is prohibited.

7-41 All electrical connections (cutting and welding machines) should be performed by the electrical repair and maintenance personnel. Welding personnel are not allowed to perform electrical connections.

7-42 Use of angle grinders without protection is prohibited.

7-43 When performing welding operations, the floor should be dry. If the floor is wet, wood, rubber, or dry lightweight clay aggregates should be used as the mat.

7-44 While performing welding operations in tight and small spaces, vessels, tanks, and metal floors, DC-powered machines should be used.

7-45 In roofed areas, especially vessels, tanks, etc., a confined space work permit should be acquired and an isolation transformer be used in cutting operations. In addition, lightning should be supplied from a 24v power source.

7- 46 Welding operations should not be performed in places where light might be reflected in the welding mask.

7-47 After the welding operation is complete, do not leave the electrodes in the electrode holders. Make sure the machine is turned off and disconnected from the power supply.

The cables should be collected, bundled, and stored in a safe place.

7- 48 While performing the welding operations, wear long leather gloves and other protective gears.

7-49 Operators should not carry or wear clothes made of flammable materials.

7-50 Gases and vapors produced by welding operations endanger the health of workers. As a result, local exhaust ventilation systems should be used in welding workshops.

7-51 Before performing any welding operation, pipes should be emptied of containing gas, vapor, and chemical liquids.

7-52 Earthing system should not be connected to the pipes, machines and other equipment.

7-53 Do not use defective cables. The connection should be near the welding spot.

7-54 Personal Protective Equipment (PPE) should be selected, procured, and used according to Document No. NIOC.HSE.SF.GU.014.00. The equipment includes the following:

- Welding uniform

- Leather gloves

- Safety shoes

- Leather gaiters

- Leather apron

- Welding mask preferably with a safety helmet)

- Respiratory protective mask (mask filter FFP2)

7- 55: Welding goggles: Welding goggles should be used to protect eyes from damage caused by Ultraviolet (UV) and infrared radiations.

Welding goggles’ lens shade should be selected according to the following chart:

(Table 1: Lens Shade Selection Chart for Welding Goggles

prepared based on the Personal Protection Equipment (PPE) Guideline, Document No. NIOC.HSE.SF.GU.014.00)

| Minimum Shade | Electric Arc Current | Electrode Size 1/32″ | Type of Activity |
| --- | --- | --- | --- |
| 7 | < 60 | < 3 | Electric Arc Welding |
| 8 | 60-160 | 3-5 |  |
| 10 | 160-250 | 5-8 |  |
| 11 | 250-550 | > 8 |  |
| 7 | < 60 |  | Electric Arc Cutting and Fluxed-Core Arc Welding |
| 10 | 60-160 |  |  |
| 10 | 160-250 |  |  |
| 10 | 250-550 |  |  |
| 8 | < 50 |  | Tungsten Welding |
| 8 | 50-150 |  |  |
| 10 | 250-500 |  |  |
| 10 | < 500 | Light | Flame Cutting |
| 11 | 500-1000 | Heavy |  |
| 6 | < 20 |  | Plasma Welding |
| 8 | 20-100 |  |  |
| 10 | 100-400 |  |  |
| 11 | 400-800 |  |  |
| 8 | < 300 | Light | Plasma Cutting |
| 9 | 300-400 | Medium |  |
| 10 | 400-800 | Heavy |  |
| 3 |  |  | Soldering Carbon Arc Welding Riveting |
| 2 |  |  |  |
| 14 |  |  |  |

7-56 Never perform welding or cutting operations in the vicinity of flammable materials or in places where items such as oil or wood chips are scattered on the floor.

Before starting the operations, all the aforementioned items should be taken out of the site and stored at a safe distance.

7-57 All degreasing and cleaning operations using chlorinated hydrocarbons and associated processes are prohibited in welding and cutting workshops.

# 8. Technical Notes on Welding

8-1 Never lubricate acetylene/oxygen cylinders’ valves and fittings or touch them with greasy hands and/or gloves.

8-2 To avoid the risk of explosion and prevent gas flow blockage and regulator damage, cylinder valves should be slowly opened and quickly closed before installation and the necessary measures should be taken to stop dust entering cylinder sockets (except hydrogen cylinders).

The regulator should be fully fastened and installed on the cylinder with utmost caution to avoid gas leakage. 8-3

8-4 The regulators installed on acetylene cylinders should be adjusted so that their pressure never exceeds 1.5 atm.

8-5 To prevent the return of gases toward the torch, use the valve specially designed to prevent gas return. Use a safety wall between the torch and the hose to prevent flashbacks.

8-6 The green gas transfer hoses (or black hoses) should be connected to the oxygen regulator, whereas the red hoses should be connected to the acetylene (or other fuel gas) regulator.

Additionally, the color of hoses and gas cylinder bodies should comply with ISIRI (Institute of Standards and Industrial Research of Iran) standards No. 3792 and 712, respectively.

8-7 When the flashback occurs due to the contact between the work or overheating, the oxygen and acetylene cylinder valves should be first closed and the torch’s top should be plunged into water.

8- 8 Use connectors to fasten the different parts of the acetylene gas pipes. If possible, use uniform hoses to prevent any potential detachment.

8-9 The torch should be appropriately tuned on and off to prevent potential flashbacks. When turning on the torch, the oxygen cylinder valve should be opened before the acetylene cylinder valve. When turning off the torch, the acetylene cylinder valve should be closed before the oxygen cylinder valve.

Only in case of flashbacks, the oxygen cylinder valve should be closed before the acetylene cylinder valve to prevent oxygen entering the torch.

8-10 To turn on the torch, the oxygen cylinder valve should be slowly and fully opened. However, the acetylene cylinder valve should only be opened 1-1.5 turns.

8-11 Use welding torch strikers to turn on the torch. Matches, hot metals or electric arcs should not be used to ignite the torch as they will cause a serious combustion if the gases are not immediately burnt.

8-12 In the event of leakage from a fitting especially the regulator fitting, the failing part should be immediately replaced.

8-13 Never fully open the oxygen cylinder valve at once as this will burn the pressure reducing valve and fire will be imminent.

8-14 The gas transfer hoses should not be put and left on the ground or where they might be walked on or run by individuals or vehicles.

8-15 Before starting the welding operation, make sure the gas hoses are intact and check them for potential folds.

8-16 Use the special wrench to open or close the cylinder valves. Avoid applying excessive pressure. Do not use pliers and/or an adjustable wrench.

8-17 When the operation is completed, fully close the acetylene and oxygen valves before opening the torch valves to let the remaining gas exit the pipeline. Finally, the gas pressure valves and the torch valve are fully closed.

8-18 A wrench should always be attached to the cylinder valve so that it can be closed if emergency arises.

8-19 The regulator’s pressure gauge should be functionally tested at least once a year.

8-20 The regulators designed for a specific kind of gas should not be used for other gases.

8-21 Acetylene cylinders should not be exposed to open flames (e.g., welding/cutting torches, electric arcs, molten debris, sparks, radiations, and sunlight) as acetylene will decompose when heated. This will increase the cylinder’s internal pressure to a dangerous level, creating the risk of explosion. The cylinders’ maximum tolerated temperature is 54° C.

8-22 Do not use direct flame to heat the cylinder and/or the regulator already frozen due to the cold weather. Use hot water instead.

8-23 During the welding operations, cylinders should be kept away from the working site.

8-24 When exposed to air, acetylene has a fairly high combustion range. Thus, utmost care should be taken when dealing with acetylene to prevent fire.

8-25 Always use soap solution when performing a leak-tight test on the poppet valve and gas outlet’s union fitting.

8-26 Never use the word “gas” to refer to “acetylene”.

8-27 The safety head should be installed on the acetylene cylinders when transporting them. Furthermore, acetylene cylinders should not be transported horizontally. Use trolleys and/or pallets to transport them vertically.

8-28 When moving cylinders with cranes and/or forklifts, use appropriate compartments and pallets. Refrain from using the crane’s chain or wire rope or the forklift’s forks to move cylinders.

8-29 If the acetylene cylinder accidentally falls on the ground, it should be placed vertically on the ground for at least one hour before being used.

8-30 Never push or lift the cylinder by its head.

8-31 The use of the cylinder as a roller, support, or retainer is strictly prohibited.

8-32 Before disconnecting the fittings, the empty cylinder’s valves should be fully closed. After installing the safety head on the empty cylinder and attaching a label onto its body indicating that the cylinder is empty, store it in an appropriate place, away from the full cylinders.

8-33 The cylinder pressure test should be performed annually.

8-34 The Cables should not twist around the welding transformer otherwise they create a magnetic field which can combine with that produced by the transformer, contributing to a potential fire.

8-35 Appropriate connectors and other equipment should be used to connect the hoses.

8-36 Safe storage of oxygen cylinders:

- Oxygen cylinders should not be exposed to open flames, electric arcs, molten debris, and sunlight.

- When moving these cylinders, take the necessary precautions to ensure they are not heavily shaken. Use the special pallets to transport the cylinders. Install the safety head before moving the cylinders.

- Oxygen cylinders should not be stored near acetylene cylinders or other flammable materials (e.g., gasoline, oil).

- Lubrication and greasing are prohibited. Do not touch oxygen and acetylene cylinders with greasy hands as this might set them on fire or contribute to their explosion.

- Never lubricate the oxygen cylinders’ movable parts of the oxygen cylinder placed before the regulator or the point where the rubber pipes connect to the oxygen and acetylene valves. Do not use greasy tools and equipment.

- Both oxygen and acetylene cylinders should be stored in a ventilated room. They should not be exposed to direct sunlight.

- Oxygen cylinders should be placed vertically and be equipped with electrical insulators.

- The oxygen cylinder’s valve should be opened slowly. Do not apply pressure or open it instantly as this might prove dangerous.

8-37 Safe storage of acetylene cylinders:

- Acetylene cylinders should be placed vertically. They should be retained by supports and special belts. Acetylene cylinders should not be placed horizontally on the ground. The belt should be placed on the cylinder’s top.

- Acetylene cylinders should be stored in a ventilated place equipped with sunshades.

- Cylinders containing acetylene or other fuel gases should not be placed near oxygen cylinders (there should be placed at a minimum of 6 m distance).

- The minimum distance between the cylinders containing acetylene or other flammable fuel gases and flammable materials (e.g, paints, solvents) should be 6 m.

- Oxygen cylinders should be equipped with safety valves to avoid explosion of exposed to heat and/or fire.

- Acetylene cylinders and tanks should be specified by writing the word “acetylene” on their bodies or attaching a special label onto them.

- Acetylene cylinders should not be placed near flammable liquids or exposed to heat.

- The regulator’s pressure gauge should be tested at least once a year.

- Do not use direct flame to heat the cylinder and/or the regulator already frozen due to the cold weather. Use hot water instead.

- Always use soap solution when performing a leak-tight test on the cylinders.

- When transporting the cylinders, avoid any impact as the acetylene gas is mixed with acetone and the smallest impact could lead to an explosion.

- The flame Arrestor (a flashback preventing system) should be installed on tip of the acetylene hose to prevent potential flashbacks.

# 9. Technical Notes on Band Saw Cutting

9-1 Check the blade to ensure that it is of the right type and has the right width. Make sure the blade is intact.

9-2 Adjust the machine before connecting it to the power outlet.

9-3 Ensure that the workpiece is free from nails or other foreign objects.

9-4 When using the band saw, wear personal protection equipment (e.g., safety shoes, safety ear plugs, safety helmet), and, if need be, a tarpaulin apron.

9-5 Ensure nothing is within the work site’s range. Eliminate the risk of falling objects. Make sure that the table is free from wood particles. Turn off the machine before cleaning the table with a brush. When cleaning the table with the brush, the power control switch should be turned off.

9-6 Make sure what you intend to do is within the saw’s capacity.

9-7 Ensure the top guard is as close as possible to the workpiece.

9-8 The guidepost guard and other accessories should be installed before starting the cutting operation.

9-9 Ask for someone’s help when cutting large pieces of lumber. The help should not guide or pull the lumber. He is only there to hold the lumber.

9-10 keep your distance while turning on the power switch. Never turn on the machine when someone is standing along it. Never stand along the machine’s wheels.

9-11 Focus on the work. Do not let minor disruptions distract your attention while working with the band saw.

Looking around or talking with others while working with the machine is fairly dangerous.

9-12 Keep your fingers at a minimum 5 cm distance from the blade while cutting. Never put your hands in front of the band saw’s blade. Exercise utmost caution while cutting the curves. It is easier to cut the wood in the direction of wood in the direction of fiber growth. Moreover, never hold the lumber in a way that your fingers or thumb is placed along the blade.

9-13 Do not place your hand near the blade. Never attempt to touch a moving blade.

9-14 Do not apply pressure on the saw. Move the lumber toward the saw with the same speed the blade’s ribs move. Unnecessary pressure might overheat or break the saw.

9-15 Round pieces should be mounted on a clamp if they are going to be cut with the band saw. When cutting, do not hold cylindrical wood pieces with your hands as it is highly dangerous. Cylindrical pieces tend to spin and will most likely push the saw.

9-16 If you need to adjust certain settings or in case of a problem, turn the power switch off. Small pieces of wood often fall into the table’s cavity. Never attempt to take them out while the saw is turned on.

Turn off the machine when the saw slot is blocked, the piece of wood is stuck and cannot be moved, and/or the belt is torn.

9-17 When the cutting operation is complete, turn off the power switch and unplug the power cord. Do not leave the site until the blade has completely stopped moving. Never leave a running saw unattended.

9-18 Avoid loose-fitting clothing when working with a band saw.

9-19 Before starting the cutting operation with the band saw, make sure the first aid kit is available at the site.

# 10. Plasma Cutting

10-1 protect all cables against damage.

10-2 Do not mount the cutting torch on the clamp.

10-3 Never use a wrench to replace the parts as it might damage the torch.

10-4 Welding and cutting fumes are harmful for your health. Thus, welding and cutting operations should only be carried out in well-ventilated places or sites equipped with exhaust hood.

10-5 Do not hold the workpiece with your hand. Use the appropriate insulated tool to hold the workpiece.

10-6 Never use the torch near or close to flammable materials.

10-7 Never use the torch to cut empty barrels as there is a risk of explosion.

10-8 Cutting operations should not be performed in wet and humid environments.

10-9 Turn off the machine before replacing or repairing the torch's parts. Never turn off the machine before the air flow cycle has stopped.

10-10 Replace the nozzle if the orifice has enlarged or loosened. Always use a nozzle with the appropriate diameter.

10-11 Fasten the exhaust nozzle with your hands.

10-12 The machine should only be repaired by the relevant skilled and experienced personnel.

10-13 Always ensure the cutting machine is equipped with earth an earthing system. Beware of sudden electric shocks.

10-14 Regularly control the welding cables to ensure the insulated parts are not broken or torn. All spotted defects should be repaired.

10-15 Always wear eye protection. Continuous exposure to ultraviolet and infrared radiation can cause eye damage.

10-16 The machine’s input power is strong enough to cause death. Thus, the input cables should always be repaired by the experienced personnel.

10-17 When cutting, always wear leather safety gloves, fire resistant sleeves, safety shoes, respirators, and safety glasses (shade 12).

10-18 Inspect and clean the machine and its joints before performing the operation and ensure it is intact and safe to use.

10-19 The cutting machine should be kept in a roofed place and protect it from rain and snow. When raining, the cutting operation should not be performed outdoors.

10-20 If the floor is wet, stand on a dry surface and wear rubber-soled shoes.

# 11. Cutting with Guillotine Machines

11-1 When working, use safety equipment, including safety shoes and safety ear plugs.

11-2 While cutting, do not cross your hand under the blade guard to Reduce waste. The machine should be handled using both hands.

11-3 When cutting the sheets, make sure they do not collide with the blade guard and protect the guard against damage.

11-4 Inform the relevant personnel of any defect. Stop working with the machine as soon as you spot any defect.

11-5 The space around the blade should be clean and free from any waste. The part and waste pallets should be arranged orderly.

11-6 When transporting sheet packages and part pallets by aerial cranes and/or forklifts, exercise utmost caution as they might collide with the personnel.

11-7 Inspect wire ropes, hooks, and other equipment used for transporting goods by overhead cranes and make sure they are intact and safe to use.

11-8 The use of overhead cranes to move goods over an area where individuals are working is prohibited.

11-9 Overhead cranes should not be used to transport unpacked sheets.

11-10 When transporting sheets with an overhead crane, the individuals working along the route should be alerted and utmost caution should be exercised.

11-11 The personnel working behind the machine, removing the cut sheets, should wear special safety equipment, including safety shoes, safety gloves, and safety ear plugs. They should be careful of the falling sheets and their potential collision with their hands.

# 12. Cutting with Handheld Grinders

12-1 Use a grinding stone wheel with the following information available on the label attached to it:

- Name of the manufacturing factory

- Stone composition and size

- Maximum allowed RPM (round/min)

12-2 The grinding stone wheel should be protected against impact and shock.

12-3 The grinding stone wheel should be kept in a dry place to avoid damage caused by humidity.

12-4 The grinding stone wheel should be mounted by a skilled worker.

12-5 A safety shield should be installed on the grinder.

12-6 When mounting the grinding stone wheel and attaching the side shields, place a rubber, cardboard, or leather sheet between them.

12-7 The grinding stone wheel should be covered by an adjustable shield made of a resistant and hard material. The distance between the covering wall or shield and the grinding stone wheel should not exceed 5 mm.

12-8 After mounting the grinding stone wheel, test it for 5 minutes. While testing, keep the personnel away from the machine.

12-9 To avoid the risk of falling apart, RPM should not exceed the predetermined RPM.

12-10 The support should be as close as possible to the grinding stone wheel’s edge (the minimum distance between the support and the edge should not exceed 3 mm).

12-11 When using the grinder, always wear safety glasses despite the shield’s transparent panel.

12-12 If you are going to use the grinder for long hours, use a special ventilator to remove the resulting dust from the site.

12-13 The grinding stone wheels that have lost their roundness should be made round with the help of a diamond pen or other relevant tools.

12-14 Never mount the grinding stone wheel on a round saw axis.

# 13. Cutting with Wirecut Machines

13-1 The hoses carrying gas and oxygen to the main regulator should be checked before using the machine.

13-2 Hoses should be checked to ensure they are securely fastened by the appropriate connectors.

13-3 Any leakage in the gas system should be controlled.

13-4 The power cables should be checked to ensure they are intact and safe to use.

13-5 The main switch should be checked to ensure its fast and proper operation.

13-6 The emergency stop switches located at the carrier’s sides should be checked to ensure they are functional.

13-7 The carrier's bumpers located at the end of the rails should be inspected.

13-8 The breakers terminating the machine’s movement (micro-switches) should be tested.

13-9 The nozzles should be checked.

13-10 The machine’s rails and body should be cleaned and become free of any potential obstacle.

13-11 The equipment controlling valves, hoses, and nozzles should be cleaned og oil and grease.

13-12 The protective fences around the machine should be intact.

13-13 The waste accumulating under the table should be regularly removed.

13-14 The cut pieces should be orderly arranged and kept at a specific place.

13-15 The individual operating the torch should wear the necessary safety equipment, including safety glasses, safety shoes, safety gloves, and safety ear plugs.

13-16 Boards or softer objects should be placed below the pieces being cut to avoid potential damage to the grinding stone wheel.

# 14. Welding and Cutting In-Service Pipelines and Equipment (HOT TAP)

The following requirements should be met when conducting safe hot welding and hot tapping operations on in-service pipelines and equipment.

14-1 HOT TAP operations are performed for safe drilling and installation of pipes for the equipment carrying oil and gas.

Thus, flammable and dangerous materials would not be exposed to air, reducing the risk of fire or explosion caused by hydrocarbonic substances.

14-2 Before starting the Hot Tap operation, the hot work permit should be obtained with all safety requirements being specified therein.

14-2 The materials used in the HOT TAP machine’s body and seals should be consistent with the content of pipes, tanks, and other containers and be confirmed by the Technical Inspection Department.

14-3 The HOT TAP machine should be resistant to pressure, temperature, and potential mechanical stresses during the operation.

14-4 The cutting piece should be suitable for effective penetration into the desired metal.

14-5 Welding operation should not be performed when the machine’s pressure and temperature are higher than the conventional pressure and temperature. Moreover, welding should not be performed under vacuum conditions.

14-6 Before starting the operation, the HOT TAP machines and all its accessories should be inspected to ensure they are suitable for the job.

14-7 The Technical Inspection Department should specify the welding method (WPS).

14-8 The potential problems occurring during the welding of in-service equipment include burn through and cracking.

Cracking: It is the result of rapid cooling of welds. Cooling can also be due to the rapid removal of heat from the pipe’s contents.

14-9 To avoid burn through, the heat transfer level during the welding operation should be taken into consideration.

14-10 The cooling procedure of the welds should be accurately evaluated to determine the appropriate input temperature needed to avoid cracking.

14-11 Before starting the operation, the containers and pipelines should be inspected to ensure that they have the appropriate thickness and are free from defects.

14-12 For specific welding operations especially those involving steel with high tensile stress, preheating operation should be performed to avoid cracking.

14-13 In some cases, where flammable mixtures might be produced due to the lack of flows, pressured steam or nonflammable gases should be run through the pipelines. These flows should be present during the welding operation.

14-14 The minimum thickness required for the operations should be documented. The minimum recommended thickness is 4.8 mm. The minimum actual thickness is a function of the thickness required for increased strength and improved safety. The minimum actual thickness required to prevent burn through is often 2.4 mm. Apart from the above-mentioned thicknesses, NIOC’s specialists might determine the necessary thickness based on the operational pressure and the metallurgical properties of the metals in question.

14-15 Welding should not be performed on pipelines or equipment with a liquid or metal temperature of less than 45 °C unless they are heated before the operation.

14-16 The HOT TAP operation should not be performed on the equipment under vacuum conditions unless the safe operation instructions are formulated after undertaking the relevant careful assessments.

14-17 Welding and HOT TAP operations should not be performed on the equipment with the following conditions:

A) The presence of air/steam or oxygen/steam close to the combustion range along with the heat produced by the welding operation might lead to the combustion and explosion of the mixture.

B) Hydrogen, carbon steel alloys, and ferrite are prone to high-temperature hydrogen attack.

C) Peroxide, chlorine, and other chemicals likely to decompose during the operation.

D) Caustics, amines, and acids with temperatures or concentrations requiring the preheat operation might contribute to the cracking of the weld’s surface or thermal effect.

14-18 Before performing the welding or HOT TAP operation, a procedure/instruction should be prepared. This procedure/instruction should include the following:

A) Connection design

B) HOT TAP operation performance method

C) Welding details, input temperature

D) Safety, health, fire-prevention, emergency response, and other relevant considerations

14-19 The operation should be performed by approved welders having adequate welding skills and experience. The machine should only be installed and set up by the skilled personnel who have gone through the relevant theoretical and on-the-job training.

14-20 Test results show that prolonged contact or repeated contact with liquids or vapors might cause injury to the personnel or hurt them.

Thus, all the substances that might be produced at the welding point should be identified.

The necessary information on the health hazards of such substances including SDS information should be provided to the users by the relevant authorities.

If the personnel are at risk of being in contact with toxic substances, the following control measures should be taken:

A) Determining the probability of contact through assessment and analysis

B) Adopting engineering or management methods or using protective equipment to minimize skin contact and inhalation of vapors or fumes

14-21 When performing HOT TAP or welding operations on in-service tanks, the following should be taken into consideration:

A) The safe distance between the welding site and the ventilator of the tanks likely to contain flammable fumes should be taken into account as the flammable fumes might reach the site and result in combustion and explosion.

B) During the hot work operations, the tank should not be emptied or filled and its content should not be altered.

The mixer’s switches should be turned off and the device disconnected.

C) All the valves connected to the pipelines should be closed and sealed with stickers.

D) When performing welding or HOT TAP operation on tanks with cone-shaped roofs, the liquid level should be maintained at least 1 m above the welding site.

E) The welding operations are performed on floating roof tanks only if it is down at a level below the liquid’s level.

How work should never be performed on the tank’s roof or available openings.

14-22 If the hot work is performed below the earth’s surface, the following should be taken into consideration:

A) Some excavation should be carried out to accelerate the workers’ movement throughout the premises.

B) Accessible exits and escape routes should be located throughout the premises.

C) Make sure the atmosphere is present at the confined space or pit where the hot work is to be carried out. The oxygen level should be measured and the area should be inspected for the present of flammable toxic substances. If oxygen level is low and flammable and toxic materials are present at the site, necessary equipment should be used for ventilation. The personnel should also wear the appropriate respiratory equipment. During the operation, the oxygen level should be continuously monitored so as to remain informed of the situation at the site.

14-23 If a pipeline passes through an intermediate casing, ensure that the welding is performed on the pipeline rather than the casing.

14-24 Before performing the Hot TAP or welding operation. The following should be considered:

A) An expert should be present at the site.

B) The spot where the connection should take place should be identified and physically marked.

C) The metal’s thickness should be approved and all the defects likely to contribute to potential problems during the welding operation should be investigated.

D) A program should be devised to monitor and control the process variables that need to be limited during the operation.

E) All the tests concerning oxygen, flammable fumes, and toxic gases should be performed to ensure safe working contortions are present at the site. All the necessary coordinations should also be made with the HSE Department to obtain the required relevant permits.

F) The work safety and health hazards should be assessed. Personal protection  and fire extinguishing equipment should be available at the site. An individual equipped with a dry powder fire extinguisher or a high-pressure fire hose should also be deployed at the site.

G) Sings and barriers should be installed at the site to inform the personnel and separate the work site, preventing unauthorized personnel’s entry.

H) Specific procedures and methods  should be developed to separate the work site from the adjacent areas in case of an emergency.

I) The personnel should be trained for HOT TAP operations and have the required skills.

14-25 Inspect all the fittings after the welding is over. Perform dye penetrant, ultrasonic, and/or magnetic testings. If the inspections are done after the first phase, the surface should be cleaned of any foreign substance before resuming the welding operation.

This method should not replace the hydrostatic or pneumatic tests and its results are not to be fully trusted.

14-26 The following steps should betaken to connect the HOT TAP machine:

A) The valve should be of the right size and material and have the appropriate metallurgic properties.  The valve should be tested for any potential leakage.

B) After the cutting machine was installed on the valve, open the small bleed fitting embedded on the machine.

The cutting blade should be passed through the opening several rime to ensure it does not get stuck inside it.

The cutting blade’s motion should be calculated with utmost precision and accuracy especially when thinner pipes are concerned. This will ensure that the other side will not be cut and damaged.

C)The drill’s clutch and the bit’s entry point should be adjusted.  When the pilot bit penetrates the cutting machine, the bleed valve should be closed.

Apart from the clutch, the cutting operation can be assessed by the resistance of the handlebars rotated manually or the increased pace of the machine powered by air or water.

D) The cutting machine should be moved back and the valve be closed. The pressure relief valve should be opened too. If the cut piece is lost, do not make any attempt to find or take it out.

In some cases, the machine should be immediately stopped to find the cut piece.

14-27 The weld’s and HOT TAP machine’s fittings should be properly checked before the cutting operation.

A) Checking whether the bolts and nuts, and seals and their nuts are securely fastened

B)  If the pipeline’s or container’s temperature allows, the hydrostatic test should be performed.

14-28 The pressure test should be at least equal to the pipeline’s or container’s pressure. However, it should not exceed approximately 10% of the internal pressure so as to avoid the pipe’s or container’s internal collapse.

If the temperature is not suitable for hydrostatic test, use air or nitrogen with soap solution to perform the test. If air is used, precise assessments should be conducted to prevent the formation of flammable mixtures.

# 15 . Appendices

Appendix A - A Sample Safety Checklist for Electric Welding operations

Appendix B - A Sample Safety Checklist for Acetylene Welding operations

Appendix C - A Sample Safety Checklist for In-Service Pipeline Welding and Cutting Operations (HOT TAP)

Appendix A - A Sample Safety Checklist for Electric Welding operations

| Comment | No | Yes | Items | Number |
| --- | --- | --- | --- | --- |
|  |  |  | Has the hot work permit been obtained for conducting the welding and cutting operations? | 1 |
|  |  |  | Has the welding and cutting equipment been inspected and was it made sure they are intact and free from defects? | 2 |
|  |  |  | Do all the personnel wear the special protection gears in accordance to their type of activity? | 3 |
|  |  |  | Has pipe’s internal pressure been relieved and the pipe’s content been safely emptied in an appropriate spot? | 4 |
|  |  |  | Have the unnecessary personnel been kept away from the work site? | 5 |
|  |  |  | Does cutting involve cold cutting? | 6 |
|  |  |  | Have the oxygen, flammable gas, and toxic substance tests been performed at the work site? | 7 |
|  |  |  | Has a suitable channel with safe entry and exit been excavated based on the relevant excavation guideline? | 8 |
|  |  |  | Has the channel been cleaned of gas liquids? | 9 |
|  |  |  | Are emergency escape breathing apparatus available at the site in cases where the gas contains hydrogen sulfide? | 10 |
|  |  |  | Are warning signs installed at the site? | 11 |
|  |  |  | Has the wind direction been taken into consideration? | 12 |
|  |  |  | Has the pipe been washed with water and the special clamp been used before starting the welding operation? | 13 |
|  |  |  | Are emergency exit routes available at the pit’s sides? | 14 |
|  |  |  | Is the work site clear of sources of spark and flame, and flammable materials up to a radius of 25 m? | 15 |
|  |  |  | Has the relevant night work permit been obtained from the regional director or acting director? | 16 |
|  |  |  | Is an appropriate generator available to provide lightning at nights? The generator should be at least 25 m away from the danger zone. | 17 |
|  |  |  | Are unused pipes filled with water to neutralize flammable and toxic gases? | 18 |
|  |  |  | Have the pipe’s ends been closed after flowing water or nitrogen through it at stage 18? | 19 |
|  |  |  | Are the personnel with fire fighting equipment available at the site? | 20 |
|  |  |  | Is an ambulance available at the site? | 21 |
|  |  |  | Are the appropriate communication equipment available at the site? | 22 |

| Comment | No | Yes | Items | Number |
| --- | --- | --- | --- | --- |
|  |  |  | Has the pipe been cleaned off of any deposits and contamination at a 30 cm (1 foot) distance? | 23 |
|  |  |  | Have the security guards been directed to alert unauthorized personnel in the vicinity of residential areas? | 24 |
|  |  |  | Does the person performing the gas test have the relevant license? | 25 |
|  |  |  | Has the gas test machine been calibrated? | 26 |
|  |  |  | Does the crane operator have the relevant special license? | 27 |
|  |  |  | Does the crane have the relevant technical inspection certificate? | 28 |
|  |  |  | Is it necessary to wash the pipe with water before starting the welding operation or does it need to be flushed with neutral gases? | 29 |
|  |  |  | Will the welding operation be conducted at height? If yes, the working at heights guideline should be closely followed. | 30 |
|  |  |  | Has the excavation and earthwork guideline been followed? | 31 |
|  |  |  | Has the safety and health hazard evaluation been conducted? | 32 |
|  |  |  | Have the personnel received the relevant training concerning the welding safety regulations? | 33 |
|  |  |  | Have the precautionary measures been taken to avoid sparks falling on flammable materials in cases where the work is done at heights? | 34 |
|  |  |  | Are the atmospheric conditions suitable for conducting welding and cutting operations? | 35 |
|  |  |  | Have the appropriate support and clamps been used for pipe fixing? | 36 |
|  |  |  | Have the tools and equipment been removed from the site after the work was over and has the environment been cleared of all environmental contaminants? | 37 |
|  |  |  | Are light and heavy vehicles kept in a safe place and ready for a potential emergency evacuation? | 38 |
|  |  |  | Has TAG LOCK been used on inlet and outlet valves in cases where the pipe’s ends cannot be detached from the facilities? | 39 |
|  |  |  | Have electrostatic hazards been taken into consideration and the necessary measures been taken to avoid them? | 40 |

Prepared by (full name):				Confirmed by (full name):

Appendix B - A Sample Safety Checklist for Acetylene Welding operations

| Comment | No | Yes | Items | Number |
| --- | --- | --- | --- | --- |
|  |  |  | Has the hot work permit been obtained? | 1 |
|  |  |  | Is there the risk of toxic fumes release? | 2 |
|  |  |  | Should breathing air cylinders be used? | 3 |
|  |  |  | Is the site well-ventilated? | 4 |
|  |  |  | Is the welder aware of the hazards of toxic fumes? | 5 |
|  |  |  | Is the welder/cutter aware of the hazards associated with his work? | 6 |
|  |  |  | Is there any welding and cutting guideline available? | 7 |
|  |  |  | Is the welder certified and approved? | 8 |
|  |  |  | Has the gas gauge been used to perform the gas test at the work site? | 9 |
|  |  |  | Is a fire extinguisher available at the welding site? | 10 |
|  |  |  | Have the adjacent areas been protected against the dangers of sparks and heat produced during the welding operation? | 11 |
|  |  |  | Have the adjacent areas where accumulation of hydrocarbon compounds is likely been inspected? | 12 |
|  |  |  | Does the hot work being undertaken outside of the closed container and/or flow lines contribute to the risk of sparks being ignited inside them? | 13 |
|  |  |  | Has the welding cable been placed at a 3 m distance from the work site and been connected to power? | 14 |
|  |  |  | Are iron binding and earthing available? | 15 |
|  |  |  | Have the electricity been turned off and the fumes, gases and dangerous substances removed from the site? | 16 |
|  |  |  | Has the wind direction been taken into consideration? | 17 |
|  |  |  | Are the welding cables damaged? | 18 |
|  |  |  | Are the gas transfer hoses (cutting) can be found along the path where vehicles travel? | 19 |
|  |  |  | Are acetylene and oxygen cylinders vertically positioned? | 20 |
|  |  |  | Are the special heads installed on the unused cylinders? | 21 |
|  |  |  | Are cylinders kept under sunshades? | 22 |
|  |  |  | Are oxygen and acetylene cylinders kept away from flammable substances? | 23 |

|  |  |  | Is there a 6 m distance between acetylene and oxygen cylinders stored in the warehouse? | 24 |
| --- | --- | --- | --- | --- |
|  |  |  | Is the place used to store the above-mentioned cylinders well-ventilated? | 25 |
|  |  |  | When welding machinery components, have their batteries been turned off and the batteries’ connectors been removed? | 26 |
|  |  |  | Are protection walls needed? | 27 |
|  |  |  | Are the cables suitable for the current to be carried through them? | 28 |
|  |  |  | Are all the cylinders safe and intact? | 29 |
|  |  |  | Are the cylinders’ valves intact? | 30 |

Prepared by (full name):				Confirmed by (full name):

Appendix C - A Sample Safety Checklist for In-Service Pipeline Welding and Cutting Operations

| Comment | No | Yes | Items | Number |
| --- | --- | --- | --- | --- |
|  |  |  | Has the hot work permit been obtained? | 1 |
|  |  |  | Have the contents of pipe or equipment been assessed in terms of their compatibility with the material the HOT TAP machine is made of? | 2 |
|  |  |  | Is the MSDS of the pipe’s or tank's content available and are you aware of the potential health hazards? | 3 |
|  |  |  | Do the pipe’s or container’s contents remain stable when exposed to heat? | 4 |
|  |  |  | Have the HOT TAP machines and its accessories been inspected? | 5 |
|  |  |  | Was the HOT TAP machine chosen according to the work’s pressure and temperature? | 6 |
|  |  |  | Is the spot to undergo the HOT TAP operation already identified and physically marked? | 7 |
|  |  |  | Has the welding spot examined in terms of thickness? Is the spot free from any defect (e.g., hydrogen attack, delimitation)? | 8 |
|  |  |  | Have oxygen, explosive gas, and toxic atmosphere tests been performed? | 9 |
|  |  |  | Are the personnel present at the site equipped with appropriate protection gears? | 10 |
|  |  |  | Do all the personnel use the appropriate protective equipment? | 11 |
|  |  |  | Are there any procedures to follow when preparing for and responding to emergencies? | 12 |
|  |  |  | Is the welding operation going to be conducted 1 m below the liquid surface for tanks with cone-shaped roofs? | 13 |
|  |  |  | Is the welding operation going to be conducted below the liquid surface for floating rood tanks? | 14 |
|  |  |  | Was the dye penetrant, ultrasonic, and/or magnetic testings been performed on all fittings when welding was over? | 15 |
|  |  |  | Have hydrostatic and pneumatic tests been performed on the weld’s fittings before starting the operation? | 16 |
|  |  |  | Are the welders approved for doing the job? | 17 |
|  |  |  | Is it necessary yo conduct the preheating operation during the welding operation? | 18 |
|  |  |  | Has the right cutting piece been selected (in terms of the material it is made of) for the effective penetration into the metal? | 19 |
|  |  |  | Has the heat transfer been assessed so as to avoid burn through and cracking? | 20 |

|  |  |  | Is it necessary to use steam or a neutral gas to produce sufficient flow when lack of flow is observed? | 21 |
| --- | --- | --- | --- | --- |
|  |  |  | Are the equipment under vacuum conditions? If yes, no HOT TAP operation should be undertaken. | 22 |
|  |  |  | Do the equipment contain air, steam, or oxygen? If they contain any flammable mixture, no HOT TAP operation should be undertaken. | 23 |
|  |  |  | Have the special signs and barriers been used at the site? | 24 |
|  |  |  | Are the personnel trained regarding the HOT TAP operations? | 25 |
|  |  |  | Has a program been devised to monitor and control the process variables (e.g., temperature, pressure)? | 26 |
|  |  |  | Has the safety and health hazard evaluation been conducted? | 27 |
|  |  |  | Is an ambulance available at the site? | 28 |
|  |  |  | Has the relevant permit been obtained from the regional director or acting director for works to be done at night? | 29 |
|  |  |  | Has the appropriate lighting been provided for the work to be done at night? | 30 |
|  |  |  | Does the person performing the gas test have the relevant license? | 31 |
|  |  |  | Has the gas test machine been calibrated? | 32 |
|  |  |  | Have the basic welding rules been observed during the welding operation? | 33 |
|  |  |  | Have the earthwork and excavation regulations been observed when excavating the pit? | 34 |
|  |  |  | Are emergency escape breathing apparatus available at the site in cases where the gas contains hydrogen sulfide? | 35 |
|  |  |  | Are emergency exit routes available at the pit’s sides? | 36 |
|  |  |  | Are the appropriate communication equipment available at the site? | 37 |
|  |  |  | Are the atmospheric conditions suitable for conducting welding and cutting operations? | 38 |

Prepared by (full name):				Confirmed by (full name):

Health, Safety, and Environment (HSE) Department

No. 17, Yaghma Alley, Jomhouri-e Eslami St. (btwn Hafez St. & Valiasr Ave.), Tehran, Tehran, Iran

Phone: +98 21 66726016, Fax: +98 21 6609723, SMS System: +98 10007615

HSEE@NIOC.IR	HTTP://HSE.NIOC.IR