National Iranian Oil Company Safety, Health and Environment Management Management of Change (MOC) Guideline Reviewing Translation; Confirmed by; Publication Date of English Version. Davood Naderi/ Laleh H. Abbasi Training & Performance Measurement Department; Mani Abdollahzadeh Rad/ HSE Manager; September 2018. Responsibility/Subject; Prepared by; Confirmed by; Approved by; Publication date. Name; Mani Abdollahzadeh/ Fereshte Miraj Head of Safety and Firefighting Department /Project Supervisor of Safety and Firefighting Department; Siavash Derafshi/ HSE Manager; Roknodin Javadi CEO; May 2016. Position. Document Code: NIOC-HSE-SF-GU-007-00 Revision date: May 11, 2016, Revision No.: 1

Contents

1. Introduction 									3

2. Purpose										3

3. Scope 										3

4. Basic documents 									4

5. Definitions 										4

6. Role and responsibilities 								5

6.1 Managing Director 								5

6.2 Project executives/heads of units and departments 				6

6.3 The HSE Department 								6

7. The implementation of the MOC process 						6

7.1 Change communication 								6

7.2 Potential effect assessment 							6

7.3 Decision-making about change 							7

7.4 Implementation of changes 							8

7.5 Follow-up measures 								8

8. Implementation Requirements of the MOC system 				8

9. The MOC system considerations in the design, construction and installation activities 9

10. References										10

11. Appendices 									10

11.1 Appendix A: Change declaration/proposal form 				10

11.2 Appendix B: Change appraisal checklist 					11

11.3 Appendix C: Review checklist after implementing the change 			12

1. Introduction

Organizational changes are typically made for such reasons as development, improvement, stakeholder needs or unpredictable events. Experience shows that undesirable, unplanned and poorly-identified and assessed changes lead to unwanted events that can consequently affect the safety of facilities, employee health and environment or can result in long-term cessation of production process, decreased quality and increased costs. To reduce events associated with such changes and their consequent costs, all subsidiaries of the National Iranian Oil Company shall establish an efficient management of change (MOC) system. MOC system is one of the main elements of the Process Safety Management (PSM) system. This system is used to evaluate and control construction design modifications, installation and operation of facilities, human resources, processes and organizational activities, before implementation, to ensure their compatibility with the existing systems in order to identify potential problematic components. The MOC system requires the establishment of a mechanism consisting of executive procedures and written instructions to properly review and manage the proposed changes in the workplace, prior to their approval and implementation.

In the present document entitled “Management of Change Guideline”, attempts were made to outline the general requirements of the MOC system as well as its establishment and implementation requirements.

Important Note: Based on this guideline, the N.I.O.C.’s subsidiaries and operating companies, operational areas, operational units and headquarters shall develop “MOC instructions” to institutionalize the requirements of the implementation of this document across their company. Companies shall compile this document with regard to the type of activities and in accordance with their organizational structure, processes, technologies, equipment, types of material, procedures and manpower.

2. Purpose

Ensuring the reduction or prevention of events and incidents caused by possible heterogeneous changes in the process, technology, equipment, types of material, procedures and organizational structure through:

- Establishment and development of MOC throughout the process/project life cycle as well as unconventional changes

- Customizing the MOC system in accordance with the specifications of facilities and expected risk, on the basis of system’s application rate and the safety culture level

- Timely and effective monitoring of the performance of the MOC system.

3. Scope

This guideline is applicable to all N.I.O.C. units, including the headquarters, subsidiaries, associates, and operational areas in the following areas:

3.1 Changes made in process equipment, such as materials used in construction operations, design parameters and equipment arrangement

3.2. Changes made in process controls, such as instrumentation, control systems, interlocks and software systems

3.3 Changes made in the type of technology, process conditions, type of material, product properties, etc.

3.4 Changes made in safety and firefighting systems (replacement, modification, improvement etc.)

3.5 Changes made in the workplace infrastructures, such as buildings, routes, facility services, etc.

3.6 Changes made in operational procedures and instructions

3.7 Changes made in human resources, such as displacement, downsizing, using contractor’s staff, etc.

3.8 Changes made in company policies, such as replacement of paper-based systems by electronic systems, changing overtime conditions, etc.

In general, any change in the equipment, processes and procedures requires the implementation of the MOC process.

4. Basic documents

Before implementing the MOC process, the following documents shall be prepared as basic requirements:

- Risk assessment documents

- Technical documents and process, instrumentation and structural drawings, Process Flow Diagrams (PFD), Piping and Instrument Diagrams (P&ID), etc.

- Administrative, operational and safety manuals and instructions

- The emergency response plan documents

- Safety Data Sheet (SDS)

- Job description

- Organizational structure and human resources information

- Ascertainment of educational requirements and qualifications

- All valid organizational documents relevant to the subject of changes

5. Definitions

- Change: Any deliberate decision made with the aim of replacing, decreasing, increasing, modifying or developing equipment, consumables, procedures, instructions, manuals, requirements, HSE rules and regulations and even the organizational structure of a company.

- Technical changes: Changes that are associated with hardware equipment and facilities and can affect occupational health, safety or environmental aspects of projects. Examples include: changes in design, construction, installation and repair, changes in buildings, removal or addition of equipment to a running system, changing consumables and purchasing, installing and using new equipment.

- Administrative changes: Modifications made in work bylaws and instructions. Examples include: changes made in laws, rules of procedure, missions and regular site inspection schedules of experts that can affect occupational safety and health.

- Organizational or structural changes: Making modifications to the organizational chart which may consequently change organizational activities. Examples include: changing shift works, utilizing contract-work services instead of salaried employees, increasing or decreasing organizational levels as well as the fields of activity.

- Changes made in the documents: Changes made in the procedures, instructions and other corporate operational documents. These changes may alter operations and possibly the associated risks with relevant activities.

- Homogenous replacement: The replacement of equipment, chemicals, procedures, organizational structure, human resources, etc., by other completely identical items [where performance and safety are not affected] is called homogenous replacement.

- Technical Committee on Change: A committee consisting of the head of the company HSE Department or his/her representative (the head of the committee), the head of the department/unit/ executor of the plan whose scope of activity is related to the intended change, the authorities of the effective/affected units or their representatives determined by the head of the HSE Department who is responsible for examining the effects of changes and making decisions about approving and implementing them.

- Temporary change: A short-term and predetermined change that is made within a limited time period.

- Emergency change: This type of change is made under specific circumstances, when time is limited and compliance with the approved MOC procedure may cause incidents or severe financial losses.

6. Role and responsibilities

The N.I.O.C.’s HSE Management, as the process owner of MOC, shall supervise its proper implementation. It is also responsible for reviewing and updating the guideline. The highest authorities of the N.I.O.C.’s subsidiaries, operating companies, and headquarters are responsible for the establishment and implementation of this guideline. According to this guideline, the N.I.O.C.’s subsidiaries, operating companies and headquarters shall develop “MOC instructions” to institutionalize the implementation requirements of this document across their company. Companies shall compile this document with regard to the type of activities and in accordance with their organizational structure.

6.1 Managing Director

- Ensuring the proper implementation of the MOC instruction, in accordance with the job description requirements defined in this guideline

- Supporting the executive team and preparing necessary infrastructures in the workplace for the implementation of the MOC system

- Determining various authority levels across the company/organization for approving changes

6.2 Project executives/heads of units and departments

- Notifying the HSE Department about changes made in processes, activities, methods, infrastructures, plans, etc. within their scopes of activity.

- Ensuring staff understanding about their role and responsibilities in implementing the MOC requirements

- Cooperation with the HSE Department in determining its work processes that might be affected by the identification of potential future risks

- Establishment and implementation of changes in accordance with the approvals of the Technical Committee on Change

6.3 The HSE Department

- Guidance, establishment, development, implementation and maintenance of the MOC system

- Technical assessment of received proposals for technical change, correspondence with HSE standards and requirements, conducting relevant risk assessments and, if necessary, holding Technical Committee on Change meetings and attending meetings as the chairman.

- Monitoring and supervising proper implementation of MOC studies in accordance with the defined requirements

- Confirming the Technical Committee on Change’s final report and submitting it to the executive units

- Ensuring the proper implementation of approvals of the Technical Committee on Change

- Ensuring the return of temporary change conditions to the initial state

- Conducting periodic inspections and audits on the MOC process and, if necessary, revising the MOC Guideline.

- Collaborating on teaching the MOC Guideline in order to create a common understanding and perception

- Keeping records on MOC documents

7. The implementation of the MOC process

7.1 Change communication:

When a change is decided to be made for any reason, the head of the department/unit or the project executor shall notify the HSE Department of the respective company to assess the impact of the change and its potential health, safety and Environmental risks, along with other necessary documents. Appendix A presents an example of a change declaration/proposal form.

7.2 Potential Effect Assessment

Using received documents, the HSE Department of the respective company shall assess the effects of the proposed change. After determining the necessity and accuracy of the change in accordance with the definition provided in the MOC system, one of the following actions shall be performed depending on the scale of the impact of the desired change on safety, health and environmental risks:

A. Procedure for changes with minor impacts on risks:

In this case, there will be no need to establish a Technical Committee on Change. The HSE department of the respective company will announce the necessary steps to the proposing department/unit. The HSE department can appoint a person to follow-up the necessary recommendations and warnings when making changes. This person will also declare the need for submitting a change report in the change declaration/proposal form (a sample form is presented in Appendix A).

B. Procedure for changes with major or important impacts on risks:

In this case, the head of the respective HSE department shall announce the need for establishing a Technical Committee on Change and determine relevant units to the nature of change in the change declaration/proposal form (a sample form is presented in Appendix B).

Before holding the Technical Committee on Change, the HSE department of the company shall cooperate with the respective department/unit to identify and assess relevant HSE risks of the desired change. The HSE department shall also send a summary of their assessment along with an invitation to the members of the Technical Committee on Change to attend the meeting.

The following measures shall be taken at the Technical Committee on Change meeting:

A) Describing the subject of the change by the representative of the department/unit or the project executor whose scope of activity is related to the intended change.

B) Reporting and analyzing the identified risks, changed and new risks and their consequences by the HSE unit/department of the relevant company.

C) Assessment of the potential change impacts on other areas, processes, units, equipment, personnel, community, etc.

D) Discussing relevant risks, accurate identification of risks and their consequences and possible problems and recording all these information.

E) Identifying and discussing predicted risk control, management, and reduction mechanisms.

F) Determining the urgency and the temporal (permanent or temporary) state of the change

Pre-designed checklists are often used and completed in the Committee discussions (an example of a change checklist is given in Appendix B).

7.3 Decision-making about change:

Based on the conducted analyses, the Technical Committee on Change shall decide whether to approve or reject the proposed changes. In the case of rejection, the reasons and a summary shall be sent to the proposing department/unit as well as other relevant units. In the case of approval, the following measures shall be determined as well as the responsible department/unit and duration for the implementation of the desired change.

- Determining change communication mechanisms to individuals or relevant companies, including contractors

- Determining the amount and method of required trainings by staff and other stakeholders

- Potential necessary technical and executive measures

- Potential necessary supervisions and controls

- The necessary measures by authorities approving the change

- Changes in relevant technical and systemic documents, including drawings, procedures, instructions, emergency response plans, etc.

7.4 Implementation of changes

The managers and heads of the departments/units in charge of making the change shall implement and document the change in accordance with the approvals of the Technical Committee on Change within a defined time period.

7.5 Follow-up measures:

- To ensure proper implementation of the change-related issues (in accordance with the clause 7.4), the results of the control measures, approved by the Technical Committee on Change, shall be followed up by the HSE department, before implementation.

- The HSE department shall also cooperate with the proposing unit to ensure the proper function of control elements. A change assessment checklist is used to examine the correspondence of changes with the Committee’s approvals (checklist B).

- Specialized training and re-training courses shall be organized and held for all staff and contractors affected by the change and relevant records shall be documented and maintained.

- After applying the proposed change, relevant documents and training courses shall be updated within a specified period (a maximum of 2 months).

- Technical Committee on Change shall, if necessary, re-evaluate relevant risks and review changes after making the proposed change (An example of a review checklist is presented in Appendix C).

8. Implementation Requirements of the MOC system

8.1 Qualified individuals shall be selected to conduct the MOC process and some basic qualifications shall be defined for determining competent qualifiers. This should be defined in the individual’s job descriptions.

8.2 The proposing unit/department shall submit the technical bases and necessities for the accurate appraisal of proposed change as well as other required information and documents to the HSE department and Technical Committee on Change, before implementing the MOC process.

8.3 All employees in the production (operation), repair, maintenance and other relevant units, whose activities and duties will be affected by the proposed change, shall be informed and trained before making the change. If necessary, the Education department shall conduct need assessments and hold required training courses.

8.4. Before making any change in human resources, the department of human resources of each company shall ensure the proper training, knowledge and competence of new staff as well as their physical and mental health (through the industrial medicine unit) in accordance with the predetermined qualifications.

8.5 When changing the number of employees, their job descriptions shall be reviewed (to avoid interference of duties) and training sessions shall be planned and held for affected groups.

8.6 The change management requirements, as one of the contractor’s HSE requirements or HSE-Plan’s requirements, shall be defined and fulfilled in all contracts.

8.7 Any change in contractors’ human resources shall be approved according to the requirements specified by the employer’s HSE department as well as a relevant supervisor.

8.8 All process safety information affected by the change shall be reviewed and revised.

8.9 Prestart-up Safety Reviews (PSSR) shall be conducted when making major changes to the process and operational units. The MOC system replaces the PSSR in cases of minor change.

8.10 When issuing a permit for a temporary change, the duration for change implementation and reversion to the initial state shall be specified.

8.11 When issuing a permit for an urgent change, a process shall be defined for evaluating the change after its implementation and relevant documents shall be reviewed.

8.12 Periodic internal/external audits shall be planned and conducted to ensure the realization of the requirements of the change management system.

8.13 MOC records shall be maintained for a specified period (1 to 5 years). These records include:

- Records of change identification, decision-making, announcement and proposal

- Records of reviews and identifications and evaluations of HSE risks

- Records of predicted risk evaluations and controls

- Records of the impacts of modifications and relevant approvals

- Records of pre-commissioning checklists and relevant signatures

- Records of change commissioning operations

- Records of change reversion to the initial state in the case of temporary changes

- Records of pre- and post-change documents

- Records of unapproved changes

- Records of planned and implemented trainings

8.14 The MOC system, as one of the requirements of the HSE management system, shall be monitored and audited periodically in order to ensure their effectiveness and quality performance.

9. The MOC system considerations in the design, construction and installation activities

The MOC system differs in early stages of project development e.g. design, construction, and installation in terms of the number, frequency, and type of changes, available information, and time frame compared with the production and operation stages. A typical MOC procedure may be simpler and less complicated.

The following procedures are conducted in this regard:

9.1 In the case of minor changes, the engineering/project management department reviews the initial proposal and asks the HSE department for relevant recommendations. The HSE department shall provide them with necessary recommendations and warnings to help them decide whether to reject or approve the change.

9.2 After making the change, technical documents shall be revised and updated.

9.3 In the event of special or inevitable conditions of change imposed in construction or installation stages, the relevant supervisor shall cooperate with the HSE department and other relevant departments to decide about the change.

9.4 If the proposed change alters technical engineering documents in construction or installation stages, the final approval shall be obtained from the engineering/project management department and as-built documentation shall be prepared.

9.5. In cases where the proposed change makes huge differences in the design, construction or installation stages and affects various units, a Technical Committee on Change shall be established to assess various dimensions of the change and associated risks. The engineering/project management department shall decide whether to establish the Committee or not.

10. References

- Guideline for Management of Change for Process Safety, CCPS, 2008.

- Management of Change (MOC) procedure, ASPC 11.

11. Appendices

11.1 Appendix A: Change declaration/proposal form

11.2 Appendix B: Change appraisal checklist

| No. | Subject | Description |
| --- | --- | --- |
| 1 | Is it possible to eliminate relevant risks or lower their levels, after making the proposed change? What new risks might be imposed on the system? |  |
| 2 | Does the proposed change comply with relevant rules, regulations and standards? |  |
| 3 | What factors or systems are required to ensure the continuity of compliance of the change with relevant standards and regulations? |  |
| 4 | To what extent the desired change will cause interferences in job descriptions of human resources? |  |
| 5 | What new problems may arise from the proposed change? Is the risk number significant? |  |
| 6 | Will the current risk number decrease or increase? |  |
| 7 | Is there any applicable control factor? |  |
| 8 | Which elements of the company’s HSE system shall be changed? (executive procedures, human resources, equipment and devices, etc.) |  |
| 9 | Have the necessary changes been made in organizational letters and notifications, new permits, licenses and certificates and safety instructions? What are those changes? |  |
| 10 | What changes shall be made in drawings, procedures and other technical and non-technical documents of the organization? |  |
| 11 | What are the requirements for changing the current function of existing buildings, industrial facilities and equipment? |  |
| 12 | What repair and maintenance requirements will be added to the system? |  |
| 13 | Enumerate the new requirements for technical inspection and testing? |  |
| 14 | Do the lists of existing spare parts and consumables need to be changed? |  |
| 15 | What training needs and professional qualifications should be added to the system? |  |
| 16 | Does this change affect the activities and processes of other companies/units/operational areas? |  |

Note: The provisions of this checklist can be increased or decreased at the Technical Committee on Change meeting.

11.3 Appendix C: Review checklist after implementing the change

| No. | Subject | Description |
| --- | --- | --- |
| 1 | To what extent does the new situation conforms to the proposed change? |  |
| 2 | Has the proposed change been altered itself? |  |
| 3 | If the proposed change has been altered, has the new risk identification, risk assessment and control factor determination been conducted? |  |
| 4 | What are the new potential risk factors, following the establishment of the change plan? |  |
| 5 | Does the proposed change cause negative environmental impacts? |  |
| 6 | Are the changed documents applicable and enforceable? |  |
| 7 | Has the change improved the function of existing buildings, industrial facilities and equipment? |  |
| 8 | What are the new technical inspection, repair and maintenance and testing requirements? Are their records available? |  |
| 9 | How effective have been the changes made in the lists of existing spare parts and consumables? |  |
| 10 | Are the identified educational needs sufficient? |  |

Note: The provisions of this checklist can be increased or decreased at the Technical Committee on Change meeting.

No. 17, Yaghma Alley, Jomhuri Eslami Avenue, Tehran.

Tel. 66726016	Fax 66709723	SMS 10007615

HSEE@NIOC.IR	HTTP://HSE.NIOC.IR